| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0050-v1 |
| --- | --- |

| Title: | AHG9: Activation of a neural-network post-processing filter for multiple pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Kai Zhang1, Li Zhang1, Chaoyi Lin2, Junru Li2, Yue Li1 18910 University Center Lane, San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Email: | yekui.wang@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com linchaoyi.cy@bytedance.com lijunru@bytedance.com yue.li@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Currently, one neural-network post-filter activation (NNPFA) SEI message is used to activate a neural-network post-processing filter for one and only one picture. However, it is asserted that this is inefficient when a particular filter applies to a set of consecutive pictures.

This contribution proposes to enable activating a filter for more than one picture by one NNPFA SEI message. It is proposed to use one of the following three options for the purpose:

- Add one flag: nnpfa_on_flag.

- Add a new SEI message for deactivating a filter.

- Add two flags: nnpfa_cancel_flag and nnpfa_persistence_flag.

# Proposal option 1: add one flag

The proposed text changes for this option are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts, same for proposed changes in Sections 2 to 3):

8.29.1 Neural-network post-filter activation SEI message syntax

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_id | ue(v) |
| nnpfa_on_flag | u(1) |
| } |  |

8.29.2 Neural-network post-filter activation SEI message semantics

This SEI message specifies the neural-network post-processing filter that may be used for post-processing filtering for the current picture.

The neural-network post-processing filter activation SEI message persists only for the current picture.

A neural-network post-filter activation (NNPFA) SEI message activates or de-activates the possible use of the target neural-network post-processing filter, specified by nnpfa_id, for post-processing filtering of a set of pictures.

NOTE 1 – There may be several neural-network post-filter activation NNPFA SEI messages present for the same picture, for example, when the post-processing filters are meant for different purposes or filter different colour components.

nnpfa_id specifies that the target neural-network post-processing filter, which is specified by one or more neural-network post-filter characteristics SEI messages that pertain to the current picture and have nnpfc_id equal to nnfpa_id may be used for post-processing filtering for the current picture.

nnpfa_on_flag equal to 1 specifies that the target neural-network post-processing filter may be used for post-processing filtering for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with a NNPFA SEI message with nnpfa_on_flag equal to 0 and the same nnpfa_id as the current SEI message is output that follows the current picture in output order.

NOTE 2 – The target neural-network post-processing filter is not applied for this subsequent picture in the current layer associated with a NNPFA SEI message with nnpfa_on_flag equal to 0 and the same nnpfa_id as the current SEI message.

nnpfa_on_flag equal to 0 specifies that the persistence of the target neural-network post-processing filter is cancelled, i.e., the target neural-network post-processing filter is no longer used unless it is activated by another NNPFA SEI message with nnpfa_on_flag equal to 1 and the same nnpfa_id as the current SEI message.

# Proposal option 2: add a new SEI message

The proposed text changes for this option are as follows:

8.29.1 Neural-network post-filter activation SEI message syntax

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_id | ue(v) |
| } |  |

8.29.2 Neural-network post-filter activation SEI message semantics

This SEI message specifies the neural-network post-processing filter that may be used for post-processing filtering for the current picture.

The neural-network post-processing filter activation SEI message persists only for the current picture.

A neural-network post-filter activation (NNPFA) SEI message activates the possible use of the target neural-network post-processing filter, specified by nnpfa_id, for post-processing filtering of a set of pictures.

NOTE 1 – There may be several neural-network post-filter activation NNPFA SEI messages present for the same picture, for example, when the post-processing filters are meant for different purposes or filter different colour components.

nnpfa_id specifies that the target neural-network post-processing filter, which is specified by one or more neural-network post-processing filter characteristics SEI messages that pertain to the current picture and have nnpfc_id equal to nnfpa_id may be used for post-processing filtering for the current picture.

The target neural-network post-processing filter may be used for post-processing filtering for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with a neural-network post-filter deactivation (NNPFD) SEI message with nnpfd_id equal to nnpfa_id is output that follows the current picture in output order.

NOTE 2 – The target neural-network post-processing filter is not applied for this subsequent picture in the current layer associated with a NNPFD SEI message with nnpfd_id equal to nnpfa_id.

8.30.1 Neural-network post-filter deactivation SEI message syntax

| nn_post_filter_deactivation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfd_id | ue(v) |
| } |  |

8.30.2 Neural-network post-filter deactivation SEI message semantics

A neural-network post-filter deactivation (NNPFD) SEI message deactivates the possible use of the target neural-network post-processing filter specified by nnpfd_id. Once the target neural-network post-processing filter is deactivated, it is no longer used unless it is activated again by a subsequent NNPFA SEI message in decoding order with nnpfa_id equal to nnpfd_id of the current SEI message.

nnpfd_id specifies the target neural-network post-processing filter, which is specified by one or more neural-network post-processing filter characteristics SEI messages that pertain to the current picture and have nnpfc_id equal to nnfpa_id.

# Proposal option 3: add two flags

The proposed text changes for this option are as follows:

8.29.1 Neural-network post-filter activation SEI message syntax

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| } |  |

8.29.2 Neural-network post-filter activation SEI message semantics

This SEI message specifies the neural-network post-processing filter that may be used for post-processing filtering for the current picture.

The neural-network post-processing filter activation SEI message persists only for the current picture.

A neural-network post-filter activation (NNPFA) SEI message activates or de-activates the possible use of the target neural-network post-processing filter, specified by nnpfa_id, for post-processing filtering of a set of pictures.

NOTE 1 – There may be several neural-network post-filter activation NNPFA SEI messages present for the same picture, for example, when the post-processing filters are meant for different purposes or filter different colour components.

nnpfa_id specifies that the target neural-network post-processing filter, which is specified by one or more neural-network post-processing filter characteristics SEI messages that pertain to the current picture and have nnpfc_id equal to nnfpa_id may be used for post-processing filtering for the current picture.

nnpfa_cancel_flag equal to 1 indicates that the persistence of the target neural-network post-processing filter established by any previous NNPFA SEI message with the same nnpfa_id as the current SEI message is cancelled, i.e., the target neural-network post-processing filter is no longer used unless it is activated by another NNPFA SEI message with the same nnpfa_id as the current SEI message and nnpfa_cancel_flag equal to 0. nnpfa_cancel_flag equal to 0 indicates that the nnpfa_persistence_flag follows.

nnpfa_persistence_flag specifies the persistence of the target neural-network post-processing filter for the current layer.

nnpfa_persistence_flag equal to 0 specifies that the target neural-network post-processing filter may be used for post-processing filtering for the current picture only.

nnpfa_persistence_flag equal to 1 specifies that the target neural-network post-processing filter may be used for post-processing filtering for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with a NNPFA SEI message with the same nnpfa_id as the current SEI message and nnpfa_cancel_flag equal to 1 is output that follows the current picture in output order.

NOTE 2 – The target neural-network post-processing filter is not applied for this subsequent picture in the current layer associated with a NNPFA SEI message with the same nnpfa_id as the current SEI message and nnpfa_cancel_flag equal to 1.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).