| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0090-v1 |
| --- | --- |

| Title: | EE1-1.3 related: Lightweight and Efficient CNN In-loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hao Zhang Cheolkon Jung Yang Liu Ming Li | Email: | 13227706628@163.com zhengzk@xidian.edu.cn serena@oppo.com myron.li@oppo.com |
| Source: | [Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd.] |  |  |

_____________________________

- Abstract

This contribution presents a convolutional neural network (CNN)-based in-loop filter. In this contribution, the CNN-based in-loop filter proposed in JVET-AA0074 (AA0074 filter) is made lightweight and efficient. Moreover, a multi-stage training strategy under AI configuration is proposed to achieve an optimal trade-off between performance and complexity. The training strategy of AA0074 filter, i.e. use ground truth as the label, is used to verify the performance of the proposed CNN filter in RA configuration. Compared with EE1-anchor-2.0 (VTM-11.0_NNVC-2.0), the proposed CNN filter achieves average {7.08%, 12.46%, 12.75%} BD-rate reductions for {Y, Cb, Cr} under AI configuration and average {-%, -%, -%} BD-rate reductions for {Y, Cb, Cr} under RA configuration.

Introduction

A CNN-based in-loop filter was proposed in JVET-AA0074 for image quality enhancement (AA0074 filter) [1]. However, AA0074 filter does not show the state-of-the-art performance as well as it is not still lightweight enough. In response to these problems, some amendments are made to AA0074 filter in this contribution. Specifically, the number of skip connections, convolution layers and channels are reduced in each residual block, and correspondingly increased the number of residual blocks to ensure good learning ability. Moreover, a multi-stage training strategy is proposed to train the proposed network taking advantage of progressive learning in it. Specifically, a parameter qp_dis is set to represent the QP difference between the network input and the label. Since the smaller QP represents higher quality, the QP value of the label is lower than that of the input. First, we use smaller qp_dis to train the network, and then gradually increase qp_dis in training after the network converges. Since the loss function is multi-stage loss, we combine the loss function with the training strategy and propose a multi-stage training strategy.

Proposed method

Network architecture

The network architecture of the proposed CNN filter is shown in Fig. 1. Compared with AA0074 filter, the following improvements are mainly made. First, we reduce the convolution layer of the feature extraction part and the reconstruction part, and reduce the number of channels in the backbone. Then, some minor changes are made to WCDAB and RAB. Specifically, we reduce the number of skip connections in WCDAB and the number of convolution layers in RAB. The modified WCDAB and RAB are shown in Fig. 2(a) and Fig. 2(b), respectively.

Fig. 1 Illustration of the proposed NN-based in-loop filter. (SConv is the standard convolution)

Fig.2 (a) The architecture of WCDAB. (b) The architecture of RAB. (c) The architecture of Depthwise separable convolution. (SConv is standard convolution, DSConv is depthwise separable convolution, and DConv is depthwise convolution)

Except for the above modifications, other structures are consistent with AA0074 filter. In addition, for the RA configuration, the QP value of the current slice is used as an additional input.

Training strategy

In this contribution, a multi-stage training strategy is proposed to maximize the performance of the model during training. Specifically, a parameter qp_dis is used, which represents the QP difference between the network input and the label. Since the smaller QP represents higher quality, the QP value of the label is lower than that of the input. First, a smaller qp_dis is used to train the network, and then gradually increase qp_dis after the network converges while continuing to train the network. Since our loss function is multi-stage loss, we combine the loss function with the training strategy, i.e. a multi-stage training strategy. Specifically, first, qp_dis is set to 5 and trained the network with L1 and L2 loss functions successively. Then, qp_dis is increased to 10, and then trained the proposed network again with L1 and L2 loss functions. After that, continue to increase qp_dis and use the same method to continue training.

It is worth noting that because the proposed CNN filter is used to process YUV images, the image characteristics of UV channel and Y channel are different. When qp_dis is 5, the difference of image quality corresponding to Y channel is far greater than that of UV channel. Therefore, in actual training, qp_dis is only used for the Y channel. For the UV channel, its label uses the uncompressed image. However, when different filters are used to process Y and UV channels respectively, the problem of image difference between Y and UV channels no longer exists, and the proposed training strategy can be applied to different filters.

Training

Table 1. Network information of the proposed CNN filter in the training stage.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3090 |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 800 |
|  | Batch size: | 32 |
|  | Training time: | ~200h/model |
|  | Training data information: | BVI-DVC, DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0, QP {12,17,22, 27, 32, 37, 42} |
|  | Loss function: | Weighted L1 and L2 |
| Optional |  |  |
|  | Number of iterations | 3100 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | random cropping |
|  | Other information: |  |

Inference

Table 2: Network information for the proposed CNN filter in the inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Libtorch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 0.78M |
|  | Total Number of Parameters (All Models) | 0.78M |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 3.12M |
|  | Multiply Accumulate (MAC)/pixel | 200K |
| Optional |  |  |
|  | Total Conv. Layers | 64 Depthwise separable convolution 56 Common convolution |
|  | Total FC Layers | 72 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Experimental results

Objective comparison

The proposed CNN in-loop filter is evaluated based on VTM-11.0-NNVC-2.0 [3] according to the common test conditions (CTCs) defined in [4]. The evaluation is performed in the AI and RA configurations. Experimental results of the proposed CNN filter are shown in Tables 3 and 4. In the AI configuration, the proposed CNN filter achieves {7.08%, 12.46%, 12.75%} BD-rate reduction on average for {Y, Cb, Cr} channel. In the RA configuration, the proposed CNN filter achieves {-%, -%, -%} BD-rate reduction on average for {Y, Cb, Cr} channel.

Table 3 Performance of the proposed CNN filter in AI configuration

Visual comparison

Fig.3 Visual quality comparison on the test datasets. (a) Video frame of Class C-BasketballDrill. (b) Result of VTM-11.0_NNVC-2.0. (c) Result of the proposed CNN filter. (d) Ground truth.

Fig. 4 Visual quality comparison on the test datasets. (a) Video frame of Class A2-CatRobot. (b) Result of VTM-11.0_NNVC-2.0. (c) Result of the proposed CNN filter. (d) Ground truth.

After completing the objective performance comparison, QP is set to 42 and two sequences (Basketball in Class C and CatRobot in Class A2) are selected to verify the visual performance of the proposed CNN filter. The comparison results are shown in Figs. 3 and 4. It can be observed from the figures that the proposed CNN filter can effectively remove artifacts and blurs in the compressed image (such as the basketball in Fig. 3 and the digits in Fig. 4) while making the edges of objects clearer.

Complexity analysis

Table 4 Complexity of the proposed CNN filter in terms of parameter quantity and KMAC.

|  | Parameters (M) | KMAC (K) |
| --- | --- | --- |
| Proposed CNN filter | 0.78 | 200 |
| JVET-Z0091 [5] | 1.9 | 485 |
| JVET-Z0113 [6] | 3.12 | 539 |

This contribution evaluates the complexity of the proposed network in terms of the number of parameters and KMAC, and compares it with other in-loop filters at the 26th JVET meeting. The results are shown in Table 4. The proposed CNN filter achieves the lowest parameter quantity and computational complexity.

Conclusions

This contribution presents a CNN based in-loop filter. We have upgraded AA0074 filter, making it more lightweight and efficient. Specifically, this contribution fine-tunes the structure of AA0074 filter, and proposes a multi-stage training strategy to take advantage of progressive learning in the network training. Experimental results demonstrate that the proposed CNN filter achieve average {7.08%, 12.46%, 12.75%} and {-%, -%, -%} BD-rate reductions over VTM-11.0-NNVC-2.0 for {Y, Cb, Cr} channels in AI and RA configurations, respectively.

We recommend further study on the proposed CNN filter by including them in EE1.

Reference

[1]	H. Zhang, C. Jung, D. Zou and M. Li, “[EE1-related] Lighter WCDANN: CNN Based In-Loop Filters,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 Document JVET-AA0074, July 2022.

[2]	Di Ma, Fan Zhang, and David Bull, “BVI-DVC: A Training Database for Deep Video Compression”, arXiv:2003.13552, 2020.

[3]	https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

[4]	S. Liu, A. Segall, E. Alshina, and R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology,” JVET-W2016, April 2021.

[5] L. Wang, X. Xu, S. Liu and F. Galpin, “EE1-1.2: neural network based in-loop filter with a single model,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 Document JVET-Z0091, April 2022.

[6]	Y. Li, K. Zhang, L. Zhang, H. Wang, K. Reuze, A.M. Kotra, M. Karczewicz, “EE1-1.7: Combined Test of EE1-1.6 and EE1-1.3,” JVET-Z0113, by teleconference, April 2022.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).