| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0097-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0084 (EE1-2.3：A CNN-based Super Resolution Method with GOP Level Adaptive Resolution) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Du Liu | Email: | du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for JVET-AA-EE1-2.3, which presents neural network-based super resolution. One crosscheck test was carried out to for the super resolution network. It is reported that for AI configuration, the crosscheck results can match the results from JVET-AA-EE1-2.3 exactly. For RA configuration, the results can match with a minor difference of 0.000x, and this is probably due to floating point calculation. It is also verified that the output YUVs at the encoder side can match those at the decoder side.

# Introduction

The JVET-AA-EE1-2.3 described in JVET-AB0084 [1] proposes a super resolution network to on top of the RPR method. The inputs to the network include reconstruction samples, prediction samples, slice QP, base QP, and slice type. Different networks are used for the luma I-slice, the luma B-slice, and the chroma slices. The resampling factor x2.0 or x1.0 is selected at GOP level and applied to all pictures in that GOP.

# Results

The cross-check simulation tests the code in EE1-2.3 branch [2] with commit f74f5050. The tests with RA and AI configurations are cross-checked. The anchor is NNVC-2.0 [3]. The compiler version is GCC 7.4.0 and the libtorch version is 1.9.0. The models are floating point Pytorch models. The results are shown in the following tables.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.20% | -5.69% | -4.83% | 112% | 2500% |
| Class A2 | -1.06% | 0.89% | 0.68% | 111% | 280% |
| Class B | -0.40% | -0.01% | -0.30% | 113% | 235% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 93% |
| Class E |  |  |  |  |  |
| Overall | -1.58% | -0.96% | -0.93% | 109% | 305% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 122% | 96% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -4.23% | -1.52% | -1.73% | 144% | 763% |
| Class A2 | -2.71% | 2.74% | 1.43% | 113% | 342% |
| Class B | -0.75% | 0.03% | -0.30% | 109% | 251% |
| Class C | 0.00% | 0.00% | 0.00% | 110% | 106% |
| Class E | 0.00% | 0.00% | 0.00% | 120% | 97% |
| Overall | -1.37% | 0.21% | -0.13% | 117% | 224% |
| Class D | 0.00% | 0.00% | 0.00% | 106% | 131% |
| Class F | 0.00% | 0.00% | 0.00% | 103% | 104% |

# References

[1] R. Chang, L. Wang, X. Xu, S. Liu, J. Nam, S. Yoo, J. Lim, S. Kim, “EE1-2.3：A CNN-based Super Resolution Method with GOP Level Adaptive Resolution”, JVET-AA0084, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting, Mainz, 20-28 Oct. 2022

[2] https://vcgit.hhi.fraunhofer.de/jvet-aa-ee1/VVCSoftware_VTM/-/commits/EE1-2.3/

[3] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc-2.0

[4] E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-X2016, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 24th Meeting, by teleconference, 6–15 October 2021