| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0102-v2 |
| --- | --- |

| Title: | AHG11/EE1-related: Updates on RPR encoder and filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Sunmi Yoo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10680 Treena St, San Diego, CA 92131, USA | Tel: Email: | junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes an RPR encoder with four available scale factors {x2.0, x1.5, x1.25, x1.0}. The scale factor is selected at GOP level based on the initial QP and PSNR value of the first picture without multi-pass coding, and it is applied to all pictures within the GOP. This contribution also suggests to increase RPR filters for NNVC software to align with ECM where the luma and chroma interpolation filters are using 12-tap (non-affine luma), 10-tap (affine luma), and 6-tap (chroma), respectively. Lastly, it is proposed to increase the post-filter taps used for re-sampling the current picture to the original picture size. The experimental results are shown as follows;

- Test 1 (Adding x1.25 scale factor)

  - AI: -0.55% (Y), 5.02% (U), 3.83% (V), 99% (EncT), 90% (DecT)

  - RA: -1.07% (Y), 3.76% (U), 3.23% (V), 92% (EncT), 79% (DecT)

- Test 2 (Test1 + increasing RPR filters)

  - AI: -0.55% (Y), 5.00% (U), 3.86% (V), 100% (EncT), 91% (DecT)

  - RA: -1.11% (Y), 3.71% (U), 3.18% (V), 92% (EncT), 79% (DecT)

- Test 3 (Test2 + increasing post-filter)

  - AI: -0.57% (Y), 4.60% (U), 3.55% (V), 99% (EncT), 91% (DecT)

  - RA: -1.12% (Y), 3.36% (U), 2.81% (V), 91% (EncT), 79% (DecT)

- Test 4 (Test1 + increasing RPR filters + increasing post-filter)

  - AI: -0.57% (Y), 4.60% (U), 3.55% (V), 99% (EncT), 91% (DecT)

  - RA: -1.12% (Y), 3.34% (U), 2.80% (V), 92% (EncT), 79% (DecT)

Introduction

In previous meetings [1][2], encoder scheme on adaptive resolution for RPR had been proposed. The encoder selects one of three candidates, including original resolution, without multi-pass coding. The performance improvement for RPR was observed with the proposed scheme. In the last meeting, adaptive resolution for RPR was recommended in EE1 to evaluate the performance of the NN-based super-resolution proposals as secondary anchors for the super-resolution category.

Proposed method

Adding x1.25 scale factor

This contribution proposes to use four scale factors by adding a scale factor x1.25 to the previous works [1], [2]. Hence, candidate list is updated to support four possible scale factors {x2.0, x1.5, x1.25 and x1.0}. Most of the criteria for determining the scale factor are similar as JVET-Z0065. The scale factor is selected at GOP level, and applied to all pictures in the GOP. To determine the scale factor, a PSNR value and initial QP for the first picture of each GOP are exploited. RPR scale factor (RPR_SF) of i-th GOP is derived as follows,

RPR_SFGOPi=2.0,DnUpPSNRi>(BasePSNR)-QPi-BaseQP*0.5                                       1.5,DnUpPSNRi>BasePSNR-OffsetPSNR-QPi-BaseQP*0.5    1.25,DnUpPSNRi>BasePSNR-OffsetPSNR*2-QPi-BaseQP*0.51.0, Oherwise

where DnUpPSNRi represents down-up scaled PSNR for the first picture of i-th GOP. BasePSNR and BaseQP indicate predefined criteria for PSNR and QP, respectively. OffsetPSNR is a gap of PSNR to determine a scale factor among x2.0, x1.5, x1.25 and x1.0. QPi is the initial QP for the first picture of i-th GOP. The threshold is adjusted in multiples of 0.5 for the difference between the initial QP and the BaseQP. In this contribution, BaseQP and OffsetPSNR are set to 37 and 3. BasePSNR for RA and AI are 46 and 43, respectively. When x2, x1.5 or x1.25 scale factors are encoded, QP value is adjusted with offset −6, −4, or −2, respectively.

Increasing RPR filters

In the ECM-6.0, 12-tap and 10-tap luma interpolation filters are used in non-affine and affine blocks for RPR, respectively [3]. In the NNVC-2.0, however, luma interpolation filters for all inter blocks are identical with interpolation process in VVC specification. 8-tap and 6-tap luma interpolation filters are used in for non-affine and affine blocks, respectively. This contribution proposes to increase the luma interpolation filters for RPR as used in ECM while those of non-RPR case are unchanged. In summary, the tap of luma filter is as shown in Table 1.

Table 1. #tap of luma interpolation filters

|  |  | ECM | NNVC | Proposed |
| --- | --- | --- | --- | --- |
| Non-Affine | Normal | 12 | 8 | 8 |
|  | RPR0 (up-sampling) | 12 | 8 | 12 |
|  | RPR1/2 (down-sampling) | 12 | 8 | 12 |
| Affine | Normal | 12 | 6 | 6 |
|  | RPR0 (up-sampling) | 10 | 6 | 10 |
|  | RPR1/2 (down-sampling) | 10 | 6 | 10 |

In case of chroma interpolation filters, ECM and NNVC are used to 6-tap and 4-tap, respectively. This contribution proposes to increase the chroma interpolation filters for RPR as used in ECM while those of non-RPR case are unchanged. In summary, the tap of chroma filter is as shown in Table 2.

Table 2. #tap of chroma interpolation filters

|  | ECM | NNVC | Proposed |
| --- | --- | --- | --- |
| Normal | 6 | 4 | 4 |
| RPR0 (up-sampling) | 6 | 4 | 6 |
| RPR1/2 (down-sampling) | 6 | 4 | 6 |

Increasing post filters for RPR

A picture of which resolution has been changed requires resampling (or up-sampling) at original picture size for performance evaluation or display in manner of post-filter. For the luma component, both ECM and NNVC are utilized to same filters to VVC specification. The chroma post-filter for NNVC is also identical to VVC text. The chroma post-filter for ECM have been increased to 6-tap from 4-tap. This contribution proposes to increase the post-filters with 12-tap and 6-tap for luma and chroma, respectively. In summary, the tap of post-filter for RPR is as shown in Table 3.

Table 3. #tap of post filters for RPR

|  | ECM | NNVC | Proposed |
| --- | --- | --- | --- |
| Luma | 8 | 8 | 12 |
| Chroma | 6 | 4 | 6 |

Experimental results

The proposed method is implemented on top of NNVC-2.0 anchor [4] and the tests are conducted by NNVC CTC [5]. For RPR experiments, encoding parameters set as follows,

ScalingRatioHor                     : 2.0
ScalingRatioVer                     : 2.0
ScalingRatioHor2                    : 1.5
ScalingRatioVer2                    : 1.5
ScalingRatioHor3                    : 1.25
ScalingRatioVer3                    : 1.25
FractionNumFrames                  : 1
UpscaledOutput                      : 2

For x1.5 scale factor, encoding parameters are added with “ScalingRatioHor2” and “ScalingRatioVer2”. For x1.25 scale factor, encoding parameters are added with “ScalingRatioHor3” and “ScalingRatioVer3”.

To evaluate the proposed method, experimental tests are set as follows,

- Test 1 (Adding x1.25 scale factor)

- Test 2 (Test1 + increasing RPR filters)

- Test 3 (Test1 + increasing post-filter)

- Test 4 (Test1 + increasing RPR filters + increasing post-filter)

Test1:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -4.05% | 3.11% | 3.09% | 79% | 54% |
| Class A2 | -1.58% | 11.77% | 10.57% | 83% | 69% |
| Class B | 0.18% | 2.36% | 1.49% | 98% | 87% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -1.07% | 3.76% | 3.23% | 92% | 79% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -1.76% | 4.05% | 2.01% | 103% | 69% |
| Class A2 | -3.58% | 21.21% | 15.93% | 92% | 78% |
| Class B | 0.58% | 2.36% | 2.46% | 100% | 92% |
| Class C | 0.02% | 0.07% | 0.08% | 101% | 109% |
| Class E | 1.04% | 0.84% | 0.85% | 100% | 101% |
| Overall | -0.55% | 5.02% | 3.83% | 99% | 90% |
| Class D | 0.04% | 0.11% | 0.08% | 100% | 105% |
| Class F | 0.00% | 0.00% | 0.00% | 104% | 109% |

Test2:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -4.06% | 3.00% | 3.03% | 81% | 54% |
| Class A2 | -1.57% | 11.85% | 10.45% | 84% | 69% |
| Class B | 0.17% | 2.22% | 1.41% | 99% | 88% |
| Class C | -0.13% | 0.01% | 0.05% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -1.11% | 3.71% | 3.18% | 92% | 79% |
| Class D | -0.13% | -0.49% | -0.48% | 101% | 99% |
| Class F | -0.02% | -0.04% | 0.17% | 101% | 100% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -1.76% | 3.95% | 2.10% | 104% | 69% |  |
| Class A2 | -3.57% | 21.20% | 15.95% | 93% | 78% |  |
| Class B | 0.58% | 2.40% | 2.52% | 100% | 94% |  |
| Class C | 0.02% | 0.07% | 0.11% | 100% | 110% |  |
| Class E | 1.03% | 0.77% | 0.76% | 100% | 101% |  |
| Overall | -0.55% | 5.00% | 3.86% | 100% | 91% |  |
| Class D | 0.02% | 0.15% | 0.10% | 100% | 105% |  |
| Class F | 0.00% | 0.05% | 0.04% | 105% | 110% |  |

Test3:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -4.10% | 2.72% | 2.57% | 79% | 54% |
| Class A2 | -1.58% | 10.66% | 9.32% | 83% | 69% |
| Class B | 0.15% | 2.05% | 1.26% | 98% | 88% |
| Class C | -0.13% | 0.01% | 0.05% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -1.12% | 3.36% | 2.81% | 91% | 79% |
| Class D | -0.13% | -0.49% | -0.48% | 101% | 100% |
| Class F | -0.02% | -0.04% | 0.17% | 101% | 100% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -1.75% | 3.80% | 1.86% | 104% | 70% |
| Class A2 | -3.56% | 19.29% | 14.55% | 93% | 79% |
| Class B | 0.54% | 2.18% | 2.35% | 100% | 94% |
| Class C | 0.02% | 0.07% | 0.11% | 100% | 109% |
| Class E | 0.94% | 0.79% | 0.81% | 100% | 101% |
| Overall | -0.57% | 4.60% | 3.55% | 99% | 91% |
| Class D | 0.02% | 0.15% | 0.10% | 100% | 104% |
| Class F | 0.00% | 0.05% | 0.04% | 105% | 111% |

Test4:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -4.08% | 2.68% | 2.54% | 80% | 55% |
| Class A2 | -1.58% | 10.65% | 9.31% | 84% | 69% |
| Class B | 0.14% | 2.02% | 1.24% | 99% | 88% |
| Class C | -0.13% | 0.01% | 0.05% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -1.12% | 3.34% | 2.80% | 92% | 79% |
| Class D | -0.13% | -0.49% | -0.48% | 101% | 99% |
| Class F | -0.02% | -0.04% | 0.17% | 101% | 100% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -1.75% | 3.80% | 1.86% | 104% | 70% |
| Class A2 | -3.56% | 19.29% | 14.55% | 93% | 79% |
| Class B | 0.54% | 2.18% | 2.35% | 100% | 95% |
| Class C | 0.02% | 0.07% | 0.11% | 100% | 111% |
| Class E | 0.94% | 0.79% | 0.81% | 100% | 103% |
| Overall | -0.57% | 4.60% | 3.55% | 100% | 92% |
| Class D | 0.02% | 0.15% | 0.10% | 100% | 105% |
| Class F | 0.00% | 0.05% | 0.04% | 105% | 112% |

Conclusion

This contribution reports performance of an RPR encoder with four scale factors {x2.0, x1.5, x1.25, x1.0}. The proposed method determines a scale factor depending on the PSNR value and initial QP for the first picture of each GOP. This contribution also suggests to increase up-sampling and down-sampling filters and post-filter for RPR scenarios.

References

[1] J. Nam, S. Yoo, J. Lim, S. Kim, “EE1-2.1-related: RPR encoder with multiple scale factors”, JVET-Y0068, Meeting by Teleconference, January 2022.

[2] J. Nam, S. Yoo, J. Lim, S. Kim, “EE1-2.1: RPR encoder with multiple scale factors”, JVET-Z0065, Meeting by Teleconference, April 2022.

[3] K. Andersson, R. Yu, “EE2-2.8, 2.9, 2.10: Longer luma and chroma filters for RPR”, JVET-AA0042, Meeting by Teleconference, July 2022.

[4] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tags/VTM-11.0_nnvc-2.0

[5] E. Alshina, R-L. Liao, S. Liu, A. Segall,	“JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AA2016, Meeting by Teleconference, July 2022.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).