| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0112-v1 |
| --- | --- |

| Title: | EE2-2.6: DMVR for affine merge coded blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen Ru-Ling Liao Xinwei Li Yan Ye | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group. |  |  |

_____________________________

- Abstract

This contribution reports the results of test 2.6 of EE2. In EE2-2.6, CU level DMVR is applied on the blocks coded with affine merge mode. It is reported EE2-2.6 achieves {-0.15%(Y), -0.06%(U), -0.11%(V)} coding gain in RA with 102% encoding time and 102% decoding time on average, and luma coding gain for class A1 and A2 are -0.20% and -0.30%, respectively.

Introduction

In last meeting, DMVR is proposed to be applied on affine merge coded blocks in JVET-AA0144[1]. And it was decided to include the proposed method in Exploration Experiment 2 [2] to further study. The test was conducted on top ECM-6.0 and the test results were reported in this contribution.

Tested method

Generally, affine model can be described using the following equations

mvx=mv1x-mv0xWx+mv2x-mv0xHy+mv0xmvy=mv1y-mv0yWx+mv2y-mv0yHy+mv0y				(1)

wherein(mvx,mvy) is the motion vector at location (x,y), (mv0x, mv0y) is the base MV representing the translation motion of the affine model, and mv1x-mv0xW, mv2x-mv0xH, mv1y-mv0yW and mv2y-mv0yH are four non-translation parameters which defines rotation, scaling and other non-translation motion of the affine model. In ECM, besides 6-parameters affine model defined as (1), there are also 4-parameters affine mode described as (2) in which only two non-translation parameters are used.

mvx=mv1x-mv0xWx+mv0y-mv1yWy+mv0xmvy=mv1y-mv0yWx+mv1x-mv0xWy+mv0y				(2)

In EE2-2.6, the base MV of the affine model of the coding blocks coded with the affine merge mode is refined by only applying first step of multi-pass DMVR. That is, a translation MV offset is added to all the CPMVs of the candidate in the affine merge list if the candidate meets the DMVR condition. And the MV offset is derived by minimizing the cost of bilateral matching which is the same as conventional DMVR. And the DMVR condition is also not changed.

The MV offset searching process is similar as the first pass of multi-pass DMVR in ECM. 3x3 square search pattern is used to loop through the search range which is set as [-3, 3] to find the best integer MV offset. And then half-pel search is conducted around the best integer position and an error surface estimation is performed at last to find a optimal MV offset with 1/16 precision. For simplicity, 2-tap bilinear interpolation is used instead of 12-tap DCT-IF during the search process, which is also the same as what multi-pass DMVR does.

Simulation results

Coding performance

The method is implemented on top of ECM-6.0. The performance is evaluated based on ECM common test conditions in JVET-Y2017[3].

Table 1: Results of test 2.6 in RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.18% | -0.17% | 103% | 103% |
| Class A2 | -0.30% | -0.19% | -0.19% | 103% | 104% |
| Class B | -0.11% | 0.06% | -0.08% | 101% | 101% |
| Class C | -0.04% | -0.01% | -0.06% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.15% | -0.06% | -0.11% | 102% | 102% |
| Class D | -0.02% | 0.04% | -0.05% | 101% | 101% |

Conclusion

It is reported that EE2-2.6 achieves {-0.15%(Y), -0.06%(U), -0.11%(V)} coding gain with only 2% increase in running time on average. And for 4K class, the coding gain is -0.2% in class A1 and -0.3% in class A2. Thus, it is suggested to adopt EE2-2.6 into next version of ECM.

References

J. Chen, R. -L. Liao, X. Li, Yan. Ye, “Non-EE2: DMVR for affine merge coded blocks”, JVET-AA0144, July. 2022

V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K Zhang, “Exploration Experiment on Enhanced Compression beyond VVC Capability (EE2)”, JVET-AA2024, July. 2022

M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).