| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0121-v1 |
| --- | --- |

| Title: | AHG11: Assistant Reference Picture Method for NNVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chengming Gu， Wenjie Zou, Yi Zhou, Jiawei Fan (Xidian University) Yaxian Bai， Cheng Huang, Yuanjian Zhang (ZTE Corporation) | Email: | cmgu@stu.xidian.edu.cn wjzou@xidian.edu.cn yi.zhou@stu.xidian.edu.cn jw.fan@stu.xidian.edu.cn huang.cheng5@zte.com.cn bai.yaxian@zte.com.cn zhang.yuanjian@zte.com.cn |
| Source: | Xidian University, ZTE Corporation |  |  |

_____________________________

- Abstract

This contribution presents an assistant reference picture for NNVC to improve the coding efficiency. Besides the reconstructed picture in NNVC, the picture after DBF will put into the decoded picture buffer, using as an assistant reference picture for the future pictures. It is reported that, with NCS2.0 (NnlfOption=2) as anchor, the performance under RA configuration are as follows:

RA：

Introduction

In VVC[1], the reconstructed pictures after adaptive loop filter (ALF) are stored in decoded picture buffer (DPB) and organized as reference picture for the following encoding picture. During inter prediction, multiple pictures from DPB are selected to generate reference picture list for the motion estimation. The reference picture list has a huge impact on the performance of inter prediction.

In NNVC, the outputs of NN filters are derived through a deep neural network. The NN filters are pre-trained and the output of NN filters can provide quality improvement in most cases. While, in some cases, the quality of NN degrades and the adaption with the output of DBF or the output of DBF and SAO is needed. It is commonly agreed that content adaptivity by signalling scaling factor for residual or/and switching between different NN-based filters models on picture or/and CTU level can provide performance improvement[2].

Proposed method

Compared with the output pictures by NN, the reconstructed pictures after DBF have a stable quality. Using the pictures after DBF as a reference picture could be a supplement to the pictures filtered by NN-filters. Considering there is a limit on the size of DPB, this method will only store the pictures which have duplicates in the reference picture lists or I frame. The encoding process of the proposed method is shown in Fig. 1

Fig. 1 The coding process of the proposed assistant reference picture method

There are several limits in this contribution:

a) Luma reconstruction pictures after DBF and Chroma reconstruction pictures after ALF will be stored in DPB.

b) The pictures after DBF should be inserted in the middle of the reference picture lists. The POC of inserted pictures should be equal to the nearest POC with current picture's POC in the reference picture lists.

c) Only the temporal layer of current picture less than 5 could be inserted a picture after DBF.

Experimental results

NCS2.0 has already been published[3]. To verify the performance of this method, tests were conducted based on NCS2.0 (NnlfOption =2)[4].

- 4 Conclusions

This contribution provides more selections for those pictures which have duplicates in the reference picture lists. The results from this contribution demonstrated that the proposed method shows coding gains on BD-Rate compare with NCS2.0.

- 5 References

- ITU-T Study Group, Series H: Audiovisual and multimedia systems: Infrastructure of audiovisual services–coding of moving video-versatile video coding, in General Secretariat and Telecom Radiocommunication (ITU-R) Standardization (ITU-T), sec. H 266, 2020.

- Hongtao Wang, Marta Karczewicz, AHG11: Neural Network-based In-Loop Filter, Document JVET-T0079, online, October 2020.

- Elena Alshina, Ru-Ling Liao, JVET common test conditions and evaluation procedures for neural network-based video coding technology , Document JVET-AA2016, online, July 2022.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).