| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0140-v2 |
| --- | --- |

| Title: | EE2-2.4: Combined test of Test 2.2 and Test 2.3 on BCW weights derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang, Han Huang, Chun-Chi Chen, Vadim Seregin, Marta Karczewicz (Qualcomm) Ru-Ling Liao, Jie Chen, Yan Ye, Xinwei Li (Alibaba group) | Tel: Email: | zhizhang@qti.qualcomm.com {ruling.lrl, yan.ye}@alibaba-inc.com |
| Source: | Qualcomm Incorporated, Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the simulation results of EE2 Test 2.4, which is a combined test of template matching based BCW index derivation for merge mode and POC based BCW weights derivation. It is reported that on top of ECM-6.0, the overall coding performance impact for {Y, U, V, EncT, DecT} are:

- Test 2.4a: without negative bi-predicted weights:
{-0.17%, -0.26%, -0.28%, 107%, 100%} for RA and {-0.18%, -0.19%, 0.03%, 104%, 101%} for LDB.

- Test 2.4b: with negative bi-predicted weights:
{-0.18%, -0.21%, -0.28%, 107%, 100%} for RA and {-0.18%, -0.32%, 0.10%, 104%, 102%} for LDB.

Introduction

This contribution presents the simulation results of EE2 Test 2.4 [1] which studies the joint test of Test 2.2 and Test 2.3. In Test 2.2, the BCW index for merge coded CUs is derived based on template matching cost instead of inferring from neighboring blocks. Given a selected merge candidate, the TM cost values are calculated with different bi-predicted weights, and then, the bi-predicted weight with minimum TM cost value is used to predict the merge CU. In addition, the bi-predicted weights for merge mode are extended. In Test 2.3, an additional BCW weight is added and is derived according to POC difference. If both reference pictures are from the past or from the future relatively to the current picture, and the current picture is a non-low-delay picture, the weight pair (-2, 10) is added. Otherwise, the weight pair (2, 6) is added. The larger value from the weight pair is assigned to the closest POC reference picture or list 0 reference picture when the POC distance is the same.

Simulation results

The proposed methods are implemented on top of ECM-6.0 and are tested under CTC for enhanced compression tool testing [2] with random access and low delay B configurations. Two tests: 2.4a) without negative weights supported and 2.4b) with negative weights supported are evaluated, and the results are shown in Table 1 and Table 2, respectively.

Table 1. Test 2.4a: combined test without negative weights

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.43% | -0.32% | 109% | 100% |
| Class A2 | -0.17% | -0.21% | -0.24% | 110% | 101% |
| Class B | -0.21% | -0.26% | -0.39% | 105% | 99% |
| Class C | -0.09% | -0.17% | -0.14% | 105% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.17% | -0.26% | -0.28% | 107% | 100% |
| Class D | -0.08% | -0.28% | -0.14% | 105% | 101% |
| Class F | -0.30% | -0.41% | -0.30% | 103% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 | -0.19% | -0.38% | -0.19% | 106% | 101% |
| Class B | -0.20% | -0.14% | 0.20% | 103% | 101% |
| Class C | -0.20% | -0.14% | 0.20% | 103% | 101% |
| Class E | -0.15% | 0.04% | 0.16% | 102% | 100% |
| Overall | -0.18% | -0.19% | 0.03% | 104% | 101% |
| Class D | -0.10% | 0.26% | 0.79% | 104% | 102% |
| Class F | -0.01% | 0.04% | -0.27% | 99% | 98% |

Table 2. Test 2.4b: combined test with negative weights

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.41% | -0.34% | 109% | 100% |
| Class A2 | -0.20% | -0.21% | -0.27% | 110% | 100% |
| Class B | -0.21% | -0.16% | -0.38% | 105% | 99% |
| Class C | -0.11% | -0.13% | -0.13% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.21% | -0.28% | 107% | 100% |
| Class D | -0.07% | -0.06% | -0.05% | 106% | 101% |
| Class F | -0.33% | -0.39% | -0.45% | 103% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | -0.26% | -0.26% | 105% | 103% |
| Class C | -0.17% | -0.34% | 0.32% | 103% | 101% |
| Class E | -0.18% | -0.41% | 0.40% | 102% | 101% |
| Overall | -0.18% | -0.32% | 0.10% | 104% | 102% |
| Class D | -0.10% | 0.22% | 0.91% | 104% | 102% |
| Class F | -0.39% | -0.55% | -0.78% | 100% | 99% |

Conclusion

This contribution reports the simulation results of EE2 Test 2.4. Experimental results show that {0.17%, 0.18%} luma bitrate saving can be achieved in RA and LB configurations when negative weights are not supported. If the negative weights are supported, {0.18%, 0.18%} luma bitrate saving are obtained in RA and LB configurations. Thus, it is recommended to adopt the proposed method to ECM.

Supplemental results

A fast algorithm is applied to reduce the encoding time, and the results are shown in Table 3.

Table 3. Test 2.4b: combined test with negative weights with encoder optimization

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.20% | -0.20% | -0.39% | 105% | 99% |
| Class E | -0.12% | -0.19% | -0.07% | 104% | 99% |
| Overall |  |  |  |  |  |
| Class D | -0.09% | -0.09% | -0.22% | 104% | 101% |
| Class F | -0.33% | -0.36% | -0.38% | 103% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.17% | -0.34% | 0.32% | 103% | 101% |
| Class E | -0.18% | -0.41% | 0.40% | 103% | 101% |
| Overall |  |  |  |  |  |
| Class D | -0.10% | 0.22% | 0.91% | 105% | 102% |
| Class F |  |  |  |  |  |

References

- V. Seregin and et al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AA2024, July 2022.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).