| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0142-v2 |
| --- | --- |

| Title: | Non-EE2: optimizing the use of available decoded reference samples |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thierry Dumas, Kevin Reuzé, Ya Chen, Karam Naser thierry.dumas@interdigital.com kevin.reuze@interdigital.com ya.chen@interdigital.com Karam.naser@interdigital.com |  |  |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This contribution proposes to optimize the use of the available decoded reference samples around the current CU during the intra prediction step. This encompasses three changes: the order of a few MPMs of the current luma CB now depends on the availability of the above-right and bottom-left decoded reference samples, the DIMD region of decoded reference samples of the current luma CB is extended towards above-right and bottom-left if available, and a few candidate wide-angle intra prediction modes are added during the TIMD derivation step if they intensively use available above-right/bottom-left decoded reference samples. The experimental results reportedly show that the average BD-rate gains of the proposal are -xxx%, -xxx%, -xxx% and xxx%, -xxx%, -xxx% under AI and RA respectively.

A bug was found and corrected in v2, which is why results differ slightly.

# Introduction

Since VVC, for the current rectangular CU, the set of allowed intra prediction modes has been adapted by the wide-angle process to better use the decoded reference samples of this CU. Specifically, if the CU height is strictly larger than the CU width, instead of heavily extending the set of decoded reference samples towards the above-right (and often padding it) for the intra prediction via the last vertical positive intra prediction modes, the last vertical modes are replaced by wide-angle horizontal positive modes. If the CU width is strictly larger than the CU height, instead of heavily extending the set of decoded reference samples towards the bottom-left (and often padding it) for the intra prediction via the first horizontal positive intra prediction modes, the first horizontal positive modes are replaced by wide-angle vertical positive modes. In the same vein, it is proposed to optimize even more the use of available decoded reference samples around the current CU.

# Proposal

## Partial MPM reordering depending on the available decoded reference samples

For the current luma CB, if none of the decoded reference samples on its above-right side is available, the last 5 vertical positive intra prediction modes (including the potential wide-angle modes) are tagged as “reserved”. If none of the decoded reference samples on its bottom-left side is available, the first 5 horizontal positive intra prediction modes (including the potential wide-angle modes) are tagged as “reserved”. During the derivation of the MPM list of the current luma CB, after appending PLANAR to this list, for each of the modes selected to predict the luma CBs as “left” (0), “above” (1), “bottom-left” (2), “above-right” (3), and “above-left” (4) illustrated in Figure 1, if this mode is tagged as “reserved” and is not already in the “reserve” list, it is appended to the “reserve” list; otherwise, it is appended to the current MPM list if it is not already in the current MPM list. Then, the “reserve” list is appended to the current MPM list. Finally, the remaining part of the derivation of the MPM list is performed.

Figure 1: proposed adaptation of the derivation of the MPM list of the current WxH luma CB.

## Extension of the DIMD region depending on the available decoded reference samples

The region of decoded reference samples of the current WxH luma CB is extended towards the above-right side if available, up to W additional columns. It is extended towards the bottom-left side if available, up to H additional rows.

## A few additional candidate wide-angle modes tested during the TIMD derivation step depending on the available decoded reference samples

Figure 2: Example of modes to add for the TIMD search.

If the current luma CB uses TIMD, the availability of the above right and bottom left reference samples are checked. If those samples are available, wide angles are added to the TIMD search. The added wide angles are the angles normally reserved for neighboring block sizes. That is, for a CU of width/height=4, TIMD will be allowed to use modes from width/height={1, 2, 4, 8, 16} for a larger variety of predictions. No extra signaling is needed.

# Experimental results

The test results are presented in Table 1.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0,07% | 0,00% | -0,03% | 106% | 110% |
| Class C | -0,06% | -0,04% | -0,13% | 106% | 113% |
| Class E | -0,04% | 0,01% | -0,19% | 106% | 112% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,06% | -0,10% | -0,08% | 102% | 108% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0,02% | -0,18% | -0,06% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,04% | 0,07% | -0,02% | 105% | 106% |
| Class F | -0,01% | 0,01% | -0,21% | 104% | 107% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 1: BD-rate gains of ECM-6.0 with the proposed optimization of the available decoded reference samples versus ECM-6.0.

# Conclusion

The experiments report that ECM-6.0 including the optimization of the available decoded reference samples yields -xxx%, -xxx%, -xxx% BD-rate gains compared to ECM-6.0. It is recommended to study this tool in EE.

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).