| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0148-v2 |
| --- | --- |

| Title: | EE2-1.11: Intra prediction fusion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang Vadim Seregin Marta Karczewicz | Tel: Email: | hongtaow@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the test results of EE2-1.11a and EE2-1.11b. EE2-1.11a studies an intra prediction method that derives predicted samples as a weighted combination of multiple predictors generated from different reference lines. EE2-1.11b studies an alternative method, which combines two reference lines into one and intra predictor is then derived from the combined reference line. Simulation results reportedly show that EE2-1.11a and EE2-1.11b achieve Y, U, V BD-rate change with encoder and decoder run time as follows:

EE2-1.11a:

AI: -0.16%, -0.04 %, -0.05 %, 102 %(EncT), 101 %(DecT)

RA: -0.07 %, -0.03 %, 0.03 %, 101 %(EncT), 99 %(DecT)

EE2-1.11b:

AI: 0.20%, 0.26%, 0.20%, 103%(EncT), 100%(DecT)

RA: 0.08%, 0.10%, 0.06%, 100%(EncT), 99%(DecT)

# Introduction

Test-1.11 studies intra prediction fusion method proposed in [3], which uses multiple predictors generated from different modes/reference lines.

In sub-test a, multiple intra predictors are generated and then fused by weighted averaging. The process of deriving the predictors to be used in the fusion process is described as follows:

- For angular intra prediction modes including the single mode case of TIMD and DIMD, the proposed method derives intra prediction by weighting intra predictions obtained from multiple reference lines represented as pfusion=w0pline+w1pline+1, where pline is the intra prediction from the default reference line and pline+1 is the prediction from the line above the default reference line. The weights are set as w0=3/4 and w1=1/4.

- For TIMD mode with blending, pline is used for the 1st mode (w0=1,w1=0) and pline+1 is used for the 2nd mode (w0=0, w1=1).

- For DIMD mode with blending, the number of predictors selected for a weighted average is increased from 3 to 6.

In sub-test b, intra prediction fusion is performed on reference lines instead of prediction blocks. Two reference lines (referred to as rline and rline+1) are used for intra prediction fusion. DeltaInt of the corresponding intra prediction angle is considered in the fusion process. Each value in the fused reference line (rfusion[i]) is derived from:

rfusioni= 3⋅rlinei+rline+1i+DeltaInt≫2

The proposed intra prediction fusion is applied to luma blocks when angular intra mode has non-integer slope (required reference samples interpolation) and the block size is greater than 16, it is used with MRL and not applied for ISP coded blocks. In the method studied in the sub-test a, PDPC is applied for the intra prediction mode using the closest to the current block reference line.

# Simulation results

The proposed methods are tested on top of ECM-6.0[1]. The test simulations were conducted using the Common Test Conditions as specified in [2]. Test results for AI and RA configurations are summarized in the following tables.

Table 1 Simulation results of EE2-1.11a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.14% | -0.09% | 103% | 100% |
| Class A2 | -0.15% | -0.03% | -0.04% | 102% | 100% |
| Class B | -0.18% | 0.03% | -0.03% | 102% | 101% |
| Class C | -0.14% | -0.08% | -0.08% | 101% | 102% |
| Class E | -0.13% | 0.01% | -0.02% | 102% | 101% |
| Overall | -0.16% | -0.04% | -0.05% | 102% | 101% |
| Class D | -0.15% | -0.04% | -0.10% | 101% | 103% |
| Class F | -0.05% | 0.10% | 0.04% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.01% | -0.02% | 101% | 99% |
| Class A2 | -0.06% | -0.02% | 0.00% | 101% | 99% |
| Class B | -0.07% | 0.06% | -0.03% | 101% | 99% |
| Class C | -0.06% | -0.17% | 0.17% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.03% | 0.03% | 101% | 99% |
| Class D | -0.06% | -0.23% | -0.31% | 101% | 100% |
| Class F | -0.03% | -0.15% | -0.18% | 101% | 100% |

Table 2 Simulation results of EE2-1.11b

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.09% | 102% | 99% |
| Class A2 | 0.13% | 0.17% | 0.17% | 103% | 98% |
| Class B | 0.16% | 0.29% | 0.26% | 103% | 101% |
| Class C | 0.24% | 0.32% | 0.17% | 102% | 100% |
| Class E | 0.45% | 0.44% | 0.32% | 103% | 98% |
| Overall | 0.20% | 0.26% | 0.20% | 103% | 100% |
| Class D | 0.19% | 0.29% | 0.02% | 102% | 100% |
| Class F | 0.21% | 0.14% | 0.24% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.09% | 0.09% | 100% | 99% |
| Class A2 | 0.09% | 0.11% | 0.05% | 100% | 100% |
| Class B | 0.11% | 0.20% | 0.00% | 101% | 100% |
| Class C | 0.12% | -0.01% | 0.13% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.10% | 0.06% | 100% | 99% |
| Class D | 0.10% | -0.12% | -0.22% | 101% | 100% |
| Class F | 0.01% | 0.13% | -0.08% | 100% | 100% |

# References

- ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-6.0

- Exploration Experiment on Enhanced Compression beyond VVC capability (EE2), V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, JVET-AA2024, July 2022.

- Non-EE2: Intra Prediction Fusion, JVET-AA0137, K. Cao, V. Seregin, M. Karczewicz, July 2022.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).