| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0159-v1 |
| --- | --- |

| Title: | [AHG11] On adjustment of residual for NNLF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhenyu Dai Yue Yu Haoping Yu Dong Wang | Tel: Email: | daizhenyu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes an adjustment of residual for NNLF’s output based on JVET-AA0088[1] to further improve the coding performance. In JVET-AA0088, a residual network based NNLF was proposed to predict the residual between the reconstructed image and the original image to get better coding performance. It is proposed to apply an offset to the residuals at the frame level, which is signaled by a flag. Compared with JVET-AA0088, the proposed method demonstrates {-0.09%, -1.06%, -0.32%} and {-0.59%, -1.63%, -0.49%} BD-rate savings with the RA and LDB configurations, respectively.

# Introduction

In JVET-AA0088[1], a neural network-based in-loop filter with constrained memory size and low complexity was presented and was adopted as one of the two NNLF sets in the AHG11 reference software.

## Network architecture

The network structure of JVET-AA0088 is shown in Fig. 1. It was designed based on the classical residual network concept, which had a skip connection line from the input reconstructed image to the output of the network, and then generated the output image (output_yuv). NNLF is trying to predict the residual between reconstructed image and original image.

Fig. 1: Network architecture of JVET-AA0088

## Training and inference

For a neural network-based in-loop filter, the consistency between the training stage and the inference stage is important for achieving a good coding performance.

Currently, the training data is generated by the VTM anchor. However, in the inference stage, the testing data especially B slices will have better quality than that in the training stage when the trained NNLF models is integrated.

To solve this inconsistency problem due to the interactions among coded frames and frames to be compressed, JVET-AA0088 designed an iterative training method which is shown in Fig .2. In the iterative training process, the latest model (LM) was integrated into the codec of the training data generator which makes the training more consistent and effective. When the training iterations complete, it will save a final model (FM) for the inference stage usage.

Fig. 2: Iterative training method of JVET-AA0088[1]

# Proposed method

## The shortcoming of iterative training

Although the iterative training method minimizes the inconsistency between training and inference, there is still some performance difference between the latest model (LM) and the final model (FM) which may impact the coding performance.

More specifically, in the training stage, FM is trained by the data generated by the codec integrating LM; and in the inference stage, FM will handle the data generated by the codec integrating FM.

For B slices, if the performance of FM is better than LM, the prediction of B slices will be better, and the residual between the reconstructed image and the original image will be smaller then.

However, since the NNLF model is trained offline, it cannot handle this inconsistency properly, which will make the output residual larger than expected.

## Residual offset

To solve this problem, this contribution proposes that a frame level residual offset can be used to adjust the output of the NNLF in the inference stage.

More specifically, the residual is adjusted by reducing the magnitude of the residual at each pixel by a small offset value. In this contribution, the offset candidates are {1, 2}. For example, if residual adjustment is performed with an offset value of 1, all positive residual values are decremented by 1, while all negative residual values are incremented by 1. An example of residual adjustment with offset value 1 is shown in Fig. 3.

Fig. 3: an example of residual offset

Two frame level flags are signalled to indicate whether the residual offset method is enabled for luma and chroma, respectively. If residual offset is enabled, another frame level flag is signalled to indicate what offset value is used for adjusting the corresponding residual colour component(s).

# Experiment results

The proposed method has been implemented on top of the AHG11 reference software with the macro NnlfOption=1, which indicates the activation of the NNLF set from JVET-AA0088. The simulations were performed by following the NNVC common test condition (CTC)[2]. Since the residual offset method is only used for B-Slice, only Random Access and Low Delay B configurations were tested.

Table 1. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over AHG11 reference software (NnlfOption=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -2.06% | -0.50% | 100% | 100% |
| Class A2 | -0.12% | -0.75% | -0.22% | 101% | 100% |
| Class B | -0.09% | -0.97% | -0.26% | 100% | 102% |
| Class C | -0.08% | -0.65% | -0.34% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -1.06% | -0.32% | 100% | 102% |
| Class D | 0.02% | -0.72% | -0.45% | 99% | 102% |
| Class F | -0.02% | -0.17% | -0.14% | 100% | 101% |

Table 2. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over AHG11 reference software (NnlfOption=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.51% | -1.65% | -0.16% | 100% | 106% |
| Class C | -0.61% | -1.60% | -0.91% | 99% | 106% |
| Class E | -0.70% | 1.19% | -0.42% | 98% | 127% |
| Overall | -0.59% | -1.63% | -0.49% | 99% | 111% |
| Class D | -0.35% | -2.09% | -2.53% | 100% | 104% |
| Class F | -0.11% | -1.91% | -1.16% | 101% | 102% |

# Conclusions

This contribution proposes an adjustment of residual for the NNLF set based on JVET-AA0088 in AHG11 reference software to further improve the coding performance. This method can provide good performance on top of JVET-AA0088 without increasing computation complexity. It is suggested to adopt this method into the NNLF set from JVET-AA0088 in the AHG11 reference software.

# References

- EE1-1.5: neural network based in-loop filter with a single model , JVET-AA0088, L. Wang, S. Lin, X. Xu, S. Liu (Tencent), F. Galpin (InterDigital)

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-AA2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).