| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0166-v2 |
| --- | --- |

| Title: | Non-EE2: Unified pruning of affine merge candidates derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to unify pruning for affine merge candidates derivation. In ECM-6.0, two different pruning logics (denoted as A and B) are used for affine merge candidates derivation, which is asserted to complicate the affine prediction design. Two solutions are tested in this proposal. In solution #1, pruning in affine merge are unified to pruning logic A. In solution #2, pruning in affine merge are unified to logic B. Simulation results based on ECM-6.0 are reported as below:

Solution #1:

RA: 0.00%, 0.05%, -0.01%, 100%, 100%

LB: -0.02%, 0.10%, 0.21%, 100%, 100%

Solution #2:

RA: -0.01%, 0.01%, -0.03%, 100%, 100%

LB: 0.04%, 0.11%, 0.42%, 100%, 100%

# Introduction

In ECM-6.0, affine candidates can be derived from adjacent based affine candidates, regression based affine candidates, history based affine candidates, and non-adjacent based affine candidates. Similarity check (a.k.a., pruning) is conducted during the affine candidates derivation. However, in ECM-6.0, different pruning logics (denoted as A and B) are used for those affine merge candidates derivation. Specifically, pruning logic A makes comparisons on BCW index, LIC flag, affine type, and reference index associated to the affine candidates; while pruning logic B does not consider the first three factors. Moreover, pruning logic A uses a fixed threshold for CPMV difference checking, while pruning logic B uses a block size adaptive threshold determination. In the current software, pruning logic A is used for adjacent based and history based affine merge candidates derivation, while pruning logic B is used for regression based and non-adjacent based affine merge candidates derivation. It is asserted such processes complicate the design logic of affine candidates derivation.

# Proposal

It is proposed to use a consistent pruning logic for all affine merge candidates. Two solutions are tested. In solution #1, pruning logic A is used for all affine merge candidates. In solution #2, pruning logic B is used for all affine merge candidates.

# Experimental Results

The proposed method was implemented on top of ECM-6.0 [1]. Experiments were conducted following the common test conditions [2] for enhanced compression tool testing. Simulation results of solution #1 and solution #2 are tabulated in Table 1 and Table 2, respectively.

Table 1: Performance of solution #1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.17% | -0.02% | 100% | 100% |
| Class A2 | -0.01% | -0.01% | 0.01% | 100% | 100% |
| Class B | 0.01% | 0.15% | -0.09% | 100% | 100% |
| Class C | 0.00% | -0.11% | 0.08% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.05% | -0.01% | 100% | 100% |
| Class D | 0.02% | -0.14% | 0.20% | 100% | 100% |
| Class F | -0.02% | 0.10% | 0.06% | 100% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.03% | -0.06% | 100% | 100% |
| Class C | -0.05% | -0.04% | 0.21% | 99% | 100% |
| Class E | -0.10% | 0.50% | 0.65% | 99% | 100% |
| Overall | -0.02% | 0.10% | 0.21% | 100% | 100% |
| Class D | -0.08% | 0.23% | 0.10% | 100% | 102% |
| Class F | -0.13% | -0.19% | -0.33% | 100% | 101% |

Table 2: Performance of solution #2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.05% | 0.14% | 100% | 100% |
| Class A2 | -0.04% | -0.01% | -0.07% | 100% | 100% |
| Class B | 0.00% | 0.03% | -0.15% | 100% | 100% |
| Class C | 0.01% | 0.05% | 0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | -0.03% | 100% | 100% |
| Class D | 0.02% | 0.07% | -0.21% | 100% | 100% |
| Class F | 0.00% | -0.04% | -0.06% | 100% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.01% | 0.46% | 100% | 100% |
| Class C | 0.01% | 0.01% | 0.05% | 100% | 101% |
| Class E | 0.05% | 0.42% | 0.84% | 100% | 100% |
| Overall | 0.04% | 0.11% | 0.42% | 100% | 100% |
| Class D | -0.16% | -0.37% | 0.90% | 100% | 101% |
| Class F | -0.06% | 0.12% | 0.07% | 100% | 101% |

# Conclusions

The contribution reports the results of a consistent pruning process for affine merge candidates derivation. It is asserted that the proposed method simplifies the design logic of affine prediction and cleans up the ECM software. It is proposed to adopt the proposed method to the ECM.

# References

- ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-6.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).