| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0178-v1 |
| --- | --- |

| Title: | EE2-related: Control-point motion vector refinement for Affine DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Chun-Chi Chen, Yan Zhang, Zhi Zhang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 USA | Email: | hanhuang@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes control point motion vector refinement for affine DMVR. The control-point motion vector is independently refined by bilateral matching of a block centered by the control-point, then it is iteratively refined to minimize the bilateral matching cost of the coding block. The proposed method is also extended to be applied in affine MMVD mode. The simulation result reports -0.39%, -0.23%, -0.29% Y, U, V, BD-rate reduction.

# Introduction

In JVET-AA0144, it is proposed to apply DMVR for the blocks coded by affine merge mode. A MV offset is added to all the control-point motion vectors (CPMV) to minimize the bilateral matching cost. The proposed method is tested in EE-2 Test 2.6.  To calculate the bilateral matching cost for the affine coded block, subblock-based affine motion compensation is performed to generate the predictors at two prediction directions.  Then bilateral matching cost is defined as the distortion between the two predictors.

In this contribution, CPMV refinement for affine DMVR is proposed, wherein a different MV offset may be applied to different control-point motion vectors.

# Proposal

Given initial control-point motion vectors initCpMvLX[cpIdx] with cpIdx=0..numCpMv – 1, wherein the numCpMv is the number of CPMVs of the current affine coding block. The proposed method can be described in the following steps:

- For each control-point, perform bilateral matching for a block that centered by the control-points to derive the refined CPMVs bmRefinedCpMvLx[cpIdx], as illustrated in Fig. 1.

- Loop over combinations of initCpMvLX[cpIdx] and bmRefinedCpMvLx[cpIdx], derive a best set of CPMVs that minimize the bilateral matching cost of the current block.

- Iteratively further refine the CPMVs to minimize the bilateral matching cost of the current block. In each iteration, one CPMV is refined while the others are fixed.

mv0’

mv1’

mv2’

Fig. 1 Illustration of independent bilateral matching search for CPMVs

# Test results

The proposed method is implemented on to of EE-2 Test 2.6. It’s also applied to the affine MMVD mode in ECM.

Table 1 shows the test results of proposed method, an average of -0.39%, -0.23%, -0.29%  Y, U, V BD-rate reduction for random access test condition. Table 2 shows test results of proposed method without applying it to affine MMVD mode, an average of -0.33%, -0.15%, -0.20% Y, U, V BD-rate reduction is reported. Runtimes in Table 1 is not inaccurate.

Table 1 Test result of proposed method

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.51% | -0.25% | -0.33% | 178% | 139% |
| Class A2 | -0.73% | -0.51% | -0.51% | 179% | 147% |
| Class B | -0.29% | -0.13% | -0.26% | 152% | 124% |
| Class C | -0.14% | -0.15% | -0.13% | 123% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.39% | -0.23% | -0.29% | 153% | 125% |
| Class D | -0.10% | -0.14% | -0.21% | 107% | 91% |
| Class F | -0.07% | -0.08% | -0.13% | 113% | 101% |

Table 2 Test result of proposed method without applied to affine MMVD

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -0.17% | -0.26% |  |  |
| Class A2 | -0.65% | -0.45% | -0.46% |  |  |
| Class B | -0.25% | -0.04% | -0.20% |  |  |
| Class C | -0.11% | -0.04% | 0.02% | 107% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.15% | -0.20% |  |  |
| Class D | -0.06% | -0.03% | -0.06% | 105% | 104% |
| Class F | -0.03% | 0.00% | -0.06% |  |  |

# Conclusion

This contribution proposed control-point motion vector refinement applied to affine DMVR and affine MMVD modes. An average of -0.39%, -0.23%, -0.29%  Y, U, V BD-rate reduction for random access test condition is reported. It is suggested to study the proposed method in the next EE.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Chen, R.-L. Liao, X. Li, Y. Ye, “Non-EE2: DMVR for affine merge coded blocks”, JVET-AA0144, July 2022.