| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0187-v1 |
| --- | --- |

| Title: | Non-EE2: No luma subsampling for CCCM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Yao-Jen Chang, Bappaditya Ray, Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ECM, convolutional cross-component intra prediction model derives the model between luma and chroma. In 4:2:0, luma component is subsampled first before deriving and applying the prediction model. In this contribution, it is proposed to disable the subsampling stage for CCCM.

Test results reportedly show BD-rate reduction for Y, U, V components as follows.

AI configuration:

Class F: -0.20, -0.45%, 0.06%; 101% enc time, 98% dec time

Class TGM: -1.35, -2.48%, -2.17%; 100% enc time, 102% dec time

# Introduction

In ECM, convolutional cross-component intra prediction model (CCCM) derives a model to predict chroma samples using reconstructed luma samples. In 4:2:0, luma component is subsampled first using 3x2 filter, then after model derivation it is applied to subsampled luma component of the current block using 7-tap filter consisting of a 5-tap spatial samples, a nonlinear term, and a bias term. The input to the spatial 5-tap component of the filter consists of a center (C) luma sample which is collocated with the chroma sample to be predicted and its above/north (N), below/south (S), left/west (W) and right/east (E) neighbors as illustrated shown on Figure 1.

Figure 1. 3x2 subsampling filter (left) and 3x3 CCCM filter (right)

# Proposal

Subsampling of luma component may not be optimal for CCCM model derivation for the content which has sharp details, such as SCC content. In this contribution it is proposed to disable luma subsampling, derive and apply model on nonsubsampled luma samples directly. CCCM model shape is diamond 5x5 if subsampling is not applied. SPS flag is signalled to indicate whether luma subsampling is applied for CCCM.

The proposed method was implemented on top of ECM-6.0 and test results are summarized in the below tables.

Table 1. CCCM with no luma subsampling

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.20% | -0.45% | 0.06% | 101% | 98% |
| Class TGM | -1.35% | -2.48% | -2.17% | 100% | 102% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).