| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0192 |
| --- | --- |

| Title: | Non-EE2: Extended partitioning mode for the inter/intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70 3508 5829 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes two extended partitioning modes for the inter/intra prediction to further enhance coding performance of ECM. The first is an extended geometric partitioning mode (GPM) that increases the discreate angles of the existing GPM from 10 to 16. The second is L-shaped partitioning mode (LSPM) that separates a coding block into two areas with two orthogonal straight lines (i.e., L-shaped partitioning), different from the existing GPM. The extended GPM and LSPM is signaled at a coding block level by extending the existing GPM partition index. The signalling overhead for them is suppressed by the template-based reordering for the GPM partition index adopted in the current ECM. Therefore, the extend GPM and LSPM can improve the prediction accuracy in m ore complicate object boundaries in addition to the existing GPM.

Simulation test results of over all classes on the top of ECM-6.0 in RA and LB are reported as follows:

- Extend GPM only enabled

  - RA (Overall): BD-rate Y/U/V: -0.03%/0.00%/-0.01%; EncT: 101%; DecT: 100%

  - LB (Overall): BD-rate Y/U/V: -0.02%/0.12%/0.19%; EncT: 102%; DecT: 101%

- LSPM only enabled

  - RA (Overall): BD-rate Y/U/V: -0.03%/0.00%/-0.04%; EncT: 102%; DecT: 100%

  - LB (Overall): BD-rate Y/U/V: -0.10%/-0.05%/0.25%; EncT: 102%; DecT: 100%

- Extended GPM + LSPM enabled

  - RA (Overall): BD-rate Y/U/V: -0.07%/0.03%/-0.02%; EncT: 104%; DecT: 100%

  - LB (Overall): BD-rate Y/U/V: -0.08%/-0.04%/0.23%; EncT: 106%; DecT: 101%

Introduction

Geometric partitioning mode (GPM) in VVC [1] is an efficient prediction tool for the boundaries of foreground and background with different motions. This is because the GPM separates a coding block into two areas with a single but 64 types of straight lines according to the object boundaries, and generates the final prediction samples by blending the prediction samples in each area. The 64 types of straight lines are determined by different discrete 10 angles and 4 distances and signaled at a coding block level. However, the existing GPM cannot always generate the highly accurate prediction samples around the object boundaries, for example, when a coding block size is large or the object boundary is not separated by the single straight line. To tackle the problem, we propose the following two extended partitioning modes for the inter/intra prediction.

Proposed methods

Extended geometric partitioning mode

We propose an extended GPM that increases the discreate angles of the existing GPM from 10 to 16 as shown in Figure 1. In this mode, the discrete distance is the same as the existing GPM. The extended GPM is signaled by extending the existing GPM partition index. This means that the additional 42 types of the GPM partition can be selectable at a coding block level. The signalling overhead for the extended GPM is suppressed by the template-based reordering for the GPM partition index adopted in the current ECM [2]. The extended GPM is applicable for both intra and inter prediction according to the GPM with inter and intra prediction adopted in the ECM [2]. Also, the adaptive blending of GPM adopted in the ECM [2] is reflected.

Figure 1 The discrete angles of the existing GPM (blue lines) and the extended GPM (orange lines).

L-shaped partitioning mode

Unlike the existing GPM, we also propose a L-shaped partitioning mode (LSPM) that separates a coding block into two areas with two orthogonal straight lines (i.e., L-shaped partitioning). The partition types of LSPM are defined by 8 partition points equally spaced at each coding block size as shown in Figure 2. Similarly to the extended GPM described in Sec. 2.1, the partition types of LSPM are signaled at a coding block level by extending the existing GPM partition index. Therefore, the signalling overhead for the LSPM is also suppressed by the template-based reordering for the GPM partition index adopted in the current ECM [2]. The LSPM is also applicable for both intra and inter prediction according to the GPM with inter and intra prediction adopted in the ECM [2]. In addition, the adaptive blending of GPM adopted in the ECM [2] is reflected in the LSPM.

The LSPM generates the final prediction samples by blending the prediction samples in each area as well as the GPM, while the derivation of the blending weights is different from the existing and extended GPM. As shown in Figure 3, the minimum weight is selected from the two weights for two partitioning boundaries of LSPM. This allows LSPM to generate highly accurate prediction samples for the boundaries.

Figure 2 The partition types of LSPM defined by 8 partition points equally spaced at each block size.

Figure 3 An example of the derivation of LSPM blending weights.

Test Results

The proposed methods are implemented on top of ECM-6.0 [3]. We evaluated the proposed methods by three testes as follows:

- Test 1: Extended GPM only enabled

- Test 2: LSPM only enabled

- Test 3: Extended GPM + LSPM enabled

The simulation tests were conducted following the common test condition and evaluation procedures for enhanced compression tool testing [4].

Tests 1, 2, and 3 results under RA and LB configurations are summarized Tables 1, 3, and 3, respectively. From Tables 1 and 2, the extended GPM and LSPM provide some coding gain in RA/LB configurations compared to ECM-6.0 with minor increments of encoder and decoder runtime. Table 1 shows the Extended GPM brings higher coding gains for the high resolution classes while the LSPM provides more gains for the low resolution classes. Table 3 shows the additive effects of the Extended GPM and LSPM.

More detailed results can be found in the attached spreadsheets.

Table 1 Test 1 results (Extended GPM only enabled) of over ECM-6.0.

|  |  |  | RA |  |  |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.05% | -0.13% | 0.05% | 101% | 100% |  |  |  |  |  |
| A2 | -0.04% | 0.03% | 0.01% | 101% | 100% |  |  |  |  |  |
| B | -0.01% | 0.07% | -0.08% | 101% | 100% | 0.03% | 0.02% | 0.24% | 102% | 101% |
| C | -0.03% | -0.03% | 0.02% | 102% | 100% | -0.10% | -0.10% | 0.16% | 103% | 101% |
| E |  |  |  |  |  | 0.01% | 0.58% | 0.16% | 100% | 101% |
| Overall | -0.03% | 0.00% | -0.01% | 101% | 100% | -0.02% | 0.12% | 0.19% | 102% | 101% |
| D | 0.03% | -0.27% | 0.06% | 102% | 100% | 0.02% | -0.14% | 0.88% | 103% | 102% |
| F | 0.00% | 0.01% | 0.04% | 100% | 100% | 0.09% | -0.37% | -0.14% | 99% | 101% |

Table 2 Test 2 results (LSPM only enabled) of over ECM-6.0.

|  |  |  | RA |  |  |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.01% | -0.05% | 0.07% | 101% | 100% |  |  |  |  |  |
| A2 | -0.06% | -0.06% | -0.07% | 101% | 100% |  |  |  |  |  |
| B | -0.01% | 0.09% | -0.09% | 102% | 100% | -0.01% | 0.06% | 0.09% | 102% | 100% |
| C | -0.05% | -0.02% | -0.03% | 102% | 100% | -0.15% | -0.16% | 0.29% | 103% | 100% |
| E |  |  |  |  |  | -0.21% | -0.08% | 0.46% | 99% | 99% |
| Overall | -0.03% | 0.00% | -0.04% | 102% | 100% | -0.10% | -0.05% | 0.25% | 102% | 100% |
| D | 0.02% | -0.29% | 0.02% | 102% | 100% | -0.18% | 0.15% | 0.34% | 104% | 100% |
| F | -0.08% | -0.10% | -0.08% | 100% | 100% | -0.03% | -0.31% | -0.18% | 100% | 100% |

Table 3 Test 3 results (Extended GPM + LSPM enabled) of over ECM-6.0.

|  |  |  | RA |  |  |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.06% | -0.10% | 0.05% | 104% | 100% |  |  |  |  |  |
| A2 | -0.09% | 0.03% | -0.06% | 104% | 100% |  |  |  |  |  |
| B | -0.06% | 0.21% | -0.09% | 104% | 100% | -0.05% | -0.39% | 0.23% | 106% | 100% |
| C | -0.09% | -0.10% | 0.03% | 105% | 100% | -0.13% | -0.06% | 0.02% | 108% | 101% |
| E |  |  |  |  |  | -0.07% | 0.57% | 0.52% | 105% | 100% |
| Overall | -0.07% | 0.03% | -0.02% | 104% | 100% | -0.08% | -0.04% | 0.23% | 106% | 101% |
| D | 0.00% | 0.05% | 0.04% | 105% | 100% | -0.04% | -0.72% | 0.52% | 108% | 102% |
| F | -0.09% | -0.12% | -0.11% | 103% | 100% | -0.26% | -0.42% | 0.13% | 103% | 101% |

Conclusion

In this contribution, as an extended partitioning modes for inter/intra prediction, the extended GPM and LSPM are proposed. The proposed methods can bring some coding gains and the additive effects are observed in the two methods. The proposed method has room to further improve the coding performance by studying RDO process, signalling, and so on. It is recommended to be further investigated in next EE2.

This work was supported by Ministry of Internal Affairs and Communications (MIC) of Japan (Grant no. JPJ000595).

Reference

- B. Bross, J. Chen, S. Liu, and Y. K. Wang, “Versatile Video Coding Editorial Refinements on Draft 10,” JVET-T2001, by teleconference, 7-16, October. 2020.

- M. Coban, F. Le Léannec, K. Naser, J. Ström, and L. Zhang, “Algorithm description of Enhanced Compression Model 6 (ECM 6),” JVET-AA2025, by teleconference, 13-22 July. 2022.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-6.0

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, by teleconference, 12-21, January. 2022.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).