| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0193-v2 |
| --- | --- |

| Title: | AHG9: A summary of proposals on NNPF SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 8910 University Center Lane, San Diego, CA 92122, USA | Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution intends to provide a summary of all proposals on NNPF SEI messages, including those included in the VSEI CDAM ballot comments in MPEG input document m60678. It is suggested that this summary is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# List of proposals on NNPF SEI messages

This category of proposals is listed in the agenda item 6.1, including 12 contributions and 58 ballot comments. These contributions and summaries of the related proposals are listed below.

Ballot comments in MPEG input document m60678

Comments 008 to 065

The technical and general comments are included below (editorial comments are not included herein):

| FI_03-009 | nnpfc_inp_format_flag and nnpfc_out_format_flag indicate a real value in the range of 0 to 1, or an unsigned integer value with a given bit depth. It would be good to leave extensibility possibilities for future versions, e.g., real value in the range of -1 to 1. | Replace the syntax elements with ue(v)-coded nnpfc_inp_format_idc and nnpfc_out_format_idc and reserve values greater than 1. |
| --- | --- | --- |
| FI_02-010 | nnpfc_uri_tag and nnpfc_uri are gated by nnpfc_purpose_and_formatting_flag equal to 1 and nnpfc_mode_idc equal to 2 and appear close to the end of the NNPFC SEI syntax. However, the nnpfc_uri_tag could be important for decoders to determine whether to process the SEI message at all, since it indicates the format of the neural network. If the format is not supported by the decoder, the SEI message could be ignored. nnpfc_purpose_and_formatting_flag is required to be equal to 1 only for nnpfc_mode_idc equal to 1 (for the first such SEI message within a CLVS). It is intentional that the purpose and formatting syntax elements do not need to be repeated in subsequent SEI messages with nnpfc_mode_idc equal to 1 and the same nnpfc_id as in the previous SEI message, which contain NNR-coded neural network updates. We think that the post-processing engine needs to have instructions for formatting the input tensors also when nnpfc_mode_idc is equal to 0 or 2. Furthermore, the syntax already requires nnpfc_purpose_and_formatting_flag to be equal to 1 when nnpfc_mode_idc is equal to 2 in order to signal nnpfc_uri_tag and nnpfc_uri. In other words, the syntax implicitly already contains the requirement that nnpfc_purpose_and_formatting_flag shall be equal to 1 when nnpfc_mode_idc is equal to 2. | Both of the following are proposed: - Move the gated syntax that contains nnpfc_uri_tag and nnpfc_uri just after nnpfc_mode_idc. - Add a requirement to have nnpfc_purpose_and_formatting_flag equal to 1 when nnpfc_mode_idc is equal to 0 or 2. |
| FI_01-011 | nnpfc_mode_idc values 0 or 2 enable identifying a base neural-network post-filter by external means or through given URIs, respectively. For many envisioned systems, this is beneficial compared to transmitting an NNR-encoded neural network representation in-band (i.e., nnpfc_mode_idc equal to 1). Adapting the neural network with NNR-encoded neural network updates has been shown to be beneficial. Such NNR-encoded updates can be used together with any means to identify the base neural-network post-filter (i.e., any value of nnpfc_mode_idc). The tag URI mechanism for nnpfc_mode_idc equal to 2 enables decentralized definitions of the format of the neural network identified by the URI nnpfc_uri. The tag URI is defined using a domain name, which most organizations have already. Thus, we think that all nnpfc_mode_idc values are well justified. | Keep all the nnpfc_mode_idc values (0, 1, 2) along with all the related syntax elements as specified in the CDAM. |
| US-017 | The syntax specification for nnpfc_complexity_element( ) does not make much sense – e.g. this syntax structure is sent whenever nnpfc_complexity_idc is greater than 0, but it doesn’t have anything in it unless nnpfc_complexity_idc is equal to 1. | Change the syntax to get rid of the part that does not make sense. |
| US-019 | When nnpfc_mode_idc is equal to 0 or 2, there does not appear to be anything technical in the draft specification that particularly associates the actual processing applied to the pictures with the name “neural-network post-filter” or that specifies what processing is to be applied to the pictures (at least without access to other information that could be unpublished). | Consider disallowing syntax/semantics combinations that may not give a semantic meaning to the provided name “neural-network post-filter” or that do not specify what processing is to be applied to the pictures. |
| US-020 | A “tag” format is specified for the nnpfc_uri_tag[ i ] identifier, with syntax and semantics as specified in IETF RFC 4151. It would be possible for Rec. ITU-T T.35 data to be used as an alternative signalling of a neural network post-filtering identifier. The standard already supports the use of T.35 for user data. | Consider allowing the use of Rec. ITU-T T.35 data as an additional alternative way of identifying processing to be applied for this SEI message, since T.35 data is already supported in the standard and can fulfil a similar purpose. |
| US-021 | The neural-network post-filter characteristics (NNPFC) SEI message specifies that the purpose of a neural-network post-processing filter may be for visual quality improvement. However, visual quality improvement may have different interpretations, such as fidelity-constrained visual quality improvement, GAN (Generative Adversarial Network)-based visual quality improvement, film grain-based visual quality improvement, etc. Different video applications may prefer different types of visual quality improvements. For example, fidelity is important in surveillance scenario, but may not be so critical in some user-generated videos. Therefore, it would be desirable to be able to identify the type of visual quality improvement. | Consider defining and signalling of different types of visual quality improvements, including e.g., the following: - General visual quality improvement, targeting at increasing either the fidelity or the subjective visual quality of the reconstructed picture after applying the neural-network post-processing filter. The improvement may be measured by any objective or subjective metric. - Objective-oriented/fidelity-oriented visual quality improvement, targeting at increasing the fidelity of the reconstructed picture after applying the neural-network post-processing filter. The fidelity may be measured by PSNR, Ms-SSIM etc. - Subjective-oriented visual quality improvement, targeting at increasing the subjective visual quality of the reconstructed picture after applying the neural-network post-processing filter. The subjective visual quality may be measured by LPIPS or MOS. - Film grain-oriented visual quality improvement, with synthesizing filter grain on the reconstructed picture after applying the neural-network post-processing filter. |
| US-022 | When the purpose of the filter involves change of either or both of the picture width and height, the picture width and height of the output of the filter are directly signalled using two ue(v) syntax elements, nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples. There are a few issues herein: 1) The value ranges for these two ue(v)-coded syntax elements are missing. 2) Both of the output picture width and height are directly signalled even when only one of them changes compared to the input value(s). This is inefficient. 3) Per the semantics of nnpfc_purpose, when nnpfc_purpose is equal to 3 or 4, at least one of the output picture width and height increases, and the width and height never become smaller. Therefore, it would be much more efficient to signal the delta values using ue(v)-coded syntax elements. | Consider making the following changes: 1) Instead of directly signal the filter output picture width and height values, the delta values of them compared to the filter input picture width and height values, respectively, are signalled using ue(v)-coded syntax elements, e.g., named nnpfc_delta_pic_width and nnpfc_delta_pic_height, respectively. 2) The value ranges of nnpfc_delta_pic_width and nnpfc_delta_pic_height are specified, e.g., as follows: The value of nnpfc_delta_pic_width shall be in the range of 0 to CroppedWidth * 15 − 1, inclusive. The value of nnpfc_delta_pic_height shall be in the range of 0 to CroppedHeight * 15 − 1, inclusive. |
| FI_08-024 | The tag URI scheme provides flexibility of identifying the format of neural network identified by nnpfc_uri without a central registration authority. However, the tag URI to be used with ISO/IEC 15938-17 (NNR) should be specified, since ISO/IEC owns the NNR format. | Specify a tag URI value (e.g., "tag:iso.org,2023:15938-17") indicating that the neural network identified by nnpfc_uri conforms to ISO/IEC 15938-17. |
| FI_05-027 | StrengthControlVal is a variable that is required to be defined when nnpfc_auxiliary_inp_idc is equal to 1. In VVC, it is defined to be the value of SliceQpY of the first slice of the picture. In Table 23, StrengthControlVal is normalized to the input tensor as follows: inputTensor[ ][ ][ ][ ] = 2(StrengthControlVal − 42) / 6 The values 42 and 6 appear to be related to common test conditions and the quantization scheme of VVC. VSEI should not have such direct dependencies on VVC. | In VSEI, Change the StrengthControlVal to be a variable that has been normalized to the range 0..1. In Table 23, insert the given StrengthControlVal directly into the input tensor: inputTensor[ ][ ][ ][ ] = StrengthControlVal (These proposed changes have a consequence that the normalization of the SliceQpY value has to be added to the derivation of StrengthControlVal within the use of the NNPFC SEI message in the VVC text.) |
| US-028 | There are two syntax elements that are specified as UTF-8 character strings. One of them says it shall be as specified in ISO/IEC 10646 and the other does not. Shouldn’t they both be as specified in ISO/IEC 10646? | Clarify. |
| US-030 | The semantics of nnpfc_id contains the following sentence: Decoders encountering a value of nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it. "It" herein is "the value of nnpfc_id". However, it is sufficient to just ignore the value of nnpfc_id. For example, if there are multiple filters specified each being associated with a particular value of nnpfc_id, and if one or more of these nnpfc_id values are within the reserved value ranges, the switch between different filters would be problematic if the ID values of these filters are ignored while the filters themselves are not ignored. | Change the following sentence: Decoders encountering a value of nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it. to the following: Decoders encountering a neural-network post-filter characteristics SEI message with nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message. |
| US-034 | Table 20 provides the definitions of the different purposes of neural network post-processing filters. Currently, the value 0 (of nnpfc_purpose) is specified as "unknown or unspecified", values 1 to 4 are specified for four different specific purposes, and other values are reserved. However, what is the usefulness of specifying this "unknown or unspecified" purpose? Note that some aspects related to the case of nnpfc_purpose equal to 0 are still specified, e.g., the values of outSubWidthC and outSubHeightC by the semantics of nnpfc_out_sub_c_flag regardless of whether nnpfc_purpose is equal to 0. In addition, if the purpose is unknown or unspecified, why was the entire SEI message included by the encoder in the bitstream at the first place and how is the decoder / client supposed to use such an SEI message? Finally, for extensibility, there are already plenty of reserved values specified for this syntax element. | Specify the value 0 for nnpfc_purpose to be a specific purpose instead of "unknown or unspecified" |
| US-035 | Is the syntax for specification of input and output formatting really necessary? Doesn’t the NNR standard itself determine its input and output format behaviour? Can this information conflict or require a conversion process that may be unnecessary? | Remove any unnecessary input/output formatting specification. |
| US-036 | The standard generally uses an array convention in which the horizontal index is first and the vertical index is second. In several of these expressions, the indexing is swapped relative to that usual convention, e.g. for InpSampleVal( ) indexing. Is the order swapping necessary? | Use the same convention consistently. If using an inconsistent convention is necessary (e.g. for compatibility with the NNR standard), highlight the difference in a NOTE to minimize reader confusion. |
| US-037 | The semantics of nnpfc_out_sub_c_flag is as follows: nnpfc_out_sub_c_flag equal to 1 specifies that outSubWidthC is equal to 1 and outSubHeightC is equal to 1. nnpfc_out_sub_c_flag equal to 0 specifies that outSubWidthC is equal to 2 and outSubHeightC is equal to 1. When nnpfc_out_sub_c_flag is not present, outSubWidthC is inferred to be equal to SubWidthC and outSubHeightC is inferred to be equal to SubHeightC. If SubWidthC is equal to 2 and SubHeightC is equal to 1, nnpfc_out_sub_c_flag shall not be equal to 0. The last sentence specifies a constraint on the value of nnpfc_out_sub_c_flag. However, it is possible that SubWidthC is equal to 2 and SubHeightC is equal to 1, while nnpfc_out_sub_c_flag is not present (when nnpfc_purpose is equal to a value that is neither 2 nor 4). In this case, per the constraint, nnpfc_out_sub_c_flag shall be equal to 1, while when nnpfc_out_sub_c_flag is equal to 1, per the first sentence in the semantics, outSubWidthC is equal to 1 and outSubHeightC is equal to 1. While on the other hand, per the second sentence in the semantics, when nnpfc_out_sub_c_flag is not present, outSubWidthC is inferred to be equal to SubWidthC and outSubHeightC is inferred to be equal to SubHeightC, and therefore in this case the values of outSubWidthC and outSubHeightC would be inferred to be equal to 2 and 1, respectively, i.e., having different values than per the first and last sentences. In short, the constraint specified by the last sentence is contradicting with the second sentence in certain condition. | Replace the following sentence: If SubWidthC is equal to 2 and SubHeightC is equal to 1, nnpfc_out_sub_c_flag shall not be equal to 0. to be the following: When ChromaFormatIdc is equal to 2 and nnpfc_out_sub_c_flag is present, the value of nnpfc_out_sub_c_flag shall be equal to 1. |
| US-040 | The neural-network post-filter characteristics (NNPFC) SEI message specifies the auxiliary input to the neural network, which only includes quantization parameter-related data. However, there are other auxiliary inputs which can also be very helpful in terms of improving the performance of post-processing filtering. Therefore, it would be desirable to enable the use of more auxiliary inputs. | Consider defining and specifying the use of more auxiliary inputs to the neural network post-processing filter, including, e.g., prediction samples. |
| US-044 | The neural-network post-filter characteristics (NNPFC) SEI message specifies that the output tensor of the filter either does not include a chroma matrix or include both chroma matrices. However, the capability of processing two chroma components separately can be very useful. Therefore, it would be desirable to enable separate processing of the two chroma components or processing only one of the two chroma components by one NNPFC SEI message. | Consider enabling separate processing of the two chroma components, including processing only one of the two chroma components by one NNPFC SEI message. |
| US-045 | The neural-network post-filter characteristics (NNPFC) SEI message specifies the process for deriving the input tensor inputTensor in Table 23. When nnpfc_inp_order_idc is equal to 3, nnpfc_component_last_flag is equal to 0, and nnpfc_auxiliary_inp_idc is equal to 0, no auxiliary input matrix is supposed to be present, otherwise, the result would be incorrect. Therefore, the assignment operation "inputTensor[ 0 ][ 6 ][ yP + overlapSize ][ xP + overlapSize ] = 2(StrengthControlVal – 42)/6'' should be removed in this case. | Consider making the following change: When nnpfc_inp_order_idc is equal to 3, nnpfc_component_last_flag is equal to 0, and nnpfc_auxiliary_inp_idc is equal to 0, no auxiliary input matrix is used. In other words, remove the assignment operation "inputTensor[ 0 ][ 6 ][ yP + overlapSize ][ xP + overlapSize ] = 2(StrengthControlVal – 42)/6'' for this case from Table 23. |
| US-046 | The value of nnpfc_constant_patch_size_flag equal to 0 specifies that the post-processing filter accepts any patch size that is a positive integer multiple of the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. nnpfc_constant_patch_size_flag equal to 1 specifies that the post-processing filter accepts exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. However, on the one hand, it is allowed for nnpfc_constant_patch_size_flag to be equal to 0, while on the other, regardless of the value of nnpfc_constant_patch_size_flag, the filtering process specified as part of the semantics of the NNPFC SEI message always use exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. In other words, the actual support of taking as input any patch size that is a positive integer multiple of the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 is missing. | Consider specifying the following: When nnpfc_constant_patch_size_flag is equal to 0, the patch size width, denoted by inpPatchWidth, and the patch size height, denoted by inpPatchHeight, are provided by external means. An example of such external means is an API that passes the values of inpPatchWidth and inpPatchHeight to the decoder and render entity in a video application system, and the values may be configured by a user through a user interface of the application. |
| US-049 | The neural-network post-filter activation (NNPFA) SEI message specifies the neural-network post-processing filter that may be used for post-processing filtering for the current picture, and the NNPFA SEI message persists only for the current picture. However, this is inefficient when a particular filter applies to a set of consecutive pictures in output order. | Consider enabling activating a neural-network post-processing filter for a set of consecutive pictures in output order in a CLVS by using one neural-network post-filter activation SEI message. |
| US-050 | The neural-network post-filter activation (NNPFA) SEI message activates a neural-network post-processing filter for an entire picture. However, it can be desirable to enable activating different neural-network post-processing filters for different regions (e.g., subpictures, slices, tiles, CTUs) of a picture. | Consider enabling activating different neural-network post-processing filters for different regions (e.g., subpictures, slices, tiles, CTUs) of a picture. |
| US-053 | The presence of a neural-network post-filter activation SEI message is only meaningful when there is an associated neural-network post-filter characteristics SEI message (with the same filter ID) present. However, currently, a neural-network post-filter activation SEI message with a particular value of nnpfa_id is allowed to be present in a picture unit even if there is no neural-network post-filter characteristics SEI message with nnpfc_id equal to the particular value of nnpfa_id present in the CLVS. | Consider specifying the following constraint: A neural-network post-filter activation (NNPFA) SEI message with a particular value of nnpfa_id shall not be present in a current PU unless within the current CLVS there is a neural-network post-filter characteristics (NNPFC) SEI message with nnpfc_id equal to the particular value of nnpfa_id present in a PU preceding the current PU in decoding order or in the current PU. |
| US-054 | Within a picture unit, a neural-network post-filter activation (NNPFA) SEI message with a particular value of nnpfa_id may be present preceding the associated neural-network post-filter characteristics (NNPFC) SEI message in decoding order. However, it would be much cleaner for an NNPFA SEI message to always follow the associated NNPFC SEI message in decoding order. | Consider specifying the following constraint: When a PU contains both an NNPFC SEI message with a particular value of nnpfc_id and an NNPFA SEI message with nnpfa_id equal to the particular value of nnpfc_id, the NNPFC SEI message shall precede the NNPFA SEI message in decoding order. |
| US-055 | As a ue(v)-coded syntax element, the value range of nnfpa_id is missing and needs to be specified. | Specify the value range of nnfpa_id similarly as for nnfpc_id, including how reserved values are handled by decoders. |
| US-056 | Is it allowed for more than one NNPFC SEI message to be present for the same picture? If so, what does this mean? Is there some limit on the number of them? If more than one is present, what happens if the nnpfc_purpose values are the same or if they are different? What happens if the nnpfc_mode_idc values are the same or if they are different? | Add appropriate clarification, explanation and/or constraints. |
| US-058 | It is our understanding that a neural network representation (NNR) can either be “intra” (coded without reference to some other NNR) or predictively/incrementally coded. Is there a way to identify which is which? In the case of incremental NNR, what is the reference? For example, is the reference the “base” representation, or could it be the accumulated result of several updates relative to a base representation? In what order are the updates applied? | Clarify |

JVET-AB0046 AHG9: On StrengthControlVal of the NNPFC SEI message [M. M. Hannuksela, M. Santamaria, F. Cricri (Nokia)]

This contribution proposes the following changes related to the neural-network post-filter characteristics (NNPFC) SEI message:

- Moving the normalization of the quantization strength input variable StrengthControlVal from the VSEI text to the use of NNPFC SEI message in the VVC text.

- Changing the normalization of StrengthControlVal from 2(StrengthControlVal − 42) / 6 to a division (÷) by 63.

It is asserted that with the proposed changes StrengthControlVal is in the range of 0..1, which could help in making StrengthControlVal applicable to codecs other than VVC too.

JVET-AB0047 AHG9: nnpfc_mode_idc related changes to the NNPFC SEI message [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

This contribution proposes the following technical changes to the neural-network post-filter characteristics (NNPFC) SEI message:

- Moving the gated syntax that contains nnpfc_uri_tag and nnpfc_uri just after nnpfc_mode_idc.

- Adding a requirement to have nnpfc_purpose_and_formatting_flag equal to 1 when nnpfc_mode_idc is equal to 0 or 2.

- Specifying a tag URI value (e.g., "tag:iso.org,2023:15938-17") indicating that the neural network identified by nnpfc_uri conforms to ISO/IEC 15938-17.

In addition, the contribution proposes editorial changes to the NNPFC SEI message.

JVET-AB0049 AHG9: Miscellaneous aspects of the two neural-network post-filtering SEI messages [Y.-K. Wang, Y. Li, C. Lin, J. Li, K. Zhang, L. Zhang (Bytedance)]

This contribution proposes the following changes for the neural-network post-filter characteristics (NNPFC) SEI message and the neural-network post-filter activation (NNPFA) SEI message:

- When encountering a NNPFC SEI message with a reserved nnpfc_id value, the decoder shall ignore the NNPFC SEI message (instead of just ignoring the nnpfc_id value).

- Specify different types of NNPF visual quality improvement, including general visual quality improvement, objective-oriented/fidelity-oriented visual quality improvement, subjective-oriented visual quality improvement, and film grain-oriented visual quality improvement.

- Specify the value 0 for nnpfc_purpose to be a specific purpose (instead of "unknown or unspecified").

- Specify the value range of nnpfc_purpose to be 0 to 255, inclusive (instead of 0 to 232 − 2, inclusive).

- Specify the following constraint to replace the existing constraint on the value of nnpfc_out_sub_c_flag, as an asserted bug fix: When ChromaFormatIdc is equal to 2 and nnpfc_out_sub_c_flag is present, the value of nnpfc_out_sub_c_flag shall be equal to 1.

- Delta code filter output picture width and height while fixing the asserted bug of missing value ranges for ue(v)-coded syntax elements.

- Specify external-means provided patch size when nnpfc_constant_patch_size_flag is equal to 0, as an asserted bug fix.

- Add the following constraint: An NNPFA SEI message with a particular value of nnpfa_id shall not be present in a current PU unless one or both of the following conditions are true: a) Within the current CLVS there is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_id present in a PU preceding the current PU in decoding order; b) There is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_id in the current PU.

- Add the following constraint: When a PU contains both an NNPFC SEI message with a particular value of nnpfc_id and an NNPFA SEI message with nnpfa_id equal to the particular value of nnpfc_id, the NNPFC SEI message shall precede the NNPFA SEI message in decoding order.

- Specify the value range and decoder handling of reserved values for nnpfa_id similarly as for nnpfc_id.

JVET-AB0050 AHG9: Activation of a neural-network post-processing filter for multiple pictures [Y.-K. Wang, K. Zhang, L. Zhang, C. Lin, J. Li, Y. Li (Bytedance)]

Currently, one neural-network post-filter activation (NNPFA) SEI message is used to activate a neural-network post-processing filter for one and only one picture. However, it is asserted that this is inefficient when a particular filter applies to a set of consecutive pictures.

This contribution proposes to enable activating a filter for more than one picture by one NNPFA SEI message. It is proposed to use one of the following three options for the purpose:

- Add one flag: nnpfa_on_flag.

- Add a new SEI message for deactivating a filter.

- Add two flags: nnpfa_cancel_flag and nnpfa_persistence_flag.

JVET-AB0058 AHG9: Frame Rate Upsampling Information in Neural-network Post-filter Characteristics SEI Message [S. Deshpande, A. Sidiya (Sharp)]

It is proposed to signal frame rate upsampling as an additional purpose for post processing filter in neural-network post-filter characteristics SEI message. For this it is proposed to conditionally signal two syntax elements related to frame rate upsampling information. Process for deriving input tensors and storing output tensors is proposed.

JVET-AB0059 AHG9: Comments on Neural-network Post-filter Characteristics SEI Message [S. Deshpande (Sharp)]

The following modifications are proposed to the neural-network post-filter characteristics SEI message:

- It is proposed to signal nnpfc_uri_tag[ i ] and nnprf_uri[ i ] syntax elements outside the conditional if statement which is conditioned on nnpfc_purpose_and_formatting_flag. It is asserted that this makes sure URI information is actually signalled in the SEI message in all cases, when nnpfc_mode_idc is specified to be equal to 2.

- It is proposed to add and modify constraints for the case when nnpfc_mode_idc is equal to 2 to allow an update of a previously signalled neural network post-processing filter.

JVET-AB0060 AHG9: On activation of the neural-network post-filter characteristics SEI message [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]

In the current VSEI working draft, the neural-network post-filter characteristics SEI message is defined on CVS and activated by the neural-network post-filter activation SEI message on PU. When there are some cases of activating all pictures in CVS, for example, in chroma format transformation, activating them for each picture is inefficient. Therefore, this contribution proposes alternative solutions. One method is to activate all pictures in the CVS, and the other is to activate pictures indicating the nuh_layer_id and the TemporalID of the NAL unit header.

JVET-AB0074 AHG9: Auxiliary input for neural-network post-processing filter [Y. Li, J. Li, C. Lin, K. Zhang, L. Zhang, Y.-K. Wang (Bytedance)]

Currently, the neural-network post-filter characteristics (NNPFC) SEI message specifies an auxiliary input to the neural network, which only includes a signal derived from the luma quantization parameter, SliceQpY. However, there is other auxiliary information that may also be helpful to improve the performance of post-processing filtering.

- This contribution proposes to feed prediction samples as an additional auxiliary input to the neural network post-processing filter.

- Furthermore, this contribution reports an asserted bug in the process for deriving the input tensor inputTensor in Table 23. It is asserted that, when nnpfc_inp_order_idc is equal to 3, nnpfc_component_last_flag is equal to 0, and nnpfc_auxiliary_inp_idc is equal to 0, no auxiliary input matrix is supposed to be present, otherwise, the result would be incorrect. Therefore, it is also proposed that the assignment operation "inputTensor[ 0 ][ 6 ][ yP + overlapSize ][ xP + overlapSize ] = 2(StrengthControlVal – 42)/6'' be removed in this case.

JVET-AB0075 AHG9: Separate processing of chroma components for neural-network post-processing filter [Y. Li, J. Li, C. Lin, K. Zhang, L. Zhang, Y.-K. Wang (Bytedance)]

Currently, the neural-network post-filter characteristics (NNPFC) SEI message specifies that the output tensor of the NN filter either does not include a chroma matrix or include both chroma matrices. In other words, the two chroma components, when present in the output tensor, are always processed at the same time. However, it is asserted that, the capability of processing two chroma components separately can be useful, e.g., when the two components possess different characteristics.

This contribution proposes to support separate processing of the two chroma components for neural-network post-processing filter.

JVET-AB0134 AHG9: On NN post-filter activation SEI message [M. Pettersson, R. Sjöberg, J. Ström (Ericsson)]

This contribution proposes two independent items for the NN post-filter activation SEI message in VSEI:

- Replace nnpfa_id with nnpfa_activation_id_plus1 and specify that nnpfa_activation_id_plus1 equal to 0 in the NN post-filter activation SEI message indicates that no NN post-filter of a NN post-filter characteristics SEI message is specified for the current picture.

- Signal a list of regions in the NN post-filter activation SEI message for which the NN post-filter is to be applied. Two alternatives are proposed:

- Signal the number of active regions, and for each region the width, height, top and left positions are signalled. If the number of active regions is equal to 0, the NN post-filter is applied to the whole picture.

- This alternative provides means for compressing the signalling of the region sizes. First, the number of regions is signalled. If the number of regions is equal to 0, the NN post-filter is applied to the whole picture. Otherwise, region sizes are signalled. Three orthogonal methods for compressing the regions are provided:

    - One flag that specifies whether all regions have the same size. If this flag is equal to 1, only one width and one height value is signalled.

    - One flag that specifies whether all regions are signaled in raster scan order. If this flag is equal to 1, the top and left position for regions is computed from the width, height and positions of previous regions. Also, if equal to 1, a flag is sent for each region that specifies whether the NN post-filter should be applied to that region or not.

    - A nnpfa_scale_factor_minus1 syntax element that specifies a scale factor that the signalled width, height, top and left positions are multiplied with.

JVET-AB0135 AHG9: On complexity metrics for NN post-filter characteristics SEI message [M. Pettersson, R. Sjöberg, J. Ström (Ericsson)]

This contribution proposes to add two additional syntax elements descriptive of the complexity of the neural network signalled by the NN postfilter characteristics SEI message. The proposed additional syntax elements are nnpfc_total_kilobyte_size and nnpfc_num_layers.

In addition, it is asserted that the current extension mechanism in the syntax structure for the complexity elements does not allow for signalling additional complexity syntax elements while also signaling the first set of complexity syntax elements. It is proposed to modify this such that each bit in the binary representation of nnpfc_complexity_idc corresponds to one group of syntax elements.

JVET-AB0152 AHG9: Regional on/off control and selection of NNPFs [J. Li, C. Lin, K. Zhang, L. Zhang, Y.-K Wang, Y. Li (Bytedance)]

Currently, both the neural-network post-filtering on/off control and the selection of a neural-network post-filter (NNPF) for use are both at the picture level. However, neural-network post-filtering may be effective only for a part of a picture. Furthermore, allowing the use of different NNPFs for different regions of a picture could further improve the performance. EE1 was conducted to evaluate the NN-based video coding technologies and the NNVC common SW base (NCS) has been established in JVET-AA meeting. In NCS-1.0, regional on/off control and regional selection of neural-network filters are supported.

This contribution proposes to support regional on/off control and/or regional selection of NNPFs for different regions within a picture. Two options are proposed:

- Option #1: Activate one NNPF in an NNPFA SEI message and the NNPF can be used for the entire current picture, some slices of the current picture, or some CTUs of the current picture.

- Option #2: Activate one or more NNPFs in an NNPFA SEI message, and a selected NNPF can be used for the entire current picture, some slices of the current picture, or some CTUs of the current picture.

# List of design questions

In below, "BC#nnn" refers to ballot comment #nnn from MPEG input document m60678.

- On NNPF input and output format

  - Is the syntax for specification of input and output formatting really necessary? Doesn’t the NNR standard itself determine its input and output format behaviour? Can this information conflict or require a conversion process that may be unnecessary? Remove any unnecessary input/output formatting specification. (BC#035)

  - Replace nnpfc_inp_format_flag and nnpfc_out_format_flag with ue(v)-coded nnpfc_inp_format_idc and nnpfc_out_format_idc and reserve values greater than 1. (BC#009)

- (On the external-means mode and the URI mode of NNPF) Keep the external-means mode and the URI mode (the modes with nnpfc_mode_idc equal to 0 and 2)?

  - Yes (BC#011)

    - Move nnpfc_uri_tag[ i ] and nnprf_uri[ i ] to be outside the conditional if statement based on nnpfc_purpose_and_formatting_flag?

      - Yes, move the gated syntax that contains nnpfc_uri_tag and nnpfc_uri just after nnpfc_mode_idc. (BC#010 item 1, JVET-AB0047 item 1)

      - Yes, keep the syntax elements just after the nnpfc_complexity_element( ) syntax structure. (JVET-AB0059 item 1)

    - Add a requirement to have nnpfc_purpose_and_formatting_flag equal to 1 when nnpfc_mode_idc is equal to 0 or 2. (BC#010 item 2, JVET-AB0047 item 2)

    - Consider allowing the use of Rec. ITU-T T.35 data as an additional alternative way of identifying processing to be applied for this SEI message, since T.35 data is already supported in the standard and can fulfil a similar purpose. (BC#020)

    - Specify a tag URI value (e.g., "tag:iso.org,2023:15938-17") indicating that the neural network identified by nnpfc_uri conforms to ISO/IEC 15938-17. (BC#024, JVET-AB0047 item 3)

    - Add and modify constraints for the case when nnpfc_mode_idc is equal to 2 to allow an update of a previously signalled NNPF. (JVET-AB0059 item 2)

  - No, remove the external-means mode and the URI mode (the modes with nnpfc_mode_idc equal to 0 and 2). (BC#019)

- On NNPF complexity

  - Change the syntax of nnpfc_complexity_element( ) to get rid of the part that does not make sense. It was commented that the syntax specification for nnpfc_complexity_element( ) does not make much sense – e.g. the syntax structure is sent whenever nnpfc_complexity_idc is greater than 0, but it doesn’t have anything in it unless nnpfc_complexity_idc is equal to 1. (BC#017)

  - It is asserted that the current extension mechanism in the syntax structure for the complexity elements does not allow for signalling additional complexity syntax elements while also signaling the first set of complexity syntax elements. Modify the nnpfc_complexity_element( ) syntax structure such that each bit in the binary representation of nnpfc_complexity_idc corresponds to one group of syntax elements. (JVET-AB0135 item 1)

  - Add two additional syntax elements (nnpfc_total_kilobyte_size and nnpfc_num_layers) descriptive of the complexity of the neural network signalled by the NNPFC SEI message. (JVET-AB0135 item 1)

- On NNPF purposes

  - Define and signal different types of visual quality improvements (note that visual quality improvement is one of the specified NNPF purposes), including the following (BC#021, JVET-AB0049 item 2):

    - General visual quality improvement, targeting at increasing either the fidelity or the subjective visual quality of the reconstructed picture after applying the neural-network post-processing filter. The improvement may be measured by any objective or subjective metric.

    - Objective-oriented/fidelity-oriented visual quality improvement, targeting at increasing the fidelity of the reconstructed picture after applying the neural-network post-processing filter. The fidelity may be measured by PSNR, MS-SSIM etc.

    - Subjective-oriented visual quality improvement, targeting at increasing the subjective visual quality of the reconstructed picture after applying the neural-network post-processing filter. The subjective visual quality may be measured by LPIPS or MOS.

    - Film grain-oriented visual quality improvement, with synthesizing filter grain on the reconstructed picture after applying the neural-network post-processing filter.

  - Specify the value 0 for nnpfc_purpose to be a specific purpose instead of "unknown or unspecified". (BC#034, JVET-AB0049 item 3)

  - Specify the value range of nnpfc_purpose to be 0 to 255, inclusive (instead of 0 to 232 − 2, inclusive). (JVET-AB0049 item 4)

  - Add frame rate upsampling as an additional purpose. (JVET-AB0058)

    - Conditionally signal two syntax elements related to frame rate upsampling information.

    - Add processes for deriving input tensors and storing output tensors.

- (On NNPF output picture width and height signalling) Make the following changes (BC#022, JVET-AB0049 item 6):

  - Instead of directly signal the filter output picture width and height values, the delta values of them compared to the filter input picture width and height values, respectively, are signalled using ue(v)-coded syntax elements, e.g., named nnpfc_delta_pic_width and nnpfc_delta_pic_height, respectively.

  - The value ranges of nnpfc_delta_pic_width and nnpfc_delta_pic_height are specified, e.g., as follows: The value of nnpfc_delta_pic_width shall be in the range of 0 to CroppedWidth * 15 − 1, inclusive. The value of nnpfc_delta_pic_height shall be in the range of 0 to CroppedHeight * 15 − 1, inclusive.

- On NNPF auxilary inputs

  - Move the normalization of the quantization strength input variable StrengthControlVal from the VSEI text to the use of NNPFC SEI message in the VVC text. (JVET-AB0046 item 1)

  - Change the normalization of StrengthControlVal from 2(StrengthControlVal − 42) / 6 to a division (÷) by 63. With this change, it is asserted that StrengthControlVal becomes a variable that has been normalized to the range 0..1. In Table 23, insert the given StrengthControlVal directly into the input tensor: inputTensor[ ][ ][ ][ ] = StrengthControlVal. (JVET-AB0046 item 2, BC#027)

  - Define and specify the use of more auxiliary inputs to the neural network post-processing filter, e.g., prediction samples. (BC#040, JVET-AB0074 item 1)

  - Make the following change (BC#045, JVET-AB0074 item 2):

When nnpfc_inp_order_idc is equal to 3, nnpfc_component_last_flag is equal to 0, and nnpfc_auxiliary_inp_idc is equal to 0, no auxiliary input matrix is used. In other words, remove the assignment operation "inputTensor[ 0 ][ 6 ][ yP + overlapSize ][ xP + overlapSize ] = 2(StrengthControlVal – 42)/6'' for this case from Table 23.

- (On syntax elements specified as UTF-8 character strings UTF-8 ) There are two syntax elements that are specified as UTF-8 character strings. One of them says it shall be as specified in ISO/IEC 10646 and the other does not. Shouldn’t they both be as specified in ISO/IEC 10646? Clarify. (BC#028)

  - Clarified by removing explicit reference to ISO/IEC 10646 from the semantics of nnpfc_uri. Both of the syntax elements are of type st(v), which is specified in VSEI to be null-terminated string encoded as universal coded character set (UCS) transmission format-8 (UTF-8) characters as specified in ISO/IEC 10646. (JVET-AB0047, one of the editorial changes)

- On value ranges for nnpfc_id and nnpfa_id and decoder hanlding of their reserved values

  - When encountering an NNPFC SEI messages with a reserved value of nnpfc_id, the decoder shall ignore the NNPFC SEI message instead of ignoring nnpfc_id value. (BC#030, JVET-AB0049 item 1)

  - Specify the value range and decoder handling of reserved values for nnpfa_id similarly as for nnpfc_id. (BC#055, JVET-AB0049 item 10)

- (On array convention) The standard generally uses an array convention in which the horizontal index is first and the vertical index is second. In several of these expressions, the indexing is swapped relative to that usual convention, e.g. for InpSampleVal( ) indexing. Is the order swapping necessary? Use the same convention consistently. If using an inconsistent convention is necessary (e.g. for compatibility with the NNR standard), highlight the difference in a NOTE to minimize reader confusion. (BC#036)

- On the value of nnpfc_out_sub_c_flag (BC#037, JVET-AB0049 item 5)

Replace the following sentence:

If SubWidthC is equal to 2 and SubHeightC is equal to 1, nnpfc_out_sub_c_flag shall not be equal to 0.

with the following:

When ChromaFormatIdc is equal to 2 and nnpfc_out_sub_c_flag is present, the value of nnpfc_out_sub_c_flag shall be equal to 1.

- (On coupled or separate filtering of chroma components) Enable separate processing of chroma components, including processing only one of the chroma components by one NNPFC SEI message. (BC#044, JVET-AB0075)

- (On NNPF patch size) Specify the following: When nnpfc_constant_patch_size_flag is equal to 0, the patch size width, denoted by inpPatchWidth, and the patch size height, denoted by inpPatchHeight, are provided by external means. An example of such external means is an API that passes the values of inpPatchWidth and inpPatchHeight to the decoder and render entity in a video application system, and the values may be configured by a user through a user interface of the application. (BC#046, JVET-AB0049 item 7)

- On NNPF activation, on/off control, and filter selection

  - Enable activating an NNPF for a set of consecutive pictures in output order in a CLVS by using one NNPFA SEI message. (BC#049, JVET-AB0050, JVET-AB0060)

    - Yes, enable it by one of the following (JVET-AB0050):

      - Add a flag: nnpfa_on_flag.

      - Add a new SEI message for deactivating a filter.

      - Add two flags: nnpfa_cancel_flag and nnpfa_persistence_flag.

    - Yes, enable it by not changing the NNPFA SEI message but adding an activation type to the NNPFC SEI message, with the following specified activation types: 0 - the NNPFA SEI message is applied; 1 – all pictures in the current CVS use this NNPF; 2 – all pictures in the current CVS with the same nuh_layer_id and TempralId, respectively, as the NAL unit containing the NNPFC SEI message use this NNPF. (JVET-AB0060)

  - Replace nnpfa_id with nnpfa_activation_id_plus1 and specify that nnpfa_activation_id_plus1 equal to 0 in the NNPFA SEI message indicates that no NNPF is activated for the current picture, to be able to explicitly indicate that no NNPF is activated for the current picture. (JVET-AB0134 item 1)

  - Enable activating different NNPFs for different regions (e.g., subpictures, slices, tiles, CTUs) of a picture? (BC#050, JVET-AB0152 option 2)

    - Yes, activate one or more NNPFs in an NNPFA SEI message, and a selected NNPF can be used for the entire current picture, some slices of the current picture, or some CTUs of the current picture. (JVET-AB0152 option 2)

  - Enable regional on/off control of NNPF? (JVET-AB0134 item 2, JVET-AB0152)

    - Yes, activate one NNPF in an NNPFA SEI message and the NNPF can be used for the entire current picture or a list of explicitly signalled regions (with region width, height, top and left positions signalled) of the current picture. (JVET-AB0134 item 2a)

      - Alternatively, same functionality, but add four more syntax elements targeting at reducing the signalling overhead. (JVET-AB0134 item 2b)

    - Yes, activate one NNPF in an NNPFA SEI message and the NNPF can be used for the entire current picture, some slices of the current picture, or some CTUs of the current picture. (JVET-AB0152 option 1)

    - Yes, also enable activating different NNPFs for different slices or CTUs of a picture. (JVET-AB0152 option 2)

- On presence and decoding order of NNPFC and NNPFA SEI messages

  - Specify the following constraint ("no NNPFC, no NNPFA"): An NNPFA SEI message with a particular value of nnpfa_id shall not be present in a current PU unless within the current CLVS there is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_id present in a PU preceding the current PU in decoding order or in the current PU. (BC#053, JVET-AB0049 item 8)

  - Specify the following constraint ("NNPFC precedes NNPFA"): When a PU contains both an NNPFC SEI message with a particular value of nnpfc_id and an NNPFA SEI message with nnpfa_id equal to the particular value of nnpfc_id, the NNPFC SEI message shall precede the NNPFA SEI message in decoding order. (BC#054, JVET-AB0049 item 9)

  - Is it allowed for more than one NNPFC SEI message to be present for the same picture? If so, what does this mean? Is there some limit on the number of them? If more than one is present, what happens if the nnpfc_purpose values are the same or if they are different? What happens if the nnpfc_mode_idc values are the same or if they are different? Add appropriate clarification, explanation and/or constraints. (BC#056)

- (On NNR base and update) It is our understanding that a neural network representation (NNR) can either be “intra” (coded without reference to some other NNR) or predictively/incrementally coded. Is there a way to identify which is which? In the case of incremental NNR, what is the reference? For example, is the reference the “base” representation, or could it be the accumulated result of several updates relative to a base representation? In what order are the updates applied? Clarify. (BC#058)

- Editorial changes not covered above:

  - BC#008, 012~016, 018, 023, 025, 026, 029, 031~033, 038, 039, 041~043, 047, 048, 051, 052, 057, 059~065

  - JVET-AB0047, excluding the technical changes in items 1 to 3