| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0242-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AB0102 (AHG11/EE1-related: Updates on RPR encoder and filters) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | K. Andersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson A |  |  |

_____________________________

# Abstract

The software has been inspected and Test 1 of the software has been tested. Compared to previous versions of the software 1.25x scaling has been added and kicks in when similarity after rescaling for a range of PSNR just below the range of PSNR where 1.5x and 2x scaling were used. It is noted that JVET-AB0080 also is very similar to Test 1 in the aspect of using 1.25x, 1.5x and 2x scaling’s.

The numbers in JVET-AB0102 were successfully verified mostly all 4 decimals match occasionally difference in 4:th decimal.  Except for CatRobot where the cross-checker used the longer version of CatRobot but encoded at 300 frames and the proponent used a 300 frames version which explains the minor differences for that sequence. It can be noted that there are some differences in the reconstruction on the encoder side than the decoder side that the proponent is looking into.

# Introduction

In comparison to previous software used in EE1 tests this software include 1.25x scale factor as one candidate resolution in addition to 1.5x and 2x. This makes the fills the gap between encoding in full resolution and encoding in 2/3 resolution which make sense.

# Comparison between JVET-AB0102 and JVET-AB0080

It is noted that JVET-AB0080 [2] is very similar to Test 1 in JVET-AB0102 [1]. The main difference is that the PSNR thresholds in JVET-AB0080 is a bit more conservative than the thresholds in JVET-AB0102. Furthermore, the thresholds in JVET-AB0102 are higher for AI than for RA. In JVET-AB0080 the thresholds are same for all conditions. In JVET-AB0102 the rescaling is tested using the downscaling filter also for upscaling. In JVET-AB0080 the rescaling test uses the same upscaling filter that the reference software uses, e.g. the 8-tap MC filter. Another difference is that the QP change when selecting to encode in a lower resolution is made before the derivation of the final QP in JVET-AB0102 and in JVET-AB0080 the QP change is performed after the final QP is derived.

# Results

Results for Test1 is compared to NNVC-2.0 CTC [3,4] for RA and AI. The encoding and decoding numbers are not accurate. It can be noted significant gains for PSNR as well as MSSSIM for luma but also some losses for chroma.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-2.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -4,05% | 3,11% | 3,09% | -7,72% | 1,06% | -0,03% | 72% | 81% |
| Class A2 | -1,58% | 11,77% | 10,57% | -5,37% | 9,26% | 7,80% | 77% | 88% |
| Class B | 0,18% | 2,36% | 1,49% | -1,23% | 2,02% | 1,28% | 90% | 89% |
| Class C | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 88% | 87% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1,07% | 3,76% | 3,23% | -3,03% | 2,74% | 1,98% | 83% | 87% |
| Class D | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 87% | 88% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-2.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -1,76% | 4,05% | 2,01% | -4,93% | 4,71% | 2,70% | 87% | 79% |
| Class A2 | -3,58% | 21,21% | 15,93% | -7,25% | 11,12% | 9,13% | 82% | 81% |
| Class B | 0,58% | 2,36% | 2,46% | -1,45% | 3,34% | 4,15% | 88% | 86% |
| Class C | 0,02% | 0,07% | 0,08% | -0,02% | 0,10% | 0,14% | 99% | 97% |
| Class E | 1,04% | 0,84% | 0,85% | -1,41% | 1,73% | 1,86% | 93% | 90% |
| Overall | -0,55% | 5,02% | 3,83% | -2,67% | 3,88% | 3,47% | 90% | 87% |
| Class D | 0,04% | 0,11% | 0,08% | -0,03% | 0,16% | 0,16% | 95% | 94% |

# Reference

[1] J. Nam, S. Yoo, J. Lim, “AHG11/EE1-related: Updates on RPR encoder and filters”, JVET-AB0102, Mainz, Germany, October 2022.

[2] K. Andersson, R. Yu, “AHG10: GOP-based RPR encoder control”, JVET-AB0080, Mainz, Germany, October 2022.

[3] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tags/VTM-11.0_nnvc-2.0

[4] E. Alshina, R-L. Liao, S. Liu, A. Segall,	“JVET common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AA2016, Meeting by Teleconference, July 2022.

# Patent rights declaration(s)

Ericsson AB may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).