| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0245-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0182 (Non-EE2: Bi-predictive local illumination compensation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Guichun Li 2747 Park Boulevard Palo Alto, CA 94306, U.S.A. | Email: | guichunli@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results of Bi-predictive local illumination compensation proposed in JVET-AB0182. The test results match with the results provided by the proponent.

# Introduction

Bi-predictive local illumination compensation was proposed in JVET-AB0182. The implementation is made on top of ECM-6.0 software tool-on configuration, where LIC tool is enabled. The test was also conducted to compare with ECM-6.0 with LIC tool-on configuration.

In current ECM, LIC is only applied on inter uni-prediction. The proposed method extended LIC design to also apply on inter bi-prediction. Two different linear models are derived to compensate the illumination changes that exist between the current block and its two prediction blocks. The final bi-prediction of the current block is calculated as the combination of two uni-prediction blocks after the LIC adjustment, i.e.,

P'x,y=1-ω∙p0'x,y+ω∙p1'x,y

and

p0'x,y=α0∙P0x,y+β0

p1'x,y=α1∙P1x,y+β1

where α0 and β0, and α1 and β1 indicate the scales and the offsets in L0 and L1, respectively; ω indicates the weight (as indicated by the CU-level BCW index) that is applied when combining the two uni-prediction blocks. Same to the current LIC design, one control flag is signaled for AMVP bi-predicted CUs to indicate the enabling/disabling of the LIC while the flag is inherited from one neighboring block for merge inter CUs. Additionally, the LIC is disabled when any of combined inter and intra (CIIP), decoder-side motion vector refinement and bi-directional optical flow (BDOF) is applied.

# Crosscheck results

Simulation was conducted according to the proposed tool-on configuration as in JVET-AB0182.

It was observed that the BD-rate match with the results provided by the proponent. Encoding time and decoding time are similar to the results from the proponent.

Table 1-Simulation Results over ECM-6.0 LIC Tool-on

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-6.0 LIC Tool On |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.39% | -0.43% | 118% | 99% |
| Class A2 | -0.24% | -0.26% | -0.21% | 120% | 99% |
| Class B | -0.35% | -0.24% | -0.15% | 117% | 100% |
| Class C | -0.26% | -0.10% | -0.20% | 117% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.24% | -0.23% | 118% | 100% |
| Class D | -0.19% | -0.07% | 0.09% | 115% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  |  | LDB |  |  |
|  |  |  | Over ECM-6.0 LIC Tool On |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.33% | -0.63% | -0.49% | 116% | 104% |
| Class E | -0.22% | -0.06% | 0.48% | 117% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.40% | -0.10% | 116% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |