| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB00271 |
| --- | --- |

| Title: | AHG3: report of latest HM performance on HDR content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | E. François (InterDigital) T. Lu (Dolby) S. Iwamura (NHK) A. Segall (Amazon) | Tel: Email: | edouard.francois@interdigital.com tlu@dolby.com iwamura.s-gc@nhk.or.jp asegall@amazon.com |
| Source: | InterDigital, Dolby, NHK, Amazon |  |  |

_____________________________

# Abstract

This contribution reports performance results of HM16.26 for HDR CTCs content.

# Introduction

Latest report on HM coding performance for HDR content (JVET-U0007) was based on HM16.18. Since then, changes in HM software, HM RA configuration and class H2 content, have been made. This document reports HDR results for latest HM version, HM16.26, using latest test conditions and most recent HDR CTCs content.

# Overview of changes impacting HM performance

The following changes related to HM or HDR content have been made since JVET-U meeting.

- Addition of MCTF feature in HM, and activation in HM CTCs.

- Changes of the GOP structure for Random Access, to conform the maximum DPB size specified for HEVC.

- New version of the class H2 (HDR-HLG) clips, to conform type 2 chroma alignment; the old version has type 0 chroma alignment.

JVET-U0007 reported results from HM without these three modifications. In addition, in JVET-U0007, HM16.18 with high-bit depth precision (HM16.18HBD, macro “RExt__HIGH_BIT_DEPTH_SUPPORT” set to 1) was used for class H1 (HDR-PQ), while HM16.18 without high-bit depth precision was used for class H2.

A first task was to reproduce the JVET-U0007 results. Regarding class H1, Dolby and Interdigital were able to get mutual matching results, using HM16.18HBD, but those results slightly differ from the ones reported in JVET-U0007, as depicted in Table 1. Regarding class H2 (old version), NHK and Interdigital were able to get mutual matching results, using HM16.18, but those results also slightly differ from the ones reported in JVET-U0007, as depicted in Table 1.

Table 1. HM16.18 Oct. 2022 over JVET-U0007.

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.18 JVET-U |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V |
| Class H1 | 0.01% | 0.02% | 0.00% | 0.01% | -0.13% | 0.00% | -0.02% | -0.12% |
| Class H2 |  |  |  |  |  | 0.00% | 0.01% | -0.05% |
| Overall | 0.01% | 0.02% | 0.00% | 0.01% | -0.13% | 0.00% | -0.01% | -0.10% |
|  | All Intra |  |  |  |  |  |  |  |
|  | Over HM16.18 JVET-U |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V |
| Class H1 | -0.02% | -0.02% | -0.01% | -0.02% | -0.01% | -0.01% | -0.01% | 0.00% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% |
| Overall | -0.02% | -0.02% | -0.01% | -0.02% | -0.01% | -0.01% | -0.01% | 0.00% |

# HM16.26 performance

HM16.26 has been evaluated for class H1 and class H2 (new version), using both non-HBD and HBD versions. Results are reported in tables below.

In Table 2, test is HM16.26HBD (using configuration files attached to HM16.26), anchor is HM16.18HBD (using configuration files attached to HM16.18). Gains are observed in RA for all metrics, mostly due to the activation of MCTF.

In Table 3, test is HM16.26HBD, anchor is HM16.26. Minor differences are observed.

Corresponding xls files are attached to the contribution.

Table 2. HM16.26HBD over HM16.18HBD.

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.18 HBD |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V |
| Class H1 | -3.65% | -3.41% | -3.15% | -1.64% | 1.37% | -3.20% | -1.78% | 0.83% |
| Class H2 new |  |  |  |  |  | -4.42% | -3.72% | -3.30% |
| Overall | -3.65% | -3.41% | -3.15% | -1.64% | 1.37% | -3.65% | -2.49% | -0.67% |
|  | All Intra |  |  |  |  |  |  |  |
|  | Over HM16.18 HBD |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V |
| Class H1 | 0.00% | 0.00% | 0.00% | -0.07% | -0.04% | 0.00% | -0.06% | -0.07% |
| Class H2 new |  |  |  |  |  | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | -0.07% | -0.04% | 0.00% | -0.04% | -0.04% |

Table 3. HM16.26HBD over HM16.26.

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.26 |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V |
| Class H1 | 0.05% | 0.01% | 0.03% | 0.14% | -0.11% | 0.03% | 0.17% | -0.15% |
| Class H2 new |  |  |  |  |  | 0.01% | -0.10% | -0.02% |
| Overall | 0.05% | 0.01% | 0.03% | 0.14% | -0.11% | 0.02% | 0.07% | -0.10% |
|  | All Intra |  |  |  |  |  |  |  |
|  | Over HM16.26 |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V |
| Class H1 | -0.10% | 0.04% | 0.02% | -0.31% | -0.22% | 0.02% | -0.35% | -0.20% |
| Class H2 new |  |  |  |  |  | 0.00% | -0.04% | -0.09% |
| Overall | -0.10% | 0.04% | 0.02% | -0.31% | -0.22% | 0.01% | -0.24% | -0.16% |

# Remaining issue

A mismatch between parallel and sequential coding is observed with the HM16.18HBD and HM16.26HBD versions.

# Recommendations

It is suggested:

- to add to the HM package a folder comprising the HDR per-sequence configuration files (see attached file “per-sequence-HDR.zip”).

- to use HM HBD version for both H1 and H2 classes.

# Annex – additional results

Table 4. HM16.18HBD over HM16.18.

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.18 IDC |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V |
| Class H1 | -6.93% | -0.78% | -0.07% | -3.55% | -6.41% | -0.05% | -2.48% | -4.73% |
| Class H2 old |  |  |  |  |  | -0.01% | -0.10% | -0.30% |
| Class H2 new |  |  |  |  |  | 0.01% | -0.16% | -0.42% |
|  | All Intra |  |  |  |  |  |  |  |
|  | Over HM16.18 IDC |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V |
| Class H1 | -5.81% | -0.43% | -0.01% | -2.32% | -4.49% | 0.00% | -1.63% | -3.50% |
| Class H2 old |  |  |  |  |  | 0.00% | -0.12% | -0.33% |
| Class H2 new |  |  |  |  |  | 0.00% | -0.09% | -0.24% |

Table 5. HM16.26 over HM16.18.

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.18 IDC |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V |
| Class H1 | -10.24% | -4.15% | -3.24% | -5.25% | -4.98% | -3.28% | -4.37% | -3.74% |
| Class H2 new |  |  |  |  |  | -4.42% | -3.76% | -3.67% |
| Overall | -10.24% | -4.15% | -3.24% | -5.25% | -4.98% | -3.70% | -4.15% | -3.72% |
|  | All Intra |  |  |  |  |  |  |  |
|  | Over HM16.18 IDC |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V |
| Class H1 | -5.75% | -0.47% | -0.03% | -2.10% | -4.37% | -0.02% | -1.35% | -3.43% |
| Class H2 new |  |  |  |  |  | -0.01% | -0.05% | -0.15% |
| Overall | -5.75% | -0.47% | -0.03% | -2.10% | -4.37% | -0.01% | -0.88% | -2.23% |

Table 6. HM16.26HBD over HM16.18HBD.

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.18 HBD IDC |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V |
| Class H1 | -3.65% | -3.41% | -3.15% | -1.64% | 1.37% | -3.20% | -1.78% | 0.83% |
| Class H2 new |  |  |  |  |  | -4.42% | -3.72% | -3.30% |
| Overall | -3.65% | -3.41% | -3.15% | -1.64% | 1.37% | -3.65% | -2.49% | -0.67% |
|  | All Intra |  |  |  |  |  |  |  |
|  | Over HM16.18 HBD IDC |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V |
| Class H1 | 0.00% | 0.00% | 0.00% | -0.07% | -0.04% | 0.00% | -0.06% | -0.07% |
| Class H2 new |  |  |  |  |  | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | -0.07% | -0.04% | 0.00% | -0.04% | -0.04% |