| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC_Notes_d0 |
| --- | --- |

| Title: | Meeting Report of the 29th Meeting of the Joint Video Experts Team (JVET), by teleconference, 11–20 January 2023 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chair of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: | +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chair of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29 held its twenty-ninth meeting during 11–20 January 2023 as an online-only meeting. For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the tenth meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.14 of this document. It is further noted that work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued in JVET as a single joint team, and explorations towards possible future need of standardization in the area of video coding are also conducted by JVET, as negotiated by the parent bodies.

The JVET meeting began at approximately 1300 hours UTC on Wednesday 11 January 2023. Meeting sessions were held on all days except the weekend days of Saturday and Sunday 14 and 15 January 2023, until the meeting was closed at approximately 00XX hours UTC on Saturday 21 January 2023.  Approximately XXX people attended the JVET meeting, and approximately XXX input documents (not counting crosschecks), 15 AHG reports, 2 EE summary reports, X BoG reports, and X incoming liaison documents were discussed. The meeting took place in coordination with a meeting of various SC29 Working Groups and Advisory Groups – where WG 5 is representing the Joint Video Coding Team(s) and their activities from the perspective of the SC 29 parent body, under whose auspices this JVET meeting was held. The subject matter of the JVET meeting activities consisted of work on further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, reference software and conformance testing packages. Further important goals were reviewing the results of the Exploration Experiment (EE) on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and to plan next steps for investigation of candidate technology towards further standard development.

As a primary goal, the JVET meeting reviewed the work that had been performed in the interim period since the twenty-eighth JVET meeting in producing the following documents:

- JVET-AB1004 Errata report items for VVC, VSEI, HEVC, AVC, Video CICP, and CP usage TR

- JVET-AB1008 Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 2), also issued as WG 5 preliminary WD

- JVET-AB1012 Overview of IT systems used in JVET

- JVET-AB2002 Algorithm description for Versatile Video Coding and Test Model 18 (VTM 18)

- JVET-AB2006 Additional SEI messages for VSEI (Draft 3), also issued as WG 5 DAM

- JVET-AB2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video

- JVET-AB2016 Common Test Conditions and evaluation procedures for neural network-based video coding technology

- JVET-AB2019 Description of algorithms and software in neural network-based video coding (NNVC)

- JVET-AB2020 Film grain synthesis technology for video applications (Draft 3), also issued as WG 5 WD

- JVET-AB2021 Draft verification test plan for VVC multilayer coding

- JVET-AB2022 Draft plan for subjective quality testing of FGC SEI message

- JVET-AB2023 Exploration Experiment on neural network-based video coding (EE1)

- JVET-AB2024 Exploration Experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AB2025 Algorithm description of Enhanced Compression Model 7 (ECM 7)

- JVET-AB2027 SEI processing order SEI message in VVC (draft 2), also issued as WG 5 preliminary WD

- JVET-AB2028 Additional conformance bitstreams for VVC multilayer configurations

- JVET-AB2029 Visual quality comparison of ECM/VTM encoding

As main results, the JVET produced XX output documents from the current meeting (to be updated):

- JVET-AB1004 Errata report items for VVC, VSEI, HEVC, AVC, Video CICP, and CP usage TR

- JVET-AB1008 Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 2), also issued as WG 5 preliminary WD

- JVET-AB1012 Overview of IT systems used in JVET

- JVET-AB2002 Algorithm description for Versatile Video Coding and Test Model 18 (VTM 18)

- JVET-AB2006 Additional SEI messages for VSEI (Draft 3), also issued as WG 5 DAM

- JVET-AB2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video

- JVET-AB2016 Common Test Conditions and evaluation procedures for neural network-based video coding technology

- JVET-AB2019 Description of algorithms and software in neural network-based video coding (NNVC)

- JVET-AB2020 Film grain synthesis technology for video applications (Draft 3), also issued as WG 5 WD

- JVET-AB2021 Draft verification test plan for VVC multilayer coding

- JVET-AB2022 Draft plan for subjective quality testing of FGC SEI message

- JVET-AB2023 Exploration Experiment on neural network-based video coding (EE1)

- JVET-AB2024 Exploration Experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AB2025 Algorithm description of Enhanced Compression Model 7 (ECM 7)

- JVET-AB2027 SEI processing order SEI message in VVC (draft 2), also issued as WG 5 preliminary WD

- JVET-AB2028 Additional conformance bitstreams for VVC multilayer configurations

- JVET-AB2029 Visual quality comparison of ECM/VTM encoding

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, 2 Exploration Experiments (EE) were defined. The next eight JVET meetings were planned for 21 – 28 April 2023 under ISO/IEC JTC 1/‌SC 29 auspices, in Antalya, TR; during 11 – 19 July 2023 under ITU-T SG16 auspices in Geneva, CH; during October 2023 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; during January 2024 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; during April 2024 under ITU-T SG16 auspices, date and location t.b.d.; during 12 – 19 July 2024 under ISO/IEC JTC 1/‌SC 29 auspices in Sapporo, JP; during October 2024 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; and during January 2025 under ITU-T SG16 auspices, date and location t.b.d.

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. It was noted that the previous sites http://phenix.int-evry.fr/jvet/, http://phenix.int-evry.fr/jct/, and http://phenix.int-evry.fr/jct3v/ are still accessible, but were converted to read-only.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and ISO/IEC JTC 1/‌SC 29/‌WG 5. The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29.

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29 held its twenty-ninth meeting during 11–20 January 2023 as an online-only meeting. For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the tenth meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of the Versatile Video Coding (VVC) and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI) standards, as well as associated conformance test sets, reference software, verification testing, and non-normative guidance information. Furthermore, starting from the twentieth meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on:

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

Furthermore, explorations towards possible future need of standardization in the area of video coding are also conducted by JVET. Currently, the following topics are under investigation:

- Exploration on Neural Network-based Video Coding

- Exploration on Enhanced Compression beyond VVC capability

This report contains three important annexes, as follows:

- Annex A contains a list of the documents of the JVET meeting

- Annex B contains a list of the meeting participants, as recorded by the teleconferencing tool used for the meeting

- Annex C contains the meeting recommendations of ISO/IEC JTC 1/‌SC 29/‌WG 5 for purposes of results reporting to ISO/IEC.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2023_01_AC_Virtual/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the twenty-eighth JVET meeting in producing the following documents:

- JVET-AB1004 Errata report items for VVC, VSEI, HEVC, AVC, Video CICP, and CP usage TR

- JVET-AB1008 Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 2), also issued as WG 5 preliminary WD

- JVET-AB1012 Overview of IT systems used in JVET

- JVET-AB2002 Algorithm description for Versatile Video Coding and Test Model 18 (VTM 18)

- JVET-AB2006 Additional SEI messages for VSEI (Draft 3), also issued as WG 5 DAM

- JVET-AB2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video

- JVET-AB2016 Common Test Conditions and evaluation procedures for neural network-based video coding technology

- JVET-AB2019 Description of algorithms and software in neural network-based video coding (NNVC)

- JVET-AB2020 Film grain synthesis technology for video applications (Draft 3), also issued as WG 5 WD

- JVET-AB2021 Draft verification test plan for VVC multilayer coding

- JVET-AB2022 Draft plan for subjective quality testing of FGC SEI message

- JVET-AB2023 Exploration Experiment on neural network-based video coding (EE1)

- JVET-AB2024 Exploration Experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AB2025 Algorithm description of Enhanced Compression Model 7 (ECM 7)

- JVET-AB2027 SEI processing order SEI message in VVC (draft 2), also issued as WG 5 preliminary WD

- JVET-AB2028 Additional conformance bitstreams for VVC multilayer configurations

- JVET-AB2029 Visual quality comparison of ECM/VTM encoding

Further important goals were reviewing the results of the EE on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and planning next steps for investigation of candidate technology towards further standard development.

## Documents and document handling considerations

### General

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. It was noted that the previous site http://phenix.int-evry.fr/jvet/ was still accessible, but had been converted to read-only.

Document registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting follow the CEST timezone (local time in Mainz), except as otherwise noted.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect one of the various software packages but have no normative effect on text are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in one of the test model descriptions such as VTM, HM, etc. (an error, oversight, or messiness) or in the associated software package are marked by the string “Decision (BF):”.

- Decisions that are merely editorial without effect on the technical content of a draft standard are marked by the string "Decision (Ed.):". Such editorial decisions are merely suggestions to the editor, who has the discretion to determine the final action taken if their judgment differs.

- Some decisions are recorded with the word “agreed” rather than “Decision:”, especially for non-normative, editorial and planning matters.

This meeting report is based primarily on notes taken by the JVET chair. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report. Expressions such as “X.XX%” indicate that the desired results were not available at the time the information was recorded.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Wednesday, 4 January 2023. Any documents uploaded after 1159 hours Paris/Geneva time on Thursday 5 January 2023 were considered “officially late”, with a grace period of 12 hours (to accommodate those living in different time zones of the world). The deadline does not apply to AHG reports and other such reports which can only be produced after the availability of other input documents.

All contribution documents with registration numbers higher than JVET-AC0200 were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “late” range might include break-out activity reports that were generated during the meetings, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions has been to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This has been agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:

- JVET-AC0XXX (a proposal on …), uploaded 01-XX,

- … .

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline.

The following other documents not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-AC0XXX (a document on …), uploaded 01-XX,

- … .

Most cross-verification reports at this meeting were registered late, and/or uploaded late (except for JVET-AC0117, JVET-AC0133, and JVET-AC0168). In the interest of brevity, the late ones are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following contribution registrations were noted that were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-AC0102, JVET-AC0157, … .

The following cross-verification reports were still missing three weeks after the end of the meeting: JVET-AC0XXX, … . They were thus marked by the JVET chair as withdrawn.

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then be counted as a late contribution if the update was after the document deadline). At the current meeting, this situation did apply with documents JVET-AC0XXX and …, which were both categorized as late in the list above, based on the time of the first reasonable document upload.

Contributions that had significant problems with uploaded versions were not observed.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE and HLS topic summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there may have been some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision, although participants tend to seldom notice what is recorded there.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors may have also noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-AB1000, the Errata report items for VVC, VSEI, HEVC, AVC, Video CICP, and CP usage TR JVET-AB1004, the Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 2) JVET-AB1008, the Overview of IT systems used in JVET JVET-AB1012, the Algorithm description for Versatile Video Coding and Test Model 18 (VTM 18) JVET-AB2002, the Additional SEI messages for VSEI (Draft 3) JVET-AB2006, the VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video JVET-AB2010, the Common test conditions and evaluation procedures for neural network-based video coding technology JVET-AB2016, the Description of algorithms and software in neural network-based video coding (NNVC) JVET-AB2019, the Film grain synthesis technology for video applications (Draft 3) JVET-AB2020, the Draft verification test plan for VVC multilayer coding JVET-AB2021, the Draft plan for subjective quality testing of FGC SEI message JVET-AB2022, the Description of the EE on Neural Network-based Video Coding JVET-AB2023, the Description of the EE on Enhanced Compression beyond VVC capability JVET-AB2024, the Algorithm description of Enhanced Compression Model 7 (ECM 7) JVET-AB2025, the SEI processing order SEI message in VVC (Draft 1) JVET-AB2027, the Additional conformance bitstreams for VVC multilayer configurations JVET-AB2028, and the Visual quality comparison of ECM/VTM encoding JVET-AB2029, had been completed and were approved. The software implementations of HM (version 17.0), VTM (version 19.0), ECM (version 7.0), and NNVC (version 3.0) were also approved.

Only minor editorial issues were found in the meeting report JVET-AB1000; no need to produce an update was identified (see section 2.12 for details).

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 5 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

It was further announced that it is necessary to register for the meeting through the ISO Meetings website for ISO/IEC experts or through the Q6/16 rapporteur for ITU-T experts. The password for meeting access had been sent to registered participants via these channels. Links to the Zoom sessions (without the necessary password) were available in the posted meeting logistics information and the calendar of meeting sessions in the JVET web site.

The following rules were established for those participating remotely via Zoom teleconference meeting:

- Use the “hand-raising” function to enter yourself in the queue to speak (unless otherwise instructed by the session chair). If you are dialed in by phone, request your queue position verbally. The online queue will be interleaved with the room queue, though it may not always be guaranteed that the sequence perfectly follows the sequence by which hand raising occurred.

- Stay muted unless you have something to say. People are muted by default when they join and need to unmute themselves to speak. The chair may mute anyone who is disrupting the proceedings (e.g. by forgetting they have a live microphone while chatting with their family or by causing bad noise or echo).

- Identify who you are and your affiliation when you begin speaking. The same applies for speakers in the room to let online participants know who is speaking.

- Use your full name and company/organization and country affiliation in your joining information, since the participation list of Zoom would also be used to compile the online part of attendance records.

- Turn on the chat window and watch for chair communication and side commentary there as well as by audio.

- Generally do not use video for the teleconferencing calls in order to avoid overloading people’s internet connections; enable only voice and screen sharing.

- Extensive use of screen sharing is encouraged, to enable participants to view the presented material and the meeting notes. At times, multiple sources of screen sharing may be enabled, so it may be necessary for participants to understand that this is happening and to understand how to select which one they want to watch.

## Agenda

The agenda for the meeting, for the further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, software and conformance packages, was as follows:

Opening remarks and review of meeting logistics and communication practices

Roll call of participants

Adoption of the agenda

Code of conduct policy reminder

IPR policy reminder and declarations

Contribution document allocation

Review of results of the previous meeting

Review of target dates

Reports of ad hoc group (AHG) activities

Report of exploration experiments on neural-network-based video coding

Report of exploration experiments on enhanced compression beyond VVC capability

Consideration of contributions on high-level syntax

Consideration of contributions and communications on project guidance

Consideration of video coding technology contributions

Consideration of contributions on conformance and reference software development

Consideration of contributions on coding-independent code points for video signal type identification

Consideration of contributions on film grain synthesis technology

Consideration of contributions on optimization of encoders and receiving systems for machine analysis of coded video content

Consideration of contributions on errata relating to standards in the domain of JVET

Consideration of contributions on technical reports relating to standards and exploration study activities in the domain of JVET

Consideration of contributions providing non-normative guidance relating to standards and exploration study activities in the domain of JVET

Consideration of information contributions

Consideration of future work items

Coordination of visual quality testing

Liaisons, coordination activities with other organizations

Review of project editor and liaison assignments

Approval of output documents and associated editing periods

Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, future meeting planning, other planning issues

Other business as appropriate for consideration

Closing

- The agenda was approved as suggested.

- The plans for the times of meeting sessions were established as follows, in UTC (which for this meeting was 1 hour behind the time in Geneva and Paris; 8 hours ahead of the time in Los Angeles, etc.). No session was scheduled to last longer than 2 hrs.

- 1300–1500 1st “afternoon” session [break after 2 hours]

- 1520–1720 2nd “afternoon” session

- [“evening” break – nearly 4 hours]

- 2100–2300 1st “night” session [break after 2 hours]

- 2320–0120+1 2nd “night” session

- It was also pointed out that the session times had been changed from meeting to meeting, such that different time zones of the world might be treated approximately equally fairly either in one meeting or another. For the current meeting, the same UTC session times were used as in the 25th JVET meeting (which had been the eighth meeting conducted as an online meeting, whereas the current one is the eleventh).

  - ISO and IEC Code of Conduct reminders

Participants were reminded of the ISO and IEC Codes of Conduct, found at

https://www.iso.org/publication/PUB100397.html.

https://www.iecapc.jp/F/IEC_Code_of_Conduct.pdf

These include points relating to:

- Respecting others

- Behaving ethically

- Escalating and resolving disputes

- Working for the net benefit of the international community

- Upholding consensus and governance

- Agreeing to a clear purpose and scope

- Participating actively and managing effective representation

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM and ECM software implementation packages use the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is also used in the process of designing the VTM and ECM software, and for evaluating proposals for technology to be potentially included in these designs. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well. The SADL and NNVC packages for neural network-based video coding use the same licensing terms.

Software packages that had been developed in prior work of the JVT, the JCT-VC and the JCT-3V have similar considerations and are maintained according to the past practice in that work.

## Communication practices

The documents for the meeting can be found at https://jvet-experts.org/. It was noted that the previous site http://phenix.int-evry.fr/jvet/ is still accessible, but was converted to read-only. It was reminded to send a notice to the chairs in cases of changes to document titles, authors, etc.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers on the JVET email list was 1184 (as of 10 January 2022). All discussions (including those on AVC, HEVC, VVC, CICP, etc.) shall be conducted on the JVET reflector rather than any of the old reflectors (including JVT, JCT-VC, and JCT-3V) which are retained for archiving purposes.

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

It is further emphasized that the document JVET-AB1012 (to be updated during this meeting) gives valuable hints about communication practices as well as other IT resources used in JVET, such as software, conformance, and test materials.

## Terminology

- ACT: Adaptive colour transform

- AFF: Adaptive frame-field

- AI: All-intra

- AIF: Adaptive interpolation filtering

- ALF: Adaptive loop filter

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component)

- AMVP: Adaptive motion vector prediction

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection

- AMVR: (Locally) adaptive motion vector resolution

- APS: Adaptation parameter set

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR)

- ARMC: Adaptive re-ordering of merge candidates

- ARSS: Adaptive reference sample smoothing

- ATM: AVC-based multiview and 3D test model

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction

- AU: Access unit

- AUD: Access unit delimiter

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10

- BA: Block adaptive

- BC: See CPR or IBC

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001)

- BDOF: Bi-directional optical flow (formerly known as BIO)

- BDPCM: Block-wise DPCM

- BL: Base layer

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardzation

- BoG: Break-out group

- BR: Bit rate

- BT: Binary tree

- BV: Block vector (used for intra BC prediction)

- CABAC: Context-adaptive binary arithmetic coding

- CBF: Coded block flag(s)

- CC: May refer to context-coded, common (test) conditions, or cross-component

- CCALF: Cross-component ALF

- CCLM: Cross-component linear model

- CCCM: Cross-component convolutional model

- CCP: Cross-component prediction

- CCSAO: Cross-component SAO

- CE: Core Experiment – a coordinated experiment conducted toward assessment of coding technology

- CG: Coefficient group

- CGS: Colour gamut scalability (historically, coarse-grained scalability)

- CIIP: Combined inter/intra prediction

- CIPF: CABAC initialization from the previous frame

- CL-RAS: Cross-layer random-access skip

- CPB: Coded picture buffer

- CPMV: Control-point motion vector

- CPMVP: Control-point motion vector prediction (used in affine motion model)

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- CST: Chroma separate tree

- CTC: Common test conditions

- CVS: Coded video sequence

- DCI: Decoder capability information

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics)

- DCTIF: DCT-derived interpolation filter

- DF: Deblocking filter

- DIMD: Decoder intra mode derivation

- DMVR: Decoder motion vector refinement

- DoCR: Disposition of comments report

- DPB: Decoded picture buffer

- DPCM: Differential pulse-code modulation

- DPS: Decoding parameter sets

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR)

- DT: Decoding time

- DQ: Dependent quantization

- ECS: Entropy coding synchronization (typically synonymous with WPP)

- EMT: Explicit multiple-core transform

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element)

- ECM: Enhanced compression model – a software codebase for future video coding exploration

- ECV: Extended Colour Volume (up to WCG)

- EL: Enhancement layer

- EOS: End of (coded video) sequence

- ET: Encoding time

- FRUC: Frame rate up conversion (pattern matched motion vector derivation)

- GCI: General constraints information

- GDR: Gradual decoding refresh

- GLM: Gradient linear model

- GOP: Group of pictures (somewhat ambiguous)

- GPM: Geometry partitioning mode

- GRA: Gradual random access

- HBD: High bit depth

- HDR: High dynamic range

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2

- HLS: High-level syntax

- HM: HEVC Test Model – a video coding design containing selected coding tools that conforms to the HEVC standard design (possibly with under-development extensions) – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM)

- HMVP: History based motion vector prediction

- HRD: Hypothetical reference decoder

- HTM: HEVC-based multiview and 3D test model (developed by JCT-3V)

- HyGT: Hyper-cube Givens transform (a type of NSST)

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample)

- IBF: Intra boundary filtering

- ILP: Inter-layer prediction (in scalable coding)

- ILRP: Inter-layer reference picture

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC)

- IRAP: Intra random access picture

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JCT-3V: Joint collaborative team on 3D video (for AVC and HEVC)

- JCT-VC: Joint collaborative team on video coding (for HEVC)

- JEM: Joint exploration model – a software codebase previously used for video coding exploration

- JM: Joint model – the primary software codebase that has been developed for the AVC standard

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions

- JVET: Joint video experts team (initially for VVC, later expanded)

- JVT: Joint video team (for AVC)

- KLT: Karhunen-Loève transform

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation

- LM: Linear model

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames

- LUT: Look-up table

- LTRP: Long-term reference picture

- MANE: Media-aware network element

- MC: Motion compensation

- MCP: Motion compensated prediction

- MCTF: Motion compensated temporal pre-filtering

- MDNSST: Mode dependent non-separable secondary transform

- MIP: Matrix-based intra prediction

- MMLM: Multi-model (cross component) linear mode

- MMVD: Merge with MVD

- MPEG: Moving picture experts group (an alliance of working groups and advisory groups in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET)

- MPM: Most probable mode (in intra prediction)

- MRL: Multiple reference line intra prediction

- MV: Motion vector

- MVD: Motion vector difference

- NAL: Network abstraction layer

- NNVC: Neural network-based video coding (experimental software package)

- NSQT: Non-square quadtree

- NSST: Non-separable secondary transform

- NUH: NAL unit header

- NUT: NAL unit type (as in AVC and HEVC)

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F)

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value

- OLS: Output layer set.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- operation point: A temporal subset of an OLS.

- PDPC: Position-dependent (intra) prediction combination.

- PERP: Padded equirectangular projection (a 360° projection format).

- PH: Picture header.

- PHEC: Padded hybrid equiangular cubemap (a 360° projection format).

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set (as in AVC and HEVC).

- PTL: Profile/tier/level combination.

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading (type of picture).

- RASL: Random-access skipped leading (type of picture).

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RDPCM: Residual DPCM

- ROT: Rotation operation for low-frequency transform coefficients.

- RPL: Reference picture list.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SADL: Small adhoc deep learning library

- SAO: Sample-adaptive offset.

- SBT: Subblock transform.

- SbTMVP: Subblock based temporal motion vector prediction.

- SCIPU: Smallest chroma intra prediction unit.

- SD: Slice data; alternatively, standard-definition.

- SDH: Sign data hiding.

- SDT: Signal-dependent transform.

- SE: Syntax element.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIF: Switchable (motion) interpolation filter.

- SIMD: Single instruction, multiple data.

- SMVD: Symmetric MVD.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- STRP: Short-term reference picture.

- STSA: Step-wise temporal sublayer access.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- TIMD: Template-based intra mode derivation

- TM: Template matching.

- TMVP: Temporal motion vector prediction.

- TS: Transform skip.

- TSRC: Transform skip residual coding.

- TT: Ternary tree.

- UCBDS: Unrestricted center-biased diamond search.

- UGC: User-generated content.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VCM: Video coding for machines.

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- VQA: Visual quality assessment.

- VT: Verification testing.

- VTM: VVC Test Model.

- VUI: Video usability information.

- VVC: Versatile Video Coding, the standardization project developed by JVET.

- WAIP: Wide-angle intra prediction

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size as a CU in the VVC context.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size as a CU in the VVC context.

## Opening remarks

Remarks during the opening session of the meeting Wednesday 11 January at 1300 UTC were as follows.

- Timing and organization of the meeting and online access, calendar posting of session plans

- Plans for subsequent F2F meetings in April (Antalya) and July (Geneva)

- Standards, TRs, supplements and technical papers approval and publication status

  - AVC

    - H.264 V14 Consented at 22nd meeting on 2021-04-30 (with annotated regions, shutter interval, and miscellaneous corrections), approved 2021-08-22, published 2021-10-13

    - ISO/IEC 14496-10:2020 (Ed. 9) FDIS ballot closed 2020-11-27, published 2020-12-15

    - ISO/IEC 14496-10:2022 (Ed. 10), had been forwarded from DIS directly for publication 2022-01-21 (with annotated regions, shutter interval, and miscellaneous corrections) with an editing period, submitted to ITTF in 2022-05 after consultation with ISO staff on format of graphics files, upgraded to “DIS approved for registration” in ISO Project system 2022-07-04, published 2022-11-07

    - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting, second draft including SMPTE ST 2128 issued at 28th meeting (not yet formally requested as a project)

    - Conformance testing

      - H.264.1 V6 Approved 2016-02-13, published 2016-06-17

      - Various amendments of ISO/IEC 14496-4:2004, including:

        - ISO/IEC 14496-4:2004/AMD 6:2005 Advanced Video Coding conformance

        - ISO/IEC 14496-4:2004/AMD 9:2006 AVC fidelity range extensions conformance

        - ISO/IEC 14496-4:2004/AMD 30:2009 Conformance testing for new profiles for professional applications

        - ISO/IEC 14496-4:2004/AMD 31:2009 Conformance testing for SVC profiles

        - ISO/IEC 14496-4:2004/AMD 38:2010 Conformance testing for Multiview Video Coding

        - ISO/IEC 14496-4:2004/AMD 41:2014 Conformance testing of MVC plus depth extension of AVC

        - ISO/IEC 14496-4:2004/AMD 42:2014 Conformance testing of Multi-Resolution Frame Compatible Stereo Coding extension of AVC

        - ISO/IEC 14496-4:2004/AMD 43:20153D-AVC conformance testing

        - ISO/IEC 14496-4:2004/AMD 45:2016 Conformance Testing for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps Extension of AVC

    - Reference software

      - H.264.2 V7 Approved 2016-02-13, published 2016-05-30

      - Various amendments of ISO/IEC 14496-5:2001, including:

        - ISO/IEC 14496-5:2001/AMD 6:2005 Advanced Video Coding (AVC) and High Efficiency Advanced Audio Coding (HE AAC) reference software

        - ISO/IEC 14496-5:2001/AMD 8:2006 AVC fidelity range extensions reference software

        - ISO/IEC 14496-5:2001/AMD 15:2010 Reference software for Multiview Video Coding

        - ISO/IEC 14496-5:2001/AMD 18:2008 Reference software for new profiles for professional applications

        - ISO/IEC 14496-5:2001/AMD 19:2009 Reference software for Scalable Video Coding

        - ISO/IEC 14496-5:2001/AMD 33:2015 Reference software for MVC plus depth extension of AVC

        - ISO/IEC 14496-5:2001/AMD 34:2014 Reference software of the multi-resolution frame compatible stereo coding of AVC

        - ISO/IEC 14496-5:2001/AMD 35:2015 3D-AVC Reference software

        - ISO/IEC 14496-5:2001/AMD 39:2016 Reference software for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps of AVC

        - ISO/IEC 14496-5:2001/AMD 42:2017 Reference software for the alternative depth information SEI message extension of AVC

  - HEVC

    - H.265 V7 approved 2019-11-29, published 2020-01-10

    - ISO/IEC 23008-2:2020 (Ed. 4) FDIS ballot closed 2020-07-16, published 2020-08-27

    - H.265 V8 Consented at the 22nd meeting (shutter interval information SEI message and miscellaneous corrections), published 2020-10-13

    - ISO/IEC 23008-2:2020/AMD 1:2021 (shutter interval information SEI message) published 2021-07-12

    - ISO/IEC 23008-2:202x (Ed. 5) began as CDAM 2 High-range levels output of 25th meeting of January 2022, CDAM ballot closed 2022-04-15, conversion to 5th edition with miscellaneous corrections planned at 26th meeting of April 2022, text submitted for DIS ballot 2022-07-10, DIS ballot closed 2023-01-10

    - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting, second draft including SMPTE ST 2128 issued at 28th meeting (not yet formally requested as a project)

    - Conformance testing

      - H.265.1 V3 approved 2018-10-14, published 2019-01-15

      - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06

      - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

    - Reference software

      - H.265.2 V4 approved 2016-12-22, published 2017-04-10

      - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, published 2017-03-01

      - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, published 2017-11-09

  - VVC

    - H.266 V1 approved 2020-08-29, published 2020-11-10

    - ISO/IEC 23090-3:2021 (Ed. 1) published 2021-02-16

    - H.266 V2 with operation range extensions, Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-06-06, published 2022-07-12

    - ISO/IEC 23090-3:2022 (Ed. 2) with operation range extensions, approval at WG level to proceed to FDIS 2022-01-21, published 2022-09-25

    - ISO/IEC 23090-3:202x (Ed. 2) / Amd.1 New level and systems-related supplemental enhancement information, CDAM 1 issued from 26th meeting, ballot closed 2022-07-14, DAM 1 issued from 27th meeting, ballot closed 2023-01-03 (ready for action at this meeting)

    - Conformance testing

      - H.266.1 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

      - ISO/IEC 23090-15:2022 V1 approval at WG level to proceed to FDIS 2022-10-15, upgraded to “DIS approved for registration” in ISO Projects system 2021-10-24, upgraded to “FDIS registered for formal approval” 2022-07-11, FDIS ballot closed 2022-11-04, published 2022-11-24

      - ISO/IEC 23090-15:202x Amd.1 Operation range extensions – DAM 1 issued from 25th meeting 2022-01-21, upgraded to “CD approved for registration as DIS” status in ISO Projects system 2022-05-31, upgraded to “DIS registered” 2022-06-22, DAM ballot closed 2022-11-15 (ready for action at this meeting)

    - Reference software

      - H.266.2 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

      - ISO/IEC 23090-16:2022 V1 approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-04-21, upgraded to “FDIS registered for formal approval” 2022-04-22, FDIS ballot initiated 2022-07-24, FDIS ballot closed 2022-09-19, published 2022-10-23

  - VSEI

    - H.274 V1 approved 2020-08-29, published 2020-11-10

    - ISO/IEC 23002-7:2021 (Ed. 1) published 2021-01-28

    - H.274 V2 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-05-22 (after 1 Last Call comment and Additional Review), pre-published 2022-06-17, published 2022-07-25

    - ISO/IEC 23002-7:2022 (Ed. 2) approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-05-05 and “FDIS registered for formal approval” 2022-05-08, FDIS ballot closed 2022-09-27, published 2022-10-30

    - ISO/IEC 23002-7:202x (2nd Ed.) Amd.1 Request for new edition and CD for additional SEI messages issued at 27th meeting, ballot closed 2022-10-10, DAM registered 2022-11-13, pending DAM ballot

  - CICP

    - ISO/IEC 23091-2:2021 (Ed. 2) had been forwarded from DIS directly for publication in 2021-04 and published 2021-10-18

    - H.273 V2 (with 4:2:0 sampling alignment and corrections for range of values for sample aspect ratio, ICTCP equations for HLG, and transfer characteristics function for sYCC of IEC 61966-2-1) Consented on 2021-04-30, Last Call closed during the 23rd meeting with approval on 2021-07-14, published 2021-09-24

    - ISO/IEC 23091-2:202x (Ed. 3) Request for new edition and CD for new edition (including YCgCo-Re and YCoCg-Ro) issued at 27th meeting, ballot closed 2022-10-10, DIS registered 2022-11-13, pending DAM ballot (no action at this meeting)

    - Preliminary draft text for including SMPTE ST 2128 issued at 28th meeting (not yet formally requested as a project)

  - Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics

    - H.Sup15 V1, approved 2017-01-27, published 2017-04-12

    - ISO/IEC TR 23008-14:2018 published 2018-08

  - Signalling, backward compatibility and display adaptation for HDR/WCG video coding

    - H.Sup18 V1, approved 2017-10-27, published 2018-01-18

    - ISO/IEC TR 23008-15:2018 published 2018-08

  - Usage of video signal type code points

    - H.Sup19 V3 approved 2021-04-30, published 2021-06-04

    - ISO/IEC TR 23091-4 (Ed. 3) published 2021-05-23

  - Working practices using objective metrics for evaluation of video coding efficiency experiments

    - HSTP-VID-WPOM V1: approved 2020-07-03, published 2020-11

    - ISO/IEC TR 23002-8 (Ed. 1) published 2021-05-20

  - Film grain synthesis technologies for video applications

    - ISO/IEC TR 23002-9 Request for subdivision and WD 1 issued at 25th meeting 2022-01-21, WD 2 issued at 27th meeting, WD 3 issued at 28th meeting

  - The following freely available standards are published here in ISO/IEC:
https://standards.iso.org/ittf/PubliclyAvailableStandards/index.html

    - Various amendments of ISO/IEC 14496-4:2004

    - Various amendments of ISO/IEC 14496-5:2001

    - ISO/IEC 23008-2:2020 (Ed. 4) HEVC

  - The following standards that have been intended by JVET to be publicly available were not available at https://standards.iso.org/ittf/PubliclyAvailableStandards/index.html as of 2023-01-08. (Please see below for record of previously issued requests.)

    - ISO/IEC 23091-2:2021 (Ed. 2) Video CICP (was requested in April 2021, and the 2019 previous edition was also not available there)

    - ISO/IEC 23008-2:2020 (Ed. 4) Amd.1:2021: Shutter interval information SEI message, published 2021-07-12 (has not been requested)

    - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, published 2017-03-01

    - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, published 2017-11-09

    - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08, published 2018-08-06

    - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

    - ISO/IEC 14496-10:2022 (Ed. 10) – AVC – final text issued and public availability requested at the 25th meeting (January 2022)

    - ISO/IEC 23002-7:2022 (Ed. 2) – VSEI – FDIS issued and public availability requested at the 25th meeting (January 2022)

    - ISO/IEC 23090-3:2022 (Ed. 2) – VVC – FDIS issued and public availability requested at the 25th meeting (January 2022)

    - ISO/IEC 23090-15:2022 (Ed. 1) – VVC conformance – FDIS issued and public availability requested at the 24th meeting (October 2021)

    - ISO/IEC 23090-16:2022 (Ed. 1) – VVC reference software – FDIS issued and public availability requested at the 25th meeting (January 2022)

  - It appears necessary to check if all older software and conformance packages are publicly available – it might be that it was never requested, e.g. for those that were produced by JCT-3V. This topic was left TBD until the next meeting – perhaps it would be best to compile a list of all relevant software and conformance parts of AVC, HEVC, MPEG-2 aka H.262, CICP, and request these in bulk.

- Draft standards progression status

  - AVC and HEVC colour type indicators for YCgCo-Re, YCgCo-Ro, and SMPTE ST 2128 are drafted and pending formal action

  - HEVC new levels (from JVET-Z1005) – ISO/IEC 23008-2 DIS of new edition of HEVC was issued from 26th meeting, incorporating Amd.1 and corrigenda items (ballot closed 2023-01-10, ballot comments in m61834); note that Amd.1 = shutter interval SEI is already included in latest ITU-T edition of H.265. It is noted that there are potential additional items (corrigenda+tickets, YCgCo-Re and YCgCo-Ro draft, SMPTE ST 2128, multiview profiles draft) which might potentially be included based on ballot comments, ITU-T consent is planned for July. It was noted that the referencing of VSEI is also somewhat different [Ed. Clarify] in the ITU-T and ISO/IEC versions of HEVC and AVC, which might be aligned at the next convenient time (basically editorial). Proceeding to FDIS should be considered at the current meeting.

  - VVC new level and systems-related supplemental enhancement information (from JVET-AA2005) – VVC DAM was issued from 27th meeting, ballot closed 2023-01-03, ballot comments in m61833. The plan is to convert this into FDIS of new edition at the current meeting. ITU-T consent is planned for July 2023 when a new edition of VSEI is also consented, to keep VVC and VSEI aligned.

  - VVC Conformance testing for operation range extensions – (from JVET-Y2026) – the DAM ballot closed 2022-11-15 (ballot comments in m61832), and this could progress to FDAM or a new edition at the current meeting. ITU consent is planned for July 2023.

  - VSEI additional SEI messages (from JVET-AB2006) – VSEI DAM (JVET draft 3) was issued from the 28th meeting and was pending a DAM ballot to be issued. To be converted into FDIS of new edition in April (if possible) or July, ITU-T consent in July 2023. No action on this is expected at the current meeting (but there are numerous input contributions in particular on NNPF).

  - Film grain synthesis technology for video applications (from JVET-AA2020 and JVET-AB0042) – JVET draft 3 was issued at the 28th meeting, and the ISO/IEC 23002-9 PDTR is planned to be issued at the current meeting (a request to start work on the TR had been made at the 25th meeting). The publication limit date is 2023-08-09, so an extension request may be needed.

  - Video CICP new edition with for YCgCo-Re and YCgCo-Ro (from JVET-Z1003), an ISO/IEC 23091-2 DIS was issued from the 28th meeting and is pending its ballot, so no action on this was expected at the current meeting). ITU-T consent is planned for July 2023.

  - Video CICP colour type indicator for SMPTE ST 2128 is drafted and pending formal action.

  - A request for free availability in ISO/IEC has to be made for each edition, amendment and corrigendum, and the request needs to be approved in the WG 5 Recommendations. A request form also needs to be filled out (but the form does not need to be issued as a WG 5 document). A freely available URL for the ITU publication should be provided for the following parts:

    - For the ongoing work items, when they become finalized

    - ISO/IEC 23008-2:2020/Amd.1:2021 – HEVC FDAM issued 20th meeting (October 2020), public availability not yet requested but may not be necessary as it becomes included in next edition

- The meeting logistics, agenda, working practices, policies, and document allocation considerations were reviewed.

  - Remote access to the meeting was provided using Zoom. This requires discipline in the meeting room (no microphone to be switched on, podium and room microphones to be under central control).

  - Having text and software available is crucial (and not just arriving at the end of the meeting).

  - There were no objections voiced in the opening plenary to the consideration of late contributions.

- The results of the previous meeting and the meeting report JVET-AB1000 were reviewed. The following small issues in the meeting report were noted and were not considered sufficient to warrant issuing a revision. These are obviously left over from a previous report, and the correct information can be found in other places of the report:

  - In section 2.3, the twenty-seventh JVET meeting should be referenced rather the twenty-fifth.

  - In section 2.4.2, the mentioning of “placeholder” documents should have been removed, as such cases did not happen in the 28th meeting.

  - In section 9, the time when AHG setup happened should have been 1600 CEST rather than 0510 UTC

- There was somewhat less of a problem of late non-cross-check documents.

- There were again a few documents registered where authors’ given names were not abbreviated, and/or company affiliation was missing in the authors’ list. Participants were reminded to stick to JVET’s conventions.

- Experts are asked to inform the chair when the title of a document is changed, or if authors are added. Otherwise, that might not be correct in the meeting notes.

- The primary goals of the meeting were

  - Conformance testing for version 2 of VVC (FDAM to be issued, DAM ballot comments in m61832)

  - New edition for HEVC (FDIS to be issued, DIS ballot comments in m61834)

  - New level and systems-related SEI for VVC (FDAM to be issued, DAM ballot comments in m61833)

  - Additional SEI in VSEI (under DAM ballot) – numerous input contributions

  - Preparation of TR for film grain (draft 4) (PDTR to be issued?)

  - Optimization of encoders and receiving systems for machine analysis of coded video content – WD of TR to be issued?

  - New edition video CICP (under DIS ballot)

  - Any action items on reference software JM/HM/VTM?

  - Additional colour type identifiers for AVC and HEVC (Draft 2 in JVET-AB1008 was issued at the last meeting) – could be included in new edition of H.265 (23008-2 DIS ballot comment on this?), no action was to be taken for AVC at this moment. It was suggested considering new editions of H.264 and ISO/IEC 14496-10 in the near future.

  - Guide to IT systems used in JVET (email reflectors, guide to MPEG and JVET web sites, calendar, ITU-T ftp, ftp for test materials, software git, conformance repositories, ticket reporting system, etc.) First version JVET-AB1012 was recently issued, update planned (also to become WG 5 output).

  - In the last meeting, public availability of experimental software packages was discussed, it was decided that the ECM branch (not the EE branches) shall be made publicly available. How about NNVC?

  - Exploration Experiments

    - Neural network-based video coding

    - Enhanced compression beyond VVC

- Liaison communication: any?

- Joint meetings: AG5 on subjective testing, WG 2 and VCEG on future video standardization activities, WG 4 (and potentially WG 1) on VCM, any other (e. g. with WG 7, regarding JVET inputs on point cloud coding)?

- The number of documents was higher than for the previous meeting (150->165)

- Scheduling was discussed, and it was agreed to avoid conducting “track” sessions in parallel (some BoG parallelism could occur)

- Principles of standards development were discussed.

## Scheduling of discussions

- The plans for the times of meeting sessions were established as follows, in UTC (1 hour behind the time in Geneva, Paris; 8 hours ahead of the time in Los Angeles, etc.). No session should last longer than 2 hrs.

1300–1500 1st “afternoon” session [break after 2 hours]

1520–1720 2nd “afternoon” session

[“evening” break – nearly 4 hours]

2100–2300 1st “night” session [break after 2 hours]

2320–0120+1 2nd “night” session

Sessions were announced via the calendar in the JVET document site at least 22 hrs. in advance. Particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Wed. 11 Jan., 1st day

  - Session 1:

    - 1300–XXXX Opening remarks, review of practices, agenda, IPR reminder

    - XXXX–1500 Reports of AHGs 1–X

  - Session 2:

    - 1520–XXXX Reports of AHGs X–15

    - XXXX–XXXX Review of EE1

  - Session 3:

    - 2100–2300 Review of EE1 and EE2

  - Session 4:

    - 2320–0120+1 Further review of EE2

- Thu. 12 Jan., 2nd day

  - Session 5:

    - 1300–XXXX Review of …

- Fri. 13 Jan., 3rd day

  - Session 9:

    - 1300–XXXX Review of …

- Mon. 16 Jan., 4th day

  - 0500–0730 MPEG information sharing session

  - Session 13:

    - 1300–1500 Review of …

    - …

- Tue. 17 Jan., 5th day

  - …

- Wed. 18 Jan., 6th day

  - 0500–0600 MPEG information sharing session

  - …

- Thu. 19 Jan., 7th day

  - …

- Fri. 20 Jan., 8th day

  - Plenary:

    - 1300–1500 Remaining business, Approval of output docs, AHGs, recommendations

  - 2100–2300 MPEG information sharing session

  - XXXX+1–XXXX+1 WG 5 Closing plenary: Approval of meeting recommendations

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks and summary reports, and may not be completely accurate; documents which are allocated to multiple sections are only counted in one of them):

- AHG reports (15) (section 3)

- Project development (section 4)

  - Deployment and advertisement of standards (2)

  - Text development and errata reporting (1)

  - Test conditions (3)

  - Subjective quality testing and verification testing (5)

  - Test Material (0)

  - Quality assessment (1)

  - Conformance test development (0)

  - Software development (3)

  - Implementation studies and complexity analysis (1)

  - AHG7: Low latency and constrained complexity (1)

  - Encoding algorithm optimization (3)

  - Profile/tier/level specification (0)

  - Proposed modification of system interface (0)

  - AHG15: Optimization of encoders and receiving systems for machine analysis of coded video content (8)

- Low-level tool technology proposals (section 5) with subtopics (number counts excluding BoG and summary reports)

  - AHG8: High bit depth and high bit rate coding (0) (section 5.1)

  - AHG11 and EE1: Neural network-based video coding (28) (section 5.2)

  - AHG12 and EE2: Enhanced compression beyond VVC capability (88) (section 5.3)

- AHG9: High-level syntax (HLS) proposals (section 6) with subtopics

  - SEI messages on neural-network post filter (19) (section 6.1)

  - SEI messages on topics other than NNPF (4) (section 6.2)

  - Film grain synthesis (1) (section 6.3)

  - Non-SEI HLS aspects (0) (section 6.4)

- Joint meetings, plenary discussions, BoG reports (0), liaison (2), summary of actions (section 7)

- Project planning (section 8)

- Establishment of AHGs (section 9)

- Output documents (section 10)

- Future meeting plans and concluding remarks (section 11)

The document counts above do not include cross-checks and summary reports.

# AHG reports (15)

These reports were discussed Wednesday 11 Jan. 2023 during 13XX–XXXX and 1520–XXXX UTC (chaired by JRO).

JVET-AC0001 JVET AHG report: Project management (AHG1) [J.-R. Ohm (chair), G. J. Sullivan (vice-chair)]

JVET-AC0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, C. Rosewarne (co-chairs), F. Bossen, J. Boyce, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs)]

JVET-AC0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs)]

JVET-AC0004 JVET AHG report: Test material and visual assessment (AHG4) [V. Baroncini, T. Suzuki, M. Wien (co-chairs), S. Liu, S. Puri, A. Segall, P. Topiwala, S. Wenger, J. Xu, Y. Ye (vice-chairs)]

JVET-AC0005 JVET AHG report: Conformance testing (AHG5) [D. Rusanovskyy, I. Moccagatta (co-chairs), F. Bossen, K. Kawamura, T. Ikai, H.-J. Jhu, K. Sühring, Y. Yu (vice-chairs)]

JVET-AC0006 JVET AHG report: ECM software development (AHG6) [V. Seregin (chair), J. Chen, F. Le Léannec, K. Zhang (vice-chairs)]

JVET-AC0007 JVET AHG report: Low latency and constrained complexity (AHG7) [A. Duenas, T. Poirier, S. Liu (co-chairs), L. Wang, J. Xu (vice-chairs)]

JVET-AC0008 JVET AHG report: High bit depth, high bit rate, and high frame rate coding (AHG8) [A. Browne, T. Ikai (co-chairs), D. Rusanovskyy, X. Xiu, Y. Yu (vice-chairs)]

JVET-AC0009 JVET AHG report: SEI message studies (AHG9) [S. McCarthy, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, C. Fogg, P. de Lagrange, G. J. Sullivan, A. Tourapis, S. Wenger (vice-chairs)]

JVET-AC0010 JVET AHG report: Encoding algorithm optimization (AHG10) [P. de Lagrange, A. Duenas, R. Sjöberg, A. Tourapis (AHG chairs)]

JVET-AC0011 JVET AHG report: Neural network-based video coding (AHG11) [E. Alshina, S. Liu, A. Segall (co-chairs), F. Galpin, J. Li, T. Shao, H. Wang, Z. Wang, M. Wien, P. Wu (vice-chairs)]

JVET-AC0012 JVET AHG report: Enhanced compression beyond VVC capability (AHG12) [M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, G. Li, X. Li, K. Naser, H. Yang (vice-chairs)]

JVET-AC0013 JVET AHG report: Film grain technologies (AHG13) [W. Husak, M. Radosavljević, W. Wan (co-chairs), D. Grois, Y. He, P. de Lagrange, A. Segall, A. Tourapis (vice-chairs)]

JVET-AC0014 JVET AHG report: NNVC software development (AHG14) [S. Eadie, F. Galpin, Y. Li, L. Wang, Z. Xie (AHG chairs)]

JVET-AC0015 JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG15) [C. Hollmann, S. Liu, S. Wang, M. Zhou (AHG chairs)]

# Project development (25)

## Deployment and advertisement of standards (2)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0020 Deployment status of the HEVC standard [G. J. Sullivan (SC 29 chair)]

JVET-AC0021 Deployment status of the VVC standard [G. J. Sullivan (SC 29 chair)]

## Text development and errata reporting (1)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0275 AHG9: Comments on VVC 2nd Ed. DAM1 [M. M. Hannuksela, M. Santamaria (Nokia)] [late]

## Test conditions (3)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0041 [AHG4] On visual volumetric video-based coding (V3C) testing conditions [S. Schwarz, M. M. Hannuksela, P. Rondao Alface, L. Kondrad, L. Ilola (Nokia)]

JVET-AC0044 [AHG4] Occupancy-only PSNR calculations for V3C V-PCC coding evaluation [S. Schwarz, M. M. Hannuksela (Nokia)]

JVET-AC0137 AHG3: On Affine AMVR setting for VTM CTC [X. Li (Google)]

JVET-AC0256 Cross-check of JVET-AC0137: On Affine AMVR setting for VTM CTC [A. Henkel (HHI)] [miss] [late]

## Subjective quality testing and verification testing (5)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0042 AHG4: Report on AHG meeting on verification test preparation for VVC multilayer coding [M. Wien (RWTH)]

JVET-AC0043 AHG4: Report on AHG meeting on subjective test preparation for film grain synthesis [M. Wien (RWTH)]

JVET-AC0145 AHG4: experiments in preparation of dual-layer VVC visual tests [P. de Lagrange (InterDigital)]

JVET-AC0181 AHG4: experiments in preparation of film grain visual tests [P. de Lagrange (InterDigital)]

JVET-AC0199 AHG13: On Film Grain Synthesis Subjective Evaluation [X. Meng, W. Zhang, S. Labrozzi (Disney Streaming)] [place] [late]

## Test material (0)

This section is kept as a template for future use.

## Quality assessment methodology (1)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0267 Training Methods in Visual Assessment: Potential Improvements for Expert Viewing Tests [M. Wien (RWTH), V. Baroncini (VABTech)] [miss] [late]

## Conformance test development (0)

This section is kept as a template for future use.

## Software development (3)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0057 AhG14 SADL v4 changes [F. Galpin, T. Dumas, P. Bordes, E. François (InterDigital)] [miss] [late]

JVET-AC0166 AHG3: Guidelines for VVC reference software development [F. Bossen, X. Li, K. Sühring (software coordinators)]

JVET-AC0204 AHG3: Guidelines for HEVC reference software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

## Implementation studies and complexity analysis (1)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0266 Update on open, optimized VVC implementations VVenC and VVdeC [A. Wieckowski, J. Brandenburg, C. Bartnik, V. George, J. Güther, G. Hege, C. Helmrich, A. Henkel, T. Hinz, C. Lehmann, C. Stoffers, B. Bross, H. Schwarz, D. Marpe, T. Schierl (HHI)] [miss] [late]

## AHG7: Low latency and constrained complexity (1)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0180 AHG 7: Reference frame padding for GDR [T. Poirier, F. Aumont (InterDigital)]

## AHG10: Encoding algorithm optimization (3)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0096 AHG10/12: Suggestion for new CTC for RPR in VTM and ECM [K. Andersson, R. Yu (Ericsson)]

JVET-AC0139 AHG10: Encoder Optimization of VTM Merge Functions [X. Li (Google)]

JVET-AC0149 AHG10/12: Reduced I-frame QP for RA [K. Andersson, P. Wennersten (Ericsson)]

JVET-AC0255 Cross-check of JVET-AC0149: AHG10/12: Reduced I-frame QP for RA [A. Henkel (HHI)] [miss] [late]

## Profile/tier/level specification (0)

This section is kept as a template for future use.

## Proposed modification of system interface (0)

This section is kept as a template for future use.

## AHG15: Optimization of encoders and receiving systems for machine analysis of coded video content (8)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0049 [AHG15] Draft technical report on optimizations for encoders and receiving systems for machine analysis of coded video content [C. Hollmann (Ericsson), S. Liu (Tencent), S. Wang (Alibaba)]

JVET-AC0050 [AHG15] Information about datasets used in VCM [C. Hollmann (Ericsson)]

JVET-AC0073 AHG15: On common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content [S. Liu (Tencent), C. Hollmann (Ericsson)]

JVET-AC0076 AHG9/AHG15: On the NNPFC SEI message for machine analysis [M. M. Hannuksela, F. Cricri, J. I. Ahonen, H. Zhang (Nokia)]

JVET-AC0077 AHG9/AHG15: On bitstreams that are potentially suboptimal for user viewing [M. M. Hannuksela, F. Cricri, H. Zhang (Nokia)]

JVET-AC0079 [AHG15] Effect of the perceptual QP adaptation (QPA) on machine task performance [C. Kim, D. Gwak, J. Lim (LGE)]

JVET-AC0086 AHG15: Feature based Encoder-only algorithms for the Video Coding for Machines [B. Li, S. Wang, Y. Ye (Alibaba), S. Wang (CityU HK)]

JVET-AC0092 AHG15: Investigations on the common test conditions of Video Coding for Machines (VCM) [S. Wang, J. Chen, Y. Ye (Alibaba), S. Wang (CityU HK)]

# Low-level tool technology proposals

## AHG8: High bit rate and high bit depth coding for VVC (0)

This section is kept as a template for future use. The possibility of removing this section and discontinuing AHG8 was considered, but AHG8 was reconstituted for another meeting cycle.

## AHG11: Neural network-based video coding (28)

### Summary, BoG reports, and information documents

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0023 EE1: Summary report of exploration experiments on neural network-based video coding [E. Alshina, F. Galpin, Y. Li, M. Santamaria, H. Wang, L. Wang, Z. Xie (EE coordinators)]

### EE1 contributions: Neural network-based video coding (17)

Beyond the EE summary report JVET-AC0023, selected contributions in this area were discussed at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

For actions decided to be taken, see section 5.2.1, unless otherwise noted.

JVET-AC0051 EE1-2.3: RPR-Based Super-Resolution Guided by Partition Information Combined with GOP Level Adaptive Resolution [Q. Han, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO), J. Nam, S. Yoo, J. Lim, S. H. Kim (LGE)]

JVET-AC0222 Crosscheck of JVET-AC0051 (EE1-2.3: RPR-Based Super-Resolution Guided by Partition Information Combined with GOP Level Adaptive Resolution) [D. Liu (Ericsson)] [miss] [late]

JVET-AC0052 EE1-2.4: CNN filter Based on RPR-based SR Combined with GOP Level Adaptive Resolution [S. Huang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO), J. Nam, S. Yoo, J. Lim, S. H. Kim (LGE)]

JVET-AC0223 Crosscheck of JVET-AC0052 (EE1-2.4: CNN filter Based on RPR-based SR Combined with GOP Level Adaptive Resolution) [D. Liu (Ericsson)] [miss] [late]

JVET-AC0055 EE1-1.11: Content-adaptive post-filter [M. Santamaria, R. Yang, F. Cricri, J. Lainema, H. Zhang, R. G. Youvalari, M. M. Hannuksela (Nokia)]

JVET-AC0056 EE1-3.1 CompressAI models integration using SADL [F. Galpin, F. Lefebvre, F. Racapé (InterDigital)]

JVET-AC0257 Crosscheck of JVET-AC0056 (EE1-3.1 CompressAI models integration using SADL) [T. Chujoh (Sharp)] [miss] [late]

JVET-AC0063 EE1-1.3: On chroma order adjustment in NNLF [Z. Dai, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AC0271 Crosscheck of JVET-AC0063 (EE1-1.3: On chroma order adjustment in NNLF) [R. Chang (Tencent)] [miss] [late]

JVET-AC0064 EE1-1.4: On adjustment of residual for NNLF [Z. Dai, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AC0272 Crosscheck of JVET-AC0064 (EE1-1.4: On adjustment of residual for NNLF) [R. Chang (Tencent)] [miss] [late]

JVET-AC0078 EE1-2.1: Updates on RPR encoder and post-filter [J. Nam, S. Yoo, J. Lim, S. Kim (LGE)]

JVET-AC0089 EE1-1.5: Combined intra and inter models for luma and chroma [D. Liu, J. Ström, M. Damghanian, P. Wennersten, K. Andersson (Ericsson)]

JVET-AC0273 Crosscheck of JVET-AC0089 (EE1-1.5: Combined intra and inter models for luma and chroma) [R. Chang (Tencent)] [miss] [late]

JVET-AC0106 EE1-1.10: Complexity Reduction on Neural-Network Loop Filter [J. N. Shingala, A. Shyam, A. Suneia, S. P. Badya (Ittiam), T. Shao, A. Arora, P. Yin, S. McCarthy (Dolby)]

JVET-AC0241 Crosscheck of JVET-AC0106 (EE1-1.10: Complexity Reduction on Neural-Network Loop Filter) [Y. Li (Bytedance)] [miss] [late]

JVET-AC0116 EE1-3.2: neural network-based intra prediction with learned mapping to VVC intra prediction modes [T. Dumas, F. Galpin, P. Bordes (InterDigital)]

JVET-AC0238 Crosscheck of JVET-AC0116 (EE1-3.2: neural network-based intra prediction with learned mapping to VVC intra prediction modes) [Y. Li (Bytedance)] [late]

JVET-AC0270 Crosscheck of EE1-3.2 (JVET-AC0116: neural network-based intra prediction with learned mapping to VVC intra prediction modes) [M. Damghanian, J. Ström (Ericsson)] [late]

JVET-AC0274 Crosscheck of JVET-AC0116 (EE1-3.2.2: Low-complexity version of the neural network-based intra prediction mode in 16-bit signed integer) [T. Shao (Dolby)] [miss] [late]

JVET-AC0118 EE1-1.8: QP-based loss function design for NN-based in-loop filter [C. Zhou, Z. Lv, J. Zhang (vivo)]

JVET-AC0215 Crosscheck of JVET-AC0118 (EE1-1.8: QP-based loss function design for NN-based in-loop filter) [C. Lin (Bytedance)] [miss] [late]

JVET-AC0143 EE1-1.6: NN chroma model without partitioning input [J. Ström, D. Liu, K. Andersson, P. Wennersten, M. Damghanian, R. Yu (Ericsson)]

JVET-AC0234 Cross-check of JVET-AC0143 (EE1-1.6: NN chroma model without partitioning input) [M. Santamaria (Nokia)] [miss] [late]

JVET-AC0155 EE1-1.9: Reduced complexity CNN-based in-loop filtering [S. Eadie, M. Coban, M. Karczewicz (Qualcomm)]

JVET-AC0177 EE1-1.7: Deep In-Loop Filter with Additional Input Information [Y. Li, K. Zhang, L. Zhang (Bytedance)]

JVET-AC0209 Crosscheck of JVET-AC0177 (EE1-1.7: Deep In-Loop Filter with Additional Input Information) [C. Zhou (vivo)] [miss] [late]

JVET-AC0194 EE1-1.1: More refinements on NN based in-loop filter with a single model [R. Chang, L. Wang, X. Xu, S. Liu (Tencent)]

JVET-AC0195 EE1-1.2: encoder-only optimization for NN based in-loop filter with a single model [R. Chang, L. Wang, X. Xu, S. Liu (Tencent)]

JVET-AC0196 EE1-2.2: GOP Level Adaptive Resampling with CNN-based Super Resolution [R. Chang, L. Wang, X. Xu, S. Liu (Tencent)]

JVET-AC0221 Crosscheck of JVET-AC0196 (EE1-2.2: GOP Level Adaptive Resampling with CNN-based Super Resolution) [D. Liu (Ericsson)] [miss] [late]

### EE1 related contributions: Neural network-based video coding (6)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0066 EE1-related: Improvement on EE1-1.7 [Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AC0081 EE1-related: A CNN Filter for RPR-Based Super-Resolution Using Wavelet Decomposition Combined with GOP Level Adaptive Resolution [H. Lan, C. Jung (Xidian Univ.), Y. Liu, M. Liu (OPPO), J. Nam, S. Yoo, J. Lim, S. H. Kim (LGE)]

JVET-AC0126 EE1-related: Reduced complexity through channel redistribution in NN head [P. Wennersten, J. Ström, D. Liu (Ericsson)]

JVET-AC0156 [EE1-related] RTNN: An In-loop Filter Based on Resblock and Transformer [H. Zhang, C. Jung (Xidian Univ.), Y. Liu, M. Li (OPPO)]

JVET-AC0178 EE1-related: In-Loop Filter with Wide Activation and Large Receptive Field [Y. Li, K. Zhang, L. Zhang (Bytedance)]

JVET-AC0197 EE1-1.1-related: More refinements on NN based in-loop filter [R. Chang, L. Wang, X. Xu, S. Liu (Tencent)]

### Improvements of NNVC software beyond EE1 (1)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0065 Non-EE1: On flipping of input and output of model in NNVC filter set 0 [Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

### Other aspects of neural network-based video coding (4)

JVET-AC0090 AhG11: Neural Network-based Reference CU Quality Enhancement for Motion Compensation Prediction [Y. Chu, Z. Wang, W. Zhang, S. Li (Hisense)]

JVET-AC0091 AhG11: Fourier Series and Laplacian Noise-based Quantization Error Compensation for End-to-End Learning-based Image Compression [S. Jiang, Z. Wang, W. Zhang, S. Li (Hisense)]

JVET-AC0114 AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement [J. Jia, Y. Zhang, H. Zhu, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent)]

JVET-AC0179 AHG11: Swin-Transformer based In-Loop Filter for Natural and Screen Contents [J. Li, K. Zhang, L. Zhang, M. Wang (Bytedance)]

## AHG12: Enhanced compression beyond VVC capability (88)

### Summary and BoG reports

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0024 EE2: Summary report of exploration experiments on enhanced compression beyond VVC capability [V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)]

### EE2 contributions: Enhanced compression beyond VVC capability (31)

Contributions in this area were discussed in the context of the EE summary report JVET-AC0024 unless otherwise noted.

For actions decided to be taken, see section 5.3.1, unless otherwise noted.

JVET-AC0045 EE2-1.7: Adaptive Reference Region DIMD [Z. Fan, Y. Yasugi, T. Ikai (Sharp)]

JVET-AC0226 Cross-check of JVET-AC0045 (EE2-1.7: Adaptive Reference Region DIMD) [P. Andrivon (Xiaomi)] [late]

JVET-AC0046 EE2-1.11c: Combination of EE2-1.7, EE2-1.8, EE2-1.9, and EE2-1.10 [Z. Fan, Y. Yasugi, T. Ikai (Sharp), S. Blasi, J. Lainema (Nokia), T. Dumas, K. Reuzé (InterDigital)]

JVET-AC0054 EE2-1.12: Gradient and location based convolutional cross-component model (GL-CCCM) for intra prediction [R. G. Youvalari, P. Astola, J. Lainema (Nokia)]

JVET-AC0060 EE2-3.4: BVP candidates clustering and BVD sign derivation for Reconstruction-Reordered IBC mode [D. Ruiz Coll, J.-K. Lee, V. Warudkar (Ofinno)]

JVET-AC0233 Crosscheck of JVET-AC0060 (EE2-3.4: BVP candidates clustering and BVD sign derivation for Reconstruction-Reordered IBC mode) [H. Zhang (Tencent)] [miss] [late]

JVET-AC0071 EE2-3.1: Direct block vector mode for chroma prediction [J.-Y. Huo, X. Hao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), M. Li, L. Zhang, J. Ren (OPPO)]

JVET-AC0245 Crosscheck of JVET-AC0071 (EE2-3.1: Direct block vector mode for chroma prediction) [X. Li (Alibaba)] [miss] [late]

JVET-AC0072 EE2-3.2: Block vector difference sign prediction for IBC blocks [J.-Y. Huo, Z.-Y. Zhang, X. Hao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), M. Li, L. Zhang, J. Ren (OPPO)]

JVET-AC0246 Crosscheck of JVET-AC0072 (EE2-3.2: Block vector difference sign prediction for IBC blocks) [X. Li (Alibaba)] [miss] [late]

JVET-AC0080 EE2-3.7: Chroma IBC method as in VTM-5.0 [Y. Wang, X. Xu, X. Zhao, R. Chernyak, S. Liu (Tencent)]

JVET-AC0133 Cross-check of JVET-AC0080 EE2-3.7 on Chroma IBC method as in VTM-5.0 [X. Li (Google)]

JVET-AC0082 EE2-1.1: Directional planar prediction [S. Yoo, J. Choi, J. Nam, M. Hong, J. Lim, S. Kim (LGE)]

JVET-AC0261 crosscheck of JVET-AC0082 (EE2-1.1 : Directional planar prediction) [K. Naser (InterDigital)] [miss] [late]

JVET-AC0083 EE2-1.4: Combination of directional planar prediction methods [S. Yoo, J. Choi, J. Nam, M. Hong, J. Lim, S. Kim (LGE), K. Kim, D. Kim, J.-H. Son, J. S. Kwak (Wilus), X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)]

JVET-AC0084 EE2-1.3: Horizontal and vertical planar modes [X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)]

JVET-AC0243 Crosscheck of JVET-AC0084 (EE2-1.3: Horizontal and vertical planar modes) [F. Wang (OPPO)] [miss] [late]

JVET-AC0094 EE2-1.10: Optimizing the use of reference samples [K. Reuzé, T. Dumas, K. Naser, Y. Chen (InterDigital)]

JVET-AC0168 Crosscheck of JVET-AC0094 (EE2-1.10: Optimizing the use of reference samples) [H.-J. Jhu (Kwai)]

JVET-AC0098 EE2-1.8: Location-dependent DIMD [S. Blasi, J. Lainema (Nokia)]

JVET-AC0227 Cross-check of JVET-AC0098 (EE2-1.8: Location-dependent DIMD) [P. Andrivon (Xiaomi)] [late]

JVET-AC0099 EE2-1.9: TIMD with directional blending [S. Blasi, J. Lainema (Nokia)]

JVET-AC0228 Cross-check of JVET-AC0099 (EE2-1.9: TIMD with directional blending) [P. Andrivon (Xiaomi)] [late]

JVET-AC0100 EE2-1.11a: Combination of Test 1.8 and Test 1.9 [S. Blasi, J. Lainema (Nokia)]

JVET-AC0229 Cross-check of JVET-AC0100 (EE2-1.11a: Combination of Test 1.8 and Test 1.9) [P. Andrivon (Xiaomi)] [late]

JVET-AC0101 EE2-1.11b: Combination of Test 1.8, Test 1.9 and Test 1.10 [S. Blasi, J. Lainema (Nokia), T. Dumas, K. Reuzé (InterDigital)]

JVET-AC0230 Cross-check of JVET-AC0101 (EE2-1.11b: Combination of Test 1.8, Test 1.9 and Test 1.10) [P. Andrivon (Xiaomi)] [late]

JVET-AC0231 Cross-check of JVET-AC0102 (EE2-1.11c: Combination of Test 1.7, Test 1.8, Test 1.9 and Test 1.10) [P. Andrivon (Xiaomi)] [late]

Cross-check of a withdrawn document?

JVET-AC0104 EE2-3.3: Block Vector Difference Prediction for IBC blocks [A. Filippov, V. Rufitskiy (Ofinno)]

JVET-AC0210 Crosscheck of JVET-AC0104 (EE2-3.3: Block Vector Difference Prediction for IBC blocks) [R.-L. Liao (Alibaba)] [miss] [late]

JVET-AC0265 crosscheck of JVET-AC0104 (EE2-3.3: Block Vector Difference Prediction for IBC blocks) [K. Naser (InterDigital)] [miss] [late]

JVET-AC0268 Crosscheck of JVET-AC0104 (EE2-3.3: Block Vector Difference Prediction for IBC blocks) [M. Radosavljević (Xiaomi)] [miss] [late]

JVET-AC0105 EE2-1.2: Improvements on planar horizontal and planar vertical mode [K. Kim, D. Kim, J.-H. Son, J.-S. Kwak (WILUS)]

JVET-AC0262 crosscheck of JVET-AC0105 (EE2-1.2: Improvements on planar horizontal and planar vertical mode) [K. Naser (InterDigital)] [miss] [late]

JVET-AC0112 EE2-3.6: IBC-CIIP, IBC-GPM, and IBC-LIC [Y. Wang, K. Zhang, L. Zhang, N. Zhang (Bytedance), C. Ma, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai)]

JVET-AC0202 Crosscheck EE2-3.6-ef: IBC-CIIP, IBC-GPM, and IBC-LIC [X. Li (Google)] [late]

JVET-AC0244 Crosscheck of JVET-AC0112 (EE2-3.6 a, b & d: IBC-CIIP, IBC-GPM, and IBC-LIC) [L. Zhang (OPPO)] [late]

JVET-AC0113 EE2-3.5: Combination of test EE2-3.1, test EE2-3.2, test EE2-3.3, and test EE2-3.4 [A. Filippov, V. Rufitskiy, D. Ruiz Coll, J.-K. Lee, V Warudkar (Ofinno), J.-Y. Huo, X. Hao, Z.-Y. Zhang, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), M. Li, L. Zhang, J. Ren (OPPO)]

JVET-AC0211 Crosscheck of JVET-AC0113 (EE2-3.5: Combination of test EE2-3.1, test EE2-3.2, test EE2-3.3, and test EE2-3.4) [R.-L. Liao (Alibaba)] [miss] [late]

JVET-AC0115 EE2-4.1a: modifications of MTS and LFNST for Intra TMP coded blocks [D. Kim, K. Kim, J.-H. Son, J. Kwak (WILUS), K. Naser, T. Poirier, F. Galpin, A. Robert (InterDigital)]

JVET-AC0240 Crosscheck of JVET-AC0115 (EE2-4.1: Experimental results of EE2-4.1a, EE2-4.1b, and EE2-4.1c) [F. Wang (OPPO)] [miss] [late]

JVET-AC0119 EE2-1.6: On Chroma Fusion improvement [C. Zhou, Z. Lv, J. Zhang (vivo)]

JVET-AC0125 EE2-3.8: Combination of chroma IBC tests [Y. Wang, X. Xu, X. Zhao, R. Chernyak, S. Liu (Tencent), J. Huo, X. Hao, Y. Ma, F. Yang (Xidian Univ.), J. Ren, M. Li (OPPO)]

JVET-AC0206 Crosscheck JVET-AC0125 EE2-3.8 on Combination of chroma IBC tests [X. Li (Google)] [late]

JVET-AC0130 EE2-4.2: Non-Separable Primary Transform for Intra Coding [P. Garus, M. Coban, B. Ray, V. Seregin, M. Karczewicz (Qualcomm), M. Koo, J. Zhao, J. Lim, S. Kim (LGE)]

JVET-AC0264 crosscheck of JVET-AC0130 (EE2-4.2: Non-Separable Primary Transform for Intra Coding) [K. Naser (InterDigital)] [miss] [late]

JVET-AC0144 EE2 Test 2.1, 2.2, 2.3 and 2.4: Affine DMVR [H. Huang, Y. Zhang, Z. Zhang, C.-C. Chen, V. Seregin, M. Karczewicz (Qualcomm), J. Chen, R.-L. Liao, X. Li, Y. Ye (Alibaba)]

JVET-AC0277 Cross-check of JVET-AC0144 tests 2.2 and 2.4b [M. Blestel (Xiaomi)] [miss] [late]

JVET-AC0147 EE2-1.13 and 1.14: CCCM using non-downsampled luma samples [H.-J. Jhu, C.-W. Kuo, X. Xiu, W. Chen, N. Yan, C. Ma, X. Wang (Kwai), V. Seregin, Y.-J. Chang, B. Ray, M. Karczewicz (Qualcomm)]

JVET-AC0117 Cross-check of EE2-1.13: CCCM using non-downsampled luma samples [T. Dumas (InterDigital)]

JVET-AC0247 Crosscheck of JVET-AC0147 (EE2-1.13 and 1.14: CCCM using non-downsampled luma samples) [X. Li (Alibaba)] [miss] [late]

JVET-AC0150 EE2-2.6: ARMC merge candidate list reordering for AMVP-merge mode [K. Cui, C. S. Coban, Z. Zhang, H. Huang, V. Seregin, M. Karczewicz (Qualcomm), H. Jang (LGE)]

JVET-AC0158 EE2-2.5: Pixel based affine motion compensation [Z. Zhang, H. Huang, Y. Zhang, P. Garus, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AC0162 EE2-5.1: Using prediction samples or residual samples for adaptive loop filter [C. Ma, X. Xiu, C.-W. Kuo, W. Chen, H.-J. Jhu, N. Yan, X. Wang (Kwai)]

JVET-AC0188 Crosscheck of JVET-AC0162 (EE2-5.1: Using prediction or residual samples for adaptive loop filter) [W. Yin (Bytedance)] [miss] [late]

JVET-AC0183 EE2-5.2: ALF with Diversified Extended Taps [W. Yin, K. Zhang, L. Zhang (Bytedance)]

JVET-AC0184 EE2-5.3: Combination Tests of 5.1 and 5.2 [W. Yin, K. Zhang, L. Zhang (Bytedance), C. Ma, X. Xiu, C.-W. Kuo, W. Chen, H.-J. Jhu, N. Yan, X. Wang (Kwai)] [place] [late]

JVET-AC0214 Crosscheck of JVET-AB0184 on EE2-5.3: Combination Tests of 5.1 and 5.2 [X. Li (Google)] [miss] [late]

JVET-AC0224 Crosscheck of JVET-AC0184 on Test 5.3b: Combination Tests of 5.1b and 5.2 [L.-F. Chen (Tencent)] [miss] [late]

JVET-AC0185 EE2-2.7: Enhanced temporal motion information derivation [L. Zhao, K. Zhang, L. Zhang (Bytedance), L.-F. Chen, R. Chernyak, X. Zhao, X. Xu, S. Liu (Tencent)]

JVET-AC0216 Crosscheck of EE2-2.7b: Enhanced temporal motion information derivation [X. Li (Google)] [miss] [late]

### EE2 related contributions (7)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0093 EE2-related: test 2.5a and PROF improvement [F. Galpin, A. Robert, K. Naser (InterDigital)] [late]

JVET-AC0121 EE2-related: on GL-CCCM improvement (test 1.12a) [P. Bordes, K. Naser, F. Galpin, E. François (InterDigital), R. G. Youvalari, P. Astola, J. Lainema (Nokia)]

JVET-AC0232 Cross-check of JVET-AC0121 (EE2-related: on GL-CCCM improvement (test 1.12a)) [P. Andrivon (Xiaomi)] [late]

JVET-AC0140 EE2-related: Additional results for test EE2-3.3 [A. Filippov, V. Rufitskiy (Ofinno)] [late]

JVET-AC0269 Crosscheck of JVET-AC0140 (EE2-related: Additional results for test EE2-3.3) [M. Radosavljević (Xiaomi)] [miss] [late]

JVET-AC0165 EE2-related: Additional results to EE2-3.4 [D. Ruiz Coll, J.-K. Lee, V Warudkar (Ofinno)]

JVET-AC0186 EE2-related: EE2-2.7 with further encoder optimizations [L. Zhao, K. Zhang, L. Zhang (Bytedance)]

JVET-AC0205 EE2-related: Modifications of MTS and LFNST for IntraTMP [R. G. Youvalari, D. Bugdayci Sansli, J. Lainema (Nokia)] [late]

JVET-AC0276 EE2-related: Complexity reduction for Decoder Side Control Point Motion Vector Refinement (test 2.3b) [M. Bestel, F. Le Léannec, P. Andrivon, M. Radosavljević (Xiaomi), H. Huang, Y. Zhang, Z. Zhang, C.-C. Chen, V. Seregin, M. Karczewicz (Qualcomm)] [late]

### ECM modifications beyond EE2 (50)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0048 Non-EE: Modification of TIMD [Y. Yasugi, T. Ikai (Sharp)] [place] [late]

JVET-AC0248 Crosscheck of JVET-AC0048 (Non-EE: Modification of TIMD) [X. Li (Alibaba)] [miss] [late]

JVET-AC0053 AHG12: Simplified linear model solver [J. Lainema, P. Astola, A. Aminlou, R. G. Youvalari (Nokia)]

JVET-AC0059 AHG12: TMP using Reconstruction-Reordered for screen content coding (RR-TMP) [J.-K. Lee, D. Ruiz Coll, V. Warudkar (Ofinno)]

JVET-AC0242 Crosscheck of JVET-AC0059 (AHG12: TMP using Reconstruction-Reordered for screen content coding (RR-TMP)) [?? (??)][miss] [late]

JVET-AC0067 Non-EE2: Modifications on template-based multiple reference line intra prediction [L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AC0068 Non-EE2: multi-candidate IntraTMP [F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AC0259 Crosscheck of JVET-AC0068 (Non-EE2: multi-candidate IntraTMP) [Z. Lv (vivo)] [miss] [late]

JVET-AC0069 Non-EE2: Intra Template-Matching Prediction Fusion [L. Zhang, F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AC0260 Crosscheck of JVET-AC0069 (Non-EE2: Intra Template-Matching Prediction Fusion) [Z. Lv (vivo)] [miss] [late]

JVET-AC0070 Non-EE2: combination of JVET-AC0068 and JVET-AC0069 [F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AC0087 Non-EE2: Intra TMP with half-pel precision [X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)]

JVET-AC0095 AHG12: Fix related to pixel copy in motion compensation [K. Andersson, R. Yu (Ericsson)]

JVET-AC0096 AHG10/12: Suggestion for new CTC for RPR in VTM and ECM [K. Andersson, R. Yu (Ericsson)]

JVET-AC0097 Non-EE2: SGPM combined with IntraTMP [C. Fang, S. Peng, D. Jiang, J. Lin, X. Zhang (Dahua)] [place] [late]

JVET-AC0103 Non-EE2: AMVP mode with subblock-based temporal motion vector prediction [R.-L. Liao, J. Chen, Y. Ye, X. Li (Alibaba)]

JVET-AC0220 Crosscheck of JVET-AC0103 (Non-EE2: AMVP mode with subblock-based temporal motion vector prediction) [X. Xiu (Kwai)] [miss] [late]

JVET-AC0107 AHG12: Fusion of Intra Template Matching [J. R. Arumugam, A. Natesan, V. Valvaiker, J. N. Shingala (Ittiam), T. Lu, P. Yin, F. Pu, T. Shao, A. Arora, S. McCarthy (Dolby)]

JVET-AC0108 Non-EE2: Extend search area in Intra Template Matching Prediction (IntraTMP) [J.-Y. Huo, H.-Q. Du, H.-L. Zhang, Z.-Y. Zhang, W.-H. Qiao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.)]

JVET-AC0249 Crosscheck of JVET-AC0108 (Non-EE2: Extend search area in Intra Template Matching Prediction (IntraTMP)) [X. Li (Alibaba)] [miss] [late]

JVET-AC0109 Non-EE2: Intra template matching (Intra TMP) based on linear filter model [J.-Y. Huo, W.-H. Qiao, X. Hao, Z.-Y. Zhang, H.-Q. Du, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.)]

JVET-AC0250 Crosscheck of JVET-AC0109 (Non-EE2: Intra template matching (Intra TMP) based on linear filter model) [X. Li (Alibaba)] [miss] [late]

JVET-AC0110 Non-EE2: A Fusion method of Intra Template Matching Prediction (Intra TMP) [J.-Y. Huo, H.-Q. Du, H.-L. Zhang, W.-H. Qiao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.)]

JVET-AC0251 Crosscheck of JVET-AC0110 (Non-EE2: A Fusion method of Intra Template Matching Prediction (Intra TMP)) [X. Li (Alibaba)] [miss] [late]

JVET-AC0111 Non-EE2: Combination of JVET-AC0108, JVET-AC0109 and JVET-AC0110 for Intra TMP [J.-Y. Huo, W.-H. Qiao, H.-Q. Du, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), H. Yuan (Shandong Univ.)]

JVET-AC0252 Crosscheck of JVET-AC0111 (Non-EE2: Combination of JVET-AC0108, JVET-AC0109 and JVET-AC0110 for Intra TMP) [X. Li (Alibaba)] [miss] [late]

JVET-AC0120 Non-EE2: template based intra MPM list construction [C. Zhou, Z. Lv, J. Zhang (vivo)]

JVET-AC0123 MTT maximum depth correction for class B sequences at QP 22 in ECM [G. Laroche, P. Onno (Canon)]

JVET-AC0235 Cross-check of JVET-AC0123 "MTT maximum depth correction for class B sequences at QP 22 in ECM" [F. Le Léannec (Xiaomi)] [late]

JVET-AC0124 Non-EE2: Bi-prediction with block-level only out-of-bound management [F. Le Léannec, M. Blestel, P. Andrivon, M. Radosavljević (Xiaomi)]

JVET-AC0219 Crosscheck of JVET-AC0124 (Non-EE2: Bi-prediction with block-level only out-of-bound management) [X. Xiu (Kwai)] [miss] [late]

JVET-AC0263 Crosscheck of JVET-AC0124 (Non-EE2: Bi-prediction with block-level only out-of-bound management) [F. Galpin (InterDigital)] [miss] [late]

JVET-AC0138 Adjusting luma/chroma BD-rate balance in ECM [E. François, Y. Chen, C. Salmon-Legagneur (InterDigital)]

JVET-AC0236 Cross-check of JVET-AC0138 [F. Le Léannec (Xiaomi)] [miss] [late]

JVET-AC0142 Non-EE2: Cross-Component Discrete Mapping Model for Screen Content Coding [B. Vishwanath, K. Zhang, L. Zhang (Bytedance)]

JVET-AC0146 AHG12: Filtered Template Matching based Intra Prediction (FTMP) [R. G. Youvalari, D. Bugdayci Sansli, P. Astola, J. Lainema (Nokia)]

JVET-AC0148 Non-EE2: CCCM using multiple downsampling filters [Y.-J. Chang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AC0149 AHG10/12: Reduced I-frame QP for RA [K. Andersson, P. Wennersten (Ericsson)]

JVET-AC0159 Non-EE2: IBC MBVD list derivation [Z. Zhang, P. Nikitin, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AC0160 Non-EE2: On the condition of OBMC [X. Li (Google)]

JVET-AC0218 Crosscheck of JVET-AC0160 (Non-EE2: On the condition of OBMC) [H.-J. Jhu, X. Xiu (Kwai)] [miss] [late]

JVET-AC0161 Non-EE2: IBC adaptation for coding of natural content [B. Ray, H. Wang, C.-C. Chen, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AC0163 Non-EE2: On non-separable primary transform for intra blocks [N. Yan, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, C. Ma, X. Wang (Kwai)]

JVET-AC0253 Crosscheck of JVET-AC0163 on Non-EE2: On non-separable primary transform for intra blocks [X. Li (Google)] [miss] [late]

JVET-AC0164 Non-EE2: Improvements on local illumination compensation in ECM7.0 [X. Xiu, N. Yan, H.-J. Jhu, W. Chen, C.-W. Kuo, C. Ma, X. Wang (Kwai)]

JVET-AC0225 Crosscheck of JVET-AC0164 (Non-EE2: on Improvements on local illumination compensation in ECM7.0) [L.-F. Chen (Tencent)] [miss] [late]

JVET-AC0167 AHG12: Using block vector derived from IntraTMP as an IBC candidate for the current block [J.-K. Lee, D. Ruiz Coll, V. Warudkar (Ofinno)]

JVET-AC0169 Non-EE2: Template Matching for RR-IBC [C.-C. Chen, H. Huang, Z. Zhang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AC0207 Crosscheck of JVET-AC0169 (Non-EE2: Template Matching for RR-IBC) [D. Ruiz Coll, J.-K. Lee, V. Warudkar (Ofinno)] [miss] [late]

JVET-AC0170 Non-EE2: Fuse intra template matching prediction with intra prediction [Y. Wang, K. Zhang, L. Zhang (Bytedance)]

JVET-AC0171 AHG12: ECM-6 Tool Off Tests [X. Li (Google)]

JVET-AC0172 Non-EE2: IBC with fractional block vectors [W. Chen, X. Xiu, C. Ma, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai)] [place] [late]

JVET-AC0173 Non-EE2: ALF classification based on residual data [I. Jumakulyyev, N. Hu, Z. Zhang, V. Seregin, M. Karczewicz, H. Huang (Qualcomm)]

JVET-AC0175 Non-EE2: Local-Boosting Cross-Component Prediction [K. Zhang, L. Zhang, Z. Deng (Bytedance)]

JVET-AC0176 Non-EE2: Non-Local Cross-Component Prediction [K. Zhang, L. Zhang, Z. Deng (Bytedance)]

JVET-AC0182 Non-EE2: High-Precision MV Refinement for BDOF [M. Salehifar, Y. He, K. Zhang, L. Zhang (Bytedance)]

JVET-AC0187 Non-EE2: Template matching-based subblock motion refinement [L. Zhao, K. Zhang, Z. Deng, L. Zhang (Bytedance)] [place] [late]

JVET-AC0189 Non-EE2: SGPM without blending [Z. Deng, L. Zhang, K. Zhang, L. Zhao (Bytedance)]

JVET-AC0190 Non-EE2: On OBMC [Z. Deng, K. Zhang, L. Zhang (Bytedance)]

JVET-AC0191 Non-EE2: Modification to MPM list derivation [H. Wang, Y.-J. Chang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AC0258 Crosscheck of JVET-AC0191 (Non-EE2: Modification to MPM list derivation) [Z. Lv (vivo)] [miss] [late]

JVET-AC0192 Non-EE2: Temporal block vector prediction [N. Zhang, K. Zhang, L. Zhang (Bytedance)]

JVET-AC0193 Non-EE2: Copy-Padding for IBC [N. Zhang, K. Zhang, L. Zhang (Bytedance)]

JVET-AC0198 Non-EE2: IntraTMP with multiple modes [P.-H. Lin, J.-L- Lin, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AC0200 AhG12 Dynamic CABAC models [F. Lo Bianco, F. Galpin, E. François (InterDigital)] [late]

JVET-AC0237 Cross-check of JVET-AC0200 [F. Le Léannec (Xiaomi)] [late]

JVET-AC0201 AHG12: CIIP extension with Intra Template Matching [K. Naser, P. Bordes, F. Galpin, K. Reuzé, A. Robert (InterDigital)] [late]

JVET-AC0203 AHG12: On LMCS luma mapping in template processing for IBC TM-based tools [A. Filippov, V. Rufitskiy (Ofinno)] [late]

JVET-AC0212 Non-EE2: Template matching for IBC BVD suffix derivation [P. Nikitin, Z. Zhang, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)] [late]

JVET-AC0213 Non-EE2: SbTMVP with MMVD [L.-F. Chen, R. Chernyak, X. Zhao, X. Xu, S. Liu (Tencent)] [late]

JVET-AC0217 Crosscheck of JVET-AC0213 on SbTMVP with MMVD [X. Li (Google)] [miss] [late]

JVET-AC0239 Non-EE2: Prediction of MVD magnitude suffix bins [A. Filippov, V. Rufitskiy, K. Suverov (Ofinno)] [miss] [late]

# High-level syntax (HLS) and related proposals (29)

## AHG9: SEI messages on neural-network post filter (20)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0208 AHG9: Summary of Proposals Related to Neural-Network Post-Filter SEI Messages [S. Deshpande (Sharp), Y.-K. Wang (Bytedance)]

JVET-AC0047 AHG9: On bugfix for NNPFC and NNPFA SEI messages [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]

JVET-AC0061 AHG9: Comments on Neural-network Post-filter Characteristics SEI Message [S. Deshpande (Sharp)]

JVET-AC0062 AHG9: On NNPFC SEI Message [S. Deshpande, A. Sidiya (Sharp)]

JVET-AC0074 AHG9: On the VVC use of the NNPFC SEI message for picture rate upsampling [M. M. Hannuksela (Nokia)]

JVET-AC0075 AHG9: On NNPFC and NNPFA SEI messages for picture rate upsampling post-filter [M. M. Hannuksela (Nokia)]

JVET-AC0076 AHG9/AHG15: On the NNPFC SEI message for machine analysis [M. M. Hannuksela, F. Cricri, J. I. Ahonen, H. Zhang (Nokia)]

JVET-AC0085 AHG9: On neural-network post-filter characteristics (NNPFC) SEI message for temporal upsampling towards machine vision [S. Wang, J. Chen, Y. Ye (Alibaba), S. Wang (CityU HK)]

JVET-AC0127 AHG9: Combination of picture rate upsampling with other NNPF purposes [Y.-K. Wang, J. Xu, Y. Li, L. Zhang, K. Zhang, J. Li, C. Lin (Bytedance)]

JVET-AC0128 AHG9: On the signalling of complexity information in NNPFC SEI message [Hendry, S. Kim (LGE)]

JVET-AC0129 AHG9: On the NNPFC SEI message update and activation [Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE)]

JVET-AC0131 AHG9: On NNPFC SEI message repetition [Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE)]

JVET-AC0132 AHG9: On design for region-based neural-network post-filter SEI message [Hendry, J. Nam, H. Jang, S. Kim, J. Lim (LGE)]

JVET-AC0134 AHG9: Signalling of NNPF quality improvement [C. Lin, Y.-K. Wang, K. Zhang, J. Li, Y. Li, L. Zhang (Bytedance)]

JVET-AC0135 AHG9: Separate activation of color components for neural-network post-filter [C. Lin, Y. Li, Y.-K. Wang, K. Zhang, J. Li, L. Zhang (Bytedance)]

JVET-AC0136 AHG9: On additional NNPF purposes [C. Lin, Y.-K. Wang, K. Zhang, Y. Li, J. Li, L. Zhang (Bytedance)]

JVET-AC0152 AHG9: Bit-masking based representation of nnpfc_purpose [J. Xu, Y.-K. Wang, L. Zhang, Y. Li (Bytedance)]

JVET-AC0153 AHG9: Bitdepth increase indication in the NNPFC SEI message [J. Xu, Y.-K. Wang, L. Zhang (Bytedance)]

JVET-AC0154 AHG9: Miscellaneous cleanups of the neural-network post-filter SEI messages [J. Xu, Y.-K. Wang, L. Zhang, C. Lin (Bytedance)]

JVET-AC0174 AHG9: Multiple input pictures for neural-network post-processing filter [Y. Li, Y.-K. Wang, C. Lin, J. Li, J. Xu, K. Zhang, L. Zhang (Bytedance)]

## AHG9: SEI messages on topics other than NNPF (5)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0058 AHG9: On the SEI processing order SEI message [Y. He, M. Coban, M. Karczewicz (Qualcomm)]

JVET-AC0077 AHG9/AHG15: On bitstreams that are potentially suboptimal for user viewing [M. M. Hannuksela, F. Cricri, H. Zhang (Nokia)]

JVET-AC0088 AHG9: Generative Face Video SEI Message [B. Chen, J. Chen, S. Wang, Y. Ye (Alibaba), S. Wang (CityU HK)]

JVET-AC0122 AHG9: Attenuation Map Information SEI for reducing energy consumption of displays [C.-H. Demarty, F. Aumont, E. Reinhard, L. Blondé, O. Le Meur, Z. Ameur, E. François (InterDigital)]

JVET-AC0141 AHG9: Alternative Picture Timing SEI [H.-B. Teo, J. Gao, C.-S. Lim, K. Abe (Panasonic)]

## Film grain synthesis (4)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

JVET-AC0043 AHG4: Report on AHG meeting on subjective test preparation for film grain synthesis [M. Wien (RWTH)]

JVET-AC0151 AHG13: Proposed text: Film grain synthesis technology for video applications (CD draft) [D. Grois (Comcast), Y. He (Qualcomm), W. Husak (Dolby), P. de Lagrange (InterDigital), A. Norkin (Netflix), M. Radosavljević (Xiaomi), A. Tourapis (Apple), W. Wan (Broadcom)]

JVET-AC0181 AHG4: experiments in preparation of film grain visual tests [P. de Lagrange (InterDigital)]

JVET-AC0199 AHG13: On Film Grain Synthesis Subjective Evaluation [X. Meng, W. Zhang, S. Labrozzi (Disney Streaming)] [place] [late]

## Non-SEI HLS aspects (0)

Contributions in this area were discussed in session X at XXXX–XXXX UTC on XXday X Jan. 2023 (chaired by XXX).

# Plenary meetings, joint meetings, BoG reports, and liaison communications

## JVET plenaries

No intermediate plenaries were held, as document review and decisions were made in single-track mode at this meeting (with some BoG activity as noted). Further detail on scheduling is recorded in section 2.13.

General plenary wrap-up discussions are recorded under sections 8, 9, and 10.

## Information sharing meetings

Information sharing sessions with other WGs and AGs of the MPEG community were held on Monday 16 Jan. 0900–1130, Wednesday 18 Jan. 0900–1000, and Friday 20 Jan. 2100–2300 UTC. The status of the work in the MPEG WGs and AGs was reviewed at these information sharing sessions.

## Joint meeting with … on XXday XX Jan. XX:XX-XX:XX UTC on …

## BoGs (0)

The following break-out groups were established at this meeting to conduct discussion and develop recommendations on particular subjects

- A BoG on …. See section XX and the report JVET-AC0XXX for details.

## Liaison communications

m60701 Liaison statement from SC 29/WG 1 to WG 5 on JPEG AI [WG 1 via SC 29 Secretariat]

TD 80/Gen LS on JPEG AI [from JPEG]

JVET, as SC 29/WG 5 and Q6/16, received similar liaison letters from SC 29/WG 1 JPEG related to its JPEG AI project. The evaluation of responses to the JPEG AI Call for Proposals had taken place at the 96th JPEG meeting of 25-29 July 2022. In total, ten teams replied to the CfP in the standard reconstruction category, two submissions were received on compressed-domain de-noising, one submission was received on compressed-domain super resolution, and three submissions were received on compressed-domain image classification.

In the standard image reconstruction category, five teams successfully passed cross-check; among them three outperformed a VVC anchor, demonstrating BD-rate gain from 18% to 32% based on seven objective metrics. In subjective quality evaluation, the best submissions outperformed the VVC anchor on average. For half of the tested rates and images, the learning-based codec outperformed the VVC anchor with non-overlapping confidence intervals.

WG1 had agreed to create a JPEG AI Verification Model under Consideration (VMuC) by combining two of the best submissions. Several core experiments exploring potential improvements over the VMuC had also been established.

In the compressed-domain image classification track, two submissions demonstrated that image classification when applied directly in the compressed domain became more efficient when compared to image classification after full decoding, while at the same time requiring significantly less computational resources.

Further information about the JPEG AI CfP results, VMuC and Core experiments could be found in the following JPEG output documents:

- wg1n100279-096-ICQ-Description of JPEG AI Verification Model under Consideration and assocated software integration procedure

- wg1n100250-096-REQ-Report on the JPEG AI Call for Proposals Results

- wg1n100284-096-ICQ-JPEG AI Core Experiment 1

WG1 looked forward to further interactions with JVET on learning-based coding and neural network based coding tools.

A reply was drafted by JVET as WG 5 N 168.

JVET thanked JPEG for its liaison reply and its update on the status of the JPEG AI project.

The reply reported that for JVET, in the exploration of neural network-based video coding (NNVC), JVET had updated our common software codebase based on improvements and further training crosschecks that had been performed on neural network-based loop filtering. JVET said a document describing algorithms and software modules contained in the NNVC common software base would be issued as JVET-AB2019 and WG 5 N 165, following an editing period and that JVET-AB0023 is a summary of our Exploration Experiment 1 evaluating performance and complexity aspects of NNVC technologies. The reply said JVET plans to continue to explore neural-network based loop filtering, super resolution, intra prediction, and intra auto-encoding in the next round of exploration experiments.

The JVET reply also reported that a DAM of the VSEI standard (Rec. ITU-T H.274 | ISO/IEC 23002-7) was issued containing definitions of neural-network post-filter characteristics (NNPFC) and neural-network post-filter activation (NNPFA) SEI messages. An NNPFC SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with NNPFA SEI messages. JVET looked forward to further communicating with JPEG on items of mutual interest.

(ITU-T SG16 also sent a reply on behalf of Q6/16 expressing similar sentiments, with somewhat less detail.)

# Project planning (update)

## Software timeline

ECM7 software (including all adoptions) was planned to be available 3 weeks after the meeting.

The NNVC 3.0 codebase software was planned to be available 3 weeks after the meeting.

VTM19.0 software was planned to be available on 2022-11-30. (Note that further updates may be released later)

HM17.0 software was planned to be available on 2022-12-16.

## Core experiment and exploration experiment planning

An EE on neural network-based video coding was established, as recorded in output document JVET-AB2023.

An EE on enhanced compression technology beyond VVC capability using techniques other than neural-network technology was also established, as recorded in output document JVET-AB2024.

Initial versions of these documents were presented and approved in the first plenary session on Friday 22 July.

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM/VTM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- VTM algorithm description text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized that those rules which had been set up or refined during the 12th JVET meeting should be observed. In particular, for some CEs of some previous meetings, results were available late, and some changes in the experimental setup had not been sufficiently discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of a draft standard by the next meeting or in the near future.

- “Exploration experiments” (EEs) are also coordinated experiments. These are conducted on technology which is not foreseen to become part of a draft standard in the near future. The investigating methodology for assessment of such technology can also be an important part of an EE. (Further general rules for EEs, as far as deviating from the CE rules below, should be discussed in a future meeting. For the current meeting, procedures as described in the EE description document are deemed to be sufficient.)

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure that the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials as described below).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

- It is mandatory to report encoder optimizations made for the benefit of a tool, and if an equivalent optimization could be applied on the anchor, a comparison against the improved anchor shall be provided.

- A new proposal can be included in a CE based on group decision, regardless if an independent party has already performed a cross-check in the meeting when it was first proposed.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments for SDR video are described in the prior output document JVET-T2010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g. not referring to methods as “improved”, “optimized”, “enhanced”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

The CE development workflow is described at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/Core-experiment-development-workflow

CE read access is available using shared accounts: One account exists for MPEG members, which uses the usual MPEG account data. A second account exists for VCEG members with account information available in the TIES informal ftp area (IFA) system at:

https://www.itu.int/ifa/t/2017/sg16/exchange/wp3/q06/vceg_account.txt

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation is requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to be marked as “withdrawn”.

T2 = Test model software release + 2 weeks: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3.

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline minus 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular contribution deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforward combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, JVET would like to see comprehensive cross-checking done, with analysis of whether the description matches the software, and a recommendation of the value of the tool and given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology into a standard or test model.

Availability of specification text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the specification will be. There must also be sufficient time to study this in detail. CE contributions without sufficiently mature draft specification text in the CE input document should not be considered for adoption.

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de). The previously approved rules for MPEG ad hoc groups established in document SC29/AG2 N 46 were agreed to apply to these ad hoc groups.

Review of AHG plans was conducted during the plenary on XXday XX January 2023 at XXXX UTC.

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise AHG and experiment studies. - Report on project status to JVET reflector. - Provide a report to the next meeting on project coordination status. - Supervise processing and delivery of output documents | J.-R. Ohm (chair), G. J. Sullivan (vice-chair) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize draft text outputs of the meeting (JVET-AB1008, JVET-AB2002, JVET-AB2006, and JVET-AB2027). - Collect reports of errata for the VVC, VSEI, HEVC, AVC, CICP, the codepoint usage TR specification and the published HDR-related technical reports and produce the JVET-AB1004 errata output collection. - Coordinate with the test model software development AhG to address issues relating to mismatches between software and text. - Collect and consider errata reports on the texts. | B. Bross, C. Rosewarne (co-chairs), F. Bossen, J. Boyce, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test models (VTM, HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, 360Lib, and HDRTools) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Enable software support for recently standardized additional SEI messages. - Discuss and make recommendations on the software development process. - Propose improvements to the guideline document for developments of the test model software. - Perform comparative tests of test model behaviour using common test conditions. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Update the CTC document JVET-AB2010, and prepare drafts of merged CTC documents for HM and VTM, as applicable. | F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Consider plans for additional verification testing of VVC capability, particularly target establishing a test plan for VVC scalability features by the next meeting. - Maintain the video sequence test material database for testing the VVC and HEVC standards and potential future extensions, as well as exploration activities. - Study coding performance and characteristics of available and proposed video test material. - Identify and recommend appropriate test material for testing the VVC standard and potential future extensions, as well as exploration activities. - Identify and characterize missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Maintain and update the directory structure for the test sequence repository, as necessary. - Collect information about test sequences that have been made available by other organizations. - Prepare and conduct remote expert viewing for purposes of subjective quality evaluation. - Coordinate with AG 5 in studying and developing further methods of subjective quality evaluation, e.g. based on crowd sourcing. - Prepare availability of viewing equipment and facilities arrangements for future meetings. | V. Baroncini, T. Suzuki, M. Wien (co-chairs), S. Liu, S. Puri, A. Segall, P. Topiwala, S. Wenger, J. Xu, Y. Ye (vice-chairs) | Y (tel., 2 weeks notice) |
| Conformance testing (AHG5) (jvet@lists.rwth-aachen.de) - Study the draft conformance testing for operation range extensions JVET-Y2026 and the additional conformance bitstreams for VVC multilayer configurations JVET-AB2028, and investigate the need for improvements. - Study the requirements of VVC, HEVC, and AVC conformance testing to ensure interoperability. - Maintain and update the conformance bitstream database, and contribute to report problems, and suggest actions to resolve these. - Study additional testing methodologies to fulfil the needs for VVC conformance testing. | D. Rusanovskyy, I. Moccagatta (co-chairs), F. Bossen, K. Kawamura, T. Ikai, H.-J. Jhu, K. Sühring, Y. Yu (vice-chairs) | N |
| ECM software development (AHG6) (jvet@lists.rwth-aachen.de) - Coordinate development of the ECM software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Prepare and deliver ECM-7.0 software version and the reference configuration encodings according to the ECMcommon test conditions. - Investigate encoder speedup and other encoder software optimization. - Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate. | V. Seregin (chair), J. Chen, F. Le Léannec, K. Zhang (vice-chairs) | Y (tel., 2 weeks notice) |
| Low latency and constrained complexity (AHG7) (jvet@lists.rwth-aachen.de) - Identify additional application scenarios and their requirements for low latency and constrained complexity, taking into account aspects of real-time encoding and decoding. - Discuss requirements of already identified scenario such as cloud gaming, game casting, video conferencing, video surveillance and remote control of systems. - Evaluate and refine new CTC for low latency and constrained complexity application scenarios, including the option of enabling GDR according to requirements of gaming applications. - Investigate a set of tools that provide a reasonable tradeoff regarding complexity vs. compression, as well as latency constraints. - Conduct tests with ECM and VTM to determine the impact of discussed configurations on coding efficiency and run time. - Refine the set of test sequences for gaming applications, and if necessary collect new test materials that are suitable for the intended application domains, and establish an applicable dataset in coordination with AHG4. - Coordinate with AHG3 and AHG12 to discuss and recommend configuration(s) applicable to ECM and VTM, taking into account complementarity with existing CTCs. | A. Duenas, T. Poirier, S. Liu (co-chairs), L. Wang, J. Xu (vice-chairs) | N |
| High bit depth, high bit rate, and high frame rate coding (AHG8) (jvet@lists.rwth-aachen.de) - Study the benefits and characteristics of VVC coding tools for high bit depth, high bit rate, and high frame rate coding. - Produce test results with JVET-AA2018 testing conditions for high bit depth and high bit rate coding in coordination with AHG3. - Contribute to the development of software and conformance testing for operation range extensions in coordination with AHG3 and AHG5. | A. Browne, T. Ikai (co-chairs), D. Rusanovskyy, X. Xiu, Y. Yu (vice-chairs) | N |
| SEI message studies (AHG9) (jvet@lists.rwth-aachen.de) - Study the SEI messages in VSEI, VVC, HEVC and AVC. - Collect software and showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing. - Identify potential needs for additional SEI messages, including the study of SEI messages defined in HEVC and AVC for potential use in the VVC context. - Coordinate with AHG3 for software support of SEI messages. | S. McCarthy, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, C. Fogg, P. de Lagrange, G. J. Sullivan, A. Tourapis, S. Wenger (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as tool adaptation and configuration, and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of non-normative techniques of pre processing for the benefit of encoder optimization. - Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality. - Study optimized encoding for reference picture resampling and scalability modes in VTM. - Consider neural network-based encoding optimization technologies for video coding standards. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding. - Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality. - Study the potential of defining default or alternate software configuration settings and test conditions optimized for either subjective quality, or higher objective quality, and coordinate such efforts with AHG3 and AHG6. - Study the effect of varying configuration parameters depending on temporal layer, such as those related to deblocking, partitioning, chroma QP. | P. de Lagrange, A. Duenas, R. Sjöberg, A. Tourapis (AHG chairs) | N |
| Neural network-based video coding (AHG11) (jvet@lists.rwth-aachen.de) - Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools and novel architectures. - Refine the test conditions for NN-based video coding in JVET-AB2016. Generate and distribute anchor encoding, and develop supporting software as needed. - Study the impact of training (including the impact of loss functions) on the performance of candidate technologies, and identify suitable material for testing and training. - Analyse complexity characteristics, perform complexity analysis, and develop complexity reductions of candidate technology. - Finalize and discuss the EE on neural network-based video coding. - Coordinate with other relevant groups, including SC29/AG5 on the evaluation and assessment of visual quality and AHG12 on the interaction with ECM coding tools. In the case of conducting subjective viewing, provide support to AHG4 for the timely availability of test materials and test subjects. - Coordinate with AHG14 on items related to NNVC software development. | E. Alshina, S. Liu, A. Segall (co-chairs), F. Galpin, J. Li, T. Shao, H. Wang, Z. Wang, M. Wien, P. Wu (vice-chairs) | Y (tel., 2 weeks notice) |
| Enhanced compression beyond VVC capability (AHG12) (jvet@lists.rwth-aachen.de) - Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC. - Discuss and propose refinements to the ECM7 algorithm description JVET-AB2025. - Study the performance and complexity tradeoff of these video coding tools. - Coordinate with AHG6 on ECM software development. - Support AHG6 in generating anchors according to the test conditions in JVET-Y2017. - Analyse the results of exploration experiments described in JVET-AB2024 in coordination with the EE coordinators. - Coordinate with AHG11 to study the interaction with neural network-based coding tools. | M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, G. Li, X. Li, K. Naser, H. Yang (vice-chairs) | Y (tel., 2 weeks notice) |
| Film grain technologies (AHG13) (jvet@lists.rwth-aachen.de) - Study the benefits and characteristics of film grain technologies, including autoregressive and frequency-filtering technologies. - Discuss the JVET-AB2020 draft of the Technical Report on Film grain synthesis technology for video applications and suggest improvements as necessary. - Study alternative film grain models and their associated documentation. - Study preprocessing and encoder technologies for determining values for FGC (Film Grain Characteristics) SEI message syntax elements. - Identify potential need for additional film grain technology and signalling, if needed. - Coordinate development of film grain technology software and configuration files. - Coordinate with AG 5 on finalizing the draft plan for subjective quality testing of the FGC SEI message JVET-AB2022, and conduct preparations for such testing. - Coordinate with AHG3 for software support of the FGC SEI message. | W. Husak, M. Radosavljević, W. Wan (co-chairs), D. Grois, Y. He, P. de Lagrange, A. Segall, A. Tourapis (vice-chairs) | Y (tel., 2 weeks notice) |
| NNVC software development (AHG14) (jvet@lists.rwth-aachen.de) - Coordinate development of the NNVC software and associated configuration files. - Prepare and deliver NNVC-3.0 software version and the reference configuration encodings according to the NNVC common test conditions JVET-AB2016. Prepare and release anchor data for all configurations of the software. - Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed. - Coordinate with NNVC algorithm and software description (JVET-AB2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate. - Coordinate with AHG11 on items related to NNVC activities. | S. Eadie, F. Galpin, Y. Li, L. Wang, Z. Xie (AHG chairs) |  |
| Optimization of encoders and receiving systems for machine analysis of coded video content (AHG15) (jvet@lists.rwth-aachen.de) - Solicit and study non-normative encoder and receiving systems technologies that enhance performance of machine analysis tasks on coded video content. - Identify and collect test materials that are suitable to be used by JVET for machine analysis tasks. - Define test conditions and generate anchors. - Define evaluation framework, procedure and methodologies. - Coordinate software development and experiments. - Evaluate proposed technologies and their suitability for machine analysis applications. - Prepare a draft technical report on optimization of encoders and receiving systems for machine analysis of coded video content. - Coordinate with MPEG VCM AHG on common interests and activities such as test materials, studying characteristics and requirements of targeted machine analysis tasks, etc. | C. Hollmann, S. Liu, S. Wang, M. Zhou (AHG chairs) |  |

It was confirmed that the rules which can be found in document ISO/IEC JTC 1/‌SC 29/‌AG 2 N 046 “Ad hoc group rules for MPEG AGs and WGs” (available at https://www.mpegstandards.org/adhoc/), are consistent with the operation mode of JVET AHGs. It is pointed out that JVET does not maintain separate AHG reflectors, such that any JVET member is implicitly a member of any AHG. This shall be mentioned in the related WG Recommendations. The list above was also issued as a separate WG 5 document (ISO/IEC JTC 1/‌SC 29/‌WG 5 N 170) in order to make it easy to reference.

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as a WG 5 output document, a separate version under the WG 5 document header should be generated. This version should be sent to GJS and JRO for upload.

The list of JVET ad hoc groups was also issued as a WG 5 output document WG 5 N 170, as noted in section 9.

JVET-AB1000 Meeting Report of the 28th JVET Meeting [J.-R. Ohm] [WG 5 N 156] (2022-11-25)

Initial versions of the meeting notes (d0 … d9) were made available on a daily basis during the meeting.

Remains valid – not updated: JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen (software coordinators)]

Remains valid – not updated: JVET-Y1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 16 [C. Rosewarne (primary editor), K. Sharman, R. Sjöberg, G. J. Sullivan (co-editors)] [WG 5 N 103]

Remains valid – not updated: JVET-Z1003 Coding-independent code points for video signal type identification (Draft 1 of 3rd edition) [WG 5 DIS N 162] [G. J. Sullivan, A. Tourapis] (2022-11-18)

The drafted revision was issued without changes as WG 5 N 162 Text of ISO/IEC DIS 23091-2:202x Coding-independent code points – Part 2: Video (3rd edition), with due date 2022-11-09. As no changes were suggested in the CD ballot, a DoCR was not issued.

JVET-AB1004 Errata report items for VVC, VSEI, HEVC, AVC, Video CICP, and CP usage TR [B. Bross, I. Moccagatta, C. Rosewarne, G. J. Sullivan, Y. Syed, Y.-K. Wang] (2022-12-17, near next meeting)

New aspects included the issues pointed out in JVET-AB0223 on the Timing / DU information SEI message in HEVC and VVC. Furthermore, the change discussed in context of JVET-AB0267 (cropped width and height) should be included.

Remains valid – not updated: JVET-Z1005 New levels for HEVC (Draft 3) [T. Suzuki, A. Tourapis, Y.-K. Wang] (2022-05-06)

Remains valid – not updated JVET-T1006 Annotated regions and shutter interval information SEI messages for AVC (Draft 2) [J. Boyce, S. McCarthy, Y.-K. Wang]

(This can be removed after publication of the new edition of ISO/IEC 14496-10.)

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye] [WG 11 N 15778]

JVET-AB1008 Additional colour type identifiers for AVC, HEVC and Video CICP (Draft 2) [G. J. Sullivan, W. Husak, A. Tourapis] (2022-11-25) [WG 5 Preliminary WD N163]

This includes identifiers for ITP-PQ-C2 colour representation proposed in JVET-AB0172. It is planned to be assigned the new codepoint number 17, to be integrated with the codepoint tables currently in JVET-Z1008 for AVC and HEVC (currently for YCoCg-R), and another table for Video CICP (only ITP-PQ-C2).

Remains valid – not updated: JCTVC-X1009 Common Test Conditions for SHVC [V. Seregin, Y. He]

Remains valid – not updated JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan]

Remains valid – not updated JVET-AA1011 HEVC multiview profiles supporting extended bit depth (draft 1) [A. Tourapis, W. Husak] [WG 5 preliminary WD N 143] (2022-08-26)

This output arises from JVET-AA0239.

Proponents of the new profiles were requested to develop conformance bitstreams, and contribute to HTM software changes if necessary.

JVET-AB1012 Overview of IT systems used in JVET [J.-R. Ohm, I. Moccagatta, K. Sühring, M. Wien] (2022-12-23)

No output: JVET-T1013

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye] [WG 11 N 16049]

Remains valid for HM – not updated: JCTVC-Z1015 Common Test Conditions for Screen Content Coding [H. Yu, R. Cohen, K. Rapaka, J. Xu]

No output: JVET-X1016 through JVET-X1019

Remains valid – not updated JVET-AA1100 Common Test Conditions for HM Video Coding Experiments [K. Sühring, K. Sharman] (2022-08-19)

This specifies only the CTC for non-4:2:0 colour formats. The corresponding document for VVC is JVET-T2013, with no unification yet.

Remains valid – not updated: JVET-T2001 Versatile Video Coding Draft 10 [B. Bross, J. Chen, S. Liu, Y.-K. Wang]

JVET-AB2002 Algorithm description for Versatile Video Coding and Test Model 18 (VTM 18) [A. Browne, Y. Ye, S. Kim] [WG 5 N 161] (2022-01-06, near next meeting)

Updates include descriptions of new SEI message processing, in particular for the SEI processing order SEI), Green metadata, and potentially some other SEI messages as listed under JVET-AB0269. Further non-normative modifications adopted for VTM are the contributions on RPR encoder, and upsanpling filters (JVET-AB0080 and JVET-AB0081).

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-N1003 Guidelines for VVC reference software development [K. Sühring]

If ever a new version is made, this should become 2003.

Remains valid – not updated: JVET-T2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 12) [Y. Ye, J. Boyce]

Remains valid – not updated JVET-AA2005 New level and systems-related supplemental enhancement information for VVC (Draft 3) [B. Bross, E. François, A. Tourapis, Y.-K. Wang] [WG 5 DAM 1 N 145] (2022-07-29)

JVET-AB2006 Additional SEI messages for VSEI (Draft 3) [S. McCarthy, T. Chujoh, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang] [WG 5 DIS N 158] (2022-11-09)

A DoCR for ballot responses on CDAM 1 (WG 5 N 157) was reviewed Thursday 27 October at 0920.

The aspect of having several NNPFC messages activated in one picture was discussed (comment #056). It was agreed to allow this, as it is the choice of the decoding device to operate those which are fitting the purpose of the local output or its capabilities. Examples are SEI messages with different purposes, or SEI messages with the same purpose and different degrees of complexity.

It was agreed to convert this into the FDIS of a new edition after DAM ballot (comment #70). Due to short editing period, it is not practical to do this at the current stage.

The elements to be modified are indicated with yellow highlights as “decision” or “agreed” in sections 6.1 and 6.2.

Remains valid – not updated: JVET-S2007 Versatile supplemental enhancement information messages for coded video bitstreams Draft 5 [J. Boyce, V. Drugeon, G. J. Sullivan, Y.-K. Wang]

Remains valid – not updated: JVET-X2008 Conformance testing for versatile video coding (Draft 7) [J. Boyce, F. Bossen, K. Kawamura, I. Moccagatta, W. Wan]

Remains valid – not updated: JVET-Y2009 Reference software for versatile video coding (Draft 3) [F. Bossen, K. Sühring, X. Li]

JVET-AB2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video [F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring] (2022-11-11)

Remains valid – not updated: JVET-Z2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy] (2022-05-13)

Remains valid – not updated: JVET-U2012 JVET common test conditions and evaluation procedures for 360° video [Y. He, J. Boyce, K. Choi, J.-L. Lin] (2021-03-31)

Remains valid – not updated: JVET-T2013 VTM common test conditions and software reference configurations for non-4:2:0 colour formats [Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu]

Remains valid – not updated: JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding [T.-C. Ma, A. Nalci, T. Nguyen]

Remains valid – not updated: JVET-Q2015 JVET functionality confirmation test conditions for reference picture resampling [J. Luo, V. Seregin]

JVET-AB2016 Common Test Conditions and evaluation procedures for neural network-based video coding technology [E. Alshina, R.-L. Liao, S. Liu, A. Segall] (2022-11-11)

Update in the methodology of kMAC/pixel computation (recommendation from JVET-AB0023).

Remains valid – not updated: JVET-Y2017 Common Test Conditions and evaluation procedures for enhanced compression tool testing [M. Karczewicz and Y. Ye]

Only a change of config file for JVET-AA0098 was noted (and probably some other CTC related decision noted elsewhere). It was further decided to enable palette mode for classes F and TGM in the CTC both for VTM anchor and ECM. As these affected only the config files rather than the document, no need to produce a revised document was identified.

Remains valid – not updated: JVET-AA2018 Common test conditions for high bit depth and high bit rate video coding [A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, Y. Yu] (2022-08-19)

JVET-AB2019 Description of algorithms and software in neural network-based video coding (NNVC) [Y. Li, H. Wang, L. Wang, F. Galpin, J. Ström] [WG 5 N 165] (2022-12-02)

Developed from JVET-AB0183 (see notes there, and under EE1 summary JVET-AB0023 about what else to added).

JVET-AB2020 Film grain synthesis technology for video applications (Draft 3) [D. Grois, Y. He, W. Husak, P. de Lagrange, A. Norkin, M. Radosavljević, A. Tourapis, W. Wan] [WG 5 N 159] (2022-12-16)

Developed from JVET-AB0042.

JVET-AB2021 Draft verification test plan for VVC multilayer coding [S. Iwamura, P. de Lagrange, M. Wien] (2022-12-16)

A draft of this document was presented in JVET at 1610 on Thursday 27 Oct. This document had been developed in the joint BoG with AG 5 (meeting at 1600-1730 on Wednesday 26 Oct.); no separate BoG report was produced, as the presented draft document reflected the results of the BoG.

JVET-AB2022 Draft plan for subjective quality testing of FGC SEI message [P. de Lagrange, W. Husak, M. Radosavljević, M. Wien] (2022-12-16)

A draft of this document was presented in JVET at 1630 on Thursday 27 Oct. This document had been developed in the joint BoG with AG 5 (meeting at 1600-1730 on Wednesday 26 Oct.), no separate BoG report was produced, as the presented draft document reflected the results of the BoG.

It was asked if also tests are planned using HM. This aspect is left open for now; it might be done if it is demonstrated that such an additional comparison would have additional benefit in terms of what the test is intending to study. Is it about showing the advantages of the SEI message (which could be used for both VVC and HEVC), or could it also demonstrate that VVC has more advantage than HEVC if the SEI message is used.

It was commented that testing the quality of the synthesized film grain is very important (goal 2 of the plan).

(It was mentioned that a merge request on the SEI message implementation on top of the HM had already been made, and it was waiting for release of a new version.)

JVET-AB2023 Exploration Experiment on Neural Network-based Video Coding (EE1) [E. Alshina, F. Galpin, Y. Li, M. Santamaria, J. Ström, H. Wang, L. Wang, Z. Xie] [WG 5 N 164] (2022-11-11)

An initial draft of this document was reviewed and approved at 0905-0945 on Friday 28 October.

Categories are loop filters, post filters, superresolution upsampling, intra coding (AE integerization), and intra prediction.

It was suggested that it would be interesting to perform subjective quality investigation with some of the super-resolution proposals in a future meeting (provided that close enough rate/quality matching with anchor is achieved).

JVET-AB2024 Exploration Experiment on Enhanced Compression beyond VVC capability (EE2) [V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang] [WG 5 N 166] (2022-11-25)

An initial draft of this document was reviewed and approved at 0945-1015 on Friday 28 October. JVET-AB0150 was added as test 2.7b. Clarification is necessary about the training set to be used in test 4.2b (referred to as “LFNST set”).

Categories are intra prediction, inter prediction, screen content coding, transforms, and in-loop filters.

JVET-AB2025 Algorithm description of Enhanced Compression Model 7 (ECM 7) [M. Coban, F. Le Léannec, R.-L. Liao, K. Naser, J. Ström, L. Zhang] [WG 5 N 167] (2022-12-31)

New elements from notes elsewhere in this report:

- Decision: Adopt JVET-AC0XXX test X.

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-Y2026 Conformance testing for VVC operation range extensions (Draft 3) [WG 5 DAM N 110] [D. Rusanovskyy, T. Hashimoto, H.-J. Jhu, I. Moccagatta, Y. Yu]

Editorial improvements are recorded in JVET-AA0109, no need for a new output at the current meeting was identified.

JVET-AB2027 SEI processing order SEI message in VVC (draft 2) [S. McCarthy, M. Hannuksela, Y.-K. Wang] [WG 5 preliminary WD 2 N 160] (2022-11-18)

New version (see discussion under JVET-AB0051 and JVET-AB0069)

JVET-AB2028 Additional conformance bitstreams for VVC multilayer configurations [S. Iwamura, I. Moccagatta] (2022-12-31)

Developed from JVET-AB0085.

JVET-AB2029 Visual quality comparison of ECM/VTM encoding [V. Baroncini, J.-R. Ohm, M. Wien] [AG 5 N75] (2022-12-09)

An initial version was reviewed on Friday 28 October at 1030.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines (assuming face-to-face meetings):

- Meeting under ITU-T SG16 auspices when it meets (ordinarily starting meetings on the Wednesday of the first week and closing it on the Wednesday of the second week of the SG16 meeting – a total of 8 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/‌SC 29 auspices when its MPEG WGs meet (ordinarily starting meetings on the Friday prior to the main week of such meetings and closing it on the same day as other MPEG WGs – a total of 8 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined. In cases of online meetings, no sessions should be held on weekend days. This may imply an earlier starting date as well.

Some specific future meeting plans (to be confirmed) were established as follows:

- Wed. 11 – Fri. 13 and Mon. 16 – Fri. 20 January 2023, 29th meeting under ISO/IEC JTC 1/‌SC 29 auspices, to be held as teleconference meeting.

- During Fri. 21 – Fri. 28 April 2023, 30th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Antalya, TR

- During Fri. 14 – Fri. 21 July 2023, 31st meeting under ITU-T SG16 auspices in Geneva, CH (date to be confirmed)

- During October 2023, 32nd meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

- During January 2024, 33rd meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

- During April 2024, 34th meeting under ITU-T SG16 auspices, date and location t.b.d.

- During Fri. 12 – Fri. 19 July 2024, 35th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Sapporo, JP

- During October 2024, 36th meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

The agreed document deadline for the 29th JVET meeting was planned to be Wednesday 4 January 2023.

Kenzler Conference Management and the entire team were thanked for the excellent hosting and organization of the 28th meeting of the JVET. Qualcomm was thanked for financially supporting the social event. Vittorio Baroncini and Mathias Wien were thanked for conducting subjective quality tests related to the ECM. VABTech was thanked for providing resources to run tests with non-expert viewers. HHI was thanked for providing equipment used in the experts viewing in Mainz. The following companies were thanked for preparing encoded bitstreams: Alibaba, Bytedance, Ericsson, InterDigital, Qualcomm, and Xiaomi. Experts who volunteered to participate in the viewing were also thanked.

It was suggested that in the next meeting, perspectives should be discussed for the ongoing JVET explorations, in terms of potentially developing standardization projects and realistic timelines. In particular, from the current status of the exploration on enhanced compression capability beyond current VVC profiles, evidence has been shown about visual quality improvement. Market needs and related requirements could be considered at the level of the parent bodies.

The 28th JVET meeting was closed at approximately 1645 hours on Friday 28 October 2022.

# Annex A to JVET report:
List of documents

# Annex B1 to JVET report:
List of meeting participants attending in person

The participants who were personally present at the meeting site of the twenty-ninth meeting of the JVET, according to an attendance sheet circulated during the meeting sessions (approximately 0 people in total), were as follows (note: Annex kept for future use):

# Annex B2 to JVET report:
List of meeting participants attending remotely

The remote participants of the twenty-eighth meeting of the JVET, according to the participation records from the Zoom teleconferencing tool used for the meeting sessions (approximately XXX people in total, not including those who had attended the meeting in person at least part-time (see annex B1), and not including those who attended only the joint sessions with other groups), were as follows:

# Annex C to JVET report:
Recommendations of the 10th meeting of
ISO/IEC JTC 1/SC 29/WG 5 MPEG Joint Video Coding Team(s) with ITU-T SG 16

ISO/IEC JTC 1/SC 29/WG 5 N XXX