| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0041-v1 |
| --- | --- |

| Title: | [AHG4] On visual volumetric video-based coding (V3C) testing conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sebastian Schwarz, Miska M. Hannuksela Patrice Rondao Alface Lukasz Kondrad Lauri Ilola Nokia Technologies Munich, Germany | Email: | {sebastian.schwarz, miska.hannuksela, patrice.rondao_alface, lukasz.kondrad, lauri.ilola}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to study coding tools for video content utilised in visual volumetric video-based coding (V3C). It includes an overview of V3C content characteristics, potential test content, and testing procedures.

# Introduction

AHG4 has the mandate to “identify and characterize missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material.” As it stands, there is already a variety of common testing conditions in JVET, e.g., for HDR/WCG, 360-degree video, lossless, etc. Nevertheless, there has been little consideration of video content representing visual volumetric assets.

Efficient coding of dynamic visual volumetric assets is a prerequisite for many AR/XR/VR applications. ISO/IEC has released two international standards addressing this need, ISO\IEC 23090-5 [1] also known as Video-based Point Cloud Compression (V-PCC), and ISO\IEC 23090-12 [2] also known as MPEG Immersive Video (MIV), establishing the Visual Volumetric Video-based Coding (V3C) family of standards. Both of these standards follow the same V3C framework, where the visual volumetric asset is segmented into coherent subsets that are projected to 2D patches, which are packed into 2D video frames and are accompanied by metadata information. Traditional 2D video codecs are used to compress 2D video frames and provide the bulk of the compression efficiency. Utilizing traditional 2D video codecs allows for a broad device deployment and efficient carriage over existing video delivery infrastructure. A third standard utilizing V3C framework, addressing coding of dynamic mesh models, is currently under development and expected to be finalized in 2024 [3]. Similarly, as with traditional 2D video codec, system level aspects of V3C standards have also been developed, e.g., carriage of V3C in ISOBMFF, DASH, and MMT is standardized in ISO/IEC 23090-10 [4], while real-time delivery aspect using RTP protocol are under development in IETF [5].

The development of V-PCC and MIV coding tools, i.e., the reprojection algorithm, packing methods, and related metadata, has been mostly carried independently from 2D video coding tools and specific 2D video codec. The intention was to emphasise the flexibility of the V3C framework by its ability to adopt to the latest and most efficient video coding technology. For example, comparisons between V-PCC coding using HM vs. VVenC have shown increases of coding efficiency of around 20-25 % [6].

However, given the unique characteristics of V3C video content, we are of the opinion that there is room for significant coding gains, given the right tools. With the constantly rising interest in visual volumetric content, we see the opportunity to develop a unique selling point for the current and upcoming generation of JVET codecs. Therefore, we propose to study coding tools targeted at V3C video content. To facilitate the study, we provide in this contribution an overview of video content characteristics generated by V3C framework, a selection of potential test content, and proposals for an initial testing framework.

# Overview of V3C video content characteristics

Detailed description of the various V3C standards can be found for example in [7, 8, 9]. To keep this contribution to a reasonable length, we will only discuss the video content characteristics for V-PCC and MIV here.

## Video-based point cloud compression (V-PCC)

In the V-PCC standard, a dynamic 3D point cloud model is segmented into coherent groups of 3D points, so called “patches” based on point surface normals in relation to a minimum of six projection planes. These patches are then packed into 2D video frames. Typically, there are three types of videos associated with a single point cloud sequence: the geometry video, the occupancy video, and at least one attribute video. An example of this process is shown in Fig. 1 below.

Figure 1: Example of the V-PCC patch generation.

Since the patches have irregular shapes, there are empty areas in the packed video frame, not associated with any point cloud information. To improve coding efficiency, these areas are padded in the geometry and attribute videos. The occupancy video carries the information which pixel coordinates carry point cloud information for 3D reconstruction. An accompanying V3C atlas bit stream provides the required additional metadata to reconstruct 3D points from the 2D video.

This atlas bitstreams carries for each patch a X, Y, Z location in 3D space, as well as its 2D location and size in the 2D video frame. At the decoder this information is used to re-project 2D planes into 3D space, as shown in Fig. 2 a).

The occupancy video is typically compressed using lossless video coding settings. To reduce bitrate requirements, the video resolution can be downscaled in relation to the nominal resolution of the geometry and attribute videos. The occupancy information is carried in the luma plane of an 8bit YUV420 video, with both chroma planes empty (i.e., coded with a constant color). A pixel value of 1 indicated an “occupied” state, a pixel value of 0 indicates an “empty” state. This information is used to reconstruct the irregular shape of the patches in the 3D reprojection, as shown in Fig. 2 b).

The geometry video represents the shift of individual 3D points in relation to their patch, as shown in Fig. 2 c) and is typically carried as 8 or 10 Bit YUV420 video, with the chroma plains left empty. The attribute video represents attribute values of each point, e.g., texture color, as shown in Fig. 2 d), and is typically carried as 8 Bit YUV420 video.

a)							b)	  		         c)			    d)

Figure 2. Point cloud reconstruction process: a – atlas; b – atlas and occupancy map; c – atlas, occupancy map and geometry, d – atlas, occupancy map, geometry, and attribute

Obviously, accurate decoding of the occupancy maps is essential, as incorrect occupancy values would lead to the reconstruction of padded (= invalid) points from the geometry video. Such incorrect points would manifest themselves as “flying pixel” in the 3D reconstruction. Therefore, the occupancy map is typically coded lossless. However, the V3C standard does not require lossless compression for the occupancy signal.

Regarding compression of the geometry and attribute videos, geometry video is typically coded at a higher quality than texture video, to avoid disturbing geometry artefacts. The exact trade-off between geometry compression and attribute compression is, however, not easily decided.

Below are two details of texture and geometry frames for the sequence Longdress.

Figure 3. Packed and padded V3C patches in geometry (left) and texture attribute (right)video.

In terms of video resolution and framerate, V-PCC videos are typically in the range of 1-2 Million luma samples at 30 fps, leading to roughly 1 million 3D points per frame at 30 frames per second.

In some cases, it might be beneficial to provide several layers for each patch, so called maps. In such cases, geometry and attribute videos may use more than one 2D frame for every frame of 3D content. However, for simplicity we will discuss such concepts at a later stage.

The V-PCC encoder tries to achieve temporal stability of the packed patches over each GOP to improve temporal prediction. However, as shown in Fig. 4, there are limits to its performance.

| POC 0 | POC 8 | POC 16 | POC 20 | POC 31 |
| --- | --- | --- | --- | --- |

Figure 4. Example of temporal patch packing for texture video of sequence Longdress (GOP size 32).

## MPEG Immersive video (MIV)

The MIV standard targets virtual and extended reality 3DoF+ use cases, in which a user has a limited area of movement to explore an immersive scene.

A typical MIV input format is a set of (360-degree) video sequences, called source views, captured by a group of cameras having arbitrary poses. Each source view represents a projection of a part of a volumetric scene onto the camera projection plane. Each source view is represented by a geometry (depth) information video sequence and the projected attribute (texture) video sequence.

Rather than compressing each source view separately, an MIV encoder typically first selects most relevant (i.e., covering most of the scene) source views, so called basic views and next, from the remaining source views, the MIV encoder takes additional information that is not available in the basic views. The basic views and the additional information are represented as 2D patches, similarly as in the V-PCC standard. 2D patches are packed in 2D frames. As the combination of several base views together with a few hundred patches may exceed typical video resolution restrictions, it is common to have more than one 2D frame per input time instance. An example of the MIV attribute 2D frames is given in Fig. 5 below, where 4 patches representing base views are packed in the first 2D frame and the patches are packed in a second 2D frame.

Figure 5:  Example illustration of two MIV 2D frames of the same time instance representing attribute atlases.

MIV geometry carry the respective geometry information, like V-PCC, as shown in Fig. 6 below. However, one key difference to V-PCC is that no additional occupancy map data is carried. Occupancy is signalled as part as the geometry. All values below a certain threshold are considered unoccupied. The MIV Extended profile allows for external occupancy maps. It is also possible to transmit geometry at a lower resolution than the attribute video. However, for simplicity we think it is better to discuss both these concepts at a later stage.

In MIV both texture and geometry video are typically coded as 10 Bit YUV420 videos, with empty chroma planes in case of the geometry video. In terms of video resolution and framerate, each MIV atlas is typically in the range of 8-9 Million luma samples at 30 fps, for a total pixel rate of over 1Gpixel/s.

Regarding compression of the geometry and attribute videos, geometry video is typically coded at a higher quality than texture video, especially as there is no external occupancy information. The exact trade-off between geometry compression and attribute compression is however not easily decided.

Regarding patch packing, MIV typically achieves much better temporal stability of the packed patches over each GOP to improve temporal prediction compared to V-PCC as shown below in Fig. 7.

Figure 6: Example illustration of two MIV geometry.

| POC 0 | POC 8 | POC 16 | POC 20 | POC 31 |
| --- | --- | --- | --- | --- |

Figure 7. Example of temporal patch packing for second texture atlas of sequence N - Chess (GOP size 32).

# Proposed testing conditions for V3C content

Both standards, V-PCC and MIV have respective Common Testing Conditions laying out the rules for reproduceable experiments [10, 11]. However, both CTCs require elaborate processes using various software and evaluation metrics. For the tasks of evaluating ECM or VVC encoding performance on V3C, we think a much more streamlined approach could be used.

Figure 8. V3C video content generation as preparatory step before the evaluation.

Instead of using the standard specific inputs, e.g., point clouds or source views, as starting point, the evaluation of V3C content encoding within JVET should start from the video files generated from the respective V3C encoders. In a preprocessing step, as illustrated in Fig. 8, a selection of test content is prepared using latest V-PCC/MIV encoders. This step can be carried out for example by experts from this AHG. The resulting video streams and atlas bit streams are then made available for evaluation of video encoding. This preparatory step eliminates the need for JVET experts to compile and run these encoders themselves and keeps the focus on the video encoding evaluation.

Fig. 9 then illustrates then the proposed evaluation framework. As video coding performance is the focus, the evaluation of coding performance should also be carried out only on the video streams themselves, as marked with “Evaluation A” in yellow in Fig. 9. We are aware that the final output of MIV and V-PCC decoders are not these video streams. Thus, new metrics are required to derive reliable correlation between distortions on the 2D videos against distortion on the 3D reconstruction. We introduce one potential metric for such an evaluation in [12], but until such metrics have been agreed on, additional evaluation may include the full V3C reconstruction, marked with “Evaluation B” in green in Fig. 9. To support such evaluation, we can provide modified V-PCC and MIV decoder which take the respective inputs and calculate various metrics directly, again without requiring JVET experts to compile and run full V3C processing chains.

Figure 9. Proposal of V3C video coding evaluation framework.

By providing initial versions for V3C videos and simplifying the evaluation chain, we hope to keep the entry barrier low to attract many new experts to this topic, as well as maintain the focus strictly on the 2D video coding performance evaluation for V3C content. The following two sub-sections propose some initial settings for creating these videos and accompanying atlas bitstreams.

## Video-based point cloud compression (V-PCC) conditions

For generating the V-PCC texture, geometry, and occupancy videos, we propose to use the latest test model TMCv19 [13] and common test conditions [10]. We limit the evaluation to lossy geometry and lossy attribute coding, and only for random access configuration (GOP size 32).

Furthermore, we propose to use full resolution occupancy maps, and only a single layer (map per patch), by setting the following configuration parameters:

occupanyPrecision: 	1

layerCountMinus1:	0

## MPEG Immersive video (MIV) conditions

For generating the VMIV texture and geometry videos, we propose to use the latest test model TMIV14 [14] and common test conditions [11]. We limit the evaluation to “MIV anchor” conditions, where some source views are packed completely while only taking only patches from other source views. Video encoding is performed in random access configuration (GOP size 32).

Furthermore, we propose to use full resolution geometry atlases maps, by setting the following configuration parameter:

geometryScaleEnableFlag: 	false

## VVC encoding testing conditions

For the initial V3C anchor encodings, we used VTM18.1 [15] with the encoder_randomaccess_vtm.cfg configuration. QP values can be derived from the Excel sheet submitted together with this contribution.

V-PCC occupancy videos where coded in two ways, either lossless (/lossless/lossless.cfg) or at QP8.

# Selection of potential V3C testing content

Both V-PCC and MIV Common Testing Conditions as well as recently carried out verification tests provide us already with a large variety of potential test content. In this selection we would like to highlight some of the sequences we consider most promising for further studies in JVET. Example viewports rendered for each sequence are made available at [16]. We believe that this selection provides a good variety of realistic test content for V3C use cases. Nonetheless, this selection should not be considered a final proposal, as everything is up for discussions.

## Video-based point cloud compression (V-PCC)

We propose the following point cloud sequences as initial selection of test sequences for V3C content video coding experiments.

| basketball_player_vox11 | football_vox10 | levi_vox10 | longdress_vox10 |
| --- | --- | --- | --- |

Figure 10. Examples of potential V-PCC test content.

Table 1: Summary of V-PCC test content characteristics.

|  |  |  |  | atlas |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| sequence | points/frame | framerate | frames | number | width | height |
| basketball_player | ~ 2,800,000 | 30 fps | 64 | 1 | 2560 | 3536 |
| football | ~ 600,000 | 30 fps | 150 | 1 | 1280 | 1280 |
| levi | ~ 600,000 | 30 fps | 150 | 1 | 1280 | 1680 |
| longdress | ~ 800,000 | 30 fps | 150 | 1 | 1280 | 1584 |

Tab. 1 summarizes the characteristics of the different V-PCC test content.

Sequences basketball_player_vox11 and longdress_vox10 are part of the V-PCC CTC. Sequences football and levi were part of the V-PCC verification tests [17].

## MPEG Immersive video (MIV)

We propose the following MIV sequences as initial selection of test sequences for V3C content video coding experiments.

| H - BabyUnicorn | N - Chess |
| --- | --- |
| O - Fan | S - CBAbasketball |
| Y - Breakfast | Z - Barn |

Figure 11. Examples of potential MIV test content.

Table 2: Summary of MIV test content characteristics.

|  |  |  | atlas |  |  |
| --- | --- | --- | --- | --- | --- |
| sequence | framerate | frames | number | framerate | frames |
| H – BabyUnicorn | 30 | 97 | 2 | 3712 | 2400 |
| N – Chess | 30 | 97 | 2 | 2048 | 4352 |
| O – Fan | 30 | 97 | 2 | 1920 | 4640 |
| S – CBAbasketball | 30 | 97 | 2 | 2048 | 4352 |
| Y – Breakfast | 30 | 97 | 2 | 1920 | 4640 |
| Z – Barn | 30 | 97 | 2 | 1920 | 4640 |

Sequences N-chess and O-Fan are part of the MIV CTC. The other sequences are of the ongoing MIV verification tests [18].

# VVC coding experiments for V3C content

Initial encoding results following the conditions and content laid out in Sections 3 and 4 are provided in the attached Excel sheet. Example viewports rendered for each sequence and test point are made available at [16].

For each sequence, 5 test points were encoded, covering broad bit rate ranges, starting at around 2 Mbit/s, and going up to over 100Mbit/s for exceptional cases. The typical bit rate range for V-PCC content was between 2-10 Mbit/s, for MIV content between 2-50 Mbit/s.

The Excel sheet also provides for each test point:

- YUV PSNR distortions between the uncompressed and compressed V3C YUV videos (texture, geometry, occupancy),

- YUV PSNR distortions of a rendered viewport compared to a viewport rendered from uncompressed V3C YUV videos (labeled “uc” for uncompressed),

- YUV PSNR distortions of a rendered viewport compared to a viewport rendered from original input data (labeled “org” for original),

- V-PCC point cloud distortion metrics D1, D2, and D1 YUV [10], compared against reconstructed point clouds from uncompressed V3C YUV videos (labeled “uc” for uncompressed),

- V-PCC point cloud distortion metrics D1, D2, and D1 YUV [10], compared against original point clouds (labeled “org” for original), and

- summarised encoding and decoding times.

# Conclusion

In this contribution, we described the characteristics of various V3C content and laid out potential testing procedures for the evaluation of coding tools targeted as such content in JVET.

We propose to establish an AHG on V3C video. The initial mandate of this AHG should be to establish common testing conditions for V3C content, possibly based on the experiments provided in this contribution. In a longer run, this AHG could study coding tools for video content utilised in visual volumetric video-based coding (V3C). We would like to invite all interested parties to join such an activity.

# References

[1]	Text of ISO/IEC DIS 23090-5 Visual Volumetric Video-based Coding and Video-based Point Cloud Compression 2nd Edition, https://dms.mpeg.expert/doc_end_user/current_document.php?id=80148&id_meeting=187

[2]	Text of ISO/IEC FDIS 23090-12 MPEG Immersive Video, https://dms.mpeg.expert/doc_end_user/current_document.php?id=79997&id_meeting=187

[3]	WD 1.0 of V-DMC, https://dms.mpeg.expert/doc_end_user/current_document.php?id=85275&id_meeting=192

[4]	Text of ISO/IEC FDIS 23090-10 Carriage of Visual Volumetric Video-based Coding Data https://dms.mpeg.expert/doc_end_user/current_document.php?id=79060&id_meeting=186#

[5] RTP Payload Format for Visual Volumetric Video-based Coding (V3C), https://datatracker.ietf.org/doc/draft-ietf-avtcore-rtp-v3c/

[6]	Coding performance evaluation of TMC2 with VVenC and VVdeC, https://dms.mpeg.expert/doc_end_user/current_document.php?id=81500&id_meeting=189

[7]	D. Graziosi, et al., "An overview of ongoing point cloud compression standardization activities: video-based (V-PCC) and geometry-based (G-PCC)", APSIPA Transactions on Signal and Information Processing: Vol. 9: No. 1, 2020.

[8]	V. K. M. Vadakital, et al., "The MPEG Immersive Video Standard—Current Status and Future Outlook," in IEEE MultiMedia, vol. 29, no. 3, pp. 101-111, 1 July-Sept. 2022.

[9]	L. Ilola et al., “An Overview of the MPEG Standard for Storage and Transport of Visual Volumetric Video-Based Coding,” in Front. Signal Process., 2022.

[10] Common testing conditions for V-PCC, https://dms.mpeg.expert/doc_end_user/current_document.php?id=76918&id_meeting=184

[11] Common test conditions for MPEG immersive video, https://dms.mpeg.expert/doc_end_user/current_document.php?id=84059&id_meeting=191

[12] JVET-AC0044, Occupancy-only PSNR calculations for V3C V-PCC coding evaluation
https://jvet-experts.org/doc_end_user/current_document.php?id=12229

[13] V-PCC Test Model v19,  https://dms.mpeg.expert/doc_end_user/current_document.php?id=83907&id_meeting=191

[14] Test Model 14 for MPEG immersive video, https://dms.mpeg.expert/doc_end_user/current_document.php?id=84070&id_meeting=191

[15] VVCSoftware_VTM, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-18.1

[16] https://mpegfs.int-evry.fr/mpegcontent/ws-mpegcontent (access through MPEG DMS) → Explorations → V3C → JVET-AC0041_viewports.zip

[17] Subjective verification test report for V-PCC, https://dms.mpeg.expert/doc_end_user/current_document.php?id=81083&id_meeting=188

[18] Verification test preparations of MPEG immersive video, https://dms.mpeg.expert/doc_end_user/current_document.php?id=84062&id_meeting=191

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).