| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0059-v1 |
| --- | --- |

| Title: | AHG12: TMP using Reconstruction-Reordered for screen content coding (RR-TMP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jung-Kyung Lee, Damian Ruiz Coll, Vikas Warudkar | Tel: Email: | jlee@ofinno.com, duizcoll@ofinno.com, vwarudkar@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution proposes TMP using Reconstruction-Reordered (RR-TMP) for screen content video coding. When RR-TMP is applied, the prediction signal is generated by matching the flipped L-shaped template causal neighbor of the current block with another reference block for intraTMP in a predefined search area. RR-TMP finds the best-matched flipped template position, flips the corresponding reference block according to the flip type, and uses it as a prediction block of the current block. RR-TMP can be further improved by allowing an adjusted RR-TMP block vector (BV) for intra block copy (IBC). Simulation results based on ECM-7.0 are reported below:

AI: Class F -0.19%, 0.05%, -0.17%, 100%, 101%; Class TGM -0.31%, -0.28%, -0.28%, 100%, 102%

RA: Class F -0.10%, -0.20%, -0.25%, 101%, 100%; Class TGM -0.11%, -0.22%, -0.10%, 101%, 100%

# Introduction

Since symmetry is often observed in screen content, a Reconstruction-Reordered IBC (RR-IBC) mode was adopted in ECM-6.0 to improve the coding efficiency of IBC further. In RR-IBC, a motion shift is added to a block vector candidate obtained from a neighbouring block according to a symmetric rule in case the neighbouring block is coded with RR-IBC.

In IntraTMP, the prediction signal is generated at the decoder side by matching the L-shaped template of the current block with another block in a predefined search area. Using IntraTMP BV of neighbouring blocks was proposed in JVET-AB0061 [2], which employs IntraTMP BV of the neighbour blocks for the IBC candidate list.

# Proposal

In this contribution, Reconstruction-Reordered TMP is proposed for screen content video coding. When RR-TMP is applied, both the encoder and decoder search for the most similar template to the flipped current template in a predefined search range, which is a reconstructed part of the current frame. The corresponding block is flipped according to flip type, horizontal or vertical flip, and used as a prediction block.

In contrast with the regular TMP, the search regions for RR-TMP are constrained. For the horizontal flip type, only a portion of the current CTU (R1) and left region (R4) are explored. Regarding the vertical flip mode, only a portion of the current CTU (R1) and the above region (R3) are explored.

The dimensions of search regions, search range width (SRW), and search range height (SRH), are set proportional to the block dimension, which is 5 times of block dimension (CbHeight, CbWidth) as shown in Figure 1.

Figure 1. RR-TMP search region for (a) horizontal flip, and (b) vertical flip.

To better utilize the symmetry property, the RR-TMP BV is stored to be used as neighbor candidate in the IBC list of the next bocks. If a current IBC block select the RR-TMP BV of neighbouring blocks as a candidate, it is adjusted in the same way it is done for the TMP BVs. For IBC AMVP, the RR-TMP BV is used only when the current block has the same RR-IBC flip type as the RR-TMP neighboring block.

As in RR-IBC, a flip-aware BV adjustment is applied to refine the block vector candidate according to two flip types, horizontal flip, and vertical flip. Figure 2 shows the flip-aware BV adjustment for horizontal flip and vertical flip in RR-TMP. (xnbr,ynbr) and (xcur,ycur) represent the coordinates of the center sample of the neighboring block and the current block, respectively, BVnbr and BVcur denotes the BV of the neighboring block and the current block, respectively.

For horizontal flipping, the horizontal component of BVcur is calculated by adding a motion shift to the horizontal component of BVnbr (denoted as BVhnbr) in case that the neighboring block is coded with a horizontal flip of RR-TMP, and the vertical component remains same:

- BVhcur=2xnbr-xcur+BVhnbr

- BVvcur=BVvnbr

Similarly, the vertical component of BVcur is calculated by adding a motion shift to the vertical component of BVnbr (denoted as BVvnbr) in case that the neighboring block is coded with a vertical flip and horizontal component remains same:

- BVhcur=BVhnbr

- BVvcur=2ynbr-ycur+BVvnbr

Figure 2. Illustration of RR-TMP BV adjustment for (a) horizontal flip, and (b) vertical flip.

However, one of the BV components is not signaled and is inferred to be equal to 0 followed by RR-IBC.

# Experimental Results

The proposed method is tested on top of ECM-7.0 software [3] under JVET ECM common test conditions [4] in AI and RA configurations. The following results were obtained:

Table I. Experimental result of the proposed method under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.19% | 0.05% | -0.17% | 100% | 100% |
| Class TGM | -0.31% | -0.28% | -0.28% | 100% | 102% |
| Overall | -0.25% | -0.11% | -0.23% | 100% | 101% |

Table II. Experimental result of the proposed method under RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.10% | -0.20% | -0.25% | 101% | 100% |
| Class TGM | -0.11% | -0.22% | -0.10% | 101% | 100% |
| Overall | -0.11% | -0.21% | -0.17% | 101% | 100% |

- References

- Zhipin Deng, Kai Zhang, Li Zhang, “EE2-3.2: Reconstruction-Reordered IBC for screen content coding”, JVET-AA0070, July. 2022.

- Woong Lim, Donghyun Kim, Jongho Kim, Sung-Chang Lim, “EE2-3.2: Using block vector derived from IntraTMP for IBC”, JVET-AB0061, Oct. 2022.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-7.0

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-Y2017, 25th meeting, by teleconference, Jan. 2022.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).