| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0158-v2 |
| --- | --- |

| Title: | EE2-2.5: Pixel based affine motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang, Han Huang, Yan Zhang, Patrick Garus, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of EE2-2.5. It is proposed to change the minimum affine subblock size from 4x4 to 1x1 for both luma and chroma components, 1x1 subblock size allows pixel based affine MC. In Test 2.5a, minimum affine subblock size is set to 1x1 for luma component when OBMC is not applied, while in Test 2.5b the minimum subblock size is set to 1x1 for all affine blocks regardless of OBMC condition. The proposed method was implemented on top of ECM-7.0 and the test results are as follows.

Test 2.5a:

RA: -0.18% (Y), -0.49% (U), -0.48% (V), 102% (EncT), 101% (DecT).

LDB: -0.05% (Y), -0.42% (U), -0.51% (V), 101% (EncT), 102% (DecT)

Test 2.5b:

RA: -0.15% (Y), -0.40% (U), -0.52% (V), 109% (EncT), 103% (DecT).

LDB: 0.01% (Y), -0.42% (U), -0.17% (V), 105% (EncT), 104% (DecT)

# Introduction

In ECM-7.0, an affine block is predicted by applying subblock based affine motion compensation. Given a W×H affine block, the minimum subblock size is set to 4×4, and the subblock size M×N is determined by checking the motion vector differences, wherein, 4≤M≤W and 4≤N≤H. The prediction block is possible to be refined by applying Prediction Refinement with Optical Flow (PROF).

When Local Illumination Compensation (LIC) flag is set to true, the affine predicted block is further refined by applying the LIC model. When Overlapped Block Motion Compensation (OBMC) flag is set to true, the affine predicted block is further refined on CU boundary and subblock boundaries by blending samples predicted by using neighbour motion information. LIC and OBMC can not be applied at same time.

# Proposal

This contribution proposes to change the minimum affine subblock size from 4x4 to 1x1 for both luma and chroma components, 1x1 subblock size allows pixel based affine MC. When affine subblock width or height is smaller than 4, PROF is disabled.

In the tests, OBMC flag is signalled only for affine AMVP coded block without MHP, and if MHP is used then OBMC flag is disabled without signalling. For affine merge mode, OBMC flag is inherited from neighbour affine block. For regular inter AMVP the OBMC flag is enabled without signalling, and OBMC is disabled for SCC classes by existed SPS flag.

In Test 2.5a, minimum affine subblock size is set to 1x1 for luma component when OBMC is not applied, minimum subblock size is always set to 1x1 for chroma components.

In Test 2.5b the minimum subblock size is set to 1x1 for all affine blocks regardless of OBMC condition and for both luma and chroma components.

# Simulation results

## Test 2.5a

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.21% | -0.08% | 103% | 102% |  |  |  |  |  |
| Class A2 | -0.32% | -0.81% | -0.95% | 103% | 101% |  |  |  |  |  |
| Class B | -0.27% | -0.55% | -0.62% | 102% | 102% | 0.00% | -0.62% | -0.69% | 101% | 103% |
| Class C | -0.09% | -0.36% | -0.27% | 101% | 99% | -0.10% | -0.15% | -0.41% | 100% | 101% |
| Class E |  |  |  |  |  | -0.05% | -0.45% | -0.34% | 101% | 101% |
| Overall | -0.18% | -0.49% | -0.48% | 102% | 101% | -0.05% | -0.42% | -0.51% | 101% | 102% |
| Class D | -0.10% | -0.48% | -0.37% | 100% | 101% | -0.17% | 0.01% | -0.51% | 100% | 101% |
| Class F | -1.13% | -1.68% | -1.75% | 101% | 99% | -0.54% | -1.04% | -1.30% | 103% | 95% |
| Class TGM | -1.97% | -1.70% | -1.66% | 100% | 101% | -2.90% | -2.52% | -2.58% | 99% | 92% |

## Test 2.5b

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.30% | -0.16% | 112% | 104% |  |  |  |  |  |
| Class A2 | -0.27% | -0.70% | -0.86% | 113% | 105% |  |  |  |  |  |
| Class B | -0.23% | -0.37% | -0.67% | 108% | 104% | 0.00% | -0.56% | -0.55% | 106% | 106% |
| Class C | -0.05% | -0.29% | -0.34% | 106% | 100% | -0.06% | -0.36% | -0.16% | 104% | 102% |
| Class E |  |  |  |  |  | 0.13% | -0.26% | 0.46% | 104% | 102% |
| Overall | -0.15% | -0.40% | -0.52% | 109% | 103% | 0.01% | -0.42% | -0.17% | 105% | 104% |
| Class D | -0.06% | -0.54% | -0.37% | 104% | 101% | 0.02% | -0.26% | -0.28% | 104% | 103% |
| Class F | -1.13% | -1.68% | -1.75% | 101% | 99% | -0.54% | -1.04% | -1.30% | 103% | 95% |
| Class TGM | -1.97% | -1.70% | -1.66% | 100% | 101% | -2.90% | -2.52% | -2.58% | 99% | 92% |

## Additional SCC results of EE2-2.5 with OBMC enabled

| Test 2.5a | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -1.19% | -2.10% | -2.22% | 100% | 100% | -0.42% | -1.15% | -1.85% | 101% | 102% |
| Class TGM | 0.52% | 0.05% | 0.07% | 102% | 99% | 1.10% | 0.60% | 0.59% | 102% | 101% |

| Test 2.5b | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -0.99% | -1.96% | -1.92% | 103% | 101% | -0.36% | -1.01% | -1.59% | 105% | 104% |
| Class TGM | 0.73% | 0.29% | 0.29% | 103% | 100% | 1.44% | 0.93% | 0.93% | 102% | 89% |

## Additional SCC results of ECM-7.0 with OBMC disabled

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | 0.45% | 0.93% | 1.07% | 97% | 81% | -0.xx% | -0.xx% | -0.xx% | 1xx% | 1xx% |
| Class TGM | -1.32% | -0.80% | -0.70% | 96% | 91% |  |  |  |  |  |

# References

[1] ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).