| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0186-v1 |
| --- | --- |

| Title: | EE2-related: EE2-2.7 with further encoder optimizations |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

This contribution reports the performance of EE2-2.7a with further encoder optimizations. It is reported that on top of ECM-7.0, simulation results of the proposed method are as below:

RA: -0.11 % (Y), -0.21 % (U), -0.23 % (V), 101% (EncT), 101% (DecT)

LB: -0.15 % (Y), -0.35 % (U), -0.15 % (V), 102% (EncT), 102% (DecT)

- Introduction

In EE2-2.7, several fast algorithms are integrated, and a SPS syntax (sps_fast_sub_tmvp_enabled_flag) is signaled to indicate whether these fast algorithms are enabled or not.

- Proposed methods

In this contribution, the determination of sps_fast_sub_tmvp_enabled_flag is modified as below:

sps.setUseFastSubTmvp = ((m_sourceWidth * m_sourceHeight) > (m_intraPeriod == -1 ? 0 : 832 * 480)).

- Experimental results

The proposed method has been implemented on top of the ECM-7.0 [1]. Results under RA/LDB configurations are presented in Table 1. The simulations were performed in accordance with the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method.

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.28% | -0.21% | 101% | 100% |  |  |  |  |  |
| Class A2 | -0.08% | -0.14% | -0.20% | 101% | 101% |  |  |  |  |  |
| Class B | -0.09% | -0.12% | -0.25% | 101% | 101% | -0.12% | -0.46% | -0.64% | 103% | 103% |
| Class C | -0.16% | -0.34% | -0.22% | 102% | 103% | -0.27% | -0.33% | -0.06% | 103% | 103% |
| Class E |  |  |  |  |  | -0.02% | -0.20% | 0.57% | 101% | 101% |
| Overall | -0.11% | -0.21% | -0.23% | 101% | 101% | -0.15% | -0.35% | -0.15% | 102% | 102% |
| Class D | -0.xx% | -0.xx% | -0.xx% | 101% | 100% | -0.02% | -0.51% | -0.79% | 104% | 103% |
| Class F | -0.xx% | -0.xx% | -0.xx% | 101% | 101% | -0.10% | 0.22% | 0.17% | 103% | 102% |

- Conclusions

This contribution reports the performance of EE2-2.7a with further encoder optimizations. It is recommended to adopt this contribution in ECM.

- References

[1] ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).