| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0077-v1 |
| --- | --- |

| Title: | AHG2: On num_l0_weights and num_l1_weights for VVC specification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoko Aono, Takeshi Chujoh, Tomohiro Ikai | Email: | aono.tomoko@sharp.co.jp chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes an improvement for the new revision of VVC. The syntax elements, num_l0_weights and num_l1_weights, specify the number of weights signalled for entries in the reference picture lists on the weighted prediction syntax table. Those values are greater than or equal to 0. However, there is an implicit condition that num_l0_weights and num_l1_weights always can never be set to 0 because there is a condition that the number of reference pictures is greater than 0 in the P slice and B slice. Therefore, this contribution proposes that the syntax elements, num_l0_weights and num_l1_weights, explicitly should change the values greater than 0. These specifications should be included in the draft document for the new revision of VVC.

Problems and solutions

Problem

The variable NumRefIdxActive[ i ] is derived as follows:

for( i = 0; i < 2; i++ ) {
	if( sh_slice_type  = =  B  | |  ( sh_slice_type  = =  P  &&  i  = =  0 ) ) {
		if( sh_num_ref_idx_active_override_flag )
			NumRefIdxActive[ i ] = sh_num_ref_idx_active_minus1[ i ] + 1	(139)
		else {
			if( num_ref_entries[ i ][ RplsIdx[ i ] ]  >=  pps_num_ref_idx_default_active_minus1[ i ] + 1 )
				NumRefIdxActive[ i ] = pps_num_ref_idx_default_active_minus1[ i ] + 1
			else
				NumRefIdxActive[ i ] = num_ref_entries[ i ][ RplsIdx[ i ] ]
		}
	} else /* sh_slice_type  = =  I  | |  ( sh_slice_type  = =  P  &&  i  = =  1 ) */
		NumRefIdxActive[ i ] = 0
}

The value of NumRefIdxActive[ i ] − 1 specifies the maximum reference index for RPL i that may be used to decode the slice. When the value of NumRefIdxActive[ i ] is equal to 0, no reference index for RPL i is used to decode the slice.

When the current slice is a P slice, the value of NumRefIdxActive[ 0 ] shall be greater than 0.

When the current slice is a B slice, both NumRefIdxActive[ 0 ] and NumRefIdxActive[ 1 ] shall be greater than 0.

Regarding the variable NumRefIdxActive[ i ], therer are following conditions.

When either of the following conditions is true, the value of NumRefIdxActive[ 0 ] shall be less than or equal to the value of NumWeightsL0:

- The value of pps_wp_info_in_ph_flag is equal to 1, pps_weighted_pred_flag is equal to 1, and sh_slice_type is equal to P.

- The value of pps_wp_info_in_ph_flag is equal to 1, pps_weighted_bipred_flag is equal to 1, and sh_slice_type is equal to B.

When pps_wp_info_in_ph_flag is equal to 1, pps_weighted_bipred_flag is equal to 1, and sh_slice_type is equal to B, the value of NumRefIdxActive[ 1 ] shall be less than or equal to the value of NumWeightsL1.

The variable NumWeightsL0 is derived as follows:

If pps_wp_info_in_ph_flag is equal to 1, the variable NumWeightsL0 is set equal to num_l0_weights. Otherwise (pps_wp_info_in_ph_flag is equal to 0), NumWeightsL0 is set equal to NumRefIdxActive[ 0 ].

The variable NumWeightsL1 is derived as follows:

if( !pps_weighted_bipred_flag  | |
		( pps_wp_info_in_ph_flag  &&  num_ref_entries[ 1 ][ RplsIdx[ 1 ] ]  = =  0 ) )
	NumWeightsL1 = 0
else if( pps_wp_info_in_ph_flag )		(145)
	NumWeightsL1 = num_l1_weights
else
	NumWeightsL1 = NumRefIdxActive[ 1 ]

When either of the following conditions is true, the syntax element num_l0_weights is encoded or decoded, and the variable NumWeightsL0 is set equal to num_l0_weights.:

- The value of pps_wp_info_in_ph_flag is equal to 1, pps_weighted_pred_flag is equal to 1, and sh_slice_type is equal to P.

- The value of pps_wp_info_in_ph_flag is equal to 1, pps_weighted_bipred_flag is equal to 1, and sh_slice_type is equal to B.

When pps_wp_info_in_ph_flag is equal to 1, pps_weighted_bipred_flag is equal to 1, and sh_slice_type is equal to B, the syntax elements num_l1_weights is encoded or decoded, and the variable NumWeightsL1 is set equal to num_l1_weights.

Since when the current slice is a P slice, the value of NumRefIdxActive[ 0 ] shall be greater than 0 and when the current slice is a B slice, both NumRefIdxActive[ 0 ] and NumRefIdxActive[ 1 ] shall be greater than 0, the values of NumWeightsL0 and the value of NumWeightsL1 shall be greater than 0

Therefore, both num_l0_weights and num_l1_weights shall be greater than 0.

However, num_l0_weights shall be in the range of 0 to Min( 15, num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] ) and num_l1_weights shall be in the range of 0 to Min( 15, num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] ).

In current VVC specification, there is an implicit condition that num_l0_weights and num_l1_weights can never be set to 0.

Solution

In subsection 7.4.7.3, it is proposed to fix num_l0_weights and num_l1_weights range as follows:

num_l0_weights specifies the number of weights signalled for entries in RPL 0 when pps_wp_info_in_ph_flag is equal to 1. The value of num_l0_weights shall be in the range of 01 to Min( 15, num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] ), inclusive.

num_l1_weights specifies the number of weights signalled for entries in RPL 1 when pps_wp_info_in_ph_flag is equal to 1. The value of num_l1_weights shall be in the range of 01 to Min( 15, num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] ), inclusive.

References

[1]	E. Francois, B. Bross, M. M. Hannuksela, A. M. Tourapis and Y.-K. Wang, “New level and systems-related supplemental enhancement information for VVC (Draft 4),” JVT-AC2005, Jan. 2023.

[2]	Recommendation H.266 (04/22).

Patent rights declaration

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).