| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0080-v1 |
| --- | --- |

| Title: | Non-EE2: Chroma-to-chroma convolutional cross-component model (C2C-CCCM) for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongkai Huo Yutian Liu | Email: | yongkai.huo@transsion.com, yutian.liu@transsion.com |
| Source: | Transsion |  |  |

_____________________________

- Abstract

This contribution introduces chroma-to-chroma convolutional cross-component model (C2C-CCCM) based on the proposal JVET-AA0057. The proposed method adds two new prediction modes that chroma red is predicted from chroma blue signals. The impacts on coding efficiency and runtimes over ECM-8.0 are reportedly {for Y, U, V, EncT, DecT}:

AI { xx.xx%, xx.xx%, xx.xx%, xxx%, xxx%}, RA { xx.xx%, xx.xx%, xx.xx%, xxx%, xxx%}

Introduction

Convolutional cross-component model (CCCM) [2] predicts chroma samples from reconstructed luma samples using a 7-tap filter. Similarly to CCLM, there is an option of using a single model or multi-model variant of CCCM. The multi-model variant uses two models, one model derived for samples above the average luma reference value and another model for the rest of the samples (following the spirit of the CCLM design). Multi-model CCCM mode can be selected for PUs which have at least 128 reference samples available. There are totally 6 CCCM modes in ECM 7.0:

- Single-model CCCM from above and left templates

- Single-model CCCM from above template

- Single-model CCCM from left template

- Multi-model CCCM from above and left templates

- Multi-model CCCM from above template

- Multi-model CCCM from left template

GL-CCCM method [1] was proposed using gradient and location information instead of the 4 spatial neighbor samples in the CCCM filter. This doubles the original 6 CCCM modes in ECM 8.0.

Proposed method

Description

In our method, the samples of chroma blue are utilized instead of reconstructed luma samples for predicting chroma red. There are two types of chroma blue samples utilized, namely the predicted samples and the reconstructed samples. In the case of utilizing reconstructed chroma blue samples, the joint coding of chroma residual (JCCR) mode is disabled. Furthermore, our proposed method is compatible with GL-CCCM mode [1].

Bitstream signalling

Usage of the mode is signalled with a CABAC coded PU level flag. One new CABAC context was included to indicate two sub-modes of CCCM. Specifically, the C2C-CCCM flag is signalled if original CCCM flag is true and non-downsampling [4] flag is false.

Experimental results

Tables below summarize the coding efficiency and runtime impact of the proposed method in ECM 8.0 using the JVET common test conditions [3].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.06% | -0.07% | 104% | 100% |
| Class C | 0.01% | 0.01% | -0.14% | 106% | 100% |
| Class E | 0.04% | -0.11% | -0.28% | 103% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.01% | -0.05% | -0.15% | 105% | 100% |
| Class F | 0.07% | -0.24% | -0.44% | 103% | 100% |
| Class TGM |  |  |  |  |  |

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Conclusion

This contribution proposes the chroma-to-chroma convolutional cross-component model (C2C-CCCM) as extensions to the CCCM tool. The results show that the gain of CCCM can be further improved using the C2C-CCCM method. It is suggested to adopt the proposed method in ECM-8.0 software.

References

- R. G. Youvalari, P. Astola, J. Lainema, “EE2-1.12: Gradient and location based convolutional cross-component model (GL-CCCM) for intra prediction,” JVET-AC0054, Jan. 2023.

- P. Astola, J. Lainema, R. G. Youvalari, A. Aminlou, and K. Panusopone, “EE2-1.1a: Convolutional cross-component intra prediction model,” JVET-AA0057, Jul. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

- H.-J. Jhu, C.-W. Kuo, X. Xiu, W. Chen, N. Yan, C. Ma, X. Wang, V. Seregin, Y.-J. Chang, B. Ray, M. Karczewicz,” EE2-1.13 and 1.14: CCCM using non-downsampled luma samples,” JVET-AC0147, Jan. 2023.

Patent rights declaration(s)

Transsion may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).