| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0093-v2 |
| --- | --- |

| Title: | AHG9: Neural-network post-processing filter with downsampling capabilities |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Jizheng Xu, Ye-Kui Wang, Chaoyi Lin, Junru Li, Kai Zhang, Li Zhang | Tel: Email: | yue.li@bytedance.com xujizheng@bytedance.com yekui.wang@bytedance.com linchaoyi.cy@bytedance.com lijunru@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

Currently, the neural-network post-filter characteristics (NNPFC) SEI message specifies purposes with upsampling capabilities, such as chroma upsampling, resolution upsampling, picture rate upsampling, and bit depth upsampling. However, in video applications, often downsampling capabilities are also needed, and like for upsampling, using neural-network filters for downsampling can also provide better performances than using conventional approaches.

This contribution proposes to define neural-network filter purposes with downsampling capabilities. It is proposed to use one of the following three options:

- Define new purposes with downsampling capabilities, including chroma downsampling, resolution downsampling, picture rate downsampling, and bit depth downsampling. Two version of text changes, option 1 and option 1a, are provided, without or with JVET-AD0091 item 3 (on signalling of the scaling ratios instead of the NNPF output picture width and height).

- Extend current purposes with upsampling capabilities to purposes with resampling capabilities, including chroma resampling, resolution resampling, picture rate resampling, and bit depth resampling. Two version of text changes, option 2 and option 2a, are provided, without or with JVET-AD0091 item 3.

- Based on option 1) or option 2), only support certain types of purposes with downsampling/resampling capabilities (e.g. resolution downsampling/resampling), with remained purposes (e.g. chroma upsampling, picture rate upsampling, bit depth upsampling) unchanged.

In this v2 version, text changes are added for option 3 according to the BoG recommendation of JVET-AD0362. The added text changes are only for extending the current purpose of resolution upsampling to resolution resampling with both upsampling capabilities and downsampling capabilities. The constraints disallowing NNPF picture width and height changing into opposite directions are removed as suggested.

Proposal option 1

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts):

### 8.28.1	Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| nnpfc_base_flag | u(1) |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( ( nnpfc_purpose & 0x02 ) != 0 ¦ ¦ ( nnpfc_purpose & 0x40 ) != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ( nnpfc_purpose & 0x20 ) != 0 ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ( nnpfc_purpose & 0x04 ) != 0 ¦ ¦ ( nnpfc_purpose & 0x80 ) != 0 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| if( ( nnpfc_purpose & 0x08 ) != 0 ¦ ¦ ( nnpfc_purpose & 0x100 ) != 0 ) { |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| } |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| nnpfc_luma_padding_val | ue(v) |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

### 8.28.2	Neural-network post-filter characteristics SEI message semantics

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified neural-network post-processing filters (NNPFs) for specific pictures is indicated with neural-network post-filter activation (NNPFA) SEI messages.

Use of this SEI message requires the definition of the following variables:

–	Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	Luma sample array CroppedYPic[ idx ] and chroma sample arrays CroppedCbPic[ idx ] and CroppedCrPic[ idx ], when present, of the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive, that are used as input for the NNPF.

–	Bit depth BitDepthY for the luma sample array of the input pictures.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	When nnpfc_auxiliary_inp_idc is equal to 1, a filtering strength control value StrengthControlVal that shall be a real number in the range of 0 to 1, inclusive.

Input picture with index 0 corresponds to the picture for which the NNPFdefined by this NNPFC SEI message is activated by an NNPFA SEI message. Input picture with index i in the range of 1 to numInputPics − 1, inclusive, precedes the input picture with index i − 1 in output order.

When nnpfc_purpose & 0x08 is not equal to 0 and the input picture with index 0 is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, all input pictures are associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

NOTE 1 – More than one NNPFC SEI message can be present for the same picture. When more than one NNPFC SEI message with different values of nnpfc_id is present or activated for the same picture, they can have the same or different values of nnpfc_purpose and nnpfc_mode_idc.

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 63 1023, inclusive, in bitstreams conforming to this edition of this document. Values of 641024 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 641024 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| nnpfc_purpose = = 0 | May be used as determined by the application |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x01 ) = = 0 | No general visual quality improvement |
| ( nnpfc_purpose & 0x01 ) != 0 | With general visual quality improvement |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x02 ) = = 0 | No chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x02 ) != 0 | With chroma upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x04 ) = = 0 | No resolution upsampling (increasing the width or height) |
| ( nnpfc_purpose & 0x04 ) != 0 | With resolution upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x08 ) = = 0 | No picture rate upsampling |
| ( nnpfc_purpose & 0x08 ) != 0 | With picture rate upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x10 ) = = 0 | No bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| ( nnpfc_purpose & 0x10 ) != 0 | With bit depth upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x20 ) = = 0 | No colourization (from the 4:0:0 chroma format to the 4:2:0, 4:2:2, or 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x20 ) != 0 | With colourization |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x40 ) = = 0 | No chroma downsampling (from the 4:4:4 chroma format to the 4:2:2 or 4:2:0 chroma format, or from the 4:2:2 chroma format to the 4:2:0 chroma format) |
| ( nnpfc_purpose & 0x40 ) != 0 | With chroma downsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x80 ) = = 0 | No resolution downsampling (decreasing the width or height) |
| ( nnpfc_purpose & 0x80 ) != 0 | With resolution downsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x100 ) = = 0 | No picture rate downsampling |
| ( nnpfc_purpose & 0x100 ) != 0 | With picture rate downsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x200 ) = = 0 | No bit depth downsampling (decreasing the luma bit depth or the chroma bit depth) |
| ( nnpfc_purpose & 0x200 ) != 0 | With bit depth downsampling |

NOTE 2– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When ChromaFormatIdc is equal to 3, nnpfc_purpose & 0x02 shall be equal to 0.

When ChromaFormatIdc is equal to 1, nnpfc_purpose & 0x40 shall be equal to 0.

When ChromaFormatIdc or nnpfc_purpose & 0x02 is not equal to 0, nnpfc_purpose & 0x20 shall be equal to 0.

When ChromaFormatIdc or nnpfc_purpose & 0x40 is not equal to 0, nnpfc_purpose & 0x20 shall be equal to 0.

When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_purpose & 0x40 shall be equal to 0.

When nnpfc_purpose & 0x04 is not equal to 0, nnpfc_purpose & 0x80 shall be equal to 0.

When nnpfc_purpose & 0x08 is not equal to 0, nnpfc_purpose & 0x100 shall be equal to 0.

nnpfc_id contains an identifying number that may be used to identify an NNPF. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFC SEI message with nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

When an NNPFC SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the following applies:

–	This SEI message specifies a base NNPF.

–	This SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS.

...

nnpfc_out_colour_format_idc, when nnpfc_purpose & 0x20 is not equal to 0, specifies the colour format of the NNPF output and consequently the values of the variables outSubWidthC and outSubHeightC. nnpfc_out_colour_format_idc equal to 1 specifies that the colour format of the NNPF output is the 4:2:0 format and outSubWidthC and outSubHeightC are both equal to 2. nnpfc_out_colour_format_idc equal to 2 specifies that the colour format of the NNPF output is the 4:2:2 format and outSubWidthC is equal to 2 and outSubHeightC is equal to 1. nnpfc_out_colour_format_idc equal to 3 specifies that the colour format of the NNPF output is the 4:2:4 format and outSubWidthC and outSubHeightC are both equal to 1. The value of nnpfc_out_colour_format_idc shall not be equal to 0.

When nnpfc_purpose & 0x02 and nnpfc_purpose & 0x20 and nnpfc_purpose & 0x40 are both equal to 0, outSubWidthC and outSubHeightC are inferred to be equal to SubWidthC and SubHeightC, respectively.

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth  >>  4 to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight  >>  4 to CroppedHeight * 16 − 1, inclusive.

When nnpfc_purpose & 0x04 is not equal to 0, at least one of the following conditions shall be true:

–	nnpfc_pic_width_in_luma_samples is greater than CroppedWidth and nnpfc_pic_height_in_luma_samples is not less than CroppedHeight.

–	nnpfc_pic_height_in_luma_samples is greater than CroppedHeight and nnpfc_pic_width_in_luma_samples is not less than CroppedWidth.

When nnpfc_purpose & 0x80 is not equal to 0, at least one of the following conditions shall be true:

–	nnpfc_pic_width_in_luma_samples is less than CroppedWidth and nnpfc_pic_height_in_luma_samples is not greater than CroppedHeight.

–	nnpfc_pic_height_in_luma_samples is less than CroppedHeight and nnpfc_pic_width_in_luma_samples is not greater than CroppedWidth.

nnpfc_num_input_pics_minus1 plus 1 specifies the number of decoded output pictures used as input for the NNPF. The value of nnpfc_num_input_pics_minus1 shall be in the range of 0 to 63, inclusive. When nnpfc_purpose & 0x08 is not equal to 0, the value of nnpfc_num_input_pics_minus1 shall be greater than 0.

...

nnpfc_out_format_idc equal to 0 indicates that the sample values output by the NNPF are real numbers where the value range of 0 to 1, inclusive, maps linearly to the unsigned integer value range of 0 to ( 1  <<  bitDepth ) – 1, inclusive, for any desired bit depth bitDepth for subsequent post-processing or displaying.

nnpfc_out_format_idc equal to 1 indicates that the luma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_luma_bitdepth_minus8 + 8 ) ) − 1, inclusive, and the chroma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 ) ) − 1, inclusive.

Values of nnpfc_out_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_out_format_idc.

nnpfc_out_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the output integer tensor. The value of nnpfc_out_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the output integer tensor. The value of nnpfc_out_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

When nnpfc_purpose & 0x10 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is greater than BitDepthY and nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is not less than BitDepthC.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is greater than BitDepthC and nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is not less than BitDepthY.

When nnpfc_purpose & 0x200 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is less than BitDepthY and nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is not greater than BitDepthC.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is less than BitDepthC and nnpfc_out_tensor_luma_bitdepth_minus1 + 1 is not greater than BitDepthY.

nnpfc_out_order_idc indicates the output order of samples resulting from the NNPF.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

...

Proposal option 2

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts):

### 8.28.1	Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| nnpfc_base_flag | u(1) |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( ( nnpfc_purpose & 0x02 ) != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ( nnpfc_purpose & 0x20 ) != 0 ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ( nnpfc_purpose & 0x04 ) != 0 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| if( ( nnpfc_purpose & 0x08 ) != 0 ) { |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| } |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| nnpfc_luma_padding_val | ue(v) |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

### 8.28.2	Neural-network post-filter characteristics SEI message semantics

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified neural-network post-processing filters (NNPFs) for specific pictures is indicated with neural-network post-filter activation (NNPFA) SEI messages.

Use of this SEI message requires the definition of the following variables:

–	Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	Luma sample array CroppedYPic[ idx ] and chroma sample arrays CroppedCbPic[ idx ] and CroppedCrPic[ idx ], when present, of the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive, that are used as input for the NNPF.

–	Bit depth BitDepthY for the luma sample array of the input pictures.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	When nnpfc_auxiliary_inp_idc is equal to 1, a filtering strength control value StrengthControlVal that shall be a real number in the range of 0 to 1, inclusive.

Input picture with index 0 corresponds to the picture for which the NNPFdefined by this NNPFC SEI message is activated by an NNPFA SEI message. Input picture with index i in the range of 1 to numInputPics − 1, inclusive, precedes the input picture with index i − 1 in output order.

When nnpfc_purpose & 0x08 is not equal to 0 and the input picture with index 0 is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, all input pictures are associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

NOTE 1 – More than one NNPFC SEI message can be present for the same picture. When more than one NNPFC SEI message with different values of nnpfc_id is present or activated for the same picture, they can have the same or different values of nnpfc_purpose and nnpfc_mode_idc.

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 63, inclusive, in bitstreams conforming to this edition of this document. Values of 64 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 64 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| nnpfc_purpose = = 0 | May be used as determined by the application |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x01 ) = = 0 | No general visual quality improvement |
| ( nnpfc_purpose & 0x01 ) != 0 | With general visual quality improvement |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x02 ) = = 0 | No chroma upresampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format, or from the 4:4:4 chroma format to the 4:2:2 or 4:2:0 chroma format, or from the 4:2:2 chroma format to the 4:2:0 chroma format) |
| ( nnpfc_purpose & 0x02 ) != 0 | With chroma upresampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x04 ) = = 0 | No resolution upresampling (increasing or deceasing the width or height) |
| ( nnpfc_purpose & 0x04 ) != 0 | With resolution upresampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x08 ) = = 0 | No picture rate upresampling |
| ( nnpfc_purpose & 0x08 ) != 0 | With picture rate upresampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x10 ) = = 0 | No bit depth upresampling (increasing or deceasing the luma bit depth or the chroma bit depth) |
| ( nnpfc_purpose & 0x10 ) != 0 | With bit depth upresampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x20 ) = = 0 | No colourization (from the 4:0:0 chroma format to the 4:2:0, 4:2:2, or 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x20 ) != 0 | With colourization |

NOTE 2– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When ChromaFormatIdc is equal to 3, nnpfc_purpose & 0x02 shall be equal to 0.

When ChromaFormatIdc or nnpfc_purpose & 0x02 is not equal to 0, nnpfc_purpose & 0x20 shall be equal to 0.

nnpfc_id contains an identifying number that may be used to identify an NNPF. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFC SEI message with nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

When an NNPFC SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the following applies:

–	This SEI message specifies a base NNPF.

–	This SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS.

...

nnpfc_out_colour_format_idc, when nnpfc_purpose & 0x20 is not equal to 0, specifies the colour format of the NNPF output and consequently the values of the variables outSubWidthC and outSubHeightC. nnpfc_out_colour_format_idc equal to 1 specifies that the colour format of the NNPF output is the 4:2:0 format and outSubWidthC and outSubHeightC are both equal to 2. nnpfc_out_colour_format_idc equal to 2 specifies that the colour format of the NNPF output is the 4:2:2 format and outSubWidthC is equal to 2 and outSubHeightC is equal to 1. nnpfc_out_colour_format_idc equal to 3 specifies that the colour format of the NNPF output is the 4:2:4 format and outSubWidthC and outSubHeightC are both equal to 1. The value of nnpfc_out_colour_format_idc shall not be equal to 0.

When nnpfc_purpose & 0x02 and nnpfc_purpose & 0x20 are both equal to 0, outSubWidthC and outSubHeightC are inferred to be equal to SubWidthC and SubHeightC, respectively.

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth  >>  4 to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight  >>  4 to CroppedHeight * 16 − 1, inclusive.

When nnpfc_purpose & 0x04 is not equal to 0, at least one of the following conditions shall be true:

–	(nnpfc_pic_width_in_luma_samples is greater than CroppedWidth and nnpfc_pic_height_in_luma_samples is not less than CroppedHeight), or (nnpfc_pic_height_in_luma_samples is greater than CroppedHeight and nnpfc_pic_width_in_luma_samples is not less than CroppedWidth).

–	(nnpfc_pic_width_in_luma_samples is less than CroppedWidth and nnpfc_pic_height_in_luma_samples is not greater than CroppedHeight), or (nnpfc_pic_height_in_luma_samples is less than CroppedHeight and nnpfc_pic_width_in_luma_samples is not greater than CroppedWidth).

nnpfc_num_input_pics_minus1 plus 1 specifies the number of decoded output pictures used as input for the NNPF. The value of nnpfc_num_input_pics_minus1 shall be in the range of 0 to 63, inclusive. When nnpfc_purpose & 0x08 is not equal to 0, the value of nnpfc_num_input_pics_minus1 shall be greater than 0.

...

nnpfc_out_format_idc equal to 0 indicates that the sample values output by the NNPF are real numbers where the value range of 0 to 1, inclusive, maps linearly to the unsigned integer value range of 0 to ( 1  <<  bitDepth ) – 1, inclusive, for any desired bit depth bitDepth for subsequent post-processing or displaying.

nnpfc_out_format_idc equal to 1 indicates that the luma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_luma_bitdepth_minus8 + 8 ) ) − 1, inclusive, and the chroma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 ) ) − 1, inclusive.

Values of nnpfc_out_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_out_format_idc.

nnpfc_out_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the output integer tensor. The value of nnpfc_out_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the output integer tensor. The value of nnpfc_out_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

When nnpfc_purpose & 0x10 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	(nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is greater than BitDepthY and nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is not less than BitDepthC), or (nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is greater than BitDepthC and nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is not less than BitDepthY)

–	(nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is less than BitDepthY and nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is not greater than BitDepthC) or (nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is less than BitDepthC and nnpfc_out_tensor_luma_bitdepth_minus1 + 1 is not greater than BitDepthY).

nnpfc_out_order_idc indicates the output order of samples resulting from the NNPF.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

...

Proposal option 1a

Option 1a provides spec text changes for the combination of option 1 plus JVET-AD0091 item 3 (for spatial resolution upsampling, signal the scaling ratios instead of the NNPF output picture width and height).

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts):

### 8.28.1	Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| nnpfc_base_flag | u(1) |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( ( nnpfc_purpose & 0x02 ) != 0 ¦ ¦ ( nnpfc_purpose & 0x40 ) != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ( nnpfc_purpose & 0x20 ) != 0 ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ( nnpfc_purpose & 0x04 ) != 0 ¦ ¦ ( nnpfc_purpose & 0x80 ) != 0 ) { |  |
| nnpfc_hor_ratio_num_minus1 | u(8) |
| nnpfc_hor_ratio_den_minus1 | u(8) |
| nnpfc_ver_ratio_num_minus1 | u(8) |
| nnpfc_ver_ratio_den_minus1 | u(8) |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| if( ( nnpfc_purpose & 0x08 ) != 0 ¦ ¦ ( nnpfc_purpose & 0x100 ) != 0 ) { |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| } |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| nnpfc_luma_padding_val | ue(v) |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

### 8.28.2	Neural-network post-filter characteristics SEI message semantics

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified neural-network post-processing filters (NNPFs) for specific pictures is indicated with neural-network post-filter activation (NNPFA) SEI messages.

Use of this SEI message requires the definition of the following variables:

–	Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	Luma sample array CroppedYPic[ idx ] and chroma sample arrays CroppedCbPic[ idx ] and CroppedCrPic[ idx ], when present, of the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive, that are used as input for the NNPF.

–	Bit depth BitDepthY for the luma sample array of the input pictures.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	When nnpfc_auxiliary_inp_idc is equal to 1, a filtering strength control value StrengthControlVal that shall be a real number in the range of 0 to 1, inclusive.

Input picture with index 0 corresponds to the picture for which the NNPFdefined by this NNPFC SEI message is activated by an NNPFA SEI message. Input picture with index i in the range of 1 to numInputPics − 1, inclusive, precedes the input picture with index i − 1 in output order.

When nnpfc_purpose & 0x08 is not equal to 0 and the input picture with index 0 is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, all input pictures are associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

NOTE 1 – More than one NNPFC SEI message can be present for the same picture. When more than one NNPFC SEI message with different values of nnpfc_id is present or activated for the same picture, they can have the same or different values of nnpfc_purpose and nnpfc_mode_idc.

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 63 1023, inclusive, in bitstreams conforming to this edition of this document. Values of 641024 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 641024 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| nnpfc_purpose = = 0 | May be used as determined by the application |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x01 ) = = 0 | No general visual quality improvement |
| ( nnpfc_purpose & 0x01 ) != 0 | With general visual quality improvement |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x02 ) = = 0 | No chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x02 ) != 0 | With chroma upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x04 ) = = 0 | No resolution upsampling (increasing the width or height) |
| ( nnpfc_purpose & 0x04 ) != 0 | With resolution upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x08 ) = = 0 | No picture rate upsampling |
| ( nnpfc_purpose & 0x08 ) != 0 | With picture rate upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x10 ) = = 0 | No bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| ( nnpfc_purpose & 0x10 ) != 0 | With bit depth upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x20 ) = = 0 | No colourization (from the 4:0:0 chroma format to the 4:2:0, 4:2:2, or 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x20 ) != 0 | With colourization |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x40 ) = = 0 | No chroma downsampling (from the 4:4:4 chroma format to the 4:2:2 or 4:2:0 chroma format, or from the 4:2:2 chroma format to the 4:2:0 chroma format) |
| ( nnpfc_purpose & 0x40 ) != 0 | With chroma downsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x80 ) = = 0 | No resolution downsampling (decreasing the width or height) |
| ( nnpfc_purpose & 0x80 ) != 0 | With resolution downsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x100 ) = = 0 | No picture rate downsampling |
| ( nnpfc_purpose & 0x100 ) != 0 | With picture rate downsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x200 ) = = 0 | No bit depth downsampling (decreasing the luma bit depth or the chroma bit depth) |
| ( nnpfc_purpose & 0x200 ) != 0 | With bit depth downsampling |

NOTE 2– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When ChromaFormatIdc is equal to 3, nnpfc_purpose & 0x02 shall be equal to 0.

When ChromaFormatIdc is equal to 1, nnpfc_purpose & 0x40 shall be equal to 0.

When ChromaFormatIdc or nnpfc_purpose & 0x02 is not equal to 0, nnpfc_purpose & 0x20 shall be equal to 0.

When ChromaFormatIdc or nnpfc_purpose & 0x40 is not equal to 0, nnpfc_purpose & 0x20 shall be equal to 0.

When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_purpose & 0x40 shall be equal to 0.

When nnpfc_purpose & 0x04 is not equal to 0, nnpfc_purpose & 0x80 shall be equal to 0.

When nnpfc_purpose & 0x08 is not equal to 0, nnpfc_purpose & 0x100 shall be equal to 0.

nnpfc_id contains an identifying number that may be used to identify an NNPF. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFC SEI message with nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

When an NNPFC SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the following applies:

–	This SEI message specifies a base NNPF.

–	This SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS.

...

nnpfc_out_colour_format_idc, when nnpfc_purpose & 0x20 is not equal to 0, specifies the colour format of the NNPF output and consequently the values of the variables outSubWidthC and outSubHeightC. nnpfc_out_colour_format_idc equal to 1 specifies that the colour format of the NNPF output is the 4:2:0 format and outSubWidthC and outSubHeightC are both equal to 2. nnpfc_out_colour_format_idc equal to 2 specifies that the colour format of the NNPF output is the 4:2:2 format and outSubWidthC is equal to 2 and outSubHeightC is equal to 1. nnpfc_out_colour_format_idc equal to 3 specifies that the colour format of the NNPF output is the 4:2:4 format and outSubWidthC and outSubHeightC are both equal to 1. The value of nnpfc_out_colour_format_idc shall not be equal to 0.

When nnpfc_purpose & 0x02 and nnpfc_purpose & 0x20 and nnpfc_purpose & 0x40 are both equal to 0, outSubWidthC and outSubHeightC are inferred to be equal to SubWidthC and SubHeightC, respectively.

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 , inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight  >>  4 to CroppedHeight * 16, inclusive.

nnpfc_hor_ratio_num_minus1 and nnpfc_hor_ratio_den_minus1 specify the horizontal luma scaling ratio nnpfcHorScalingRatio as follows:

nnpfcHorScalingRatio = (nnpfc_hor_ratio_num_minus1  + 1) ÷ ( nnpfc_hor_ratio_den_minus1 + 1 )	(X )

The value of nnpfcHorScalingRatio shall be in the range of 1/16 to 16, inclusive. When nnpfc_hor_ratio_num_minus1 and nnpfc_hor_ratio_den_minus1 are not present, the value of nnpfcHorScalingRatio is inferred to be equal to 1.

The value of the variable nnpfcOutputPicWidth, representing the width of the luma sample array of the picture(s) resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture, is derived as follows:

nnpfcOutputPicWidth = CroppedWidth * nnpfcHorScalingRatio	(X )

nnpfc_ver_ratio_num_minus1 and nnpfc_ver_ratio_den_minus1 specify the vertical luma scaling ratio nnpfcVerScalingRatio is derived as follows:

nnpfcVerScalingRatio = (nnpfc_ver_ratio_num_minus1 + 1) ÷ ( nnpfc_ver_ratio_den_minus1 + 1 )	(X )

The value of nnpfcVerScalingRatio shall be in the range of 1/16 to 16, inclusive. When nnpfc_ver_ratio_num_minus1 and nnpfc_ver_ratio_den_minus1 are not present, the value of nnpfcVerScalingRatio is inferred to be equal to 1.

The value of the variable nnpfcOutputPicHeight, representing the height of the luma sample array of the picture(s) resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture, is derived as follows:

nnpfcOutputPicHeight = CroppedHeight * nnpfcHorScalingRatio	(X )

When nnpfc_purpose & 0x04 is not equal to 0, at least one of the following conditions shall be true:

–	nnpfc_pic_width_in_luma_samples is greater than CroppedWidth and nnpfc_pic_height_in_luma_samples is not less than CroppedHeight.

–	nnpfc_pic_height_in_luma_samples is greater than CroppedHeight and nnpfc_pic_width_in_luma_samples is not less than CroppedWidth.

When nnpfc_purpose & 0x80 is not equal to 0, at least one of the following conditions shall be true:

–	nnpfc_pic_width_in_luma_samples is less than CroppedWidth and nnpfc_pic_height_in_luma_samples is not greater than CroppedHeight.

–	nnpfc_pic_height_in_luma_samples is less than CroppedHeight and nnpfc_pic_width_in_luma_samples is not greater than CroppedWidth.

nnpfc_num_input_pics_minus1 plus 1 specifies the number of decoded output pictures used as input for the NNPF. The value of nnpfc_num_input_pics_minus1 shall be in the range of 0 to 63, inclusive. When nnpfc_purpose & 0x08 is not equal to 0, the value of nnpfc_num_input_pics_minus1 shall be greater than 0.

...

nnpfc_out_format_idc equal to 0 indicates that the sample values output by the NNPF are real numbers where the value range of 0 to 1, inclusive, maps linearly to the unsigned integer value range of 0 to ( 1  <<  bitDepth ) – 1, inclusive, for any desired bit depth bitDepth for subsequent post-processing or displaying.

nnpfc_out_format_idc equal to 1 indicates that the luma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_luma_bitdepth_minus8 + 8 ) ) − 1, inclusive, and the chroma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 ) ) − 1, inclusive.

Values of nnpfc_out_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_out_format_idc.

nnpfc_out_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the output integer tensor. The value of nnpfc_out_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the output integer tensor. The value of nnpfc_out_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

When nnpfc_purpose & 0x10 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is greater than BitDepthY and nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is not less than BitDepthC.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is greater than BitDepthC and nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is not less than BitDepthY.

When nnpfc_purpose & 0x200 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is less than BitDepthY and nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is not greater than BitDepthC.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is less than BitDepthC and nnpfc_out_tensor_luma_bitdepth_minus1 + 1 is not greater than BitDepthY.

nnpfc_out_order_idc indicates the output order of samples resulting from the NNPF.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

...

Proposal option 2a

Option 2a provides spec text changes for the combination of option 2 plus JVET-AD0091 item 3 (for spatial resolution upsampling, signal the scaling ratios instead of the NNPF output picture width and height).

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts):

### 8.28.1	Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| nnpfc_base_flag | u(1) |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( ( nnpfc_purpose & 0x02 ) != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ( nnpfc_purpose & 0x20 ) != 0 ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ( nnpfc_purpose & 0x04 ) != 0 ) { |  |
| nnpfc_hor_ratio_num_minus1 | u(8) |
| nnpfc_hor_ratio_den_minus1 | u(8) |
| nnpfc_ver_ratio_num_minus1 | u(8) |
| nnpfc_ver_ratio_den_minus1 | u(8) |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| if( ( nnpfc_purpose & 0x08 ) != 0 ) { |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| } |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| nnpfc_luma_padding_val | ue(v) |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

### 8.28.2	Neural-network post-filter characteristics SEI message semantics

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified neural-network post-processing filters (NNPFs) for specific pictures is indicated with neural-network post-filter activation (NNPFA) SEI messages.

Use of this SEI message requires the definition of the following variables:

–	Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	Luma sample array CroppedYPic[ idx ] and chroma sample arrays CroppedCbPic[ idx ] and CroppedCrPic[ idx ], when present, of the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive, that are used as input for the NNPF.

–	Bit depth BitDepthY for the luma sample array of the input pictures.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	When nnpfc_auxiliary_inp_idc is equal to 1, a filtering strength control value StrengthControlVal that shall be a real number in the range of 0 to 1, inclusive.

Input picture with index 0 corresponds to the picture for which the NNPFdefined by this NNPFC SEI message is activated by an NNPFA SEI message. Input picture with index i in the range of 1 to numInputPics − 1, inclusive, precedes the input picture with index i − 1 in output order.

When nnpfc_purpose & 0x08 is not equal to 0 and the input picture with index 0 is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, all input pictures are associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

NOTE 1 – More than one NNPFC SEI message can be present for the same picture. When more than one NNPFC SEI message with different values of nnpfc_id is present or activated for the same picture, they can have the same or different values of nnpfc_purpose and nnpfc_mode_idc.

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 63, inclusive, in bitstreams conforming to this edition of this document. Values of 64 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 64 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| nnpfc_purpose = = 0 | May be used as determined by the application |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x01 ) = = 0 | No general visual quality improvement |
| ( nnpfc_purpose & 0x01 ) != 0 | With general visual quality improvement |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x02 ) = = 0 | No chroma upresampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format, or from the 4:4:4 chroma format to the 4:2:2 or 4:2:0 chroma format, or from the 4:2:2 chroma format to the 4:2:0 chroma format) |
| ( nnpfc_purpose & 0x02 ) != 0 | With chroma upresampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x04 ) = = 0 | No resolution upresampling (increasing or deceasing the width or height) |
| ( nnpfc_purpose & 0x04 ) != 0 | With resolution upresampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x08 ) = = 0 | No picture rate upresampling |
| ( nnpfc_purpose & 0x08 ) != 0 | With picture rate upresampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x10 ) = = 0 | No bit depth upresampling (increasing or deceasing the luma bit depth or the chroma bit depth) |
| ( nnpfc_purpose & 0x10 ) != 0 | With bit depth upresampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x20 ) = = 0 | No colourization (from the 4:0:0 chroma format to the 4:2:0, 4:2:2, or 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x20 ) != 0 | With colourization |

NOTE 2– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When ChromaFormatIdc is equal to 3, nnpfc_purpose & 0x02 shall be equal to 0.

When ChromaFormatIdc or nnpfc_purpose & 0x02 is not equal to 0, nnpfc_purpose & 0x20 shall be equal to 0.

nnpfc_id contains an identifying number that may be used to identify an NNPF. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFC SEI message with nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

When an NNPFC SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the following applies:

–	This SEI message specifies a base NNPF.

–	This SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS.

...

nnpfc_out_colour_format_idc, when nnpfc_purpose & 0x20 is not equal to 0, specifies the colour format of the NNPF output and consequently the values of the variables outSubWidthC and outSubHeightC. nnpfc_out_colour_format_idc equal to 1 specifies that the colour format of the NNPF output is the 4:2:0 format and outSubWidthC and outSubHeightC are both equal to 2. nnpfc_out_colour_format_idc equal to 2 specifies that the colour format of the NNPF output is the 4:2:2 format and outSubWidthC is equal to 2 and outSubHeightC is equal to 1. nnpfc_out_colour_format_idc equal to 3 specifies that the colour format of the NNPF output is the 4:2:4 format and outSubWidthC and outSubHeightC are both equal to 1. The value of nnpfc_out_colour_format_idc shall not be equal to 0.

When nnpfc_purpose & 0x02 and nnpfc_purpose & 0x20 are both equal to 0, outSubWidthC and outSubHeightC are inferred to be equal to SubWidthC and SubHeightC, respectively.

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 , inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight  >>  4 to CroppedHeight * 16, inclusive.

nnpfc_hor_ratio_num_minus1 and nnpfc_hor_ratio_den_minus1 specify the horizontal luma scaling ratio nnpfcHorScalingRatio as follows:

nnpfcHorScalingRatio = (nnpfc_hor_ratio_num_minus1 + 1) ÷ ( nnpfc_hor_ratio_den_minus1 + 1 )	(X )

The value of nnpfcHorScalingRatio shall be in the range of 1/16 to 16, inclusive. When nnpfc_hor_ratio_num_minus1 and nnpfc_hor_ratio_den_minus1 are not present, the value of nnpfcHorScalingRatio is inferred to be equal to 1.

The value of the variable nnpfcOutputPicWidth, representing the width of the luma sample array of the picture(s) resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture, is derived as follows:

nnpfcOutputPicWidth = CroppedWidth * nnpfcHorScalingRatio	(X )

nnpfc_ver_ratio_num_minus1 and nnpfc_ver_ratio_den_minus1 specify the vertical luma scaling ratio nnpfcVerScalingRatio is derived as follows:

nnpfcVerScalingRatio = (nnpfc_ver_ratio_num_minus1 + 1) ÷ ( nnpfc_ver_ratio_den_minus1 + 1 )	(X )

The value of nnpfcVerScalingRatio shall be in the range of 1/16 to 16, inclusive. When nnpfc_ver_ratio_num_minus1 and nnpfc_ver_ratio_den_minus1 are not present, the value of nnpfcVerScalingRatio is inferred to be equal to 1.

The value of the variable nnpfcOutputPicHeight, representing the height of the luma sample array of the picture(s) resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture, is derived as follows:

nnpfcOutputPicHeight = CroppedHeight * nnpfcHorScalingRatio	(X )

When nnpfc_purpose & 0x04 is not equal to 0, at least one of the following conditions shall be true:

–	(nnpfc_pic_width_in_luma_samples is greater than CroppedWidth and nnpfc_pic_height_in_luma_samples is not less than CroppedHeight), or (nnpfc_pic_height_in_luma_samples is greater than CroppedHeight and nnpfc_pic_width_in_luma_samples is not less than CroppedWidth).

–	(nnpfc_pic_width_in_luma_samples is less than CroppedWidth and nnpfc_pic_height_in_luma_samples is not greater than CroppedHeight), or (nnpfc_pic_height_in_luma_samples is less than CroppedHeight and nnpfc_pic_width_in_luma_samples is not greater than CroppedWidth).

nnpfc_num_input_pics_minus1 plus 1 specifies the number of decoded output pictures used as input for the NNPF. The value of nnpfc_num_input_pics_minus1 shall be in the range of 0 to 63, inclusive. When nnpfc_purpose & 0x08 is not equal to 0, the value of nnpfc_num_input_pics_minus1 shall be greater than 0.

...

nnpfc_out_format_idc equal to 0 indicates that the sample values output by the NNPF are real numbers where the value range of 0 to 1, inclusive, maps linearly to the unsigned integer value range of 0 to ( 1  <<  bitDepth ) – 1, inclusive, for any desired bit depth bitDepth for subsequent post-processing or displaying.

nnpfc_out_format_idc equal to 1 indicates that the luma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_luma_bitdepth_minus8 + 8 ) ) − 1, inclusive, and the chroma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 ) ) − 1, inclusive.

Values of nnpfc_out_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_out_format_idc.

nnpfc_out_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the output integer tensor. The value of nnpfc_out_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the output integer tensor. The value of nnpfc_out_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

When nnpfc_purpose & 0x10 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	(nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is greater than BitDepthY and nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is not less than BitDepthC), or (nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is greater than BitDepthC and nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is not less than BitDepthY)

–	(nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is less than BitDepthY and nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is not greater than BitDepthC) or (nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is less than BitDepthC and nnpfc_out_tensor_luma_bitdepth_minus1 + 1 is not greater than BitDepthY).

nnpfc_out_order_idc indicates the output order of samples resulting from the NNPF.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

...

Proposal option 3

The proposed text changes are a subset of the proposed text changes in option 1/1a or option 2/2a.

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts):

### 8.28.1	Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| nnpfc_base_flag | u(1) |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( ( nnpfc_purpose & 0x02 ) != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ( nnpfc_purpose & 0x20 ) != 0 ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ( nnpfc_purpose & 0x04 ) != 0 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| if( ( nnpfc_purpose & 0x08 ) != 0 ) { |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| } |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| nnpfc_luma_padding_val | ue(v) |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

### 8.28.2	Neural-network post-filter characteristics SEI message semantics

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified neural-network post-processing filters (NNPFs) for specific pictures is indicated with neural-network post-filter activation (NNPFA) SEI messages.

Use of this SEI message requires the definition of the following variables:

–	Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	Luma sample array CroppedYPic[ idx ] and chroma sample arrays CroppedCbPic[ idx ] and CroppedCrPic[ idx ], when present, of the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive, that are used as input for the NNPF.

–	Bit depth BitDepthY for the luma sample array of the input pictures.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	When nnpfc_auxiliary_inp_idc is equal to 1, a filtering strength control value StrengthControlVal that shall be a real number in the range of 0 to 1, inclusive.

Input picture with index 0 corresponds to the picture for which the NNPFdefined by this NNPFC SEI message is activated by an NNPFA SEI message. Input picture with index i in the range of 1 to numInputPics − 1, inclusive, precedes the input picture with index i − 1 in output order.

When nnpfc_purpose & 0x08 is not equal to 0 and the input picture with index 0 is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, all input pictures are associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

NOTE 1 – More than one NNPFC SEI message can be present for the same picture. When more than one NNPFC SEI message with different values of nnpfc_id is present or activated for the same picture, they can have the same or different values of nnpfc_purpose and nnpfc_mode_idc.

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 63, inclusive, in bitstreams conforming to this edition of this document. Values of 64 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 64 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| nnpfc_purpose = = 0 | May be used as determined by the application |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x01 ) = = 0 | No general visual quality improvement |
| ( nnpfc_purpose & 0x01 ) != 0 | With general visual quality improvement |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x02 ) = = 0 | No chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x02 ) != 0 | With chroma upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x04 ) = = 0 | No resolution upresampling (increasing or deceasing the width or height) |
| ( nnpfc_purpose & 0x04 ) != 0 | With resolution upresampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x08 ) = = 0 | No picture rate upsampling |
| ( nnpfc_purpose & 0x08 ) != 0 | With picture rate upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x10 ) = = 0 | No bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| ( nnpfc_purpose & 0x10 ) != 0 | With bit depth upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x20 ) = = 0 | No colourization (from the 4:0:0 chroma format to the 4:2:0, 4:2:2, or 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x20 ) != 0 | With colourization |

NOTE 2– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When ChromaFormatIdc is equal to 3, nnpfc_purpose & 0x02 shall be equal to 0.

When ChromaFormatIdc or nnpfc_purpose & 0x02 is not equal to 0, nnpfc_purpose & 0x20 shall be equal to 0.

nnpfc_id contains an identifying number that may be used to identify an NNPF. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFC SEI message with nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

When an NNPFC SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the following applies:

–	This SEI message specifies a base NNPF.

–	This SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS.

...

nnpfc_out_colour_format_idc, when nnpfc_purpose & 0x20 is not equal to 0, specifies the colour format of the NNPF output and consequently the values of the variables outSubWidthC and outSubHeightC. nnpfc_out_colour_format_idc equal to 1 specifies that the colour format of the NNPF output is the 4:2:0 format and outSubWidthC and outSubHeightC are both equal to 2. nnpfc_out_colour_format_idc equal to 2 specifies that the colour format of the NNPF output is the 4:2:2 format and outSubWidthC is equal to 2 and outSubHeightC is equal to 1. nnpfc_out_colour_format_idc equal to 3 specifies that the colour format of the NNPF output is the 4:2:4 format and outSubWidthC and outSubHeightC are both equal to 1. The value of nnpfc_out_colour_format_idc shall not be equal to 0.

When nnpfc_purpose & 0x02 and nnpfc_purpose & 0x20 are both equal to 0, outSubWidthC and outSubHeightC are inferred to be equal to SubWidthC and SubHeightC, respectively.

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth  >>  4 to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight  >>  4 to CroppedHeight * 16 − 1, inclusive.

nnpfc_num_input_pics_minus1 plus 1 specifies the number of decoded output pictures used as input for the NNPF. The value of nnpfc_num_input_pics_minus1 shall be in the range of 0 to 63, inclusive. When nnpfc_purpose & 0x08 is not equal to 0, the value of nnpfc_num_input_pics_minus1 shall be greater than 0.

...

nnpfc_out_format_idc equal to 0 indicates that the sample values output by the NNPF are real numbers where the value range of 0 to 1, inclusive, maps linearly to the unsigned integer value range of 0 to ( 1  <<  bitDepth ) – 1, inclusive, for any desired bit depth bitDepth for subsequent post-processing or displaying.

nnpfc_out_format_idc equal to 1 indicates that the luma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_luma_bitdepth_minus8 + 8 ) ) − 1, inclusive, and the chroma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 ) ) − 1, inclusive.

Values of nnpfc_out_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_out_format_idc.

nnpfc_out_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the output integer tensor. The value of nnpfc_out_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the output integer tensor. The value of nnpfc_out_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

When nnpfc_purpose & 0x10 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is greater than BitDepthY, or nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is greater than BitDepthC

nnpfc_out_order_idc indicates the output order of samples resulting from the NNPF.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

...

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).