| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0094-v1 |
| --- | --- |

| Title: | AHG9: On the NNPFC SEI message involving depth pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin1, Ye-Kui Wang2, Jizheng Xu2, Yue Li2, Junru Li3, Kai Zhang2, Li Zhang2 1Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 28910 University Center Lane San Diego, CA 92122, USA 3Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | linchaoyi.cy@bytedance.com yekui.wang@bytedance.com xujizheng@bytedance.com yue.li@bytedance.com lijunru@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Depth picture generation is an important task of computer vision, which can be used in a variety of applications, such as 3D scene modeling and VR. The depth pictures might be generated from decoded output texture pictures by an NNPF, as indicated by the following sentences in the meeting minutes of the previous JVET meeting:

"During the discussion, it was mentioned that an additional example where new pictures (or components of other modality) are generated at the output of an NNPF could be depth maps generated from video pictures."

This contribution proposes to signal depth generation task as an additional purpose for post processing filter in NNPFC SEI message. It is proposed to use one of the following three options:

- Add depth generation as an additional NNPF purpose.

- Depth pictures may be used as input to an NNPF to assist other post processing tasks.

- Combination of options 1 and 2: Add depth generation as an additional NNPF purpose, and depth pictures may be used as input to an NNPF to assist post processing tasks.

# Proposal option 1

The proposed text changes for this option are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

### 8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| nnpfc_out_format_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| if( ( nnpfc_purpose & 0x40 ) != 0 ) |  |
| nnpfc_out_tensor_depth_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| ... |  |
| } |  |

### 8.28.2 Neural-network post-filter characteristics SEI message semantics

...

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 63 127, inclusive, in bitstreams conforming to this edition of this document. Values of 64 128 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 64 128 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| nnpfc_purpose = = 0 | May be used as determined by the application |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x01 ) = = 0 | No general visual quality improvement |
| ( nnpfc_purpose & 0x01 ) != 0 | With general visual quality improvement |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x02 ) = = 0 | No chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x02 ) != 0 | With chroma upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x04 ) = = 0 | No resolution upsampling (increasing the width or height) |
| ( nnpfc_purpose & 0x04 ) != 0 | With resolution upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x08 ) = = 0 | No picture rate upsampling |
| ( nnpfc_purpose & 0x08 ) != 0 | With picture rate upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x10 ) = = 0 | No bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| ( nnpfc_purpose & 0x10 ) != 0 | With bit depth upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x20 ) = = 0 | No colourization (from the 4:0:0 chroma format to the 4:2:0, 4:2:2, or 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x20 ) != 0 | With colourization |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x40 ) = = 0 | No depth picture generation |
| ( nnpfc_purpose & 0x40 ) != 0 | With depth picture generation |

NOTE 2– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When ChromaFormatIdc is equal to 3, nnpfc_purpose & 0x02 shall be equal to 0.

When ChromaFormatIdc or nnpfc_purpose & 0x02 is not equal to 0, nnpfc_purpose & 0x20 shall be equal to 0.

...

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight to CroppedHeight * 16 − 1, inclusive.

When nnpfc_purpose & 0x40 is not equal to 0, nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples also specify the width and height, respectively, of the depth sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output depth picture.

...

nnpfc_out_format_idc equal to 0 indicates that the sample values output by the NNPF are real numbers where the value range of 0 to 1, inclusive, maps linearly to the unsigned integer value range of 0 to ( 1  <<  bitDepth ) − 1, inclusive, for any desired bit depth bitDepth for subsequent post-processing or displaying.

nnpfc_out_format_idc equal to 1 indicates that the luma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_luma_bitdepth_minus8 + 8 ) ) − 1, inclusive, and the chroma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 ) ) − 1, inclusive., and when nnpfc_purpose & 0x40 is not equal to 0, the depth sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_depth_bitdepth_minus8 + 8 ) ) − 1, inclusive.

Values of nnpfc_out_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_out_format_idc.

nnpfc_out_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the output integer tensor. The value of nnpfc_out_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the output integer tensor. The value of nnpfc_out_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

When nnpfc_purpose & 0x10 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is greater than BitDepthY.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is greater than BitDepthC.

nnpfc_out_tensor_depth_bitdepth_minus8 plus 8 specifies the bit depth of depth sample values in the output integer tensor. The value of nnpfc_out_tensor_depth_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_order_idc indicates the output order of samples resulting from the NNPF.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3 4, inclusive, in bitstreams conforming to this edition of this document. Values of 4 5 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 5 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_out_order_idc shall not be equal to 3.

Table 22 contains an informative description of nnpfc_out_order_idc values.

Table 22 – Description of nnpfc_out_order_idc values

| nnpfc_out_ order_idc | Description |
| --- | --- |
| 0 | If nnpfc_purpose & 0x40 is equal to 0, only the luma matrix is present in the output tensor, thus the number of channels is 1. Otherwise (nnpfc_purpose & 0x40 is not equal to 0), the luma and depth matrices are present in the output tensor, thus the number of channels is 2. |
| 1 | If nnpfc_purpose & 0x40 is equal to 0, only the chroma matrices are present in the output tensor, thus the number of channels is 2. Otherwise (nnpfc_purpose & 0x40 is not equal to 0), the chroma and depth matrices are present in the output tensor, thus the number of channels is 3. |
| 2 | If nnpfc_purpose & 0x40 is equal to 0, the luma and chroma matrices are present in the output tensor, thus the number of channels is 3. Otherwise (nnpfc_purpose & 0x40 is not equal to 0), the luma, chroma, and depth matrices are present in the output tensor, thus the number of channels is 4. |
| 3 | If nnpfc_purpose & 0x40 is equal to 0, four luma matrices and two chroma matrices are present in the output tensor, thus the number of channels is 6. Otherwise (nnpfc_purpose & 0x40 is not equal to 0), four luma matrices and two chroma matrices and four depth matrices are present in the output tensor, thus the number of channels is 10. nnpfc_out_order_idc equal to 3 can only be used when the output chroma format is 4:2:0. |
| 4 | Only the depth matrix is present in the output tensor, thus the number of channels is 1. nnpfc_out_order_idc equal to 4 can only be used when nnpfc_purpose & 0x40 is not equal to 0. |
| 4 5..255 | Reserved |

The process StoreOutputTensors( ), for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numOutputPics; i++ ) {
	if( nnpfc_out_order_idc  = =  0 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < nnpfc_pic_height_in_luma_samples  &&  xY < nnpfc_pic_width_in_luma_samples )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						if( ( nnpfc_purpose & 0x40 )  !=  0 )
							FilteredDPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						if( ( nnpfc_purpose & 0x40 )  !=  0 ) 
							FilteredDPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  1 ) {			(85)
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {
				xSrc = cLeft * horCScaling + xP
				ySrc = cTop * verCScaling + yP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / outSubHeightC  &&
						xSrc < nnpfc_pic_width_in_luma_samples / outSubWidthC )
					if( !nnpfc_component_last_flag ) {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					}
			}
		if( ( nnpfc_purpose & 0x40 )  !=  0 ) {
			for( yP = 0; yP < outPatchHeight; yP++)
				for( xP = 0; xP < outPatchWidth; xP++ ) {
					yY = cTop * outPatchHeight / inpPatchHeight + yP
					xY = cLeft * outPatchWidth / inpPatchWidth + xP
					if ( yY < nnpfc_pic_height_in_luma_samples  &&  xY < nnpfc_pic_width_in_luma_samples )
						if( !nnpfc_component_last_flag )
							FilteredDPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						else
							FilteredDPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
		}
	}
	else if( nnpfc_out_order_idc  = =  2 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC 
				yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC
				if ( yY < nnpfc_pic_height_in_luma_samples && xY < nnpfc_pic_width_in_luma_samples)
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 1 ][ yPc ][ xPc ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 2 ][ yPc ][ xPc ] 
						if( ( nnpfc_purpose & 0x40 )  !=  0 ) 
							FilteredDPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 1 ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 2 ] 
						if( ( nnpfc_purpose & 0x40 )  !=  0 ) 
							FilteredDPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  3 )
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / 2  &&
						xSrc < nnpfc_pic_width_in_luma_samples / 2 )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 4 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 5 ][ yP ][ xP ] 
						if( ( nnpfc_purpose & 0x40 )  !=  0 ) {
							FilteredDPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 6 ][ yP ][ xP ]
							FilteredDPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 7 ][ yP ][ xP ]
							FilteredDPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 8 ][ yP ][ xP ]
							FilteredDPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 9 ][ yP ][ xP ]
						}
					} else {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 4 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 5 ] 
						if( ( nnpfc_purpose & 0x40 )  !=  0 ) {
							FilteredDPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 6 ]
							FilteredDPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 7 ]
							FilteredDPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 8 ]
							FilteredDPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 9 ]
						}
					}
			}
	else if( nnpfc_out_order_idc  = =  4 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < nnpfc_pic_height_in_luma_samples  &&
							xY < nnpfc_pic_width_in_luma_samples )
					if( !nnpfc_component_last_flag )
						FilteredDPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
					else
						FilteredDPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
			}
}

...

# Proposal option 2

The proposed text changes for this option are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

### 8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| nnpfc_input_depth_flag | u(1) |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| if( nnpfc_input_depth_flag ) |  |
| nnpfc_inp_tensor_depth_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| ... |  |
| nnpfc_out_format_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| ... |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| nnpfc_luma_padding_val | ue(v) |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| if( nnpfc_input_depth_flag ) |  |
| nnpfc_depth_padding_val | ue(v) |
| } |  |
| ... |  |
| } |  |

### 8.28.2 Neural-network post-filter characteristics SEI message semantics

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified neural-network post-processing filters (NNPFs) for specific pictures is indicated with neural-network post-filter activation (NNPFA) SEI messages.

Use of this SEI message requires the definition of the following variables:

–	Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	Luma sample array CroppedYPic[ idx ], and chroma sample arrays CroppedCbPic[ idx ] and CroppedCrPic[ idx ], and depth sample array CroppedDPic[ idx ], when present, of the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive, that are used as input for the NNPF.

–	Bit depth BitDepthY for the luma sample array of the input pictures.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

–	Bit depth BitDepthD for the depth sample arrays, if any, of the input pictures.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	When nnpfc_auxiliary_inp_idc is equal to 1, a filtering strength control value StrengthControlVal that shall be a real number in the range of 0 to 1, inclusive.

Input picture with index 0 corresponds to the picture for which the NNPFdefined by this NNPFC SEI message is activated by an NNPFA SEI message. Input picture with index i in the range of 1 to numInputPics − 1, inclusive, precedes the input picture with index i − 1 in output order.

When nnpfc_purpose & 0x08 is not equal to 0 and the input picture with index 0 is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, all input pictures are associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

...

nnpfc_input_depth_flag equal to 1 indicates that depth pictures associated with the cropped decoded output picture are used as input to the NNPF. nnpfc_input_depth_flag equal to 0 indicates that no depth picture is used as input to the NNPF.

nnpfc_component_last_flag equal to 1 indicates that the last dimension in the input tensor inputTensor to the NNPF and the output tensor outputTensor resulting from the NNPF is used for a current channel. nnpfc_component_last_flag equal to 0 indicates that the third dimension in the input tensor inputTensor to the NNPF and the output tensor outputTensor resulting from the NNPF is used for a current channel.

NOTE 4 – The first dimension in the input tensor and in the output tensor is used for the batch index, which is a practice in some neural network frameworks. While formulae in the semantics of this SEI message use the batch size corresponding to the batch index equal to 0, it is up to the post-processing implementation to determine the batch size used as input to the neural network inference.

NOTE 5 – For example, when nnpfc_inp_order_idc is equal to 3 and nnpfc_auxiliary_inp_idc is equal to 1, there are 7 channels in the input tensor, including four luma matrices, two chroma matrices, and one auxiliary input matrix. In this case, the process DeriveInputTensors( ) would derive each of these 7 channels of the input tensor one by one, and when a particular channel of these channels is processed, that channel is referred to as the current channel during the process.

nnpfc_inp_format_idc indicates the method of converting a sample value of the cropped decoded output picture to an input value to the NNPF. When nnpfc_inp_format_idc is equal to 0, the input values to the NNPF are real numbers and the functions InpY( ) and, InpC( ), and InpD( ) are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) − 1 )			(77)

InpC( x )= x ÷ ( ( 1  <<  BitDepthC ) − 1 )			(78)

InpD( x )= x ÷ ( ( 1  <<  BitDepthD ) − 1 )			(xx)

When nnpfc_inp_format_idc is equal to 1, the input values to the NNPF are unsigned integer numbers and the functions InpY( ) and, InpC( ), and InpD( ) are specified as follows:

shiftY = BitDepthY − inpTensorBitDepthY
if( inpTensorBitDepthY  >=  BitDepthY)
	InpY( x ) = x  <<  ( inpTensorBitDepthY − BitDepthY )	(79)
else
	InpY( x ) = Clip3(0, ( 1  <<  inpTensorBitDepthY ) − 1, ( x + ( 1  <<  ( shiftY − 1 ) ) )  >>  shiftY )

shiftC = BitDepthC − inpTensorBitDepthC
if( inpTensorBitDepthC  >=  BitDepthC )
	InpC( x ) = x  <<  ( inpTensorBitDepthC − BitDepthC ) 	(80)
else
	InpC( x ) = Clip3(0, ( 1  <<  inpTensorBitDepthC ) − 1, ( x + ( 1  <<  ( shiftC − 1 ) ) )  >>  shiftC )

shiftD = BitDepthD − inpTensorBitDepthD
if( inpTensorBitDepthD  >=  BitDepthD )
	InpD( x ) = x  <<  ( inpTensorBitDepthD − BitDepthD )	(xx)
else
	InpD( x ) = Clip3(0, ( 1  <<  inpTensorBitDepthD ) − 1, ( x + ( 1  <<  ( shiftD − 1 ) ) )  >>  shiftD )

The variable inpTensorBitDepthY is derived from the syntax element nnpfc_inp_tensor_luma_bitdepth_minus8 as specified below. The variable inpTensorBitDepthC is derived from the syntax element nnpfc_inp_tensor_chroma_bitdepth_minus8 as specified below. The variable inpTensorBitDepthD is derived from the syntax element nnpfc_inp_tensor_depth_bitdepth_minus8 as specified below.

Values of nnpfc_inp_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_inp_format_idc.

nnpfc_inp_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the input integer tensor. The value of inpTensorBitDepthY is derived as follows:

inpTensorBitDepthY = nnpfc_inp_tensor_luma_bitdepth_minus8 + 8	(81)

It is a requirement of bitstream conformance that the value of nnpfc_inp_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_inp_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the input integer tensor. The value of inpTensorBitDepthC is derived as follows:

inpTensorBitDepthC = nnpfc_inp_tensor_chroma_bitdepth_minus8 + 8	(82)

It is a requirement of bitstream conformance that the value of nnpfc_inp_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_inp_tensor_depth_bitdepth_minus8 plus 8 specifies the bit depth of depth sample values in the input integer tensor. The value of inpTensorBitDepthD is derived as follows:

inpTensorBitDepthD = nnpfc_inp_tensor_depth_bitdepth_minus8 + 8	(xx)

nnpfc_inp_order_idc indicates the method of ordering the sample arrays of a cropped decoded output picture as one of the input pictures to the NNPF.

The value of nnpfc_inp_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_inp_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_inp_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_inp_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When ChromaFormatIdc is not equal to 1, nnpfc_inp_order_idc shall not be equal to 3.

Table 21 contains an informative description of nnpfc_inp_order_idc values.

Table 21 – Description of nnpfc_inp_order_idc values

| nnpfc_inp_ order_idc | Description |
| --- | --- |
| 0 | If nnpfc_input_depth_flag is equal to 0, the following applies: if If nnpfc_auxiliary_inp_idc is equal to 0, one luma matrix is present in the input tensor for each input picture, and the number of channels is 1; otherwise when nnpfc_auxiliary_inp_idc is equal to 1, one luma matrix and one auxiliary input matrix are present, and the number of channels is 2. Otherwise (nnpfc_input_depth_flag is equal to 1), the following applies: if nnpfc_auxiliary_inp_idc is equal to 0, one luma matrix and one depth matrix are present in the input tensor for each input picture, and the number of channels is 2; otherwise when nnpfc_auxiliary_inp_idc is equal to 1, one luma matrix, one depth matrix and one auxiliary input matrix are present, and the number of channels is 3. |
| 1 | If nnpfc_input_depth_flag is equal to 0, the following applies: if If nnpfc_auxiliary_inp_idc is equal to 0, two chroma matrices are present in the input tensor, and the number of channels is 2. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, two chroma matrices and one auxiliary input matrix are present, and the number of channels is 3. Otherwise (nnpfc_input_depth_flag is equal to 1), the following applies: if nnpfc_auxiliary_inp_idc is not equal to 0, two chroma matrices and one depth matrix are present in the input tensor, and the number of channels is 3. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, two chroma matrices, one depth matrix, and one auxiliary input matrix are present, and the number of channels is 4. |
| 2 | If nnpfc_input_depth_flag is equal to 0, the following applies: if If nnpfc_auxiliary_inp_idc is equal to 0, one luma and two chroma matrices are present in the input tensor, and the number of channels is 3. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, one luma matrix, two chroma matrices and one auxiliary input matrix are present, and the number of channels is 4. Otherwise (nnpfc_input_depth_flag is equal to 1), the following applies: if nnpfc_auxiliary_inp_idc is equal to 0, one luma, two chroma matrices, and one depth matrix are present in the input tensor, and the number of channels is 4. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, one luma matrix, two chroma matrices, one depth matrix and one auxiliary input matrix are present, and the number of channels is 5. |
| 3 | If nnpfc_input_depth_flag is equal to 0, the following applies: if If nnpfc_auxiliary_inp_idc is equal to 0, four luma matrices and two chroma matrices are present in the input tensor, and the number of channels is 6. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, four luma matrices, two chroma matrices, and one auxiliary input matrix are present in the input tensor, and the number of channels is 7. Otherwise (nnpfc_input_depth_flag is equal to 1), the following applies: if nnpfc_auxiliary_inp_idc is equal to 0, four luma matrices, two chroma matrices, and one depth matrix are present in the input tensor, and the number of channels is 10. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, four luma matrices, two chroma matrices, and one auxiliary input matrix are present in the input tensor, and the number of channels is 11. The luma channels are derived in an interleaved manner as illustrated in Figure 12. This nnpfc_inp_order_idc can only be used when the input chroma format is 4:2:0. |
| 4..255 | Reserved |

Figure 12 – Illustration of deriving the four luma channles (right) from the luma component (left) when nnpfc_inp_order_idc is equal to 3

A patch is a rectangular array of samples from a component (e.g., a luma or chroma component) of a picture.

nnpfc_auxiliary_inp_idc greater than 0 indicates that auxiliary input data is present in the input tensor of the NNPF. nnpfc_auxiliary_inp_idc equal to 0 indicates that auxiliary input data is not present in the input tensor. nnpfc_auxiliary_inp_idc equal to 1 specifies that auxiliary input data is derived as specified in Formula 84.

The value of nnpfc_auxiliary_inp_idc shall be in the range of 0 to 1, inclusive, in bitstreams conforming to this edition of this document. Values of 2 to 255, inclusive, for nnpfc_inp_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_inp_order_idc in the range of 2 to 255, inclusive. Values of nnpfc_inp_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When nnpfc_auxiliary_inp_idc is equal to 1, the variable strengthControlScaledVal is derived as follows:

if( nnpfc_inp_format_idc  = =  1 )
	strengthControlScaledVal = Floor ( StrengthControlVal * ( ( 1  <<  inpTensorBitDepthY ) − 1 ) )	(83)
else
	strengthControlScaledVal = StrengthControlVal

The process DeriveInputTensors( ), for deriving the input tensor inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numInputPics; i++ ) {
	if( nnpfc_inp_order_idc  = =  0 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpVal = InpY( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) ) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( nnpfc_input_depth_flag ) {
					inpDVal = InpD( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight,
						CroppedWidth, CroppedDPic[ i ] ) )
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpDVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpDVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = strengthControlScaledVal
				} else {
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpVal
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = strengthControlScaledVal
				}
			}
	else if( nnpfc_inp_order_idc  = =  1 )				(84)
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpCbVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ] ) )
				inpCrVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ] ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( nnpfc_input_depth_flag ) {
					yY = cTop + yP
					xY = cLeft + xP
					yC = yY / SubHeightC
					xC = xY / SubWidthC
					inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
							CroppedWidth / SubWidthC, CroppedCbPic[ i ] ) )
					inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
							CroppedWidth / SubWidthC, CroppedCrPic[ i ] ) ) 
					inpDVal = InpD( InpSampleVal( yY, xY, CroppedHeight,
							CroppedWidth, CroppedDPic[ i ] ) ) 
					yPovlp = yP + nnpfc_overlap
					xPovlp = xP + nnpfc_overlap
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpCbVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCrVal
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpDVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpCbVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCrVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpDVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = strengthControlScaledVal
				} else {
					inpCbVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
							CroppedWidth / SubWidthC, CroppedCbPic[ i ] ) )
					inpCrVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
							CroppedWidth / SubWidthC, CroppedCrPic[ i ] ) )
					yPovlp = yP + nnpfc_overlap
					xPovlp = xP + nnpfc_overlap
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpCbVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCrVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpCbVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCrVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = strengthControlScaledVal
				}
			}
	else if( nnpfc_inp_order_idc  = =  2 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yY = cTop + yP
				xY = cLeft + xP
				yC = yY / SubHeightC
				xC = xY / SubWidthC
				inpYVal = InpY( InpSampleVal( yY, xY, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ] ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ] ) ) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( nnpfc_input_depth_flag ) {
					inpDVal = InpD( InpSampleVal( yY, xY, CroppedHeight,
							CroppedWidth, CroppedDPic[ i ] ) )
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpYVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCbVal
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpCrVal
						inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = inpDVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpYVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCbVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpCrVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = inpDVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 4 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 4 ] = strengthControlScaledVal
				} else {
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpYVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCbVal
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpCrVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpYVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCbVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpCrVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = strengthControlScaledVal
				}
			}
	else if( nnpfc_inp_order_idc  = =  3 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yTL = cTop + yP * 2
				xTL = cLeft + xP * 2
				yBR = yTL + 1
				xBR = xTL + 1
				yC = cTop / 2 + yP
				xC = cLeft / 2 + xP
				inpTLVal = InpY( InpSampleVal( yTL, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpTRVal = InpY( InpSampleVal( yTL, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpBLVal = InpY( InpSampleVal( yBR, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpBRVal = InpY( InpSampleVal( yBR, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCbPic[ i ] ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCrPic[ i ] ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap

if( nnpfc_input_depth_flag ) {
					inpDTLVal = InpD( InpSampleVal( yTL, xTL, CroppedHeight,
							CroppedWidth, CroppedDPic[ i ] ) )
					inpDTRVal = InpD( InpSampleVal( yTL, xBR, CroppedHeight,
							CroppedWidth, CroppedDPic[ i ] ) )
					inpDBLVal = InpD( InpSampleVal( yBR, xTL, CroppedHeight,
							CroppedWidth, CroppedDPic[ i ] ) )
					inpDBRVal = InpD( InpSampleVal( yBR, xBR, CroppedHeight,
							CroppedWidth, CroppedDPic[ i ] ) )
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpTLVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpTRVal
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpBLVal
						inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = inpBRVal
						inputTensor[ 0 ][ i ][ 4 ][ yPovlp ][ xPovlp ] = inpCbVal
						inputTensor[ 0 ][ i ][ 5 ][ yPovlp ][ xPovlp ] = inpCrVal
						inputTensor[ 0 ][ i ][ 6 ][ yPovlp ][ xPovlp ] = inpDTLVal
						inputTensor[ 0 ][ i ][ 7 ][ yPovlp ][ xPovlp ] = inpDTRVal
						inputTensor[ 0 ][ i ][ 8 ][ yPovlp ][ xPovlp ] = inpDBLVal
						inputTensor[ 0 ][ i ][ 9 ][ yPovlp ][ xPovlp ] = inpDBRVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpTLVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpTRVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpBLVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = inpBRVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 4 ] = inpCbVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 5 ] = inpCrVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 6 ] = inpDTLVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 7 ] = inpDTRVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 8 ] = inpDBLVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 9 ] = inpDBRVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 10 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 10 ] = strengthControlScaledVal
				} else {
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpTLVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpTRVal
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpBLVal
						inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = inpBRVal
						inputTensor[ 0 ][ i ][ 4 ][ yPovlp ][ xPovlp ] = inpCbVal
						inputTensor[ 0 ][ i ][ 5 ][ yPovlp ][ xPovlp ] = inpCrVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpTLVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpTRVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpBLVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = inpBRVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 4 ] = inpCbVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 5 ] = inpCrVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 6 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 6 ] = strengthControlScaledVal
				}
			}
}

...

nnpfc_out_format_idc equal to 0 indicates that the sample values output by the NNPF are real numbers where the value range of 0 to 1, inclusive, maps linearly to the unsigned integer value range of 0 to ( 1  <<  bitDepth ) − 1, inclusive, for any desired bit depth bitDepth for subsequent post-processing or displaying.

nnpfc_out_format_idc equal to 1 indicates that the luma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_luma_bitdepth_minus8 + 8 ) ) − 1, inclusive, and the chroma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 ) ) − 1, inclusive.

Values of nnpfc_out_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_out_format_idc.

nnpfc_out_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the output integer tensor. The value of nnpfc_out_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the output integer tensor. The value of nnpfc_out_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

When nnpfc_purpose & 0x10 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is greater than BitDepthY.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is greater than BitDepthC.

nnpfc_out_order_idc indicates the output order of samples resulting from the NNPF.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_out_order_idc shall not be equal to 3.

Table 22 contains an informative description of nnpfc_out_order_idc values.

Table 22 – Description of nnpfc_out_order_idc values

| nnpfc_out_ order_idc | Description |
| --- | --- |
| 0 | Only the luma matrix is present in the output tensor, thus the number of channels is 1. |
| 1 | Only the chroma matrices are present in the output tensor, thus the number of channels is 2. |
| 2 | The luma and chroma matrices are present in the output tensor, thus the number of channels is 3. |
| 3 | Four luma matrices and two chroma matrices are present in the output tensor, thus the number of channels is 6. This nnpfc_out_order_idc can only be used when the output chroma format is 4:2:0. |
| 4..255 | Reserved |

The process StoreOutputTensors( ), for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numOutputPics; i++ ) {
	if( nnpfc_out_order_idc  = =  0 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < nnpfc_pic_height_in_luma_samples  &&  xY < nnpfc_pic_width_in_luma_samples )
					if( !nnpfc_component_last_flag )
						FilteredYPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
					else
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
			}
	else if( nnpfc_out_order_idc  = =  1 )				(85)
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {
				xSrc = cLeft * horCScaling + xP
				ySrc = cTop * verCScaling + yP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / outSubHeightC  &&
						xSrc < nnpfc_pic_width_in_luma_samples / outSubWidthC )
					if( !nnpfc_component_last_flag ) {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  2 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC 
				yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC
				if ( yY < nnpfc_pic_height_in_luma_samples && xY < nnpfc_pic_width_in_luma_samples)
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 1 ][ yPc ][ xPc ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 2 ][ yPc ][ xPc ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 1 ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 2 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  3 )
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / 2  &&
						xSrc < nnpfc_pic_width_in_luma_samples / 2 )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 4 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 5 ][ yP ][ xP ]
					} else {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 4 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 5 ]
					}
			}
}

...

nnpfc_padding_type indicates the process of padding when referencing sample locations outside the boundaries of the cropped decoded output picture as described in Table 23. The value of nnpfc_padding_type shall be in the range of 0 to 15, inclusive.

Table 23 – Informative description of nnpfc_padding_type values

| nnpfc_padding_type | Description |
| --- | --- |
| 0 | zero padding |
| 1 | replication padding |
| 2 | reflection padding |
| 3 | wrap-around padding |
| 4 | fixed padding |
| 5..15 | Reserved |

nnpfc_luma_padding_val indicates the luma value to be used for padding when nnpfc_padding_type is equal to 4.

nnpfc_cb_padding_val indicates the Cb value to be used for padding when nnpfc_padding_type is equal to 4.

nnpfc_cr_padding_val indicates the Cr value to be used for padding when nnpfc_padding_type is equal to 4.

nnpfc_depth_padding_val indicates the depth value to be used for padding when nnpfc_padding_type is equal to 4. The value of nnpfc_depth_padding_val shall be in the range of 0 to 2nnpfc_inp_tensor_depth_bitdepth_minus8+8-1, inclusive.

The function InpSampleVal( y, x, picHeight, picWidth, croppedPic ) with inputs being a vertical sample location y, a horizontal sample location x, a picture height picHeight, a picture width picWidth, and sample array croppedPic returns the value of sampleVal derived as follows:

NOTE 6 – For the inputs to the function InpSampleVal( ), the vertical location is listed before the horizontal location for compatibility with input tensor conventions of some inference engines.

If( nnpfc_padding_type  = =  0 )
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal = 0
	else
		sampleVal = croppedPic[ x ][ y ]			(92)
else if( nnpfc_padding_type  = =  1 )
	sampleVal = croppedPic[ Clip3( 0, picWidth − 1, x ) ][ Clip3( 0, picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  2 ) 
	sampleVal = croppedPic[ Reflect( picWidth − 1, x ) ][ Reflect( picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  3 ) 
	if( y >= 0  &&  y < picHeight )
		sampleVal = croppedPic[ Wrap( picWidth − 1, x ) ][ y ] 
else if( nnpfc_padding_type  = =  4 ) 
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth ) {
		sampleVal[ 0 ]  = nnpfc_luma_padding_val		sampleVal[ 1 ]  = nnpfc_cb_padding_val
		sampleVal[ 2 ] = nnpfc_cr_padding_val
		if( nnpfc_depth_input_flag )
			sampleVal[ 3 ] = nnpfc_depth_padding_val
	} else
		sampleVal = croppedPic[ x ][ y ]

...

# Proposal option 3

The proposed text changes for this option are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

### 8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| nnpfc_input_depth_flag | u(1) |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| if( nnpfc_input_depth_flag ) |  |
| nnpfc_inp_tensor_depth_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| ... |  |
| nnpfc_out_format_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| if( ( nnpfc_purpose & 0x40 ) != 0 ) |  |
| nnpfc_out_tensor_depth_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| ... |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| nnpfc_luma_padding_val | ue(v) |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| if( nnpfc_input_depth_flag ) |  |
| nnpfc_depth_padding_val | ue(v) |
| } |  |
| ... |  |
| } |  |

### 8.28.2 Neural-network post-filter characteristics SEI message semantics

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified neural-network post-processing filters (NNPFs) for specific pictures is indicated with neural-network post-filter activation (NNPFA) SEI messages.

Use of this SEI message requires the definition of the following variables:

–	Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	Luma sample array CroppedYPic[ idx ], and chroma sample arrays CroppedCbPic[ idx ] and CroppedCrPic[ idx ], and depth sample array CroppedDPic[ idx ], when present, of the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive, that are used as input for the NNPF.

–	Bit depth BitDepthY for the luma sample array of the input pictures.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

–	Bit depth BitDepthD for the depth sample arrays, if any, of the input pictures.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	When nnpfc_auxiliary_inp_idc is equal to 1, a filtering strength control value StrengthControlVal that shall be a real number in the range of 0 to 1, inclusive.

Input picture with index 0 corresponds to the picture for which the NNPFdefined by this NNPFC SEI message is activated by an NNPFA SEI message. Input picture with index i in the range of 1 to numInputPics − 1, inclusive, precedes the input picture with index i − 1 in output order.

When nnpfc_purpose & 0x08 is not equal to 0 and the input picture with index 0 is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, all input pictures are associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

NOTE 1 – More than one NNPFC SEI message can be present for the same picture. When more than one NNPFC SEI message with different values of nnpfc_id is present or activated for the same picture, they can have the same or different values of nnpfc_purpose and nnpfc_mode_idc.

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 63 127, inclusive, in bitstreams conforming to this edition of this document. Values of 64 128 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 64 128 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| nnpfc_purpose = = 0 | May be used as determined by the application |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x01 ) = = 0 | No general visual quality improvement |
| ( nnpfc_purpose & 0x01 ) != 0 | With general visual quality improvement |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x02 ) = = 0 | No chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x02 ) != 0 | With chroma upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x04 ) = = 0 | No resolution upsampling (increasing the width or height) |
| ( nnpfc_purpose & 0x04 ) != 0 | With resolution upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x08 ) = = 0 | No picture rate upsampling |
| ( nnpfc_purpose & 0x08 ) != 0 | With picture rate upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x10 ) = = 0 | No bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| ( nnpfc_purpose & 0x10 ) != 0 | With bit depth upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x20 ) = = 0 | No colourization (from the 4:0:0 chroma format to the 4:2:0, 4:2:2, or 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x20 ) != 0 | With colourization |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x40 ) = = 0 | No depth picture generation |
| ( nnpfc_purpose & 0x40 ) != 0 | With depth picture generation |

NOTE 2– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When ChromaFormatIdc is equal to 3, nnpfc_purpose & 0x02 shall be equal to 0.

When ChromaFormatIdc or nnpfc_purpose & 0x02 is not equal to 0, nnpfc_purpose & 0x20 shall be equal to 0.

...

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight to CroppedHeight * 16 − 1, inclusive.

When nnpfc_purpose & 0x40 is not equal to 0, nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples also specify the width and height, respectively, of the depth sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output depth picture.

...

nnpfc_input_depth_flag equal to 1 indicates that depth pictures associated with the cropped decoded output picture are used as input to the NNPF. nnpfc_input_depth_flag equal to 0 indicates that no depth picture is used as input to the NNPF.

nnpfc_component_last_flag equal to 1 indicates that the last dimension in the input tensor inputTensor to the NNPF and the output tensor outputTensor resulting from the NNPF is used for a current channel. nnpfc_component_last_flag equal to 0 indicates that the third dimension in the input tensor inputTensor to the NNPF and the output tensor outputTensor resulting from the NNPF is used for a current channel.

NOTE 4 – The first dimension in the input tensor and in the output tensor is used for the batch index, which is a practice in some neural network frameworks. While formulae in the semantics of this SEI message use the batch size corresponding to the batch index equal to 0, it is up to the post-processing implementation to determine the batch size used as input to the neural network inference.

NOTE 5 – For example, when nnpfc_inp_order_idc is equal to 3 and nnpfc_auxiliary_inp_idc is equal to 1, there are 7 channels in the input tensor, including four luma matrices, two chroma matrices, and one auxiliary input matrix. In this case, the process DeriveInputTensors( ) would derive each of these 7 channels of the input tensor one by one, and when a particular channel of these channels is processed, that channel is referred to as the current channel during the process.

nnpfc_inp_format_idc indicates the method of converting a sample value of the cropped decoded output picture to an input value to the NNPF. When nnpfc_inp_format_idc is equal to 0, the input values to the NNPF are real numbers and the functions InpY( ) and, InpC( ), and InpD( ) are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) − 1 )			(77)

InpC( x )= x ÷ ( ( 1  <<  BitDepthC ) − 1 )			(78)

InpD( x )= x ÷ ( ( 1  <<  BitDepthD ) − 1 )			(xx)

When nnpfc_inp_format_idc is equal to 1, the input values to the NNPF are unsigned integer numbers and the functions InpY( ) and, InpC( ), and InpD( ) are specified as follows:

shiftY = BitDepthY − inpTensorBitDepthY
if( inpTensorBitDepthY  >=  BitDepthY)
	InpY( x ) = x  <<  ( inpTensorBitDepthY − BitDepthY )	(79)
else
	InpY( x ) = Clip3(0, ( 1  <<  inpTensorBitDepthY ) − 1, ( x + ( 1  <<  ( shiftY − 1 ) ) )  >>  shiftY )

shiftC = BitDepthC − inpTensorBitDepthC
if( inpTensorBitDepthC  >=  BitDepthC )
	InpC( x ) = x  <<  ( inpTensorBitDepthC − BitDepthC ) 	(80)
else
	InpC( x ) = Clip3(0, ( 1  <<  inpTensorBitDepthC ) − 1, ( x + ( 1  <<  ( shiftC − 1 ) ) )  >>  shiftC )

shiftD = BitDepthD − inpTensorBitDepthD
if( inpTensorBitDepthD  >=  BitDepthD )
	InpD( x ) = x  <<  ( inpTensorBitDepthD − BitDepthD )	(xx)
else
	InpD( x ) = Clip3(0, ( 1  <<  inpTensorBitDepthD ) − 1, ( x + ( 1  <<  ( shiftD − 1 ) ) )  >>  shiftD )

The variable inpTensorBitDepthY is derived from the syntax element nnpfc_inp_tensor_luma_bitdepth_minus8 as specified below. The variable inpTensorBitDepthC is derived from the syntax element nnpfc_inp_tensor_chroma_bitdepth_minus8 as specified below. The variable inpTensorBitDepthD is derived from the syntax element nnpfc_inp_tensor_depth_bitdepth_minus8 as specified below.

Values of nnpfc_inp_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_inp_format_idc.

nnpfc_inp_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the input integer tensor. The value of inpTensorBitDepthY is derived as follows:

inpTensorBitDepthY = nnpfc_inp_tensor_luma_bitdepth_minus8 + 8	(81)

It is a requirement of bitstream conformance that the value of nnpfc_inp_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_inp_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the input integer tensor. The value of inpTensorBitDepthC is derived as follows:

inpTensorBitDepthC = nnpfc_inp_tensor_chroma_bitdepth_minus8 + 8	(82)

It is a requirement of bitstream conformance that the value of nnpfc_inp_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_inp_tensor_depth_bitdepth_minus8 plus 8 specifies the bit depth of depth sample values in the input integer tensor. The value of inpTensorBitDepthD is derived as follows:

inpTensorBitDepthD = nnpfc_inp_tensor_depth_bitdepth_minus8 + 8	(xx)

nnpfc_inp_order_idc indicates the method of ordering the sample arrays of a cropped decoded output picture as one of the input pictures to the NNPF.

The value of nnpfc_inp_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_inp_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_inp_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_inp_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When ChromaFormatIdc is not equal to 1, nnpfc_inp_order_idc shall not be equal to 3.

Table 21 contains an informative description of nnpfc_inp_order_idc values.

Table 21 – Description of nnpfc_inp_order_idc values

| nnpfc_inp_ order_idc | Description |
| --- | --- |
| 0 | If nnpfc_input_depth_flag is equal to 0, the following applies: if If nnpfc_auxiliary_inp_idc is equal to 0, one luma matrix is present in the input tensor for each input picture, and the number of channels is 1; otherwise when nnpfc_auxiliary_inp_idc is equal to 1, one luma matrix and one auxiliary input matrix are present, and the number of channels is 2. Otherwise (nnpfc_input_depth_flag is equal to 1), the following applies: if nnpfc_auxiliary_inp_idc is equal to 0, one luma matrix and one depth matrix are present in the input tensor for each input picture, and the number of channels is 2; otherwise when nnpfc_auxiliary_inp_idc is equal to 1, one luma matrix, one depth matrix and one auxiliary input matrix are present, and the number of channels is 3. |
| 1 | If nnpfc_input_depth_flag is equal to 0, the following applies: if If nnpfc_auxiliary_inp_idc is equal to 0, two chroma matrices are present in the input tensor, and the number of channels is 2. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, two chroma matrices and one auxiliary input matrix are present, and the number of channels is 3. Otherwise (nnpfc_input_depth_flag is equal to 1), the following applies: if nnpfc_auxiliary_inp_idc is not equal to 0, two chroma matrices and one depth matrix are present in the input tensor, and the number of channels is 3. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, two chroma matrices, one depth matrix, and one auxiliary input matrix are present, and the number of channels is 4. |
| 2 | If nnpfc_input_depth_flag is equal to 0, the following applies: if If nnpfc_auxiliary_inp_idc is equal to 0, one luma and two chroma matrices are present in the input tensor, and the number of channels is 3. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, one luma matrix, two chroma matrices and one auxiliary input matrix are present, and the number of channels is 4. Otherwise (nnpfc_input_depth_flag is equal to 1), the following applies: if nnpfc_auxiliary_inp_idc is equal to 0, one luma, two chroma matrices, and one depth matrix are present in the input tensor, and the number of channels is 4. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, one luma matrix, two chroma matrices, one depth matrix and one auxiliary input matrix are present, and the number of channels is 5. |
| 3 | If nnpfc_input_depth_flag is equal to 0, the following applies: if If nnpfc_auxiliary_inp_idc is equal to 0, four luma matrices and two chroma matrices are present in the input tensor, and the number of channels is 6. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, four luma matrices, two chroma matrices, and one auxiliary input matrix are present in the input tensor, and the number of channels is 7. Otherwise (nnpfc_input_depth_flag is equal to 1), the following applies: if nnpfc_auxiliary_inp_idc is equal to 0, four luma matrices, two chroma matrices, and one depth matrix are present in the input tensor, and the number of channels is 10. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, four luma matrices, two chroma matrices, and one auxiliary input matrix are present in the input tensor, and the number of channels is 11. The luma channels are derived in an interleaved manner as illustrated in Figure 12. This nnpfc_inp_order_idc can only be used when the input chroma format is 4:2:0. |
| 4..255 | Reserved |

Figure 12 – Illustration of deriving the four luma channles (right) from the luma component (left) when nnpfc_inp_order_idc is equal to 3

A patch is a rectangular array of samples from a component (e.g., a luma or chroma component) of a picture.

nnpfc_auxiliary_inp_idc greater than 0 indicates that auxiliary input data is present in the input tensor of the NNPF. nnpfc_auxiliary_inp_idc equal to 0 indicates that auxiliary input data is not present in the input tensor. nnpfc_auxiliary_inp_idc equal to 1 specifies that auxiliary input data is derived as specified in Formula 84.

The value of nnpfc_auxiliary_inp_idc shall be in the range of 0 to 1, inclusive, in bitstreams conforming to this edition of this document. Values of 2 to 255, inclusive, for nnpfc_inp_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_inp_order_idc in the range of 2 to 255, inclusive. Values of nnpfc_inp_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When nnpfc_auxiliary_inp_idc is equal to 1, the variable strengthControlScaledVal is derived as follows:

if( nnpfc_inp_format_idc  = =  1 )
	strengthControlScaledVal = Floor ( StrengthControlVal * ( ( 1  <<  inpTensorBitDepthY ) − 1 ) )	(83)
else
	strengthControlScaledVal = StrengthControlVal

The process DeriveInputTensors( ), for deriving the input tensor inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numInputPics; i++ ) {
	if( nnpfc_inp_order_idc  = =  0 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpVal = InpY( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) ) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( nnpfc_input_depth_flag ) {
					inpDVal = InpD( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight,
						CroppedWidth, CroppedDPic[ i ] ) )
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpDVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpDVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = strengthControlScaledVal
				} else {
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpVal
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = strengthControlScaledVal
				}
			}
	else if( nnpfc_inp_order_idc  = =  1 )				(84)
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpCbVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ] ) )
				inpCrVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ] ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( nnpfc_input_depth_flag ) {
					yY = cTop + yP
					xY = cLeft + xP
					yC = yY / SubHeightC
					xC = xY / SubWidthC
					inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
							CroppedWidth / SubWidthC, CroppedCbPic[ i ] ) )
					inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
							CroppedWidth / SubWidthC, CroppedCrPic[ i ] ) ) 
					inpDVal = InpD( InpSampleVal( yY, xY, CroppedHeight,
							CroppedWidth, CroppedDPic[ i ] ) ) 
					yPovlp = yP + nnpfc_overlap
					xPovlp = xP + nnpfc_overlap
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpCbVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCrVal
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpDVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpCbVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCrVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpDVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = strengthControlScaledVal
				} else {
					inpCbVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
							CroppedWidth / SubWidthC, CroppedCbPic[ i ] ) )
					inpCrVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
							CroppedWidth / SubWidthC, CroppedCrPic[ i ] ) )
					yPovlp = yP + nnpfc_overlap
					xPovlp = xP + nnpfc_overlap
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpCbVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCrVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpCbVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCrVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = strengthControlScaledVal
				}
			}
	else if( nnpfc_inp_order_idc  = =  2 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yY = cTop + yP
				xY = cLeft + xP
				yC = yY / SubHeightC
				xC = xY / SubWidthC
				inpYVal = InpY( InpSampleVal( yY, xY, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ] ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ] ) ) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( nnpfc_input_depth_flag ) {
					inpDVal = InpD( InpSampleVal( yY, xY, CroppedHeight,
							CroppedWidth, CroppedDPic[ i ] ) )
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpYVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCbVal
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpCrVal
						inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = inpDVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpYVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCbVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpCrVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = inpDVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 4 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 4 ] = strengthControlScaledVal
				} else {
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpYVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCbVal
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpCrVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpYVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCbVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpCrVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = strengthControlScaledVal
				}
			}
	else if( nnpfc_inp_order_idc  = =  3 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yTL = cTop + yP * 2
				xTL = cLeft + xP * 2
				yBR = yTL + 1
				xBR = xTL + 1
				yC = cTop / 2 + yP
				xC = cLeft / 2 + xP
				inpTLVal = InpY( InpSampleVal( yTL, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpTRVal = InpY( InpSampleVal( yTL, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpBLVal = InpY( InpSampleVal( yBR, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpBRVal = InpY( InpSampleVal( yBR, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCbPic[ i ] ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCrPic[ i ] ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap

if( nnpfc_input_depth_flag ) {
					inpDTLVal = InpD( InpSampleVal( yTL, xTL, CroppedHeight,
							CroppedWidth, CroppedDPic[ i ] ) )
					inpDTRVal = InpD( InpSampleVal( yTL, xBR, CroppedHeight,
							CroppedWidth, CroppedDPic[ i ] ) )
					inpDBLVal = InpD( InpSampleVal( yBR, xTL, CroppedHeight,
							CroppedWidth, CroppedDPic[ i ] ) )
					inpDBRVal = InpD( InpSampleVal( yBR, xBR, CroppedHeight,
							CroppedWidth, CroppedDPic[ i ] ) )
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpTLVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpTRVal
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpBLVal
						inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = inpBRVal
						inputTensor[ 0 ][ i ][ 4 ][ yPovlp ][ xPovlp ] = inpCbVal
						inputTensor[ 0 ][ i ][ 5 ][ yPovlp ][ xPovlp ] = inpCrVal
						inputTensor[ 0 ][ i ][ 6 ][ yPovlp ][ xPovlp ] = inpDTLVal
						inputTensor[ 0 ][ i ][ 7 ][ yPovlp ][ xPovlp ] = inpDTRVal
						inputTensor[ 0 ][ i ][ 8 ][ yPovlp ][ xPovlp ] = inpDBLVal
						inputTensor[ 0 ][ i ][ 9 ][ yPovlp ][ xPovlp ] = inpDBRVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpTLVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpTRVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpBLVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = inpBRVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 4 ] = inpCbVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 5 ] = inpCrVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 6 ] = inpDTLVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 7 ] = inpDTRVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 8 ] = inpDBLVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 9 ] = inpDBRVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 10 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 10 ] = strengthControlScaledVal
				} else {
					if( !nnpfc_component_last_flag ) {
						inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpTLVal
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpTRVal
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpBLVal
						inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = inpBRVal
						inputTensor[ 0 ][ i ][ 4 ][ yPovlp ][ xPovlp ] = inpCbVal
						inputTensor[ 0 ][ i ][ 5 ][ yPovlp ][ xPovlp ] = inpCrVal
					} else {
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpTLVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpTRVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpBLVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = inpBRVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 4 ] = inpCbVal
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 5 ] = inpCrVal
					}
					if( nnpfc_auxiliary_inp_idc  = =  1 )
						if( !nnpfc_component_last_flag )
							inputTensor[ 0 ][ i ][ 6 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal
						else
							inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 6 ] = strengthControlScaledVal
				}
			}
}

...

nnpfc_out_format_idc equal to 0 indicates that the sample values output by the NNPF are real numbers where the value range of 0 to 1, inclusive, maps linearly to the unsigned integer value range of 0 to ( 1  <<  bitDepth ) − 1, inclusive, for any desired bit depth bitDepth for subsequent post-processing or displaying.

nnpfc_out_format_idc equal to 1 indicates that the luma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_luma_bitdepth_minus8 + 8 ) ) − 1, inclusive, and the chroma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 ) ) − 1, inclusive., and when nnpfc_purpose & 0x40 is not equal to 0, the depth sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_depth_bitdepth_minus8 + 8 ) ) − 1, inclusive.

Values of nnpfc_out_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_out_format_idc.

nnpfc_out_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the output integer tensor. The value of nnpfc_out_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the output integer tensor. The value of nnpfc_out_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

When nnpfc_purpose & 0x10 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is greater than BitDepthY.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is greater than BitDepthC.

nnpfc_out_tensor_depth_bitdepth_minus8 plus 8 specifies the bit depth of depth sample values in the output integer tensor. The value of nnpfc_out_tensor_depth_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_order_idc indicates the output order of samples resulting from the NNPF.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3 4, inclusive, in bitstreams conforming to this edition of this document. Values of 4 5 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 5 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When nnpfc_purpose & 0x02 is not equal to 0, nnpfc_out_order_idc shall not be equal to 3.

Table 22 contains an informative description of nnpfc_out_order_idc values.

Table 22 – Description of nnpfc_out_order_idc values

| nnpfc_out_ order_idc | Description |
| --- | --- |
| 0 | If nnpfc_purpose & 0x40 is equal to 0, only the luma matrix is present in the output tensor, thus the number of channels is 1. Otherwise (nnpfc_purpose & 0x40 is not equal to 0), the luma and depth matrices are present in the output tensor, thus the number of channels is 2. |
| 1 | If nnpfc_purpose & 0x40 is equal to 0, only the chroma matrices are present in the output tensor, thus the number of channels is 2. Otherwise (nnpfc_purpose & 0x40 is not equal to 0), the chroma and depth matrices are present in the output tensor, thus the number of channels is 3. |
| 2 | If nnpfc_purpose & 0x40 is equal to 0, the luma and chroma matrices are present in the output tensor, thus the number of channels is 3. Otherwise (nnpfc_purpose & 0x40 is not equal to 0), the luma, chroma, and depth matrices are present in the output tensor, thus the number of channels is 4. |
| 3 | If nnpfc_purpose & 0x40 is equal to 0, four luma matrices and two chroma matrices are present in the output tensor, thus the number of channels is 6. Otherwise (nnpfc_purpose & 0x40 is not equal to 0), four luma matrices and two chroma matrices and four depth matrices are present in the output tensor, thus the number of channels is 10. nnpfc_out_order_idc equal to 3 can only be used when the output chroma format is 4:2:0. |
| 4 | Only the depth matrix is present in the output tensor, thus the number of channels is 1. nnpfc_out_order_idc equal to 4 can only be used when nnpfc_purpose & 0x40 is not equal to 0. |
| 4 5..255 | Reserved |

The process StoreOutputTensors( ), for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numOutputPics; i++ ) {
	if( nnpfc_out_order_idc  = =  0 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < nnpfc_pic_height_in_luma_samples  &&  xY < nnpfc_pic_width_in_luma_samples )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ] 
						if( ( nnpfc_purpose & 0x40 )  !=  0 ) 
							FilteredDPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ] 
						if( ( nnpfc_purpose & 0x40 )  !=  0 ) 
							FilteredDPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ] 					}
			}
	else if( nnpfc_out_order_idc  = =  1 ) {			(85)
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {
				xSrc = cLeft * horCScaling + xP
				ySrc = cTop * verCScaling + yP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / outSubHeightC  &&
						xSrc < nnpfc_pic_width_in_luma_samples / outSubWidthC )
					if( !nnpfc_component_last_flag ) {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					}
			}
		if( ( nnpfc_purpose & 0x40 )  !=  0 ) {
			for( yP = 0; yP < outPatchHeight; yP++)
				for( xP = 0; xP < outPatchWidth; xP++ ) {
					yY = cTop * outPatchHeight / inpPatchHeight + yP
					xY = cLeft * outPatchWidth / inpPatchWidth + xP
					if ( yY < nnpfc_pic_height_in_luma_samples  &&  xY < nnpfc_pic_width_in_luma_samples )
						if( !nnpfc_component_last_flag )
							FilteredDPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						else
							FilteredDPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ] 
		}
	}
	else if( nnpfc_out_order_idc  = =  2 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC 
				yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC
				if ( yY < nnpfc_pic_height_in_luma_samples && xY < nnpfc_pic_width_in_luma_samples)
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 1 ][ yPc ][ xPc ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 2 ][ yPc ][ xPc ] 
						if( ( nnpfc_purpose & 0x40 )  !=  0 ) 
							FilteredDPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 1 ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 2 ] 
						if( ( nnpfc_purpose & 0x40 )  !=  0 ) 
							FilteredDPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  3 )
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / 2  &&
						xSrc < nnpfc_pic_width_in_luma_samples / 2 )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 4 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 5 ][ yP ][ xP ] 
						if( ( nnpfc_purpose & 0x40 )  !=  0 ) {
							FilteredDPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 6 ][ yP ][ xP ]
							FilteredDPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 7 ][ yP ][ xP ]
							FilteredDPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 8 ][ yP ][ xP ]
							FilteredDPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 9 ][ yP ][ xP ]
						}
					} else {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 4 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 5 ] 
						if( ( nnpfc_purpose & 0x40 )  !=  0 ) {
							FilteredDPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 6 ]
							FilteredDPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 7 ]
							FilteredDPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 8 ]
							FilteredDPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 9 ]
						}
					}
			}
	else if( nnpfc_out_order_idc  = =  4 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < nnpfc_pic_height_in_luma_samples  &&  xY < nnpfc_pic_width_in_luma_samples )
					if( !nnpfc_component_last_flag )
						FilteredDPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
					else
						FilteredDPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
			}
}

...

nnpfc_padding_type indicates the process of padding when referencing sample locations outside the boundaries of the cropped decoded output picture as described in Table 23. The value of nnpfc_padding_type shall be in the range of 0 to 15, inclusive.

Table 23 – Informative description of nnpfc_padding_type values

| nnpfc_padding_type | Description |
| --- | --- |
| 0 | zero padding |
| 1 | replication padding |
| 2 | reflection padding |
| 3 | wrap-around padding |
| 4 | fixed padding |
| 5..15 | Reserved |

nnpfc_luma_padding_val indicates the luma value to be used for padding when nnpfc_padding_type is equal to 4.

nnpfc_cb_padding_val indicates the Cb value to be used for padding when nnpfc_padding_type is equal to 4.

nnpfc_cr_padding_val indicates the Cr value to be used for padding when nnpfc_padding_type is equal to 4.

nnpfc_depth_padding_val indicates the depth value to be used for padding when nnpfc_padding_type is equal to 4. The value of nnpfc_depth_padding_val shall be in the range of 0 to 2nnpfc_inp_tensor_depth_bitdepth_minus8+8-1, inclusive.

The function InpSampleVal( y, x, picHeight, picWidth, croppedPic ) with inputs being a vertical sample location y, a horizontal sample location x, a picture height picHeight, a picture width picWidth, and sample array croppedPic returns the value of sampleVal derived as follows:

NOTE 6 – For the inputs to the function InpSampleVal( ), the vertical location is listed before the horizontal location for compatibility with input tensor conventions of some inference engines.

If( nnpfc_padding_type  = =  0 )
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal = 0
	else
		sampleVal = croppedPic[ x ][ y ]			(92)
else if( nnpfc_padding_type  = =  1 )
	sampleVal = croppedPic[ Clip3( 0, picWidth − 1, x ) ][ Clip3( 0, picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  2 ) 
	sampleVal = croppedPic[ Reflect( picWidth − 1, x ) ][ Reflect( picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  3 ) 
	if( y >= 0  &&  y < picHeight )
		sampleVal = croppedPic[ Wrap( picWidth − 1, x ) ][ y ] 
else if( nnpfc_padding_type  = =  4 ) 
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth ) {
		sampleVal[ 0 ]  = nnpfc_luma_padding_val		sampleVal[ 1 ]  = nnpfc_cb_padding_val
		sampleVal[ 2 ] = nnpfc_cr_padding_val
		if( nnpfc_depth_input_flag )
			sampleVal[ 3 ] = nnpfc_depth_padding_val
	} else
		sampleVal = croppedPic[ x ][ y ]

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).