| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0124-v2 |
| --- | --- |

| Title: | AHG 12: Fix related to template type selection in Intra TMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dongcheol Kim Kyungyong (Kyle) Kim Ju-Hyung (John) Son Jin Sam Kwak | Tel: Email: | dongcheol.kim@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

It was discovered that the condition of template type selection in Intra TMP was not aligned with the intention of JVET-W0069: On Intra TMP Boundary Conditions. Specifically, for left template type selection, the condition of isLeftAboveAvailable() was being used instead of isLeftAvailable(), which resulted in a categorization of NO_TEMPLATE even when Left Template was available. To address this issue, it is proposed to replace the isLeftAboveAvailable() with the isLeftAvailable() for selecting the LEFT_TEMPLATE in Intra TMP.

It is reported that on top of ECM-8.0, the overall coding performance is as below:

For AI configuration: x.xx%, x.xx % and x.xx % with xxx% EncT, xxx% DecT.

For RA configuration: x.xx%, x.xx% and x.xx% with xxx% EncT, xxx% DecT.

Introduction

At the JVET-W meeting, JVET-W0069 related to Intra TMP boundary conditions was submitted and adopted, which introduced the left template and above template types. The left template is used when only the left template is available at the top boundary of the picture/slice, and the above template is used when only the above template is available at the left boundary of the picture/slice.

Figure 1: Left: templates for intra TMP, Right: partial template for Intra TMP at picture/slice boundaries.

Currently, in ECM-8.0, the isLeftAboveAvailable() function is used to determine the left template type. Therefore, this condition may categorize the left template as NO_TEMPLATE even when the left template is available.

Proposal

The proposed fix is to replace the existing condition of isLeftAboveAvailable() with the new condition of isLeftAvailable() to select LEFT_TEMPLATE in Intra TMP.

#if JVET_W0069_TMP_BOUNDARY if (isAboveLeftAvailable(cu, chType, posLT) && isAboveAvailable(cu, chType, posLT, numAboveUnits, unitWidth, (neighborFlags + totalLeftUnits + 1)) && isLeftAvailable(cu, chType, posLT, numLeftUnits, unitHeight, (neighborFlags + totalLeftUnits - 1))) { return L_SHAPE_TEMPLATE; } else if (isAboveLeftAvailable(cu, chType, posLT) isLeftAvailable(cu, chType, posLT, numLeftUnits, unitHeight, (neighborFlags + totalLeftUnits - 1))) { return LEFT_TEMPLATE; } else if (isAboveAvailable(cu, chType, posLT, numAboveUnits, unitWidth, (neighborFlags + totalLeftUnits + 1))) { return ABOVE_TEMPLATE; } else { return NO_TEMPLATE; }

Experimental results

All Tests were implemented on top of ECM-8.0 [1]. Simulations were performed by following the common test conditions [2]. More detailed results could be found int the attached excel sheets. The test results are summarized below.

Table 1: performance of xxx over ECM-8.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.09% | 102% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.04% | 0.02% | 101% | 99% |
| Class C | -0.01% | -0.04% | 0.06% | 101% | 99% |
| Class E | 0.00% | 0.02% | 0.15% | 101% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | 0.06% | -0.09% | 100% | 100% |
| Class F | -0.01% | 0.03% | -0.01% | 101% | 100% |
| Class TGM | -0.01% | 0.00% | 0.00% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.12% | 0.10% | 101% | 99% |
| Class C | -0.03% | 0.04% | 0.16% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | 0.06% | -0.33% | 101% | 100% |
| Class F | 0.02% | 0.04% | 0.06% | 103% | 101% |
| Class TGM | 0.00% | -0.01% | -0.01% | 103% | 101% |

References

- M. Coban, and et al., “Algorithm description of Enhanced Compression Model 8 (ECM 8),” JVET-AC2025, Jan. 2023.

[2] M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Feb. 2022

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).