| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0215-v2 |
| --- | --- |

| Title: | Non-EE2: Extension of IBC-GPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Ning Yan, Xianglin Wang | Email: | machangyue@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In ECM-8.0, IBC-GPM is applied which divides a CU into two geometry partitions based on a sample-wise combination of two prediction signals. In the current design, one of the IBC-GPM prediction signals is generated from the IBC prediction while the other is generated from the intra prediction. In this contribution, the existing IBC-GPM is further extended to allow the predictions of both partitions are generated using the IBC. The simulation results on top of ECM-8.0 are summarized as follows.

AI:

Class F : {-0.16%, -0.27%, -0.28%, 100%, 100%}, Class TGM : {-0.43%, -0.34 %, -0.31%, 99%, 99%};

RA:

Class F : {-0.17%, -0.86 %, -0.25%, 100%, 101%}, Class TGM : {-0.36%, -0.18%, -0.25%,100%, 100%};

Introduction

In JVET-AC meeting, intra block copy with geometry partition mode (IBC-GPM) [1] was adopted. IBC-GPM splits a CU into two geometry partitions and generates the prediction sample of the whole block based on a sample-wise combination of two prediction signals. In the current design, one of the IBC-GPM prediction signals is generated from the IBC prediction while the other is generated from the intra prediction. The IBC part of the IBC-GPM is generated based on the candidate list of regular IBC merge mode or IBC template matching (TM) merge mode. Additionally, the same intra prediction mode (IPM) candidate list, which is used for the GPM with inter and intra prediction for intra prediction, is applied to generate the intra part of the IBC-GPM. The IPM candidate list size is pre-defined as 3. In the current IBC-GPM design, there are totally 48 geometry partitioning modes, which are grouped into two sets as shown:

Table 1: Geometry partitioning modes in the first geometry partitioning mode set

| ibc_gpm_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| angleIdx | 0 | 0 | 8 | 8 | 16 | 16 | 24 | 24 |
| distanceIdx | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 |

Table 2: Geometry partitioning modes in the second geometry partitioning mode set

| ibc_gpm_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| angleIdx | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 4 | 5 |
| distanceIdx | 0 | 1 | 3 | 0 | 1 | 3 | 0 | 1 | 3 | 0 |
| ibc_gpm_partition_idx | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| angleIdx | 5 | 5 | 11 | 11 | 11 | 12 | 12 | 12 | 13 | 13 |
| distanceIdx | 1 | 3 | 0 | 1 | 3 | 0 | 1 | 3 | 0 | 1 |
| ibc_gpm_partition_idx | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| angleIdx | 13 | 14 | 14 | 14 | 18 | 18 | 19 | 19 | 20 | 20 |
| distanceIdx | 3 | 0 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 |
| ibc_gpm_partition_idx | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| angleIdx | 21 | 21 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 30 |
| distanceIdx | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 |

For the signaling, when IBC-GPM is selected, one flag is firstly signalled to indicate whether the first or the second set is selected, followed by the index of the selected partition mode in the set. After that, one flag is signalled to indicate the prediction modes of two partitions. When the flag is true, it indicates the first partition is predicted based on the intra prediction and the second partition is predicted based on the IBC. Otherwise (when the flag is false), it indicates the first partition is predicted based on the IBC and the second partition is predicted based on the intra prediction. When the intra prediction is applied to one partition, an IPM index is further signalled to select the intra mode from the IPM list for the partition. Similarly, when the IBC is applied to one partition, a merge index is signalled.

Proposal

To further improve the efficiency, the contribution proposes to extend the IBC-GPM by allowing the generation of the prediction samples of both partitions using the IBC. Specifically, in the proposed scheme, the core IBC-GPM design (e.g., 48 GPM modes, the IBC merge candidate list, the IPM list) is kept the same as the ones in ECM-8.0. Only the signaling of the existing IBC-GPM is slightly modified to enable the proposed IBC-IBC combination. Specifically, two flags are signaled to indicate the prediction modes of two partitions. The first flag is always signaled to indicate whether the first partition is intra predicted or not. The second flag only needs to be signaled when the first flag is false (i.e., the first partition is IBC predicted) to indicate whether the intra prediction is applied to the second partition. Additionally, when both flags are false (indicating that both partitions are generated using the IBC), the maximum codeword of the second partition is reduced by 1 due to the fact that the merge indices of two partitions cannot be identical.

Experimental results

The proposed method was implemented on top EMC-8.0 [2] and tests were performed following ECM CTC [3]. The test results of the proposed method are summarized in the following table.

Table 1: Test result of GPM with IBC and IBC prediction

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -0.16% | -0.27% | -0.28% | 100% | 100% |
| Class TGM | -0.43% | -0.34% | -0.31% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.17% | -0.86% | -0.25% | 100% | 101% |
| Class TGM | -0.36% | -0.18% | -0.25% | 100% | 100% |

Conclusion

The contribution proposes to extend the IBC-GPM to allow both two partitions of IBC-GPM are generated using the IBC prediction. Simulation results show the coding benefits without noticeable enc/dec runtime impact. The proposed method is asserted to have minor implementation cost on the existing IBC-GPM. It is recommended to study the proposed method in the next round of EE.

References

- Y. Wang, K. Zhang, L. Zhang, N. Zhang, C. Ma, X. Xiu, W. Chen, H.-H. Jhu, C.-W. Kuo, N. Yan, X. Wang, “EE2-3.6: IBC-CIIP, IBC-GPM, and IBC-LIC”, JVET-AC0112, January 2023.

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).