| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0320-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0106 (EE1-1.6: In-loop filter with wide activation and large receptive field) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tong Shao | Email: | Tong.Shao@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the training and inference cross-check results for JVET-AD0106, Test EE1-1.6.1: to evaluate techniques proposed in JVET-AC0178 on top of  NNVC-4.0. Due to the crosschecker’s time and computing resource limit, only Intra model and AI results crosscheck is performed and finished. Compared to the results in EE1-1.6, the AI inference results using Intra model trained by the crosschecker show less than 0.1% difference, at {0.08%	, -0.12%,	0.32%}, which could be concluded that for Intra model and AI results, it has passed the training and inference crosschecks.

# Training information

The training scripts in the EE1-1.6/ training-JVET-AC0178 were used in the training procedure. And SADL int16 results are reported.

The crosschecker has no issue with launching the training. However, the first training run was uxpectedlys topped, and it seems that the code has no attribute for resume training after being stopped with certain epochs. It’s suggested that this feature could be added in the training script. This has no influence on the correctness of the training or the conclusion of the crosscheck.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-A100 40GB |
|  | Framework: | PyTorch v1.11 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 60 |
|  | Batch size: | 64 |
|  | Training time: | 1.6.1: |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Experimental results

Results are presented in table 1 and 2.

- Inference results

Table 1. Performance of EE1-1.6.1AI (SADL int16, NNVC-4.0 anchor)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -10.91% | -21.01% | -20.88% |  |  |
| Class A2 | -9.89% | -21.31% | -17.36% |  |  |
| Class B | -10.13% | -20.39% | -21.03% |  |  |
| Class C | -11.24% | -21.36% | -23.47% |  |  |
| Class E | -14.99% | -24.67% | -25.35% |  |  |
| Overall | -11.28% | -21.58% | -21.66% |  |  |
| Class D | -10.81% | -21.29% | -23.29% |  |  |
| Class F | -6.79% | -15.24% | -15.25% |  |  |

- Training results

Table 2. Performance of EE1-1.6.1 with retrained AI model (SADL int16, proponent’s model as anchor)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 1.08% | 1.08% |  |  |
| Class A2 | 0.04% | 0.05% | -0.45% |  |  |
| Class B | 0.02% | -1.30% | 0.21% |  |  |
| Class C | 0.15% | 0.31% | 0.41% |  |  |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -0.12% | 0.32% |  |  |
| Class D | 0.22% | -0.13% | 0.33% |  |  |
| Class F | 0.36% | -0.89% | 0.32% |  |  |

# References

- F. Galpin, T. Dumas, P. Bordes, E. François. AHG11: Small Ad-hoc Deep-Learning Library (SADL) update. JVET-AA0086, 27th Meeting, by teleconference, 13–22 July 2022..

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AB2016, Mainz, DE, October 2022.