| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0184 |
| --- | --- |

| Title: | AHG12: TIMD with IntraTMP and IBC candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Fabrice Le Léannec Philippe Bordes Franck Galpin Antoine Robert | Email: | karam.naser@interdigital.com, |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

TIMD is a an intra tool that uses template analysis to generate the prediction signal. Specifically, a set of intra candidate modes are tested on the template and the template distance (SATD) is measured to deduce up to two best modes. This contribution proposes extending the candidate modes with IBC and IntraTMP candidates obtained from neighboring blocks. The proposed modification is tested on top of ECM-9.0 and the following results are obtained:

AI: -0.10%	-0.13%	-0.16%	with EncT 101% and DecT 102%

# Introduction

In TIMD mode, SATD between the prediction samples and reconstruction samples of the template is calculated. The prediction is generated for each mode in MPMs and wide-angle. The two intra prediction modes with the minimum SATD are selected as the TIMD modes. These two TIMD modes are blended with the specific weights according to their SATD score.

It is asserted that TIMD is limited to ‘regular’ intra modes, while neglecting other modes that can improve the prediction quality. In this contribution, it is proposed to introduce the other modes IBC and IntraTMP to TIMD mode selection process.

# Proposal

This contribution proposed augmenting TIMD template analysis with IntraTMP and IBC modes. That is, the IntraTMP and IBC information of the current block and its neighbors can be used to predict the current template. Each prediction from IBC and IntraTMP is included in the SATD comparison to deduce the best two TIMD modes. If selected by TIMD process, IntraTMP/IBC prediction is performed for the current block and the blending process is used.

# Experimental results

The proposal is implemented on ECM-9.0. The following results are obtained in using the ECM common testing conditions:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.24% | 0.11% | 100% | 102% |
| Class A2 | 0.00% | -0.08% | 0.13% | 101% | 102% |
| Class B | -0.14% | 0.13% | -0.10% | 101% | 102% |
| Class C | -0.04% | -0.09% | -0.36% | 102% | 102% |
| Class E | -0.17% | -0.50% | -0.94% | 100% | 101% |
| Overall | -0.10% | -0.12% | -0.23% | 101% | 102% |
| Class D | -0.10% | -1.86% | 1.14% | 99% | 101% |
| Class F | -0.10% | -1.74% | 0.50% | 100% | 102% |
| Class TGM | -0.11% | -0.27% | -0.49% | 101% | 103% |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).