| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0045-v1 |
| --- | --- |

| Title: | AHG2/AHG9: On the use of NNPF SEI messages in AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

JVET-AE1016 includes the draft text for changes of the AVC, including the new subclause D.3.3 specifying the use of the neural-network post-filter characteristics (NNPFC) and neural-network post-filter activation (NNPFA) SEI messages in AVC. This input document proposes the following changes to subclause D.3.3:

- It is proposed to specify that the non-nested NNPFC and NNPFA SEI messages apply to decoded pictures resulting from the decoding process specified in Clauses 2 to 9 and to remove references to nuh_layer_id, which does not exist in AVC.

- It is proposed to remove the constraint and the NOTE that refer to output_flag, since output_flag is present only in the NAL unit header SVC extension and is therefore meaningless unless subclause D.3.3 would be extended to apply to layer representations of SVC.

- Since AVC does not define picture unit, it is proposed to replace the term picture unit with the term access unit in constraints related to the occurrence and content of NNPFC SEI messages within a picture unit.

- It is proposed to fix the following syntax element and variable names:

  - fp_arrangement_type  frame_packing_arrangement_type

  - SliceQpY  SliceQPY

# On nuh_layer_id

## Problem statement

Subclause D.3.3 (use of NNPFC and NNPFA SEI messages) has 5 occurrences of nuh_layer_id. However, the AVC syntax of the NAL unit header does not include nuh_layer_id.

## Overview of possible fixes

It is possible to overcome the asserted problem with the following:

- Specify that the non-nested NNPFC and NNPFA SEI messages apply to decoded pictures resulting from the decoding process specified in Clauses 2 to 9.

- Specify that the nested NNPFC and NNPFA SEI messages to the layer representation, view, or texture or depth view defined by the containing scalable nesting SEI message, MVC scalable nesting SEI message, or MVCD scalable nesting SEI message.

It is proposed to adopt only item 1.

Item 2 could be adopted later, should there be companies interested in deploying neural-network post-filtering with SVC, MVC, MVC+D, or 3D-AVC.

## Proposed specification text changes relative to JVET-AE1016

### D.3.3	Use of the neural network post-filter characteristics SEI message and the neural-network post-filter activation SEI message

Let currPic be the cropped decoded output picture that results from the decoding process specified in Clauses 2 to 9 and for which the neural-network post-processing filter (NNPF) defined by the neural-network post-filter characteristics (NNPFC) SEI message is activated by a neural-network post-filter activation (NNPFA) SEI message and currLayerId be the nuh_layer_id value of currPic.

It is a requirement of bitstream conformance that when an NNPFA SEI message is associated with a picture, the value of output_flag, when present, of that picture shall be equal to 1.

NOTE – Since only cropped decoded output pictures are used as input pictures of the NNPF, the value of output_flag, when present, of the coded picture corresponding to each input picture of the NNPF is equal to 1.

The variable pictureRateUpsamplingFlag is set equal to ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1: 0.

The variable numInputPics is set equal to nnpfc_num_input_pics_minus1 + 1.

The variable numInferences is derived as follows:

–	If all of the following conditions are true, the variable numPostRoll is set equal to the value of i such that nnpfc_interpolated_pics[ i ] is greater than 0 and the variable numInferences is set equal to 1 + numPostRoll:

–	nnpfc_purpose is equal to 8 (i.e., the only purpose for the NNPF is picture rate upsampling).

–	The NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1.

–	nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i that is greater than 0.

–	Either of the following conditions is true:

–	currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId.

–	currPic is the last picture in the coded video sequence in output order and nnpfa_no_foll_clvs_flag is equal to 1.

–	Otherwise, if all of the following conditions are true, the variable numPostRoll is set equal to InpIdx[ i ] for the value of i such that nnpfa_output_flag[ i ] is equal to 1, and the variable numInferences is set equal to 1 + numPostRoll:

–	pictureRateUpsamplingFlag is equal to 0.

–	numInputPics is greater than 1.

–	The NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1.

–	nnpfa_output_flag[ idx ] is equal to 1 for a single value of idx in the range of 0 to NumInpPicsInOutputTensor − 1, inclusive, and for that single value of idx, InpIdx[ idx ] is greater than 0.

–	Either of the following conditions is true:

–	currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId.

–	currPic is the last picture in the coded video sequence in output order and nnpfa_no_foll_clvs_flag is equal to 1.

–	Otherwise, the variable numInferences is set equal to 1.

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies:

–	The arrays inputPic[ i ] and inputPresentFlag[ i ] for i in the range of 0 to numInputPics − 1, inclusive, representing all the input pictures and the presence of input pictures, respectively, are specified as follows:

–	When j is greater than 0, for each value of k in the range of 0 to j − 1, inclusive, inputPic[ k ] is set to be currPic and inputPresentFlag[ k ] is set equal to 0.

–	The j-th input picture, inputPic[ j ], is set to be currPic and inputPresentFlag[ j ] is set equal to 1.

–	When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − 1, inclusive, in increasing order of i:

–	If both of the following conditions are true, inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1:

–	Either of the following conditions is true:

–	pictureRateUpsamplingFlag is equal to 1 and currPic is associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and a particular value of current_frame_is_frame0_flag, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId, precede inputPic[ i − 1 ] in output order, and are associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and the same value of current_frame_is_frame0_flag.

–	pictureRateUpsamplingFlag is equal to 0 or currPic is not associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] in output order.

–	nnpfa_no_prev_clvs_flag is equal to 0 or the coded picture corresponding to prevPic and the current picture are present in the same coded video sequence.

–	Otherwise, the following applies:

–	inputPic[ i ] is set to be the same picture as inputPic[ i − 1 ] and inputPresentFlag[ i ] is set equal to 0.

–	It is a requirement of bitstream conformance that, when pictureRateUpsamplingFlag is equal to 1, nnpfc_interpolated_pics[ i − 1 ] shall be equal to 0.

–	It is a requirement of bitstream conformance that when inputPresentFlag[ i ] is equal to 0 and nnpfc_input_pic_output_flag[ i ] is equal to 1, the value of nnpfa_output_flag[ idx ] shall be equal to 0 for the value of idx such that InpIdx[ idx ] is equal to i.

…

# On output_flag

## Problem statement

Subclause D.3.3 refers to output_flag, which is present only in the NAL unit header SVC extension. However, subclause D.3.3 is currently not sufficient to apply to layer representations and hence the text referring to output_flag is also meaningless.

## Proposed specification text changes relative to JVET-AE1016

It is a requirement of bitstream conformance that when an NNPFA SEI message is associated with a picture, the value of output_flag, when present, of that picture shall be equal to 1.

NOTE – Since only cropped decoded output pictures are used as input pictures of the NNPF, the value of output_flag, when present, of the coded picture corresponding to each input picture of the NNPF is equal to 1.

# On picture unit

## Problem statement

Subclause D.3.3 (use of NNPFC and NNPFA SEI messages) refers to the term picture unit in constraints related to the occurrence and content of NNPFC SEI messages within a picture unit. However, AVC does not define picture unit.

## Overview of the proposed fix

As discussed in Section 1, the non-nested NNPFC and NNPFA SEI messages apply to the "base layer", i.e., the decoded pictures resulting from the decoding process of Clauses 2 to 9. Consequently, it is proposed to use term access unit instead of the term picture unit.

If NNPFC and NNPFA SEI messages were enabled as nested SEI messages, the respective constraints would need to be added for a layer representation (SVC), a view component (MVC), and per texture or depth view component (MVC+D and 3D-AVC). However, this document does not propose specifying NNPFC and NNPFA SEI messages as nested SEI messages.

## Proposed specification text changes relative to JVET-AE1016

There shall not be more than two NNPFC SEI messages present in a picture unit an access unit with the same value of nnpfc_id. When there are two NNPFC SEI messages present in a picture unit an access unit with the same value of nnpfc_id, these SEI messages shall have different content. When two NNPFC SEI messages with the same nnpfc_id and different content are present in the same picture unit access unit, both of these NNPFC SEI messages shall be in the same SEI NAL unit.

# Editorial syntax element and variable name fixes

The editorial following changes are proposed to match with syntax element names and variable names in AVC:

- fp_arrangement_type  frame_packing_arrangement_type

- SliceQpY  SliceQPY

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).