| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0048-v1 |
| --- | --- |

| Title: | A Study on Decoder Interoperability of Generative Face Video Compression |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bolin Chen, Shanzhi Yin, Jie Chen, Yan Ye, Shiqi Wang | Email: | chenbolin.chenboli@alibaba-inc.com shanzhyin3-c@my.cityu.edu.hk jiechen.cj@alibaba-inc.com yan.ye@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba and City University of Hong Kong |  |  |

_____________________________

- Abstract

At the 29th JVET meeting, JVET-AC0088 proposed a generative face video SEI message to allow VVC-coded pictures to be used as base pictures to drive a deep generative network to generate additional images, therefore enabling ultra low rate face video compression. At the 30th JVET meeting, JVET-AD0051 generalized the generative face video SEI message to include syntax elements that correspond to a variety of facial representations, including 2D keypoints, 2D landmarks, 3D keypoints, temporal motion features, facial semantics and other formats. At the 31st JVET meeting, JVET-AE0080, JVET-AE0083 and JVET-AE0280 were proposed to support more common syntax design of generative face video SEI message and define the interface between the decoder (which decodes the base pictures and parses the SEI message) and the generative neural network.

One question that was raised during the discussion of these contributions was the interoperability between different facial representations and their associated decoder networks. This contribution attempts to shed some lights on how the interoperability problem might be solved. Specifically, we further analyze the decoder design of generative face video SEI message, and discuss how to support the decoder interoperability and parameter translatability in the two following schemes:

- Scheme 1: recognizing that the end-to-end decoder consists of two independent modules, optical flow estimation module and generator module, we retrain the original end-to-end decoder as independent modules, where we fix the generator module and train the optical flow estimation module to adapt to different types of parameters.

- Scheme 2: we establish a generalized face parameter translator so that different facial representations can be converted into the required type that can be accepted by a fixed decoder.

Introduction

Recently, deep generative models, especially generative adversarial network (GAN) [1], have shown great potentials in face video compression [2]-[9]. Figure 1 illustrates the general block diagram of generative face video compression (GFVC). As shown in Figure 1 , the generative encoder uses an analysis network to economically model the input face video with a small number of transmitted symbols (e.g., semantic parameters, facial landmarks, motion features, etc.). At the decoder, a deep generative model uses the decoded symbols and a base picture to reconstruct high quality face images. Compared to the early work on model-based video coding [10][11][12], which had to rely on much simpler synthesis models, nowadays deep generative networks provide signifantly higher capability to infer and synthesize the reconstructed video, resulting in much better reconstruction quality. It should be mentioned that most of the work in this area use a traditional video codec such as the VVC or HEVC to compress the first picture (or a set of key pictures) of a talking face video sequence as the base picture to provide essential texture reference. Then, the subsequent face pictures can be effectively reconstructed by providing these base pictures and the motion parameters to the GAN models.

Figure 1 - General block diagram of generative face video compression.

In Figure 2, we provide a summary of a few typical facial representations for GFVC algorithms. Because the face images exhibit strong statistical regularities, they can be economically characterized with 2D landmarks, 2D keypoints, region matrix, 3D keypoints, compact feature matrix or facial semantics. Each of these facial description strategies can lead to reduced coding bit-rate and improved coding efficiency, but also face some drawbacks and challenges. In particular, each of these GFVC approaches with different facial representations requires a pair of specifically-trained encoder and decoder. If the encoder and decoder do not match, then the reconstruction of face video cannot be successfully realized. Such incompatibility reduces the practicability of these approaches.

Figure 2 – Examples of facial representations used in generative face video compression.

In this contribution, we investigate the issue of interoperability among different GFVC schemes that rely on different representations. We explore the parameter translatability and decoder interoperability for generative face video compression. We propose two schemes that attempt to improve interpretability and compatibility in practical applications. Experimental results are provided to show the performance of these two proposed schemes.

Exploration on Decoder Interoperability

To overcome the issue of decoder incompatibility among different GFVC systems, we analyze the GFVC decoder design, and provide two viable solutions to improve decoder interoperability and parameter translatability: flow translator and parameter translator.

All GFVC decoders in [2]-[9] are composed of motion estimation module and frame generation module, where the motion estimation module transforms the decoded facial representation parameters into the dense motion flow and the frame generation module utilizes the transformed flow and the decoded texture reference to reconstruct face images. As shown in Figure 3 (a), the original schemes need a pair of specifically-trained encoder and decoder, where the two modules in the decoder are jointly trained in an end-to-end fashion and are regarded as an united decoder. As a result, when a given decoder receives a different type of facial representation produced by an unpaired encoder, the issue of incompatibility will arise. This reduces the practicability of GFVC systems.

To better meet real application requirements, interoperability of GFVC decoders is investigated. As shown in Figure 3 (b) and (c), two solutions are provided as follows:

- Solution 1 (Figure 3 (b)): the original decoder is split into two parts, including motion estimator and frame generator. Then, the generator module is fixed and the motion estimator is retrained as the flow translator to convert different types of facial parameters into dense motion flow. For different generative face video compression algorithms, they all need their own flow translator such that the corresponding face representation can be transformed into the dense flow required by the fixed generator.

- Solution 2 (Figure 3 (c)): the decoder is fixed – this could be the decoder from any of the GFVC algorithms. Then, a separate parameter translator is trained so that different facial representations can be converted into the required representation of the selected fixed decoder.

- Original Scheme: United Decoder

- Proposed Solution 1: Separate Flow Translator and Fixed Generator

- Proposed Solution 2: Separate Parameter Translator and Fixed United Decoder

Figure 3 – Two proposed solutions for GFVC decoder compatibility

More specifically, face images can be compactly modelled as different facial representations, including 2D keypoints, 3D keypoints, compact feature, facial semantics and so on. Denote the input face image as x. It is encoded to face paramters by face analysis model,

f=ε(x)

where ε and f denotes face encoder and its extracted parameters, respectively. ε={εA,εB, ⋯ ,εN} and f={fA,fB, ⋯ ,fN} represents the different analysis models and facial parameter representations of various GFVC approaches.

Flow Translator

As shown in in Figure 3 (b), to convert these facial representations into a common flow format, we propose the flow translator for the fixed generator scenario in Figure 3 (b). As shown in Figure 4, the decoded facial representation f is fed into the corresponding flow translator to be converted into dense optical flow. Each flow translator follows the original motion estimation architecture (i.e., MotionNet(⋅)) of the GFVC approaches. As such, the pixel-wise dense motion map (i.e. Mdense) can be obtained for the inference of the final video by the generator module,

Mdense= MotionNet(f),

Different GFVC approaches need to carry different flow translators. The proposed solution does not involve any changes to the network architecture. It simply splits the original decoder into two parts, with one part being a fixed generator regardless of the specific GFVC approach. Thus, the proposed flow translator improves the decoder interoperability.

Figure 4 – Flowchart of flow translator for GFVC

Parameter Translator

As shown in Figure 3 (c), to convert different facial representations into a specific and common facial representation format, we propose the parameter translator for a given fixed united decoder. As shown in Figure 5, the fixed decoder receives one specific type of facial representation parameters, e.g. 3D keypoints, and activates the corresponding motion estimation module and frame generation module to reconstruct the face images. If another different type of facial representation parameters, such as 2D keypoints, compact feature and facial semantics, are sent by the encoder, a specific parameter translator will be used to convert the received parameters into those required by the fixed decoder, in this example, 3D keypoints. More generally, assume the decoder is designated to decode face image from face parameters of type B, the received parameters of type A  fA should be translated to fB using the face translator T(∙) as follows,

fB=T(fA).

Then, the transformed face representation fB can be utilized to reconstruct face images via the fixed united decoder. The proposed solution does not change the decoder; it only adds a new translator module for parameter transformation, thus making different GFVC systems interoperable with each other.

Figure 5 – Flowchart of parameter translator for GFVC

Experimental Results

To validate the performance of the proposed flow translator or parameter translator for GFVC, 50 talking face video sequences are selected as testing sequences from the dataset VoxCeleb [13]. Each of these testing sequences is cropped from the original sequence into 10-second long video (i.e. 250 frames at 25 fps) with the resolution of 256*256. The coded color format is RGB 444.

- For VVC, the Low-Delay-Bidirectional (LDB) configuration in VTM-10.0 reference software is adopted to encode these testing sequences, where the quantization parameters (QP) are set to 45, 47, 50 and 52.

- For GFVC schemes, the first picture is taken as the base bicture, which is compressed as an I picture using the VTM-11.0 reference software, and all other pictures are represented with a series of facial parameters and compressed via a GFVC method. In particular, the QPs of the base picture are set to 37, 42, 47 and 52.

Flow Translator

In this section, we carry the related experiments to verify the effectiveness of the proposed flow translator. The experiment uses the generator module in the first order motion model (FOMM) system [5] as the fixed decoder as an example. And the motion estimators of FOMM and compact feature for temporal evolution (CFTE) [8] are retrained as the indepent flow translator. Figure 6 illustrates that although the proposed flow translator slightly reduces the rate distortion performance of the original end-to-end solution, the overall results are still significantly better than the VVC codec.

Figure 6 - Rate-distortion performance for flow translator in terms of Rate-DISTS and Rate-LPIPS.

Parameter Translator

In this section, we carry out the related experiments to verify the effectiveness of the proposed parameter translator. In particular, we regard different generative face decoders as the fixed decoder as follows,

- The neural talking head decoder  [7] is regarded as the fixed decoder. We use open source implementation of [7] available at https://github.com/zhanglonghao1992/One-Shot_Free-View_Neural_Talking_Head_Synthesis and refer to this decoder as FV2V in this contribution. The FOMM [5] and CFTE [8] parameters will be transformed into the 3D keypoint representation required by the FV2V  decoder via the parameter translator. Figure 7 (a) illustrates that the proposed parameter translator can achieve promising RD performance with fixed FV2V decoder.

- The CFTE decoder [8] is regarded as the fixed decoder and the FOMM [5] and FV2V [8] parameters will be transformed into the compact feature representation required by the fixed CFTE decoder via the parameter translator. Figure 7 (b) illustrates that the proposed parameter translator can achieve promising RD performance with fixed CFTE decoder.

- The FOMM decoder [5] is regarded as the fixed decoder and the Nvidia [7] and CFTE [8] parameters will be transformed into the 2D keypoint and Jacobian matrix representation required by the fixed FOMM decoder via the parameter translator. Figure 7 (c) illustrates that the proposed parameter translator can achieve promising RD performance with fixed FOMM decoder.

As illustrated in Figure 7, the reconstruction quality of incompatible encoder-decoder pair with the parameter translator is slightly worse than that of the original encoder-decoder pair due to the loss caused by parameter conversion. Nevertheless, all pairs of GFVCs (i.e., the original encoder-decoder pair or the incompatible encoder-decoder pair with the newly-added translator) can significantly outperform the traditional VVC codec in terms of rate-DISTS and rate-LPIPS.

- Fixed FV2V Decoder [7]

- Fixed CFTE Decoder [8]

- Fixed FOMM Decoder [5]

Figure 7 - Rate-distortion performance for parameter translator in terms of Rate-DISTS and Rate-LPIPS for different fixed decoders.

Application to GFVC SEI Message

At the 29th /30th /31st JVET meetings, JVET-AC0088, JVET-AD0051, JVET-AE0080, JVET-AE0083 and JVET-AE0280 proposed various syntax designs for a GFVC SEI message. One question that was raised during the discussion of these contributions was the interoperability between different facial representations and their associated decoder networks. In sections 2 and 3 of this contribution, we discussed how to improve decoder interoperability and parameter translatability for GFVC. Herein, we further analyze how the proposed solutions could affect the GFVC SEI message.

In GFVC SEI message, one can send the decoder neural network to ensure proper decoding is performed. Based on the flow translator and the parameter translator methods, instead of the entire decoder, only flow translator or parameter translator needs to be sent. As shown in Table 1 and Table 2, the number of parameters that need to be sent for the flow translator and for the parameter translator is smaller than that of the entire decoder for all GFVC schemes, especially, the parameter translator size is approximately only 2% of the entire decoder, resulting in significantly reduced overhead.

Table 1 – Number of parameters of flow translator vs. entire decoder

|  | FOMM [5] | CFTE [8] |
| --- | --- | --- |
| Facial representation | 2D keypoints & Jacobian matrices | compact feature martix |
| Entire decoder | 45.58 M | 43.89 M |
| Flow translator | 37.73 M | 36.04 M |
| FOMM generator | 7.85 M |  |

Table 2 - Number of parameters of flow translator vs. entire decoder.

|  | FOMM [5] | FV2V [7] | CFTE [8] |
| --- | --- | --- | --- |
| Facial representation | 2D keypoints & Jacobian matrices | 3D keypoint | compact feature martix |
| Entire decoder | 45.58 M | 53.11 M | 43.89 M |
| Parameter translator | 1.2 M |  |  |

Conclusion

This contribution investigates the issue of decoder interoperability and parameter translatability in GFVC. The proposed flow translator and parameter translators are asserted to be beneficial for the practical deployment of generative face video compression approaches, and achieve ultra-low bitrate face video communication.

References

- Goodfellow I, Pouget-Abadie J, Mirza M, et al. “Generative adversarial nets,” Advances in neural information processing systems, vol. 27, 2014.

- Oquab M, Stock P, Haziza D, et al. Low bandwidth video-chat compression using deep generative models[C]//Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2021: 2388-2397.

- Konuko G, Valenzise G, Lathuilière S. Ultra-low bitrate video conferencing using deep image animation[C]//ICASSP 2021-2021 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP). IEEE, 2021: 4210-4214.

- Feng D, Huang Y, Zhang Y, et al. A generative compression framework for low bandwidth video conference[C]//2021 IEEE International Conference on Multimedia & Expo Workshops (ICMEW). IEEE, 2021: 1-6.

- Siarohin A, Lathuilière S, Tulyakov S, et al. First order motion model for image animation[J]. Advances in Neural Information Processing Systems, 2019, 32.

- Siarohin A, Woodford O J, Ren J, et al. Motion representations for articulated animation[C]//Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2021: 13653-13662.

- Wang T C, Mallya A, Liu M Y. One-shot free-view neural talking-head synthesis for video conferencing[C]//Proceedings of the IEEE/CVF conference on computer vision and pattern recognition. 2021: 10039-10049.

- Chen B, Wang Z, Li B, et al. Beyond keypoint coding: Temporal evolution inference with compact feature representation for talking face video compression[C]//2022 Data Compression Conference (DCC). IEEE, 2022: 13-22.

- Chen B, Wang Z, Li B, et al. Interactive Face Video Coding: A Generative Compression Framework[J]. arXiv preprint arXiv:2302.09919, 2023.

- Pearson D E, Robinson J A. Visual communication at very low data rates[J]. Proceedings of the IEEE, 1985, 73(4): 795-812.

- Musmann H G, Hötter M, Ostermann J. Object-oriented analysis-synthesis coding of moving images[J]. Signal processing: Image communication, 1989, 1(2): 117-138.

- Nakaya Y, Chuah Y C, Harashima H. Model-based/waveform hybrid coding for videotelephone images[C]//Acoustics, Speech, and Signal Processing, IEEE International Conference on. IEEE Computer Society, 1991: 2741, 2742, 2743, 2744-2741, 2742, 2743, 2744.

- Chung J S, Nagrani A, Zisserman A. , “Voxceleb2: Deep speaker recognition,” in Interspeech, 2018

- Chen B, Chen J, et al. “AHG9: Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AC0088, January 2023.

- Chen B, Chen J, et al. “AHG9: Common SEI Message for Generative Face Video,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AD0051, April 2023.

- McCarthy S, et al. “AHG9: Generative Face Video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0080, July 2023.

- Chen B, Chen J, et al. “AHG9: Common SEI Message of Generative Face Video,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0083, July 2023.

- Chen B, Chen J, et al. “AHG9: Common Text for Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0280, July 2023.

- Ding K, Ma K, Wang S, et al. Image quality assessment: Unifying structure and texture similarity[J]. IEEE transactions on pattern analysis and machine intelligence, 2020, 44(5): 2567-2581.

- Zhang R, Isola P, Efros A A, et al. The unreasonable effectiveness of deep features as a perceptual metric[C]//Proceedings of the IEEE conference on computer vision and pattern recognition. 2018: 586-595.

Patent rights declaration(s)

Alibaba and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).