| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0051-v1 |
| --- | --- |

| Title: | AHG9: Signalling the gain provided by NNPFs and NNPF groups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Antti Hallapuro Nokia Technologies, Tampere, Finland | Email: | {miska.hannuksela, francesco.cricri, antti.hallapuro}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Signalling the quality improvement or the gain resulting from applying a neural-network post-filter (NNPF) enables the decoding system to select whether to apply the NNPF given its signalled complexity. Moreover, if there are multiple alternative NNPFs, by signaling the gain and complexity of the NNPFs, the decoding system can choose the NNPF that provides a good trade-off between the gain and complexity given the resources of the decoding system. This document proposes the following items to indicate the gain of NNPFs and NNPF groups:

- An NNPF gain syntax structure that implements the following design decisions:

- As a starting point, it is proposed to specify only the following PSNR-based metric in the semantics:

  - Specify the formula for PSNR identically to the common test conditions.

  - The improvement of between the average PSNRs before and after post-filtering is indicated as the gain value.

- Since new metrics may evolve over time and it may be tedious for JVET and its parent organizations to standardize metrics, it is proposed to allow URI-based identification of metrics.

- Signalling multiple metrics is enabled, since a decoder may understand one metric but not the other, or the same NNPF may provide gains for different purposes that could be characterized by different metrics.

- It is proposed to indicate whether the indicated gain concerns colour components collectively, and, if not, the components (Y, Cb, or Cr) that the indicated gain concerns.

- The representation format of the signalled post-filter gain, such as a fixed-point value or a floating-point value of a certain precision, is proposed to depend on the metric itself. The size of the signalled post-filter gain is proposed to be indicated in 16-bit words so that metrics that are not recognized by a decoder can be skipped.

- Either of the following options for carrying the NNPF gain syntax structure for individual NNPFs:

- An NNPF gain SEI message.

- Metadata added to NNPFC message using nnpfc_reserved_metadata_extension bits.

- Either of the following options for carrying the NNPF gain syntax structure for NNPF groups:

- Enabling the use of the NNPF gain SEI message for NNPF groups.

- Metadata added to NNPF group characteristics (NNPFGC) SEI message.

# Introduction

JVET-AC0134 [1] introduced the idea of signalling NNPF quality improvement for enabling the decoder to select whether to apply an NNPF given its signalled complexity. Further study was encouraged, as documented in the breakout group minutes JVET-AC0324 [2] as follows:

It was asked whether we would specify the formulae for computing the metrics. The proposal does not provide such equations. Providing the data would also imply a relationship to some “original” content representation. We do not have any other SEI message data that tries to provide quality measurements.

It was commented that such information could hypothetically be provided in a separate SEI message. Such an SEI message could hypothetically be developed on a longer timeline. It was thus suggested that no action be taken at this time other than to encourage further study for potential use beyond the current planned timeline.

In such further study, other types of quality measurement could also be considered (e.g. MOS-based subjective quality scores, MS-SSIM, etc.).

This document presents a study on these questions and other design decisions.

It is asserted that indicating the gain provided by a post-filter group is as important as the gain of an individual NNPF. For example, a decoder may compare the complexity and gain of a group of alternating NNPFs (NNPF A for PicListA, NNPF B for PicListB, where PicListA is non-overlapping with PicListB) with those of a single NNPF (NNPF C) to select either the group of alternating NNPFs or the single NNPF C.

The document is organized as follows:

- Section 2 proposes the NNPF gain syntax structure that includes one or more values characterizing the gain obtained by the NNPF or the NNPF group, depending on the syntax structure that contains the NNPF gain syntax structure.

- Section 3 proposes options to carry the NNPF gain syntax structure for individual NNPFs.

- Section 4 proposes options to carry the NNPF gain syntax structure for NNPF groups.

If a VSEI amendment working draft is resulting from the JVET-AF meeting, it is proposed to add the proposed syntax and semantics in the working draft. Otherwise, it is proposed to add the proposed syntax and semantics in the technologies under consideration of future extensions of VSEI.

# NNPF gain syntax structure

## Design decisions

The following is proposed:

- As a starting point, it is proposed to specify only the following PSNR-based metric in the semantics:

  - Specify the formula for PSNR identically to the common test conditions, i.e.:

    - For 10-bit video, PSNR is calculated as 10*(log10((1020 * 1020) / MSE).

    - 8-bit content is converted to 10-bit input by shifting 2 bits to the left and 10-bit PSNR calculation is used.

  - The improvement of between the average PSNRs before and after post-filtering is reported in the SEI message.

- Since new metrics may evolve over time and it may be tedious for JVET and its parent organizations to standardize metrics, it is proposed to allow URI-based identification of metrics.

- Signalling multiple metrics is enabled, since a decoder may understand one metric but not the other, or the same NNPF may provide gains for different purposes that could be characterized by different metrics.

- It is proposed to indicate whether the indicated gain concerns colour components collectively, and, if not, the components (Y, Cb, or Cr) that the indicated gain concerns.

- The representation format of the signalled post-filter gain, such as a fixed-point value or a floating-point value of a certain precision, is proposed to depend on the metric itself. The size of the signalled post-filter gain is proposed to be indicated in 16-bit words so that metrics that are not recognized by a decoder can be skipped.

## Proposed text

| nnpf_gain( ) { | Descriptor |
| --- | --- |
| nnpf_gain_num_entries_minus1 | ue(v) |
| while( !byte_aligned( ) ) |  |
| nnpf_gain_alignment_zero_bit | u(1) |
| for( i = 0; i <=nnpf_gain_num_entries_minus1; i++ ) |  |
| nnpf_gain_metric_uri[ i ] | st(v) |
| for( i = 0; i <=nnpf_gain_num_entries_minus1; i++ ) { |  |
| nnpf_gain_per_component_flag[ i ] | u(1) |
| if( nnpf_gain_per_component_flag[ i ] ) { |  |
| nnpf_gain_y_flag[ i ] | u(1) |
| nnpf_gain_cb_flag[ i ] | u(1) |
| if( nnpf_gain_y_flag[ i ] + nnpf_gain_cb_flag[ i ] > 0 ) |  |
| nnpf_gain_cr_flag[ i ] | u(1) |
| } |  |
| nnpf_gain_word_cnt_minus1[ i ] | ue(v) |
| for( j = 0; j < nnpfGainNumValuesPerEntry[ i ]; j++ ) |  |
| nnpf_gain_value[ i ][ j ] | u(v) |
| } |  |
| } |  |

nnpf_gain_num_entries_minus1 plus 1 specifies the number of entries in this syntax structure.

nnpfc_gain_alignment_zero_bit shall be equal to 0.

nnpf_gain_metric_uri[ i ], when indicating a non-empty character string, contains a URI with syntax and semantics as specified in IETF Internet Standard 66 identifying the metric used in the gain values provided for the i-th entry in this syntax structure. nnpf_gain_metric_uri[ i ] that is an empty null-terminated string specifies that the i-th entry in this syntax structure provides one or more gain values using a peak signal-to-noise ratio based metric specified below.

nnpf_gain_per_component_flag[ i ] equal to 1 specifies that gain values are provided on colour component basis. nnpf_gain_per_component_flag[ i ] equal to 0 specifies that the i-th entry in this syntax structure has one and only one gain value, which may be derived collectively from several colour components.

nnpf_gain_y_flag[ i ], when present and equal to 1, specifies that the 0-th gain value for the i-th entry in this syntax structure is derived from the luma component. nnpf_gain_y_flag[ i ], when present and equal to 0, specifies that the i-th entry in this syntax structure does not have a gain value derived from the luma component.

nnpf_gain_cb_flag[ i ], when present and equal to 1, specifies that the ( nnpf_gain_y_flag[ i ] )-th gain value for the i-th entry in this syntax structure is derived from the Cb component. nnpf_gain_cb_flag[ i ], when present and equal to 0, specifies that the i-th entry in this syntax structure does not have a gain value derived from the Cb component.

nnpf_gain_cr_flag[ i ], when present and equal to 1 or when inferred to be equal to 1, specifies that the ( nnpf_gain_y_flag[ i ] +nnpf_gain_cb_flag[ i ] )-th gain value for the i-th entry in this syntax structure is derived from the Cr component. nnpf_gain_cr_flag[ i ], when present and equal to 0, specifies that the i-th entry in this syntax structure does not have a gain value derived from the Cr component. When nnpf_gain_y_flag[ i ] and nnpf_gain_cb_flag[ i ] are present and both equal to 0, nnpf_gain_cr_flag[ i ] is inferred to be equal to 1.

The value of nnpfGainNumValuesPerEntry[ i ], which specifies the count of gain values for the i-th entry in this syntax structure, is derived as follows:

nnpfGainNumValuesPerEntry[ i ] = nnpf_gain_per_component_flag[ i ] ?
	nnpf_gain_y_flag[ i ] + nnpf_gain_cb_flag[ i ] + nnpf_gain_cr_flag[ i ] : 1	(xx)

nnpf_gain_word_cnt_minus1[ i ] plus 1 specifies the length of each gain value for the i-th entry in units of 16 bits.

nnpf_gain_value[ i ][ j ] specifies the j-th gain value for the i-th entry in this syntax structure. The semantics and the representation format of the gain value are determined by the value of nnpf_gain_metric_uri[ i ]. The length of nnpf_gain_value[ i ][ j ] is 16 * ( nnpf_gain_word_cnt_minus1[ i ] + 1 ) bits.

When nnpf_gain_metric_uri[ i ] is an empty null-terminated string, it is a requirement of bitstream conformance that all the following constraints apply:

–	nnpf_gain_per_component_flag[ i ] is equal to 1.

–	nnpf_gain_word_cnt_minus1[ i ] is equal to 0.

–	nnpfc_out_format_idc is equal to 1.

–	When nnpf_gain_y_flag[ i ] is equal to 1, all of the following constraints apply:

–	outTensorBitDepthY is less than or equal to 10.

–	nnpfcOutputPicWidth is equal to CroppedWidth.

–	nnpfcOutputPicHeight is equal to CroppedHeight.

–	When nnpf_gain_cb_flag[ i ] is equal to 1 or nnpf_gain_cr_flag[ i ] is equal to 1, all of the following constraints apply:

–	outTensorBitDepthC is less than or equal to 10.

–	nnpfcOutputPicWidth / outSubWidthC is equal to CroppedWidth / SubWidthC.

–	nnpfcOutputPicHeight / outSubHeightC is equal to CroppedHeight / SubHeightC.

The function Log10( x ) is defined as the base-10 logarithm of x.

Given 10-bit sample arrays origSampleArray and testSampleArray, the function Psnr( origSampleArray, testSampleArray, width, height) returns the value of psnrVal, which is the average peak signal-to-noise ratio of testSampleArray in comparison to origSampleArray, derived as follows:

sumSquaredError = 0
for( y = 0; y < height; y++ )
	for( x = 0; x < width; x++ ) {
		sampleError = testSampleArray[ x ][ y ] − origSampleArray[ x ][ y ]	(xx)
		sumSquaredError += sampleError * sampleError
	}
if( sumSquaredError  = =  0 )
	psnrVal = 999.99
else
	psnrVal = 10 * Log10( 1020 * 1020  ÷ ( sumSquaredError ÷ width ÷ height ) )

When nnpf_gain_metric_uri[ i ] is an empty null-terminated string, the following applies

–	Let OriginalPictures be the list of pictures given as input to encoding in output order.

–	CroppedDecodedPictures and ListNnpfOutputPics are derived as specified in subclause 8.28.1.1.

–	When the bit-depth bitDepth in OriginalPictures, CroppedDecodedPictures, or ListNnpfOutputPics is less than 10, each sample sampleVal is converted to 10-bit by shifting to the left, i.e., sampleVal is set equal to sampleVal << ( 10 − bitDepth ).

–	It is a requirement of bitstream conformance that the number of pictures, assigned to variable numPics, in OriginalPictures, CroppedDecodedPictures and ListNnpfOutputPics is the same.

–	nnpf_gain_value[ i ][ j ] >> 8 specifies the integer part of avgPsnrDiff and nnpf_gain_value[ i ][ j ] & 255 specifies the fractional part of avgPsnrDiff derived as follows:

sumPsnrDiff = 0
for( a = 0; a < numPics; a++ ) {
	if( j  = = 0  &&  nnpf_gain_y_flag [ i ]  = =  1 )
		cIdx = 0
	else if( ( j  = = 0  &&  nnpf_gain_y_flag[ i ]  = =  0  &&  nnpf_gain_cb_flag[ i ]  = =  1 )  | | 
		( j  = =  1  &&  nnpf_gain_y_flag[ i ]  = =  1  &&  nnpf_gain_cb_flag[ i ]  = =  1 )
		cIdx = 1
	else
		cIdx = 2
	if( cIdx  = =  0 ) {			(xx)
		arrayWidth = CroppedWidth
		arrayHeight = CroppedHeight
	} else {
		arrayWidth = CroppedWidth / SubWidthC
		arrayHeight = CroppedHeight / SubHeightC
	}
	sumPsnrDiff += Psnr( OriginalPictures[ a ][ cIdx ], ListNnpfOutputPics[ a ][ cIdx ], width, height ) −
		Psnr( OriginalPictures[ a ][ cIdx ], CroppedDecodedPictures[ a ][ cIdx ], width, height )
}
avgPsnrDiff = sumPsnrDiff ÷ numPics

# Where to carry the NNPF gain syntax structure?

## Design decisions

Two options for carrying nnpf_gain( ) are identified:

- An NNPF gain SEI message. The same SEI NAL unit that carries the NNPFC SEI message shall also include the NNPF gain SEI message, when present. This enables using the gain information for NNPF selection.

- Metadata added to NNPFC message using nnpfc_reserved_metadata_extension bits.

Unlike what was proposed in JVET-AC0134 [1], no metadata is proposed to be added to the NNPFA SEI message, because it is asserted that the NNPFC SEI message would be used for NNPF selection, whereas NNPFA SEI messages would not suit that purpose.

## Proposed text for NNPF gain SEI message

### VSEI

| nn_post_filter_gain( payloadSize ) { | Descriptor |
| --- | --- |
| nnpf_gain_id | ue(v) |
| nnpf_gain( ) |  |
| } |  |

nnpf_gain_id indicates the nnpfc_id value of the NNPF for which this SEI message indicates one or more values characterizing the gain obtained by the NNPF.

### VVC

When present, the NNPF gain SEI message shall be present in an SEI NAL unit that contains an NNPFC SEI message with nnpfc_id equal to nnpf_gain_id.

## Proposed text for adding metadata to NNPFC SEI message

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| nnpfc_num_metadata_extension_bits | ue(v) |
| if( nnpfc_num_metadata_extension_bits > 0 ) { |  |
| nnpfc_num_gain_bits | ue(v) |
| if( nnpfc_num_gain_bits > 0 ) |  |
| nnpf_gain( ) |  |
| if( nnpfc_num_metadata_extension_bits > nnpfcNumDefinedMetadataBits ) |  |
| nnpfc_reserved_metadata_extension | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

nnpfc_num_metadata_extension_bits equal to 0 specifies that nnpfc_num_gain_bits nnpfc_reserved_metadata_extension is not present. nnpfc_num_metadata_extension_bits greater than 0 specifies the sum of the value of nnpfc_num_gain_bits and the length, in bits, of nnpfc_num_gain_bits and nnpfc_reserved_metadata_extension, when present.

The value of nnpfc_num_metadata_extension_bits shall be in the range of 0 to 2 048, inclusive. Values in the range of 1 to 2 048, inclusive, for nnpfc_num_metadata_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of nnpfc_num_metadata_extension_bits in the range of 0 to 2 048, inclusive.

nnpfc_num_gain_bits greater than 0 shall be equal to the size of nnpf_gain( ) in bits. nnpfc_num_gain_bits equal to 0 specifies that nnpf_gain( ) is not present.

If nnpfc_num_metadata_extension_bits is equal to 0, the variable nnpfcNumDefinedMetadataBits is set equal to 0. Otherwise, the variable nnpfcNumDefinedMetadataBits is set equal to the length of nnpfc_num_gain_bits plus the value of nnpfc_num_gain_bits.

nnpfc_reserved_metadata_extension shall not be present in bitstreams conforming to this version of this Specification. However, decoders conforming to this version of this Specification shall ignore the presence and value of nnpfc_reserved_metadata_extension. When present, the length, in bits, of nnpfc_reserved_metadata_extension is equal to nnpfc_num_metadata_extension_bits − nnpfcNumDefinedMetadataBits.

# Indicating the gain resulting from NNPF groups

## Design decisions

Indicating the gain is proposed to be allowed for grouping types 0 (cascaded NNPFs) and 2 (alternating NNPFs). For these grouping types, the encoding system controls the activation and hence the gain can be unambiguously derived in the encoding system.

Indicating the gain for grouping types 1, 3 and 4 (alternative, parallel, and optional NNPFs or NNPF groups, respectively) is proposed to be disallowed, because a decoding system makes the dynamic decisions which NNPF or NNPF group is activated and hence an encoding system cannot derive the gain. Furthermore, the gain can be indicated for each member of the NNPF group either in its respective NNPFC or NNPFGC SEI message or its respective NNPF gain SEI message, depending on which design choice is taken where to carry the gain information.

Respectively to indicating the gain for individual NNPFs, two options for carrying nnpf_gain( ) for NNPF groups are identified:

- Enabling the use of the NNPF gain SEI message for NNPF groups.

- Metadata added to NNPF group characteristics (NNPFGC) SEI message.

## Proposed text for enabling NNPF gain SEI message for NNPF groups

The proposed text changes are relative to those proposed in Section 3.2.

### VSEI

nnpf_gain_id indicates the nnpfc_id value of the NNPF or the nnpfgc_id value of the NNPF group for which this SEI message indicates one or more values characterizing the gain obtained by the NNPF or the NNPF group, respectively. It is a requirement of bitstream conformance that nnpfgc_grouping_type is not equal to 1, 3, or 4 in the NNPFGC SEI message with nnpfgc_id equal to nnpf_gain_id.

### VVC

When present, the NNPF gain SEI message shall be present in an SEI NAL unit that contains an NNPFC SEI message with nnpfc_id equal to nnpf_gain_id or an NNPFGC SEI message with nnpfgc_id equal to nnpf_gain_id.

## Proposed text for adding metadata to NNPFGC SEI message

| nn_post_filter_group_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfgc_id | ue(v) |
| nnpfgc_grouping_type | ue(v) |
| if( nnpfgc_grouping_type = = 0 ¦ ¦ nnpfgc_grouping_type = = 2 ) |  |
| nnpfgc_purpose | u(16) |
| nnpfgc_num_members_minus2 | ue(v) |
| for( i = 0; i <= nnpfgc_num_members_minus2 + 1; i++ ) |  |
| nnpfgc_member_id[ i ] | ue(v) |
| nnpfgc_complexity_info_present_flag | u(1) |
| if( nnpfgc_complexity_info_present_flag ) { |  |
| nnpfgc_parameter_type_idc | u(2) |
| if( nnpfgc_parameter_type_idc != 2 ) |  |
| nnpfgc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfgc_num_parameters_idc | u(6) |
| nnpfgc_num_kmac_operations_idc | ue(v) |
| nnpfgc_total_kilobyte_size | ue(v) |
| } |  |
| nnpfgc_gain_info_present_flag | u(1) |
| if( nnpfgc_gain_info_present_flag ) |  |
| nnpf_gain( ) |  |
| } |  |

nnpfgc_gain_info_present_flag equal to 1 specifies that nnpf_gain( ) is present and indicates one or more values characterizing the gain obtained by the NNPF group. nnpfgc_gain_info_present_flag equal to 0 specifies that nnpf_gain( ) is not present. When nnpfgc_grouping_type is equal to 1, 3, or 4, nnpfgc_gain_info_present_flag shall be equal to 0.

# References

- Lin, Y.-K. Wang, K. Zhang, J. Li, Y. Li, L. Zhang, "AHG9: Signalling of NNPF quality improvement", JVET-AC0134, Jan. 2023.

- G. J. Sullivan (BoG coordinator), "BoG report on neural-network based post-filtering contributions", JVET-AC0324, Jan. 2023.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).