| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0055-v1 |
| --- | --- |

| Title: | AHG9: On Source Picture Timing Information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson-Allendes Sachin Deshpande | Tel: Email: | +46721882169 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This input document relates to the Source Picture Timing Information (SPTI) SEI message included in the Technology under Consideration for future extensions of VSEI (draft 1). The following items are proposed:

- Syntax and semantics modification is proposed such that an SPTI message cannot contain information about pictures in lower temporal layers.

- Syntax modification is proposed to condition the presence of the sublayer indicator on that persistence flag equals 1.

- It is proposed not to signal spti_sublayer_source_picture_interval_scale_factor[ spti_max_sublayers_minus1 ] for the highest temporal sublayer and instead infer its value.

- It is proposed to signal the time scale (spti_time_scale) with a minus 1 coding to syntactically avoid signalling the value 0 for time scale.

- Addition of text for sections “Use of source picture timing information” for the relevant standards is proposed.

The items are independent of each other, except for item 2 which builds on top of item 1.

# Introduction

The Source Picture Timing Information (SPTI) SEI message was proposed in [1] and included in the Technology under Consideration (TuC) [2].

This document proposes modifications relative to the TuC [2].

## Information about lower temporal layers

We believe that it is undesirable to allow for an SPTI SEI message with TemporalId greater than 0 to contain information about source picture timing of pictures with lower TemporalId than the TemporalId of the SPTI SEI message. If this would be allowed, the information about source picture timing in lower temporal layers could be different depending on if all temporal layers are received or if a subset of the temporal layers are received. We believe this to be highly undesirable and suggest to avoid this by introducing the following modifications to the syntax and semantics:

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_picture_timing_type | u(8) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_time_scale | u(32) |
| spti_num_units_in_elemental_source_picture_interval | u(32) |
| spti_max_sublayers_minus_1 | u(3) |
| for( i = TemporalId0; i <= spti_max_sublayers_minus1; i++ ) { |  |
| spti_sublayer_source_picture_interval_scale_factor[ i ] | ue(v) |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

spti_max_sublayer specifies the highest temporal sublayer that this SPTI SEI message contains source picture timing information about. The value of spti_max_sublayer shall be in the range of TemporalId to 7, inclusive.

## Condition the presence of sublayer indicator on that persistence flag is 1

The text in [2] specifies that when spti_persistence_flag is equal to 0, the SPTI SEI message applies to the current decoded picture only. Thus there is no need to signal any information related to other pictures than the current picture. Specifically, we propose to condition the presence of spti_max_sublayer on that spti_persistence_flag is equal to 1. These modifications are shown below on top of the proposed changes from item 1:

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_picture_timing_type | u(8) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_time_scale | u(32) |
| spti_num_units_in_elemental_source_picture_interval | u(32) |
| if( spti_persistence_flag ) |  |
| spti_max_sublayer | u(3) |
| for( i = TemporalId; i <= spti_max_sublayer; i++ ) { |  |
| spti_sublayer_source_picture_interval_scale_factor[ i ] | ue(v) |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

spti_max_sublayer specifies the highest temporal sublayer that this SPTI SEI message contains source picture timing information about. The value of spti_max_sublayer shall be in the range of TemporalId to 7, inclusive. When spti_max_sublayer is not present, it is inferred to be equal to TemporalId.

## Signalling of source picture interval scale factor

We assert that the values for the syntax elements spti_time_scale and spti_num_units_in_elemental_source_picture_interval can always be selected such that the scale factor for the highest temporal layer that is signalled is equal to 1. Thus, it is proposed not to signal spti_sublayer_source_picture_interval_scale_factor[ spti_max_sublayers_minus1 ] for the highest temporal sublayer and instead infer its value.. The proposed syntax and semantics modifications are shown below.

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_picture_timing_type | u(8) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_time_scale | u(32) |
| spti_num_units_in_elemental_source_picture_interval | u(32) |
| spti_max_sublayers_minus_1 | u(3) |
| for( i = 0; i <= spti_max_sublayers_minus1; i++ ) { |  |
| if ( i != spti_max_sublayers_minus1 ) |  |
| spti_sublayer_source_picture_interval_scale_factor[ i ] | ue(v) |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

spti_sublayer_source_picture_interval_scale_factor[ i ], when present, specifies a scale factor used in determining the source picture interval of corresponding consecutive pictures in output order in the CLVS having TemporalId less than or equal to i. The value 0 may be used to indicate that the source picture corresponding to the current decoded output picture is identical to the source picture corresponding to the previous decoded output picture. When spti_sublayer_source_picture_interval_scale_factor[ i ] is not present, it is inferred to be equal to 1.

## Time scale signalling with a minus 1 coding

It is proposed to signal the time scale (spti_time_scale) with a minus 1 coding to syntactically avoid signalling the value 0  for time scale.

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_picture_timing_type | u(8) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_time_scale_minus1 | u(32) |
| spti_num_units_in_elemental_source_picture_interval | u(32) |
| spti_max_sublayers_minus_1 | u(3) |
| for( i = 0; i <= spti_max_sublayers_minus1; i++ ) { |  |
| spti_sublayer_source_picture_interval_scale_factor[ i ] | ue(v) |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

spti_time_scale_minus1  plus 1 specifies the number of time units that pass in one second. The value of spti_time_scale shall not be equal to 0. For example, a time coordinate system that measures time using a 27 MHz clock has an spti_time_scale_minus1  of 27,000,000 -1.

spti_num_units_in_elemental_source_picture_interval specifies the number of time units of a clock operating at the frequency (spti_time_scale_minus1+1) Hz that corresponds to the indicated elemental source picture interval of consecutive pictures in output order in the CLVS.

The indicated elemental source picture interval, also to be denoted by the variable ElementalSourcePictureInterval, in units of seconds, is equal to the quotient of spti_num_units_in_elemental_source_picture_interval divided by (spti_time_scale_minus1+1). For example, to represent an elemental source picture interval equal to 0.04 seconds, (spti_time_scale_minus1+1) may be equal to 27,000,000 and spti_num_units_in_elemental_source_picture_interval may be equal to 1,080,000.

## Text for sections “Use of source picture timing information”

The SPTI SEI message is intended to be used with VVC, HEVC and AVC. In order to use the SPTI SEI message in these standards, a section needs to be added with description of how to use the SEI message in the context of the respective standard. In particular, variables used in the SEI message need to be assigned values from the respective standard. We propose to include the text for this in order to ensure that there is a good understanding of how the SPTI SEI message can be used in each standard. The following text, marked with red text is proposed:

### X.YY.ZZ	Use of the source picture timing SEI message in VVC

For purposes of interpretation of the source picture timing SEI message, the following variable is specified:

–	TemporalId is set equal to TemporalId.

### X.YY.ZZ	Use of the source picture timing SEI message in HEVC

For purposes of interpretation of the source picture timing SEI message, the following variable is specified:

–	TemporalId is set equal to TemporalId.

### X.YY.ZZ	Use of the source picture timing SEI message in AVC

For purposes of interpretation of the source picture timing SEI message, the following variable is specified:

–	If the bitstream conforms to any of the profiles defined in Annex G, H, I, or J, TemporalId is set equal to temporal_id.

–	Otherwise (the bitstream conforms to a profile defined in Annex A), TemporalId is set equal to ( nal_ref_idc = = 0 ? 1 : 0 ).

# References

- JVET-AE0079, AHG8/AHG9: Source picture timing information SEI message, S. McCarthy, G. J. Sullivan, P. Yin (Dolby) July 2023

- JVET-AE2032, M. M. Hannuksela, J. Chen, S. Deshpande, S. McCarthy, Technologies under consideration for future extensions of VSEI (draft 1) July 2023

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.