| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting: Hannover, Germany, 13–20 October 2023 | Document: JVET-AF0065-v1 |
| --- | --- |

| Title: | AHG9: On the SEI processing order SEI message (Part 1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen Olena Chubach Yu-Wen Huang Shawmin Lei | Email: | {lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek, Inc. |  |  |

_____________________________

# Abstract

This contribution addresses comments related to the JVET-AE2027 made at the recent JVET-AE meeting. It is proposed to add following aspects in the JVET-AE2027 - SEI processing order SEI message in VVC (draft 5):

- Scope of persistence

- Processing order type, such as Cascade, Parallel, and Alternate etc.

- Processing purpose, such as general visual quality improvement, processing for human viewing, and processing for machine etc.

- Support of the neural network post filter (NNPF) SEI messages with two options

# Introduction

JVET-AE2027 from the recent JVET meeting was updated from JVET-AE0156. There are some comments from the JVET AE meeting on JVET-AE0156:

“Another comment is made about persistence. Previously it was the entire CVS, which would no longer be clear when the SEI message is changed. Further study on this.”

“It was commented that this SEI message would be relevant not only for VVC, but also for AVC and HEVC, where new standardization actions were initiated. It was however discussed that it would be premature to include processing order in AVC CD and HEVC CDAM at this meeting, as in the context of discussion about NNPF grouping, it might be further extended for additional functionality in the context of NNPF.”

The updated JVET-AE2027 is as follows.

## D.11 SEI processing order SEI message

D.11.1 SEI processing order SEI message syntax

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| if( po_sei_wrapping_flag[ i ] ) { |  |
| reserved_alignment_6bits | u(6) |
| sei_message( ) |  |
| } else { |  |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| if( po_sei_prefix_flag[ i ]) { |  |
| po_num_prefix_bytes[ i ] | b(8) |
| for( j = 0; j < po_num_prefix_bytes[ i ]; j++ ) |  |
| po_prefix_byte[ i ][ j ] | b(8) |
| } |  |
| } |  |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| } |  |

### D.11.2 SEI processing order SEI message semantics

The SEI processing order SEI message carries information indicating the preferred processing order, as determined by the encoder (i.e., the content producer), for different types of SEI messages that may be present in a CVS.

…

po_sei_processing_order[ i ] indicates the preferred order of processing of the i-th SEI message type for which preferred processing order information is provided in the SEI processing order SEI message. For any two different integer values of m and n that are greater than or equal to 0, po_sei_processing_order[ m ] less than po_sei_processing_order[ n ] indicates any SEI message type with payloadType equal to po_sei_payload_type[ m ] and, when present, bytes po_prefix_byte[ m ][ p ] for p ranging from 0 to po_num_prefix_bytes[ m ] − 1, inclusive, should be processed before any SEI message type with payloadType equal to po_sei_payload_type[ n ], and, when present, bytes po_prefix_byte[ n ][ q ] for q ranging from 0 to po_num_prefix_bytes[ n ] − 1, inclusive, and po_sei_processing_order[ m ] equal to po_sei_processing_order[ n ] indicates that there is no preferred order of processing between the SEI message types. When there are multiple SEI messages with the same values of po_sei_payload_type[ i ], po_num_prefix_bytes [ i ], and bytes po_prefix_ byte[ i ][ j ] for j ranging from 0 to po_num_prefix_bytes [ i ] − 1, inclusive, they shall have the same value of po_sei_processing_order[ i ].

po_sei_processing_order[ 0 ] shall be equal to 0, and for i greater than 0, po_sei_processing_order[ i ] shall be equal to po_sei_processing_order[ i − 1 ] or po_sei_processing_order[ i − 1 ] + 1.

The value of po_sei_processing_order[ po_num_sei_messages_minus2 + 1 ] shall not be equal to 0.

# Proposal

To address the comments from the last meeting, this contribution proposes to add following aspects for the SEI processing order SEI message.

- Scope of persistence

- Processing order type, such as Cascade, Parallel, and Alternate etc.

- Processing purpose, such as general visual quality improvement, processing for human viewing, and processing for machine etc.

- Support of the neural network post filter (NNPF) SEI messages with two options

## Scope of persistence, processing order type, and processing purpose

Some related syntax elements and semantics text are added as highlighted below.

D.11 SEI processing order SEI message

D.11.1 SEI processing order SEI message syntax

| sei_processing_order ( payloadSize ) { | Descriptor |
| --- | --- |
| po_cancel_flag | u(1) |
| if( !po_cancel_flag ) { |  |
| po_persistence_flag | u(1) |
| po_purpose | u(8) |
| po_type | u(4) |
| reserved_alignment_2bits | u(2) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| if( po_sei_wrapping_flag[ i ] ) { |  |
| reserved_alignment_6bits | u(6) |
| sei_message( ) |  |
| } else { |  |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| if( po_sei_prefix_flag[ i ]) { |  |
| po_num_prefix_bytes[ i ] | b(8) |
| for( j = 0; j < po_num_prefix_bytes[ i ]; j++ ) |  |
| po_prefix_byte[ i ][ j ] | b(8) |
| } |  |
| } |  |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| } |  |
| } |  |

D.11.2 SEI processing order SEI message semantics

The SEI processing order SEI message carries information indicating the preferred processing order type, as determined by the encoder (i.e., the content producer), for different types of SEI messages that may be present in a CVS.

When an SEI processing order SEI message is present in any access unit of a CVS, an SEI processing order SEI message shall be present in the first access unit of the CVS. The SEI processing order SEI message persists in decoding order from the current access unit until the end of the CVS. When there are multiple SEI processing order SEI messages present in a CVS, they shall have the same content.

It is a requirement of bitstream conformance that, within an SEI processing order SEI message, there shall be at least two pairs of the syntax elements po_sei_payload_type[ i ] and po_sei_processing_order[ i ], and there shall be at least two values of po_sei_processing_order[ i ] that are not equal.

The SEI processing order SEI message can carry one or more SEI prefix indications of a particular payloadType. Each SEI prefix indication is a byte string that follows the SEI payload syntax of that value of payloadType and contains a number of complete syntax elements starting from the first syntax element in the SEI payload, and may be followed by bits that do not represent any complete syntax element of the SEI payload.

These SEI prefix indications should provide sufficient information to determine the specific processing order for SEI messages having the same value of payloadType but different preferred processing order.

po_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous SEI processing order SEI message in output order that applies to the current layer. po_cancel_flag equal to 0 indicates that processing order related information follows.

po_persistence_flag specifies the persistence of the SEI processing order SEI message for the current layer. The persistence flag value in any of the indicated SEI messages shall have the same value as the value of po_persistence_flag.

NOTE – Some SEI messages may not have the persistence flag syntax element, thus the scope of persistence of those SEI messages is indicated by po_persistence_flag in the SEI processing order SEI message.

po_persistence_flag equal to 0 specifies that the SEI processing order SEI message applies to the current decoded picture only.

po_persistence_flag equal to 1 specifies that the SEI processing order SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

—	A new CLVS of the current layer begins.

—	The bitstream ends.

—	A picture in the current layer in an AU associated with a SEI processing order SEI message is output that follows the current picture in output order.

po_purpose indicates the collective purpose of the indicated SEI messages as specified in Table X.1. The value of po_purpose shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values in the range of 4 to 255, inclusive, for po_purpose are reserved for future use by ITU T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders shall ignore SEI processing order SEI messages with the value of po_purpose in the range of 4 to 255, inclusive.

Table X.1 – po_purpose values

| Value | Description |
| --- | --- |
| 0 | Unspecified or specified by external means |
| 1 | General visual quality improvement |
| 2 | Processing for human viewing |
| 3 | Processing for machine |

po_type indicates the processing order type for the indicated SEI messages and has the semantics of po_type as specified in Table X.2. The value of po_type shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 7, inclusive, for po_type are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore SEI processing order (SPO) SEI messages with the value of po_type in the range of 4 to 7, inclusive.

Table X.2 – Definition of po_type

| Value | Description |
| --- | --- |
| 0 | Unspecified or Option – Application is optional |
| 1 | Cascade - The indicated SEI messages in an SPO SEI message with po_type is applied in the sequential order with the index i, which means in a way that the indicated SEI messages are ordered with increasing values of the index i for processing. |
| 2 | Alternate – The indicated SEI messages in an SPO SEI message with po_type is applied alternatively (only one is chosen). |
| 3 | Parallel – The indicated SEI messages in an SPO SEI message withs po_type is applied in parallel. |

po_num_sei_messages_minus2 plus 2 indicates the number of SEI messages that have a processing order type po_type as specified in Table X.2 indicated in the SEI processing order SEI message.

po_sei_importance_flag[ i ] indicates the degree of importance determined by the encoder for the SEI message with index i.

If the decoding system cannot interpret or does not support any indicated SEI message that has po_sei_importance_flag[ i ] equal to 1, it should ignore the entire SEI processing order SEI message.

reserved_alignment_6bits has no meaning and shall be equal to 0 in bitstreams conforming to this version of this Specification. Decoders shall allow this syntax element to have other values and shall ignore the value.

If po_sei_wrapping_flag[ i ] is equal to 0, an SEI message should be present outside of the SEI processing order SEI message with payloadType equal to po_sei_payload_type[ i ]. However, if po_sei_wrapping_flag[ i ] is equal to 0 and no SEI message is present with payloadType equal to po_sei_payload_type[ i ], the following applies:

- If po_sei_importance_flag[ i ] is equal to 1, the decoder should ignore the entire SEI processing order SEI message.

- Otherwise, the decoder should ignore all data associated with the loop variable value of i.

NOTE – po_sei_wrapping_flag[ i ] equal to 1 enables SEI messages to be carried within the SEI processing order SEI message to prevent such SEI messages from being incorrectly interpreted by decoders that do not process the SEI processing order SEI message. Thus, po_sei_wrapping_flag[ i ] equal to 1 is intended to be used when po_sei_wrapping_flag[ i ] equal to 0 can lead to unintended results being produced by such decoders.

po_sei_prefix_flag[ i ] equal to 1 specifies that po_num_prefix_bytes[ i ] is present. po_sei_prefix_flag[ i ] equal to 0 specifies that po_num_prefix_bytes[ i ] is not present.

po_sei_payload_type[ i ] specifies the payloadType value of the i-th SEI message type for which preferred processing order information is provided in the SEI processing order SEI message. For any two different non-negative integer values of m and n, the values of po_sei_payload_type[ m ] and po_sei_payload_type[ n ] shall not be identical unless po_sei_prefix_flag[ m ] and po_sei_prefix_flag[ n ] are both equal to 1.

SeiProcessingOrderSeiList is set to consist of the payloadType values specified in clause D.2.1, except the values 137, 144, 147, 148, 179, 180, 200, 201, 208, and 213. The value of po_sei_prefix_flag[ i ] shall be equal to 0 when po_sei_payload_type[ i ] is not equal to any value among SeiProcessingOrderSeiList.

po_num_prefix_bytes[ i ], when present, specifies the number of bytes associated with the i-th SEI message for which preferred processing order information is provided in the SEI processing order SEI message. When not present, the value of po_num_prefix_bytes[ i ] is inferred to be equal to 0.

po_prefix_byte[ i ][ j ], when present, specifies the j-th byte value of the i-th SEI message.

po_sei_processing_order[ i ] indicates the preferred order of processing of the i-th SEI message type for which preferred processing order information is provided in the SEI processing order SEI message. For any two different integer values of m and n that are greater than or equal to 0, po_sei_processing_order[ m ] less than po_sei_processing_order[ n ] indicates any SEI message type with payloadType equal to po_sei_payload_type[ m ] and, when present, bytes po_prefix_byte[ m ][ p ] for p ranging from 0 to po_num_prefix_bytes[ m ] − 1, inclusive, should be processed before any SEI message type with payloadType equal to po_sei_payload_type[ n ], and, when present, bytes po_prefix_byte[ n ][ q ] for q ranging from 0 to po_num_prefix_bytes[ n ] − 1, inclusive, and po_sei_processing_order[ m ] equal to po_sei_processing_order[ n ] indicates that there is no preferred order of processing between the SEI message types. When there are multiple SEI messages with the same values of po_sei_payload_type[ i ], po_num_prefix_bytes [ i ], and bytes po_prefix_ byte[ i ][ j ] for j ranging from 0 to po_num_prefix_bytes [ i ] − 1, inclusive, they shall have the same value of po_sei_processing_order[ i ].

po_sei_processing_order[ 0 ] shall be equal to 0, and for i greater than 0, po_sei_processing_order[ i ] shall be equal to po_sei_processing_order[ i − 1 ] or po_sei_processing_order[ i − 1 ] + 1.

The value of po_sei_processing_order[ po_num_sei_messages_minus2 + 1 ] shall not be equal to 0.

The syntax element po_sei_processing_order[ i ] from JVET-AE02027 can be removed, since the processing order type defined in Table X.2 includes the sequential order type of JVET-AE0227, as well as other types of processing order, allowing to have additional functionality (as commented in the last JVET-AE meeting).

## Support of the neural network post filter (NNPF) SEI messages

To support NNPF SEI messages as indicated SEI messages in the SPO SEI message, two options are proposed. Option 1 is that the i-th SEI message in the SPO SEI message is an NNPF activation SEI message and targeted NNPF characteristic SEI message is carried outside the SPO SEI message. Option 2 is that both NNPF characteristic and activation SEI messages may be carried as a pair inside the SPO SEI message.

### Option 1: the i-th SEI message is an activation SEI message

The i-th SEI message in the SPO SEI message is an NNPF activation SEI message, as it is in Section 2.1, and targeted NNPF characteristic SEI message is carried outside the SPO SEI message. In this case, encoder loses the wrapping option for the NNPF characteristic SEI message.

The following semantics text is added and no syntax change in Section 2.1.

In the case that the i-th SEI message is an activation SEI message, po_sei_wrapping_flag[ i ] shall be equal to 1, and the targeted SEI message outside this SEI message for activation by the i-th SEI message shall precede the first VCL NAL unit of the current picture and this SEI message in decoding order.

### Option 2: the i-the SEI message is a pair of the characteristic and activation SEI messages

Both NNPF characteristic and activation SEI messages may be carried as a pair in the SPO SEI message. In this case, encoder keeps the wrapping option for the NNPF characteristic SEI message. The following changes are made on top of the syntax proposed above in Section 2.1.

| sei_processing_order( payloadSize ) { |  | Descriptor |  |
| --- | --- | --- | --- |
| po_cancel_flag |  | u(1) |  |
| if( !po_cancel_flag ) { |  |  |  |
| po_persistence_flag |  | u(1) |  |
| po_purpose |  | u(8) |  |
| po_type |  | u(4) |  |
| reserved_alignment_2bits |  | u(2) |  |
| po_num_sei_messages_minus2 |  | u(8) |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++) { |  |  |  |
| po_sei_wrapping_flag[ i ] |  | u(1) |  |
| po_sei_importance_flag[ i ] |  | u(1) |  |
| if( po_sei_wrapping_flag[ i ] ) { |  |  |  |
| reserved_alignment_6bits |  | u(6) |  |
| sei_message( ) |  |  |  |
| sei_activation_present_flag[ i ] |  | u(1) |  |
| if( sei_activation_present_flag[ i ] ) { |  |  |  |
| reserved_alignment_7bit |  | u(7) |  |
| sei_message( ) //The corresponding activation SEI message, if applicable. |  |  |  |
| } |  |  |  |
| } else { |  |  |  |
| po_sei_prefix_flag[ i ] |  | u(1) |  |
| … |  |  |  |
| sei_activation_present_flag[ i ] |  | u(1) |  |
| if( sei_activation_present_flag[ i ] ) { |  |  |  |
| reserved_alignment_7bit |  | u(7) |  |
| sei_message( ) //The corresponding activation SEI message, if applicable. |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

sei_activation_present_flag[ i ] equal to 1 indicates the activation SEI message following the i-th SEI message is present. sei_activation_present_flag[ i ] equal to 0 indicates the activation SEI message following the i-th SEI message is not present. The activation SEI message, when present, is applicable for the i-th SEI message as the activation for use.

In the case that the i-th SEI message is an activation SEI message, po_sei_wrapping_flag[ i ] shall be equal to 1 and sei_activation_present_flag[ i ] shall be equal to 0, and the targeted SEI message for activation by the i-th SEI message shall precede the first VCL NAL unit of the current picture and this SEI message in decoding order.

The Proponents prefer Option 2, as it covers Option 1 (see the above paragraph) and may be used for implementation equivalent to Option 1.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).