| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0068-v1 |
| --- | --- |

| Title: | AHG8/AHG9: Signalling of encoder preprocessing information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Jia, Yue Li, Ye-Kui Wang, Kai Zhang, Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | wei.jia@bytedance.com yue.li@bytedance.com yekui.wang@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add, to the encoder optimization information SEI message in the TuC document JVET-AE2032, the signalling of the type of the spatial subsampling filters used in encoder preprocessing when the optimization_type indicates spatial subsampling.

# Introduction

At the 31th JVET meeting in July 2023, JVET-AE0064 adopted into the TuC JVET-AE2032 a new SEI message, named the encoder optimization information SEI message. The SEI message contains information of encoding pre-processing type applied to picture(s) associated with the SEI message.

The knwoledge of the filter type used in the encoder preprocessing is beneficial for the quality of the reconstruction in the decoder significantly. However, currently the optimizing filters employed in the encoder preprocessing are not signalled to the decoder. Without filter type knowledge, the usefullness of signalling of the preprocessing type itself apperas highly questionable.

# Proposed signalling

It is proposed to signal the filter type of the optimization in the preprocessing when optimization_type & 0x04 is not equal to 0, i.e., indicating spatial subsampling. The possible filter types include bilinear interpolation, bicubic interpolation, nearest neighbor interpolation, Lanczos interpolation, and mask for interpolation codes.

# Proposed text changes

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

    - Encoder optimization information SEI message syntax

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| optimization_cancel_flag | u(1) |
| if( !optimization_cancel_flag ){ |  |
| optimization_persistence_flag | u(1) |
| optimization_for_human_viewing_flag | u(1) |
| optimization_for_machine_analysis_flag | u(1) |
| optimization_type | u(16) |
| if ( optimization_type & 0x04 ) != 0 ) |  |
| optimization_spatial_subsampling_filter_type | u(7) |
| } |  |
| } |  |

    - Encoder optimization information SEI message semantics

…

optimization_type indicates the types of optimization method as specified in Table x.

Table x – Definition of optimization_type

| Value | Interpretation |
| --- | --- |
| optimization_type = = 0 | May be used as determined by the application |
| optimization_type > 0 && ( optimization_type & 0x01 ) = = 0 | No object-based optimization |
| ( optimization_type & 0x01 ) != 0 | With object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| optimization_type > 0 && ( optimization_type & 0x02 ) = = 0 | No temporal subsampling optimization |
| ( optimization_type & 0x02 ) != 0 | With temporal subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x04 ) = = 0 | No spatial subsampling optimization |
| ( optimization_type & 0x04 ) != 0 | With spatial subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x08 ) = = 0 | No temporal quality optimization |
| ( optimization_type & 0x08 ) != 0 | With temporal quality optimization |
| optimization_type > 0 && ( optimization_type & 0x10 ) = = 0 | No spatial quality optimization |
| ( optimization_type & 0x10 ) != 0 | With spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |

optimization_spatial_subsampling_filter_type indicates the type of spatial subsampling filter as specified in Table 25.

The value of optimization_spatial_subsampling_filter_type shall be in the range of 0 to 5, inclusive, in bitstreams conforming to this edition of this document. Values of 6 to 127, inclusive, for optimization_spatial_subsampling_filter_type are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore optimization_spatial_subsampling_filter_type in the range of 6 to 127, inclusive.

Table 25 – Definition of optimization_spatial_subsampling_filter_type

| Value | Interpretation |
| --- | --- |
| 0 | May be used as determined by the application |
| 1 | Bilinear interpolation |
| 2 | Bicubic interpolation |
| 3 | Nearest neighbor interpolation |
| 4 | Lanczos interpolation |
| 5 | Mask for interpolation codes |

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).