| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0084-v1 |
| --- | --- |

| Title: | Non-EE2: Update on IBC-LIC Model Merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lai Zhang, Yue Yu, Haoping Yu, Dong Wang | Email: | zhanglai@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes an IBC-LIC model merge mode that inherits IBC-LIC model parameters from spatial adjacent and non-adjacent neighbors, history candidates, or default models. The proposed method is implemented on top of ECM-10.0 software, and the performance is reported as follows:

AI: { -0.47% Y, -0.51% U, -0.36% V, 101.0% EncT, 100.6% DecT } for class F

{ -0.15% Y, -0.13% U, -0.11% V, 100.4% EncT, 102.6% DecT} for class TGM

RA: { -0.29% Y, -0.21% U, -0.33% V, 100.5% EncT, 104.4% DecT } for class F

{ -0.03% Y, -0.02% U, -0.02% V, 100.0% EncT, 104.4% DecT } for class TGM

Introduction

Intra block copy with local illumination compensation (IBC-LIC) is an adopted tool in the current ECM. It compensates the local illumination variation within a picture between the CU coded with IBC and its prediction block with a linear equation α*p[x]+β. The parameters of the linear equation are derived using the same method as LIC for inter prediction except that the reference template is generated with a block vector in IBC-LIC.

IBC-LIC can be applied in both the IBC-AMVP mode and the IBC-merge mode. In the IBC-AMVP mode, an IBC-LIC flag is signaled and there are four modes available when IBC-LIC flag is true, including L-shape, Multi-models, Top-only, and Left-only IBC-LIC modes. In the IBC-merge mode, the IBC-LIC flag is inferred from the merge candidate and only the L-shape IBC-LIC mode is used.

IBC-LIC model merge mode is a method proposed in JVET-AE0073 [1]. It is proposed to inherit IBC-LIC model parameters from previously coded blocks, update the β parameter of the selected model, and then apply it to the reference block. The original method in JVET-AE0073 achieved -0.56% Y in class F and -0.17% Y in class TGM with around 102% encoding time in AI configuration over ECM-9.0. But the performance dropped to -0.29% Y in class F and -0.13% Y in class TGM with around 104% encoding time over EE2-2.11b, which was an IBC-LIC related EE test reported in JVET-AE0084 and adopted in ECM-10.0.

Proposed methods

In this contribution, an IBC-LIC model merge mode is proposed to inherit IBC-LIC model parameters from previously coded blocks. More specifically, an IBC-LIC model for an IBC-LIC model merge mode can be obtained as follows:

- Construct a model candidate list which consists of model parameters from spatial adjacent and non-adjacent neighbors, history candidates, and default models. The size of the candidate list is twelve.

Similar to the CCP merge method, IBC-LIC models are collected from the previously coded IBC-LIC and IBC-LIC model merge modes in both adjacent and non-adjacent positions. When an IBC-LIC model candidate is a multi-model IBC-LIC coded block, the parameters from two linear models and the classification threshold are inherited accordingly.

Besides, a history IBC-LIC model table with a size of six is maintained similar to the HMVP table. The IBC-LIC models from spatial neighbors and the history IBC-LIC model table are added to the IBC-LIC model merge candidate list. If the list is not full, a default model and the scaled models are then added to the list. To avoid redundant models, a pruning operation is also applied.

- Select an IBC-LIC model from the candidate list and signal its index in the bitstream.

This proposal has the following new aspects over JVET-AE0073 [1]:

- The β parameter of the LIC model is inherited and would not be modified.

- In IBC-merge mode, the selected LIC model is applied to the reference template during merge list reordering. Then the IBC merge list is further sorted by the inherited IBC LIC flags. The proposed mode is used only when there is an adjacent block that is coded as IBC-LIC or the proposed mode.

- In the IBC-AMVP mode, the selected LIC model is applied to the reference template during BVD prediction.

A flag is signaled to indicate whether the proposed IBC-LIC merge mode is applied or not. If this flag is true, an index is further signaled to indicate which candidate model is used by the current block. The flag is only signaled when the current block isn’t coded as IBC-CIIP, IBC-GPM, TM-Merge, or skip mode.

Experimental results

The proposed method is implemented on top of ECM-10.0 [2]. The test results are reported as follows by following the common test conditions [3]:

Table 1. The proposed method on top of ECM-10.0

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | -0.47% | -0.51% | -0.36% | 101.0% | 100.8% | 100.7% |
| Class TGM | -0.15% | -0.13% | -0.11% | 100.4% | 102.3% | 100.8% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | -0.29% | -0.21% | -0.33% | 100.5% | 104.4% | 102.2% |
| Class TGM | -0.03% | -0.02% | -0.02% | 100.0% | 104.4% | 102.9% |

Conclusion

This contribution proposes an IBC-LIC model merge mode. The results show that the proposed method can improve the coding efficiency of ECM with minor complexity increases. It is suggested to further study this method in EE2.

References

- L. Zhang, Y. Yu, H. Yu, D. Wang, “Non-EE2: IBC-LIC Model Merge mode”, JVET-AE0073, July, 2023.

- ECM10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-10.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, July, 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).