| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0099-v1 |
| --- | --- |

| Title: | [AHG9] On signaling of source picture interval scale factor and sublayer synthesized flag in source picture timing information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that syntax elements spti_max_sublayers_minus1, spti_sublayer_source_picture_interval_scale_factor[ i ] and spti_sublayer_synthesized_picture_flag[ i ] are not needed when the SEI message persists only for one picture (i.e., for picture in the same access unit that contains the SEI message). Furthermore, when the persistence of the SPTI SEI applies only to the current picture, the computation of SourcePictureInterval[ ] and SourcePictureTime[ ] is not necessary and may cause confusion.

This contribution proposes the following modifications to overcome the asserted problem:

- Signal syntax elements spti_max_sublayers_minus1, spti_sublayer_source_picture_interval_scale_factor[ i ] and spti_sublayer_synthesized_picture_flag[ i ] only when the value of spti_persistence_flag is equal to 1.

- Update the description related to SourcePictureInterval[ i ] and SourcePictureTime[ n ] such that they apply only when spti_persistence_flag is equal to 1.

- Specify that when persistence of the SPTI SEI applies only to the current picture, SourcePictureInterval is set to be equal to ElementalSourcePictureInterval.

# Problem Statement

In the 31st JVET meeting, the first design of source picture timing information (SPTI) SEI message was agreed to be included in the Technologies under Consideration for future extensions of VSEI (JVET-AE2032).

In the current design of SPTI SEI message, syntax element spti_max_sublayers_minus1, spti_sublayer_source_picture_interval_scale_factor[ i ], and spti_sublayer_synthesized_picture_flag[ i ] are signaled regardless the persistence of the SEI message. It is asserted that these syntax elements are not needed when the SEI message persists only for one picture (i.e., for picture in the same access unit that contains the SEI message). Furthermore, when the persistence of the SPTI SEI applies only to the current picture, the computation of SourcePictureInterval[ ] and SourcePictureTime[ ] is not necessary and may cause confusion.

# Proposal

To overcome the asserted problem above, we propose the following modification to the syntax and semantics of the SEI message:

- Signal syntax elements spti_max_sublayers_minus1, spti_sublayer_source_picture_interval_scale_factor[ i ] and spti_sublayer_synthesized_picture_flag[ i ] only when the value of spti_persistence_flag is equal to 1.

- Update the description related to SourcePictureInterval[ i ] and SourcePictureTime[ n ] such that they apply only when spti_persistence_flag is equal to 1.

- Specify that when persistence of the SPTI SEI applies only to the current picture, SourcePictureInterval is set to be equal to ElementalSourcePictureInterval.

# Text implementation

…

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_picture_timing_type | u(8) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_time_scale | u(32) |
| spti_num_units_in_elemental_source_picture_interval | u(32) |
| if( spti_persistence_flag ) { |  |
| spti_max_sublayers_minus1 | u(3) |
| if( spti_max_sublayers_minus1 > 0 ) { |  |
| for( i = 0; i <= spti_max_sublayers_minus1; i++ ) { |  |
| spti_sublayer_source_picture_interval_scale_factor[ i ] | ue(v) |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

…

spti_max_sublayers_minus_1 plus 1 specifies the maximum number of temporal sublayers that may be present in the CLVS. When not present and spti_persistence_flag is equal to 0, the value of spti_max_sublayers_minus1 is inferred to be equal to 0.

spti_sublayer_source_picture_interval_scale_factor[ i ], when present, specifies a scale factor used in determining the source picture interval of corresponding consecutive pictures in output order in the CLVS having TemporalId less than or equal to i. The value 0 may be used to indicate that the source picture corresponding to the current decoded output picture is identical to the source picture corresponding to the previous decoded output picture.

When spti_persistence_flag is equal to 1, the following applies:

The indicated source picture interval associated with output pictures having TemporalId less than or equal to i, denoted by the variable SourcePictureInterval[ i ], in units of seconds, is derived as follows:

SourcePictureInterval[ i ] = ElementalSourcePictureInterval * spti_sublayer_source_picture_interval_scale_factor[ i ]

When picture n is a picture that is output having TemporalId less than or equal to i and is not the first picture of the bitstream that is output, the value of the variable SourcePictureTime[ n ] is derived as follows:

–	If temporalReversalFlag is equal to 0, SourcePictureTime[ n ] = SourcePictureTime[ previousPicInOutputOrder ] + SourcePictureInterval[ i ]

–	Otherwise, temporalReversalFlag is equal to 1, SourcePictureTime[ n ] = SourcePictureTime[ previousPicInOutputOrder ] - SourcePictureInterval[ i ]

where previousPicInOutputOrder is the last picture that is output having TemporalId less than or equal to i that precedes picture n in output order (if any). When the value of SourcePictureTime[ 0 ] is not provided by external means not specified in this document, the value of SourcePictureTime[ 0 ] is inferred to be 0.

Otherwise (i.e., spti_persistence_flag is equal to 0), the following applies:

The indicated source picture interval for source picture associated with output picture corresponds to the current picture and source picture associated with the output picture corresponds to the picture that follows the current picture in output order, denoted by the variable SourcePictureInterval, in units of seconds, and is set to be equal to ElementalSourcePictureInterval.

spti_sublayer_synthesized_picture_flag[ i ], when present, equal to 1 indicates that decoded output pictures belonging to the ith temporal sublayer are synthesized and do not correspond to unmodified original source pictures. spti_sublayer_synthesized_picture_flag[ i ] equal to 0 provides no such indication. When not present, the value of spti_sublayer_synthesized_picture_flag[ i ] is inferred to be equal to 0.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).