| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0114-v2 |
| --- | --- |

| Title: | AHG12: Local-Boosting on Cross-Component Merge Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongdong Qin, Keqin Ding, Zhuowei Xu, Jacek Konieczny Block G1, TCL International E City 1001 Zhongshanyuan Road Shenzhen, Guangdong, China 518051 | Email | {hongdong.qin, keqin.ding, zhuowei.xu, jacek.k}@tcl.com |
| Source: | TCL |  |  |

_____________________________

# Abstract

It is proposed to enable local-boosting cross-component prediction (LBCCP) in cross-component merge mode. At cross-component merge mode candidate reordering stage, for each MM-CCLM/MM-CCCM candidate, a filter is tried on template cost calculation in order to determine whether LBCCP is used on the corresponding candidate.

The experimental results over ECM-10.0 are summarised as follows:

AI:		x.xx% (Y), x.xx % (U), x.xx % (V), 100 % (EncT), 100 % (DecT)

RA:	x.xx% (Y), x.xx % (U), x.xx % (V), 100 % (EncT), 100 % (DecT)

# Introduction

In JVET-AD0188 [1], a merge candidate list for cross-component prediction was proposed. Meanwhile in JVET-AD0120 [2] aspect #1, a low-pass filter being applied on CCP predicted results when MM-CCLM/MM-CCCM is used was proposed to boost CCP prediction results. These proposals were independently adopted into ECM [3].

# Proposed methods

It is proposed to enable LBCCP aspect #1 upon CCP merge mode. When a candidate uses MM-CCLM/MM-CCCM, its model costs before and after LBCCP filtering are estimated. If the cost after filtering is lower, the candidate shall be labeled as LBCCP filter-enabled. The filter for estimating template cost is 3-tap and coefficients are [3, 10, 3]/16.

In CCP merge mode at candidate reordering stage, if MM-CCLM is used, MM model is tested instead of CCLM.

Before

cost += xUpdateOffsetsAndGetCostCCLM<false>(pu, COMPONENT_Cb, pu.Cb(), cclmModels, 1, ccpCand.glmIdc); cost += xUpdateOffsetsAndGetCostCCLM<false>(pu, COMPONENT_Cr, pu.Cr(), cclmModels, 1, ccpCand.glmIdc);

After

cost += xUpdateOffsetsAndGetCostCCLM<false>(pu, COMPONENT_Cb, pu.Cb(), cclmModels, (ccpCand.type & CCP_TYPE_MMLM) ? 2 : 1, ccpCand.glmIdc, (ccpCand.type & CCP_TYPE_MMLM), filteredSAD); cost += xUpdateOffsetsAndGetCostCCLM<false>(pu, COMPONENT_Cr, pu.Cr(), cclmModels, (ccpCand.type & CCP_TYPE_MMLM) ? 2 : 1, ccpCand.glmIdc, (ccpCand.type & CCP_TYPE_MMLM), filteredSAD);

There is no additional signal since LBCCP on cross-component merge mode is derived at decoder side.

# Experimental results

The proposed method was tested on top of ECM-10.0 under the common test conditions [4]. The table below shows the performance results.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.00% | -0.07% | 0.12% | 100.2% | 100.9% | 100.0% |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.02% | 0.03% | -0.06% | 100.3% | 98.8% | 100.0% |
| Class C | -0.02% | -0.03% | 0.04% | 100.3% | 101.6% | 100.0% |
| Class E | -0.02% | 0.00% | -0.38% | 100.0% | 101.4% | 100.0% |
| Overall |  |  |  |  |  |  |
| Class D | -0.02% | -0.03% | 0.08% | 100.2% | 100.1% | 100.0% |
| Class F |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |

# References

- Z. Deng, K. Zhang, L. Zhang, “EE2-1.3: Local-Boosting Cross-Component Prediction,” JVET-AD0120, Apr. 2023.

- K. Zhang, L. Zhang, Z. Deng, C.-M. Tsai, H.-Y. Tseng, C.-Y. Chuang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Chen, Y.-W. Huang, S.-M. Lei, “EE2-1.6: Non-local cross-component prediction and cross-component merge mode”,

- ECM-10.0 reference software, https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, Jul. 2023.

# Patent rights declaration(s)

TCL may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).