| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0117-v2 |
| --- | --- |

| Title: | Non-EE2: extensions of IBC GPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Ning Yan, Xianglin Wang | Email: | machangyue@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to extend IBC GPM scheme in ECM10.0 from two aspects: First, IBC GPM with block vector difference (IBC GPM BVD) is introduced. Second, the split mode of IBC GPM is reordered with template matching methods. The simulation results on top of ECM-10.0 are summarized as follows:

- IBC GPM BVD:

Screen Content:

AI:

Class F : {-0.20%, -0.12%, 0.01%, 103.4%, 99.1%, 100.0%}, Class TGM : {-0.26%, -0.24 %, -0.23%, 103.3%, 99.8%, 100.0%};

RA:

Class F : {-0.10%, 0.04%, 0.02%, 102.3%, 99.8%, 99.9%}, Class TGM : {-0.21%, -0.14%, -0.17%, 101.4%, 99.4%, 100.0%};

Natural Content :

AI : {0.00%, 0.00%, 0.00%, 100.0%, 100.0%, 100.0%}, RA : {0.00%, 0.00 %, 0.00%, 100.0%, 100.0%, 100.0%};

- IBC GPM BVD + Split mode reordering :

Screen Content:

AI:

Class F : {-0.21%, -0.22%, -0.05%, 105.6%, 99.9%, 101.4%}, Class TGM : {-0.39%, -0.34 %, -0.36%, 106.4%, 99.5%, 101.1%};

RA:

Class F : {-0.16%, 0.22 %, -0.04%, 103.4%, 99.8%, 101.2%}, Class TGM : {-0.26%, -0.16%, -0.26%, 102.3%, 99.1%, 100.7%};

Natural Content :

AI : {0.00%, 0.00%, 0.00%, 100.0%, 100.0%, 100.0%}, RA : {0.00%, 0.00 %, 0.00%, 100.0%, 100.0%, 100.0%};

Introduction

In ECM-10.0, IBC geometry partition (IBC-GPM) mode [1], [2] is applied, where each of two IBC-GPM predictions are generated from IBC prediction or intra prediction. When one GPM partition is IBC-predicted, the corresponding block vector (BV) is selected from the candidate list of regular IBC merge mode or IBC template matching (TM) merge mode. Otherwise, when one GPM partition is intra-predicted, the corresponding intra mode is selected from the same MPM list of GPM-inter-intra mode. Additionally, there are totally 48 geometry partitioning modes in IBC-GPM, which are grouped into two sets as shown:

Table 1: Geometry partitioning modes in the first geometry partitioning mode set

| ibc_gpm_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| angleIdx | 0 | 0 | 8 | 8 | 16 | 16 | 24 | 24 |
| distanceIdx | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 |

Table 2: Geometry partitioning modes in the second geometry partitioning mode set

| ibc_gpm_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| angleIdx | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 4 | 5 |
| distanceIdx | 0 | 1 | 3 | 0 | 1 | 3 | 0 | 1 | 3 | 0 |
| ibc_gpm_partition_idx | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| angleIdx | 5 | 5 | 11 | 11 | 11 | 12 | 12 | 12 | 13 | 13 |
| distanceIdx | 1 | 3 | 0 | 1 | 3 | 0 | 1 | 3 | 0 | 1 |
| ibc_gpm_partition_idx | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| angleIdx | 13 | 14 | 14 | 14 | 18 | 18 | 19 | 19 | 20 | 20 |
| distanceIdx | 3 | 0 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 |
| ibc_gpm_partition_idx | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| angleIdx | 21 | 21 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 30 |
| distanceIdx | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 |

Proposal

In this contribution, the IBC-GPM scheme is further extended from two aspects: First, IBC-GPM with block vector difference is introduced. Specifically, one additional BV difference (BVD), which is selected from the BVD set {1/4-pel, 1/2-pel, 1-pel, 2-pel, 4-pel, 8-pel, 16-pel, 32-pel}, is allowed to be added on top of the BV of one IBC partition when it is generated based on regular IBC merge mode. For the signaling, one flag is signaled for each IBC partition to indicate whether additional BVD is applied. When the flag is true, the corresponding BVD is further signaled. Second, the split mode of IBC GPM is reordered with template matching methods. Moreover, to achieve a better performance/complexity trade-off, the reordering is only applied to the first set of IBC-GPM split modes.

Experimental results

The proposed methods were implemented on top EMC-10.0 [3] and tests were performed following ECM CTC [4]. The test results of the proposed method are summarized in the following table.

Table 1: Test result of IBC GPM BVD

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | -0.20% | -0.12% | 0.01% | 103.4% | 99.1% | 100.0% |
| Class TGM | -0.26% | -0.24% | -0.23% | 103.3% | 99.8% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | -0.10% | 0.04% | 0.02% | 102.3% | 99.8% | 99.9% |
| Class TGM | -0.21% | -0.14% | -0.17% | 101.4% | 99.4% | 100.0% |

Table 2: Test result of IBC GPM BVD plus split mode reordering

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | -0.21% | -0.22% | -0.05% | 105.6% | 99.9% | 101.4% |
| Class TGM | -0.39% | -0.34% | -0.36% | 106.4% | 99.5% | 101.1% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | -0.16% | 0.22% | -0.04% | 103.4% | 99.8% | 101.2% |
| Class TGM | -0.26% | -0.16% | -0.26% | 102.3% | 99.1% | 100.7% |

Conclusion

The contribution proposes to improve IBC GPM from two aspects: 1) adding block vector difference to the block vector of the IBC prediction results; 2) reordering the split modes with template matching methods. Simulation results show noticeable coding benefits with reasonable enc/dec run-time impact. It is recommended to study the proposed methods in the next round of EE.

References

[1] Y. Wang, K. Zhang, L. Zhang, N. Zhang, C. Ma, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang, “EE2-3.6: IBC-CIIP, IBC-GPM, and IBC-LIC”, JVET-AC0112, January 2023.

[2] Y. Kidani, H. Kato, K. Kawamura, C. Ma, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang, G. Verba, Z. Zhang, P. Nikitin, H. Huang, V. Seregin, M. Karczewicz, “EE2-2.2/EE2-2.3/EE2-2.4: Bi-predictive IBC GPM, bi-predictive IBC, and IBC MBVD list derivation for camera captured and screen contents”, JVET-AE0169, July 2023.

[3] ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-10.0.

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluationJVET procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).