| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0138-v1 |
| --- | --- |

| Title: | AHG9: Truncated bit depth support SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ding Ding, Xin Zhao, Shan Liu, Gilles Teniou, Stephan Wenger | Tel: Email: | {ddding, xinzzhao, shanl, teniou, swenger}@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Currently, video content is typically represented with eight or higher bit depth, such as 8-bit or 10-bit, which has been widely adopted for video communication perceived by human vision. However, it is observed through experimental results that, for machine vision/tasks, such as object tracking and detection, the quality score, e.g., measured by detection rate, shows lower sensitivity to the bit depth. Therefore, a lower sample value range, which can be represented by fewer bits per sample than 8 bits, can achieve a better trade-off between bitrate and machine task score.

In this contribution, it is proposed to include SEI messages signaling the actual sample value range (or bit depth) of the input video, which is represented by 8-bit or higher bit depth.

# Introduction

The following figures show an example of converting a video frame with 10 bit sample representation to video frame with a lower bit depth for luma (luma represented by 10 bit sample with sample values range from 0 to 512, i.e., 9 bit range, no change on chroma samples) and decoded frame after converting back to 10 bits (represented by 10 bit sample with sample values range from 0 to 1023, i.e., 10 bit range).

Figure 1: From left to right: original frame with 10 bit value range, pre-processed frame with 9-bit value range, decoded frame converted back to 10 bit value range. From top to down: all color planes, luma plane, histograms of luma, Cb and Cr planes.

It is observed that, better trade-off between bitrates and object detection/tracking accuracy may be achieved by removing the Least Significant Bits (LSB) of the luma component, which is implemented (for coding efficiency reasons) as a shift of sample values of pre-defined number of bits and this number needs to be signaled to the decoder, so that the opposite shift is correctly applied before machine tasks. By converting the luma component of original video frame to lower value range before feeding the sequence into the encoder, the coding bits can be significantly lower, but the decoded video frame can keep a similar machine task score.

# Proposed method #1

It is proposed to signal the bit depth truncation information for each color component separately. The proposed text changes for this method #1 are described as follows, all the text changes described below are added texts on top of the current VSEI spec text.

## x.x Bit depth truncation SEI message

### x.x.x	Bit depth truncation SEI message syntax

| bit_depth_truncation ( payloadSize ) { | Descriptor |
| --- | --- |
| bit_depth_truncation_cancel_flag | u(1) |
| if( !bit_depth_truncation_cancel_flag ) { |  |
| bit_depth_truncation_param | u(3) |
| } |  |
| } |  |

### x.x.x	Bit depth truncation SEI message semantics

The bit depth truncation (BDT) SEI message specifies various parameters of the left shift values that may be applied on the reconstructed frame samples before sending the reconstructed frame samples for further processing logic such as object detection or object tracking.

bit_depth_truncation_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous bit depth scaling SEI message in output order that applies to the current layer. bit_depth_truncation_cancel_flag equal to 0 indicates that bit depth shift information follows.

bit_depth_truncation_param specifies the left shift value applied the sample values of the input image. The value of this syntax should be kept the same throughout a CVS or the coding efficiency may be significantly sacrificed.

Depending on the value of bit_depth_truncation_param, the sample value shifting is specified as follows:

–	If bit_depth_truncation_param is not equal to 0, the sample value shifting processing as specified by:

v = I[ c ][ x ][ y ] << bit_depth_truncation_param	(xx)

I[ c ][ x ][ y ] = v > ( 1  <<  bitDepth[ c ] ) − 1) ? ( 1  <<  bitDepth[ c ] ) − 1) : I[ c ][ x ][ y ]	(xx)

–	Otherwise (bit_depth_truncation_param is equal to 0), no further processing applied on I[ c ][ x ][ y ].

where I[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the input image I, bitDepth[ c ] is the number of bits used for each sample in a fixed-length unsigned binary representation of the arrays I[ c ][ x ][ y ], where c = 0..2, x = 0..PicWidthInLumaSamples − 1, and y = 0..PicHeightInLumaSamples − 1.

# Proposed method #2

It is proposed to signal the bit depth truncation information for each color component separately. The proposed text changes for this method #2 are described as follows, all the text changes described below are added texts on top of the current VSEI spec text.

## x.x Bit depth truncation SEI message

### x.x.x	Bit depth truncation SEI message syntax

| bit_depth_truncation( payloadSize ) { | Descriptor |
| --- | --- |
| bit_depth_truncation_cancel_flag | u(1) |
| if( !bit_depth_truncation_cancel_flag) { |  |
| bit_depth_truncation_param_luma | u(3) |
| bit_depth_truncation_param_chroma | u(3) |
| } |  |
| } |  |

### x.x.x	Bit depth truncation SEI message semantics

The bit depth truncation (BDT) SEI message specifies various parameters of the left shift values that may be applied on the reconstructed image samples before sending the reconstructed image samples for further processing logic such as object detection or object tracking.

bit_depth_truncation_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous bit depth truncation SEI message in output order that applies to the current layer. bit_depth_offset_cancel_flag equal to 0 indicates that bit depth shift information follows.

bit_depth_truncation_param_luma, bit_depth_truncation_param_chroma, specifies the left shift value applied the sample values of the luma and chroma color components of the input image, respectively. The value of these syntaxes should be kept the same throughout a CVS or the coding efficiency may be significantly sacrificed.

Depending on the value of bit_depth_truncation_param_luma and bit_depth_truncation_param_chroma, the sample value shifting is specified as follows:

–	An array of variable bitDepthShiftFactor[ c ] is derived as follows:

bitDepthShift[ 0 ] = bit_depth_truncation_param_luma	(xx)

bitDepthShift[ 1 ] = bit_depth_truncation_param_chroma	(xx)

bitDepthShift[ 2 ] = bit_depth_truncation_param_chroma	(xx)

For c = 0..2, the followings apply:

–	If bitDepthShift[ c ] is not equal to 0, the sample value shifting processing as specified by:

v = I[ c ][ x ][ y ] << bitDepthShift[ c ]		(xx)

I[ c ][ x ][ y ] = v > ( 1  <<  bitDepth[ c ] ) – 1) ? ( 1  <<  bitDepth[ c ] ) – 1) : I[ c ][ x ][ y ]	(xx)

–	Otherwise (bitDepthShift[ c ] is equal to 0), no further processing applied on I[ c ][ x ][ y ].

Where I[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the input image I, bitDepth[ c ] is the number of bits used for each sample in a fixed-length unsigned binary representation of the arrays I[ c ][ x ][ y ], where c = 0..2, x = 0..PicWidthInLumaSamples – 1, and y = 0..PicHeightInLumaSamples – 1.

# Proposed method #3

It is proposed to signal the bit depth truncation information for each color component separately. The proposed text changes for this method #3 are described as follows, all the text changes described below are added texts on top of the current VSEI spec text.

## x.x Bit depth truncation SEI message

### x.x.x	Bit depth truncation SEI message syntax

| bit_depth_truncation( payloadSize ) { | Descriptor |
| --- | --- |
| bit_depth_truncation_cancel_flag | u(1) |
| if( !bit_depth_truncation_cancel_flag) { |  |
| bit_depth_truncation_param_luma | u(3) |
| bit_depth_truncation_param_cb | u(3) |
| bit_depth_truncation_param_cr | u(3) |
| } |  |
| } |  |

### x.x.x	Bit depth truncation SEI message semantics

The bit depth truncation (BDT) SEI message specifies various parameters of the left shift values that may be applied on the reconstructed image samples before sending the reconstructed image samples for further processing logic such as object detection or object tracking.

bit_depth_truncation_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous bit depth truncation SEI message in output order that applies to the current layer. bit_depth_offset_cancel_flag equal to 0 indicates that bit depth shift information follows.

bit_depth_truncation_param_luma, bit_depth_truncation_param_cb and bit_depth_truncation_param_cr, specifies the left shift value applied the sample values of the luma, Cb and Cr color components of the input image, respectively. The values of these syntaxes should be kept the same throughout a CVS or the coding efficiency may be significantly sacrificed.

Depending on the value of bit_depth_truncation_param_luma and bit_depth_truncation_param_chroma, the sample value shifting is specified as follows:

–	An array of variable bitDepthShiftFactor[ c ] is derived as follows:

bitDepthShift[ 0 ] = bit_depth_truncation_param_luma	(xx)

bitDepthShift[ 1 ] = bit_depth_truncation_param_cb	(xx)

bitDepthShift[ 2 ] = bit_depth_truncation_param_cr	(xx)

For c = 0..2, the followings apply:

–	If bitDepthShift[ c ] is not equal to 0, the sample value shifting processing as specified by:

v = I[ c ][ x ][ y ] << bitDepthShift[ c ]		(xx)

I[ c ][ x ][ y ] = v > ( 1  <<  bitDepth[ c ] ) – 1) ? ( 1  <<  bitDepth[ c ] ) – 1) : I[ c ][ x ][ y ]	(xx)

–	Otherwise (bitDepthShift[ c ] is equal to 0), no further processing applied on I[ c ][ x ][ y ].

Where I[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the input image I, bitDepth[ c ] is the number of bits used for each sample in a fixed-length unsigned binary representation of the arrays I[ c ][ x ][ y ], where c = 0..2, x = 0..PicWidthInLumaSamples – 1, and y = 0..PicHeightInLumaSamples – 1.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).