| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0139-v2 |
| --- | --- |

| Title: | EE1-3.1: neural network-based intra prediction with reduced complexity |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Dumas F. Galpin P. Bordes | Author(s) or Contact(s): | thierry.dumas@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

Following the 29th JVET meeting, three additional neural network-based coding tools, see JVET-AC0055, JVET-AC0196, and JVET-AC0116, were integrated into VTM-11-NNVC. In JVET-AC0116, a neural network-based intra prediction mode is added to the set of intra prediction modes in VTM-11-NNVC. To limit the complexity of the neural network-based intra prediction mode, each neural network in this mode features a relatively small architecture and weight sparsity is enforced. However, the architecture and weight sparsity policy are not tuned to each neural network.

To address this, JVET-AD0212 proposed a refined architecture and weight sparsity policy adapted to each neural network. This contribution reports EE1-3.1 associated to JVET-AD0212.

It is reported that NNVC-6 in which the models of the neural network-based intra prediction mode are replaced by the models with refined architecture and weight sparsity policy yields 0.22%, 0.16%, 0.10% and 0.08%, -0.01%, 0.02% in AI and RA respectively with respect to NNVC-6. In NNVC-6, the worst-case complexity of the neural network-based intra prediction mode is estimated at 7.8 kMACs/pixel whereas, in NNVC-6 containing the models with refined architecture and weight sparsity policy, the worst-case complexity of this mode becomes 4.8 kMACs/pixel.

# Architecture and weight sparsity policy

The neural network-based intra prediction mode contains 7 neural networks, each neural network predicting blocks of a different size in 4×4,8×4,16×4,32×4,8×8,16×8,16×16.

## Architecture policy

The evolution of the architecture policy from [3] to EE1-3.1 is presented in Table 1 using the definitions in Figure 1. Note that all the mentioned layers are fully-connected. No convolutional layers exist in the presented neural networks.

Figure 1: prediction of the current w×h block from its context of reference samples via the neural network fh, w . , θh,w, parametrized by θh,w, belonging to the neural network-based intra prediction mode. Here, w=8 and h=4. The dimensions of the context of reference samples, “preprocessing”, “postprocessing”, repIdx, grpIdx1, grpIdx2 are explained in [3].

Table 1: architecture policy for each neural network in [3] and EE1-3.1.

| architecture | [3] | EE1-3.1 |
| --- | --- | --- |
| Number of hidden layers | For 16x16, 3. For the other six neural networks, 2. | Same. |
| Number of neurons per hidden layer | 1216. | For 4x4, 8x4, and 16x16, 1216 for the last hidden layer, and 640 for the other hidden layers. For the other four neural networks, 1216. |
| Biases | For all layers. | Only for the layer returning repIdx and the layer returning grpIdx1 and grpIdx2. |

## Weight sparsity policy

Compared to [3], in EE1-3.1, the percentage of non-zero weights in the layer returning repIdx becomes 10% for the neural network predicting blocks of size 4×4, and 50% for the other six neural networks.

# AHG11 template

| Network information in training stage |  |  |
| --- | --- | --- |
| Mandatory | GPU type | GPU: Tesla-P100-16GB |
|  | Framework | Tensorflow 2.6 |
|  | Number of GPUs per task | 1 |
|  | Epoch | 15 |
|  | Batch size | 2048 |
|  | Training time | 8 hours to 3 days, depending on the training cycle. |
|  | Training data information | During the generation of the training sets for training a neural network for intra prediction, pairs of a block and its context are extracted from the partitioning of YUV frames encoded/decoded via NNVC-6.0. |
|  | Training configurations for generating compressed training data | QP belonging to {18, 19, 20, 21, 22, 27, 32, 37}. |
|  | Patch size | The neural network predicting w×h blocks is trained on pairs of a w×h block and its context of reference samples. |
|  | Learning rate | Decreasing from 1e-4 to 5e-8. |
|  | Optimizer | ADAM. |
|  | Loss function | SATD between the original block and its neural network prediction. |
|  | Preprocessing | See [3]. |

| Network information (inference) |  |  |
| --- | --- | --- |
| Parameter number | block size | Parameter number |
|  | 4×4 8×4 16×4 32×4 8×8 16×8 16×16 | 77477 120816 182356 229048 194023 238768 291311 |
| Parameter precision | 16-bit signed integer. |  |
| MAC/pixel | block size | MACs/pixel |
|  | 4×4 8×4 16×4 32×4 8×8 16×8 16×16 | 4837 3773 2848 1789 3030 1865 1138 |
| Total conv. layers |  |  |
| Total FC layers | 4 cascaded fully-connected layers for the neural network predicting 16×16 blocks. 3 cascaded fully-connected layers for the other six neural networks. |  |
| Mem (MB) | 4.9 MB for the whole neural network-based intra prediction mode. |  |
| GPU Type | No GPU |  |
| CPU Type | Xeon 2.4GHz |  |
| Framework | SADL (C++ with a single thread) |  |

## Worst-case complexity in MACs/pixel

Given Table 1, from [3] to EE1-3.1, the worst-case number in MACs/pixel of the neural network-based intra prediction mode drops from 7.8 kMACs/pixel to 4.8 kMACs/pixel.

## Average complexity in MACs/pixel

For EE1-3.1, in AI, over the CTC,

- average number in MACs/pixel of the neural network-based intra prediction mode, considering only the blocks predicted by this mode = 630 MACs/pixel

- average number in MACs/pixel, considering all the blocks predicted in intra = 339 MACs/pixel.

# Training with reduced amount of training data

The original training of the neural network-based intra prediction mode can be finished in about 15 days, provided that many CPUs (about 1600 cores) are available to generate the training sets at each training cycle. To speed-up the reproduction of the training of the neural network-based intra prediction mode, an alternative training with reduced amount of raw data is tested. Section 4 presents the results of both the original training and the alternative training with reduced amount of raw data.

# Experiments

The experimental results are presented in Figure 2, Figure 3, Figure 4, Figure 5, Figure 6, Figure 7, Figure 8, and Figure 9.

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0, --NnIntraPred=0, --NnlfOption=0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -4.04% | -3.60% | -3.91% | -4.23% | -3.41% | -3.67% | 166% | 371% |
| Class A2 | -2.70% | -2.45% | -2.79% | -2.66% | -2.00% | -2.22% | 170% | 468% |
| Class B | -3.02% | -2.58% | -2.86% | -2.82% | -2.28% | -2.37% | 171% | 509% |
| Class C | -3.00% | -2.73% | -2.89% | -2.98% | -2.36% | -2.32% | 162% | 552% |
| Class E | -4.50% | -4.03% | -3.75% | -4.38% | -3.64% | -2.42% | 168% | 526% |
| Overall | -3.38% | -3.00% | -3.18% | -3.32% | -2.67% | -2.56% | 167% | 488% |
| Class D | -2.83% | -2.51% | -2.68% | -2.82% | -2.15% | -1.76% | 156% | 594% |
| Class F | -2.04% | -1.76% | -1.70% | -1.94% | -1.65% | -1.39% | 142% | 322% |

Figure 2: NNVC-6 (NN-intra activated and LCNN deactivated) in which the models of the neural network-based intra prediction mode uses the refined architecture and weight sparsity policy versus NNVC-6 (NN-intra deactivated and LCNN deactivated) in AI. The original training of the neural network-based intra prediction mode is used.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0, --NnIntraPred=0, --NnlfOption=0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -2.20% | -0.92% | -1.49% | -2.02% | -0.57% | -1.05% | 130% | 155% |
| Class A2 | -1.39% | -0.28% | -0.93% | -1.35% | 0.27% | -0.35% | 128% | 139% |
| Class B | -1.70% | -0.39% | -0.71% | -1.59% | 0.07% | 0.18% | 129% | 156% |
| Class C | -1.63% | -0.88% | -1.03% | -1.47% | 0.01% | -0.01% | 130% | 176% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1.72% | -0.61% | -0.99% | -1.60% | -0.03% | -0.22% | 129% | 157% |
| Class D | -1.34% | -1.07% | -1.06% | -1.42% | -1.05% | -0.39% | 127% | 161% |
| Class F | -1.12% | -0.78% | -0.75% | -0.88% | -0.38% | 0.01% | 131% | 115% |

Figure 3: NNVC-6 (NN-intra activated and LCNN deactivated) in which the models of the neural network-based intra prediction mode uses the refined architecture and weight sparsity policy versus NNVC-6 (NN-intra deactivated and LCNN deactivated) in RA. The original training of the neural network-based intra prediction mode is used.

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0, --NnIntraPred=1, --NnlfOption=3 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.28% | 0.14% | 0.13% | 0.34% | 0.20% | 0.25% | 98% | 99% |
| Class A2 | 0.11% | -0.09% | 0.01% | 0.18% | -0.11% | 0.06% | 97% | 99% |
| Class B | 0.20% | 0.17% | 0.08% | 0.24% | 0.16% | -0.06% | 95% | 96% |
| Class C | 0.18% | 0.07% | 0.13% | 0.24% | -0.06% | -0.04% | 92% | 94% |
| Class E | 0.37% | 0.56% | 0.18% | 0.48% | 0.68% | -0.03% | 97% | 95% |
| Overall | 0.22% | 0.16% | 0.10% | 0.29% | 0.16% | 0.02% | 95% | 96% |
| Class D | 0.15% | 0.14% | 0.24% | 0.08% | -0.17% | 0.38% | 93% | 96% |
| Class F | 0.17% | 0.01% | 0.16% | 0.26% | -0.24% | 0.12% | 99% | 96% |

Figure 4: NNVC-6 (NN-intra activated and LCNN activated) in which the models of the neural network-based intra prediction mode uses the refined architecture and weight sparsity policy versus NNVC-6 (NN-intra activated and LCNN activated) in AI. The original training of the neural network-based intra prediction mode is used.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0, --NnIntraPred=1, --NnlfOption=3 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.13% | -0.04% | 0.09% | 0.16% | -0.31% | 0.23% | 100% | 100% |
| Class A2 | 0.02% | -0.24% | -0.06% | 0.08% | -0.25% | -0.16% | 99% | 100% |
| Class B | 0.11% | -0.05% | -0.02% | 0.03% | -0.07% | -0.21% | 99% | 100% |
| Class C | 0.06% | 0.23% | 0.09% | 0.10% | 0.24% | -0.41% | 95% | 95% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.08% | -0.01% | 0.02% | 0.09% | -0.07% | -0.17% | 98% | 98% |
| Class D | 0.05% | 0.08% | 0.26% | 0.24% | -0.42% | 0.02% | 103% | 98% |
| Class F | 0.03% | 0.14% | -0.20% | 0.16% | 0.10% | 0.00% | 101% | 102% |

Figure 5: NNVC-6 (NN-intra activated and LCNN activated) in which the models of the neural network-based intra prediction mode uses the refined architecture and weight sparsity policy versus NNVC-6 (NN-intra activated and LCNN activated) in RA. The original training of the neural network-based intra prediction mode is used.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0, --NnIntraPred=0, --NnlfOption=0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.82% | -3.17% | -3.49% | -3.93% | -3.00% | -3.38% | 174% | 372% |
| Class A2 | -2.64% | -2.02% | -2.37% | -2.62% | -1.84% | -2.04% | 180% | 477% |
| Class B | -2.79% | -2.26% | -2.36% | -2.63% | -2.01% | -2.01% | 180% | 512% |
| Class C | -2.27% | -1.67% | -1.67% | -2.42% | -1.36% | -1.41% | 172% | 510% |
| Class E | -3.98% | -3.65% | -3.25% | -3.85% | -3.24% | -2.03% | 176% | 530% |
| Overall | -3.02% | -2.47% | -2.54% | -3.00% | -2.21% | -2.11% | 177% | 482% |
| Class D | -1.96% | -1.37% | -1.37% | -1.99% | -0.74% | -0.80% | 167% | 553% |
| Class F | -1.65% | -1.27% | -1.14% | -1.64% | -1.06% | -0.84% | 150% | 300% |

Figure 6: NNVC-6 (NN-intra activated and LCNN deactivated) in which the models of the neural network-based intra prediction mode uses the refined architecture and weight sparsity policy versus NNVC-6 (NN-intra deactivated and LCNN deactivated) in AI. The training with reduced amount of raw data is used.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0, --NnIntraPred=0, --NnlfOption=0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -2.07% | -0.59% | -1.36% | -1.90% | -0.49% | -1.11% | 127% | 151% |
| Class A2 | -1.36% | -0.12% | -0.68% | -1.31% | 0.10% | -0.15% | 125% | 139% |
| Class B | -1.62% | -0.08% | -0.19% | -1.48% | 0.39% | 0.78% | 127% | 157% |
| Class C | -1.25% | -0.71% | -0.52% | -1.14% | -0.10% | -0.03% | 129% | 175% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1.56% | -0.36% | -0.61% | -1.44% | 0.03% | 0.00% | 127% | 157% |
| Class D | -0.98% | -0.25% | -0.58% | -1.00% | -0.01% | -0.09% | 126% | 165% |
| Class F | -0.80% | -0.43% | -0.39% | -0.64% | -0.21% | 0.00% | 130% | 115% |

Figure 7: NNVC-6 (NN-intra activated and LCNN deactivated) in which the models of the neural network-based intra prediction mode uses the refined architecture and weight sparsity policy versus NNVC-6 (NN-intra deactivated and LCNN deactivated) in RA. The training with reduced amount of raw data is used.

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0, --NnIntraPred=1, --NnlfOption=3 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.48% | 0.55% | 0.55% | 0.61% | 0.62% | 0.57% | 98% | 97% |
| Class A2 | 0.12% | 0.28% | 0.29% | 0.15% | 0.06% | 0.18% | 96% | 99% |
| Class B | 0.37% | 0.46% | 0.56% | 0.39% | 0.46% | 0.31% | 94% | 94% |
| Class C | 0.81% | 0.95% | 1.22% | 0.75% | 0.86% | 0.82% | 93% | 91% |
| Class E | 0.74% | 0.96% | 0.56% | 0.94% | 1.15% | 0.38% | 97% | 93% |
| Overall | 0.51% | 0.64% | 0.66% | 0.56% | 0.62% | 0.46% | 95% | 94% |
| Class D | 0.94% | 0.71% | 1.28% | 0.82% | 0.95% | 1.42% | 94% | 92% |
| Class F | 0.51% | 0.49% | 0.62% | 0.54% | 0.39% | 0.55% | 99% | 93% |

Figure 8: NNVC-6 (NN-intra activated and LCNN activated) in which the models of the neural network-based intra prediction mode uses the refined architecture and weight sparsity policy versus NNVC-6 (NN-intra activated and LCNN activated) in AI. The training with reduced amount of raw data is used.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0, --NnIntraPred=1, --NnlfOption=3 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.22% | 0.12% | 0.26% | 0.30% | -0.26% | 0.37% | 100% | 99% |
| Class A2 | 0.02% | -0.32% | -0.01% | 0.09% | -0.54% | -0.08% | 100% | 100% |
| Class B | 0.16% | 0.09% | 0.19% | 0.14% | 0.17% | 0.22% | 100% | 100% |
| Class C | 0.39% | 0.40% | 0.43% | 0.48% | 0.06% | -0.21% | 96% | 97% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.21% | 0.10% | 0.23% | 0.25% | -0.09% | 0.08% | 99% | 99% |
| Class D | 0.38% | 0.46% | 0.37% | 0.56% | 0.08% | -0.28% | 99% | 98% |
| Class F | 0.29% | 0.29% | 0.14% | 0.33% | 0.22% | -0.08% | 99% | 100% |

Figure 9: NNVC-6 (NN-intra activated and LCNN activated) in which the models of the neural network-based intra prediction mode uses the refined architecture and weight sparsity policy versus NNVC-6 (NN-intra activated and LCNN activated) in RA. The training with reduced amount of raw data is used.

# Conclusion

NNVC-6 in which the models of the neural network-based intra prediction mode are replaced by the models featuring refined architecture and weight sparsity policy yields 0.22%, 0.16%, 0.10% and 0.08%, -0.01%, 0.02% in AI and RA respectively with respect to NNVC-6. The introduction of the refined architecture and weight sparsity policy decreases the worst-case complexity of the neural network-based intra prediction mode from 7.8 kMACs/pixel to 4.8 kMACs/pixel.

It is suggested to integrate the models of the neural network-based intra prediction mode with refined architecture and weight sparsity policy into the next version of VTM-11-NNVC.

# References

[1] M. Santamaria, R. Yang, F. Cricri, J. Lainema, H. Zhang, R. G. Youvalari, M. Hannuksela. EE1-1.11 : Content-adaptive post-filter. JVET-AC0055, 29th JVET meeting, Teleconference, 11-20 January 2023.

[2] R. Chang, L. Wang, X. Xu, S. Liu. EE1-2.2: GOP Level Adaptive Resampling with CNN-based Super Resolution. JVET-AC0196, 29th JVET meeting, Teleconference, 11-20 January 2023.

[3] T. Dumas, F. Galpin, P. Bordes. EE1-3.2: neural network-based intra prediction with learned mapping to VVC intra prediction modes. JVET-AC0116, 29th JVET meeting, Teleconference, 11-20 January 2023.

[4] T. Dumas, F. Galpin, P. Bordes. AHG11: neural network-based intra prediction with reduced complexity. JVET-AD0212, 30th JVET meeting, Antalya, 21-28 April 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).