| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0144-v1 |
| --- | --- |

| Title: | [AHG9] Adaptive Film grain synthesis using Alpha Channel Information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gilles Teniou Stephan Wenger | Tel: Email: (teniou,swenger) [at]global.tencent.com |  |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Film Grain Synthesis (aka FGS) is a tool aimed at maintaining the original film grain while preserving the encoding efficiency due to its random nature and complexity. While being used to maintain the artistic intent or to mitigate visual artefacts, film grain is not seen relevant for the entire sequence or even within an image. This contribution proposes to address the film grain synthesis applied locally on a per image basis by using the auxiliary data as a mask.

# Problem statement

Film grain is well none feature originated from the small particles of silver-halide crystals on the film that became easy to recognize as a movie artistic specificity. The film grain effect also gives to games a cinematic feel by simulating the grainy visuals that are present in some movies. It creates realism if used softly and locally where it has value for the viewer. In such a case those artificial imperfections make synthetic contents more realistic. Finally, film grain may also be used to soften the visual artefacts incurred in the capture process or during the compression of the video signal.

In many cases, the film grain provides a certain level of realism only in some parts of the picture, for example on the skin texture, while keeping a soft background.

In its simplest approach the spatial adaptation of the film grain synthesis combines the calculated film grain values with a binary mask conveyed in the auxiliary data. In this case each film grain value is applied as is or not at all in the image.

Another approach is to use the mask to define weighting factors following the CTU structure of the main picture and apply more film grain in areas with a lower frequency.

An alternate approach, still relying on the same process is to use a mask with weighting factors on the film grain values. Such a mask may be a depth map transmitted as auxiliary data for 3D rendering purposes. In such a scenario, the film grain is applied to forefront objects while the background remains unchanged. This configuration is often used to increase the depth perception.

# Proposed approach

The film grain characteristics SEI message has been defined to synthetize the grain of the content without encoding it. Today, it lacks the spatial adaptation to the image. As shown in the example below:

The VSEI specification [1] also specifies the Alpha Channel Information (ACI) SEI messages that defines how to combine the auxiliary data with the output image for alpha blending.

The ACI SEI message has 2 defined modes in alpha_channel_use_idc: mode 0: multiplicative and mode 1: non multiplicative. A 3rd mode (mode 3, since mode 2 is set to "unspecified") can be defined as the multiplicative use of the auxiliary data with the corresponding simulated film grain, defined by the Film Grain characteristics SEI message.

While interpreting the film grain characteristics SEI message, if the ACI SEI message is also present for the same picture with alpha_channel_use_idc equal to 3, then the associated auxiliary picture is used as a weighting factor of the film grain values.

Original, FGC only and FGC+ACI modes

The following clause depicts the impacted changes on the VSEI specification.

# Proposal

Change 1

## 8.5	Film grain characteristics SEI message

### 8.5.2	Film grain characteristics SEI message semantics

fg_blending_mode_id identifies the blending mode used to blend the simulated film grain with the input images as specified in Table 6. fg_blending_mode_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for fg_blending_mode_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain characteristic SEI messages with fg_blending_mode_id equal to 2 or 3.

Table 6 – fg_blending_mode_id values

| Value | Description |
| --- | --- |
| 0 | Additive |
| 1 | Multiplicative |

Depending on the value of fg_blending_mode_id, the blending mode is specified as follows:

–	If fg_blending_mode_id is equal to 0, the blending mode is additive as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Î[ c ][ x ][ y ] + G[ c ][ x ][ y ] )	(21)

–	Otherwise (fg_blending_mode_id is equal to 1), the blending mode is multiplicative as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Î[ c ][ x ][ y ] +	(22)
			Round( ( Î[ c ][ x ][ y ] * G[ c ][ x ][ y ] ) ÷ ( ( 1  <<  fgBitDepth[ c ] ) − 1 ) ) )

where Î[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the input image Î, G[ c ][ x ][ y ] is the simulated film grain value at the same position and colour component, and fgBitDepth[ c ] is the number of bits used for each sample in a fixed-length unsigned binary representation of the arrays Igrain[ c ][ x ][ y ], Î[ c ][ x ][ y ], and G[ c ][ x ][ y ], where c = 0..2, x = 0..PicWidthInLumaSamples − 1, and y = 0..PicHeightInLumaSamples − 1.

If the current PU contains an ACI SEI message (as defined in clause 8.23.2) with alpha_channel_use_idc equal to 3, the simulated film grain value shall be weighted by the auxiliary data value as follow:

–	If fg_blending_mode_id is equal to 0, the blending mode is additive as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Î[ c ][ x ][ y ] + G[ c ][ x ][ y ]* Iaux[ c ][ x ][ y ])

–	Otherwise (fg_blending_mode_id is equal to 1), the blending mode is multiplicative as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Î[ c ][ x ][ y ] +
			Round( ( Î[ c ][ x ][ y ] * G[ c ][ x ][ y ]* Iaux[ c ][ x ][ y ]) ) ÷ ( ( 1  <<  fgBitDepth[ c ] ) − 1 ) ) )

Where Iaux[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the decoded auxiliary picture.

End of change 1

Change 2

## 8.23	Alpha channel information SEI message

### […]

### 8.23.2	Alpha channel information SEI message semantics

The alpha channel information (ACI) SEI message provides information about alpha channel sample values and post-processing applied to the decoded alpha planes coded in auxiliary pictures of type AUX_ALPHA and one or more associated primary pictures.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i, no picture in the CVS shall be associated with an ACI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i and an ACI SEI message, the SDI SEI message shall precede the ACI SEI message in decoding order.

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as an alpha auxiliary layer by an SDI SEI message, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

–	A CLVS containing the auxiliary picture picA ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the ACI SEI message applies.

alpha_channel_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous ACI SEI message in output order that applies to the current layer. alpha_channel_cancel_flag equal to 0 indicates that ACI follows.

Let currPic be the picture that the ACI SEI message is associated with. The semantics of ACI SEI message persist for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an ACI SEI message is output that follows the current picture in output order.

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the decoded auxiliary picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the decoded auxiliary picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. alpha_channel_use_idc equal to 3 indicates that the simulated film grain value at the same position and colour component samples should be multiplied by the interpretation sample values of the decoded auxiliary picture prior to calculating the film grain values. Values greater than 3 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2. Decoders shall ignore alpha channel information SEI messages in which alpha_channel_use_idc is greater than 3.

End of change 2

# References

[1]	Recommendation ITU-T H.274 (V2) Versatile supplemental enhancement information messages for coded video bitstream, May 2022.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).