| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0147-v1 |
| --- | --- |

| Title: | AHG8/AHG9: On Picture Modality Type |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingying Gao Han Boon Teo Chong Soon Lim Kiyofumi Abe | Tel: Email: | +65-6550-5406 jingying.gao@sg.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposes to add picture modality types in VSEI. The picture modality type such as visible picture and infrared picture tells the nature of the picture data. This distinction can be crucial for applications where the processing, interpretation, or display of the decoded picture is influenced by its spectral properties. Hence, it is proposed to add three picture modality types (visible, infrared and thermal infrared) in VSEI by one of the following options:

-	Option 1:  Signal picture modality type in Encoder Optimization Information (EOI) SEI message in JVET-AE2032.

-	Option 2:  Signal picture modality type in VUI parameters.

-	Option 3:   Signal picture modality type in Scalability Dimensional Information (SDI) SEI message in VSEI.

It is proposed to adopt one of the proposed option to VSEI.

# Introduction

Traditional video coding methods, such as Versatile Video Codec (VVC/H.266), were designed primarily for human vision. With the rise of machine learning/neural network-based applications and the abundance of image sensors, more and more video data are consumed by machines rather than humans. There is an increasing demand for videos catering to machine vision, which is beyond the limitations of human sight. Human vision is restricted to the visible light spectrum, while machine vision extends beyond these boundaries. More and more videos are consumed by machines rather than humans. Today's advanced image sensors, which operate across a variety of spectral ranges, are being integrated into cameras. These image sensors are widely used in applications including autonomous vehicles, UAVs, industrial automation, and more [1].

As illustrated in Figure 1, the picture can be captured from various ranges of spectrum, such as visible light and infrared light. The source of light defines the picture modality type, and pictures of different modality types serve different purposes. Table 1 shows examples of picture modality types, visible pictures have rich detailed color information and are used for both human vision and machine vision; active infrared (IR) sensors can assist in distinguishing objects under low light conditions or obscured by the scattering effects of dust and fog; thermal infrared (TIR) sensors have the capability to capture videos based on heat, and the thermal infrared images can be used to detect hidden crack which is invisible by human vision. Different picture modality types may affect the processing, interpretation, or display of the decoded picture. It’s beneficial to signal the picture modality type in the video bitstream. This proposal aims to improve the machine analysis accuracy by distinguishing different modality types of pictures.

Figure 1: An example of the electromagnetic spectrum.

Table 1: Examples of picture modality types

| Picture modality type | Source of light | Possible application |
| --- | --- | --- |
| Visible (VIS) picture | Visible light | Human vision, machine vision, etc. |
| Infrared (IR) picture | External infrared light | Night Vision, Obstacle detection, etc. |
| Thermal Infrared (TIR) picture | Thermal radiation | Building inspections, Electrical inspections, etc. |

# Problem Statement

Different modality types of pictures (i.e., different-sensor data) are widely used in various vision tasks for more accurate or robust prediction. Examples of different picture modality types include visible pictures, infrared pictures, thermal infrared pictures, etc. Visible picture from the visible spectrum provides detailed color information under good lighting conditions; infrared pictures are often used for night vision; thermal infrared imaging can detect living beings and other heat sources even in complete darkness. Various picture modality types have unique characteristics, and as such, different machine vision models are often developed and trained to analyze and interpret these distinct modality types of pictures. However, VSEI doesn’t specify the type of picture modality. If the decoder is not aware of the picture modalities, an appropriate machine vision network model may not be selected, hence, resulting in a drop in accuracy.

# Proposal

## Option 1: Signal picture modality type in encoder optimization information (EOI) SEI message

Modifications to Encoder optimization information SEI message in JVET-AE2032 [2].

Syntax:

| encoder_optimization_info( payloadSize ) { | Descriptor |
| --- | --- |
| optimization_cancel_flag | u(1) |
| if( !optimization_cancel_flag ){ |  |
| optimization_persistence_flag | u(1) |
| optimization_for_human_viewing_flag | u(1) |
| optimization_for_machine_analysis_flag | u(1) |
| optimization_type | u(16) |
| optimization_modality_info_present_flag | u(1) |
| if(optimization_modality_info_present_flag) { |  |
| optimization_modality_type | u(8) |
| } |  |
| } |  |
| } |  |

Table 2 – Mapping of optimization_modality_type to the type of picture modalities

| optimization_modality_type | type of picture modality |
| --- | --- |
| 0 | Visible Picture |
| 1 | Infrared Picture |
| 2 | Thermal Infrared Picture |
| 0..255 | Reserved for future use |

Semantic:

optimization_modality_info_present_flag equal to 1 specifies that  optimization_modality_type is present in the encoder optimization information SEI message. optimization_modality_info_present_flag equal to 0 specifies that optimization_modality_type is not present in the encoder optimization information SEI message. When not present, the value of optimization_modality_info_present_flag is inferred to be equal to 0.

optimization_modality_type indicates the type of modality of the decoded picture as specified in Table 2. When not present, the value of optimization_modality_type is inferred to be equal to 0.

NOTE - The decoded color component could be ignored when optimization_modality_type is equal to 1 or 2.

## Option 2: Signal picture modality type in VUI parameters.

Modifications to VUI.

Syntax:

| vui_parameters( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| vui_modality_info_present_flag | u(1) |
| if(vui_modality_info_present_flag) { |  |
| vui_modality_type | u(8) |
| } |  |
| } |  |

Table 3 – Mapping of vui_modality_type to the type of picture modalities

| vui_modality_type | type of picture modality |
| --- | --- |
| 0 | Visible Picture |
| 1 | Infrared Picture |
| 2 | Thermal Infrared Picture |
| 0..255 | Reserved for future use |

Semantic:

vui_modality_info_present_flag equal to 1 specifies that vui_modality_type is present in the VUI parameters. vui_modality_info_present_flag equal to 0 specifies that vui_modality_type is not present in the VUI parameters. When not present, the value of vui_modality_info_present_flag is inferred to be equal to 0.

vui_modality_type indicates the type of modality of the decoded picture as specified in Table 3. When not present, the value of vui_modality_type is inferred to be equal to 0.

NOTE - The decoded color component could be ignored when vui_modality_type is equal to 1 or 2.

## Option 3: Signal picture modality type in Scalability Dimensional Information (SDI) SEI message.

Modifications to sdi_aux_id[i] in Scalability dimension information SEI message in VSEI.

Semantic:

The scalability dimension information (SDI) SEI message provides the SDI for each layer in the current CVS, i.e., the CVS containing the SDI SEI message, such as 1) when there may be multiple views, the view ID of each layer; and 2) when there may be auxiliary information (such as depth or alpha) carried by one or more layers, the auxiliary ID of each layer.

When an SDI SEI message is present in any AU of a CVS, an SDI SEI message shall be present for the first AU of the CVS. All SDI SEI messages in a CVS shall have the same content.

sdi_aux_id[ i ] equal to 0 indicates that the i-th layer in the current CVS does not contain auxiliary pictures. sdi_aux_id[ i ] greater than 0 indicates the type of auxiliary pictures in the i-th layer in the current CVS as specified in Table 4. When sdi_auxiliary_info_flag is equal to 0, the value of sdi_aux_id[ i ] is inferred to be equal to 0.

Table 4 – Mapping of sdi_aux_id[ i ] to the type of auxiliary pictures

| sdi_aux_id[ i ] | Name | Type of auxiliary pictures |
| --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| 3 | AUX_VISIBLE | Visible picture |
| 4 | AUX_INFRARED | Infrared picture |
| 5 | AUX_THERMAL_INFRARED | Thermal infrared picture |
| 6..127 |  | Reserved |
| 128..159 |  | Unspecified |
| 160..255 |  | Reserved |

NOTE 1 – The interpretation of auxiliary pictures associated with sdi_aux_id[ i ] in the range of 128 to 159, inclusive, is specified through means other than the sdi_aux_id[ i ] value.

sdi_aux_id[ i ] shall be in the range of 0 to 5, inclusive, or 128 to 159, inclusive, for bitstreams conforming to this version of this Specification. Although the value of sdi_aux_id[ i ] shall be in the range of 0 to 5, inclusive, or 128 to 159, inclusive, in this version of this Specification, decoder shall also allow other values of sdi_aux_id[ i ] in the range of 0 to 255, inclusive.

If sdi_aux_id[ i ] is equal to 0, the i-th layer is referred to as a primary layer. Otherwise, the i-th layer is referred to as an auxiliary layer. When sdi_aux_id[ i ] is equal to 1, the i-th layer is also referred to as an alpha auxiliary layer. When sdi_aux_id[ i ] is equal to 2, the i-th layer is also referred to as a depth auxiliary layer. When sdi_aux_id[ i ] is equal to 3, the i-th layer is also referred to as a visible auxiliary layer. When sdi_aux_id[ i ] is equal to 4, the i-th layer is also referred to as an infrared auxiliary layer. When sdi_aux_id[ i ] is equal to 5, the i-th layer is also referred to as a thermal infrared auxiliary layer.

# Use Case

Example 1

In certain scenarios, sensor switching becomes necessary due to the inherent limitations of the sensors. In a multi-sensor setup incorporating an RGB sensor and an IR sensor (such as the Panasonic i-PRO Extreme WV-S2131L), the camera system has the capability to activate and deactivate these sensors in response to environmental conditions. For example, the detection of object motion, low light, fog, or rain triggers the enabling of the IR sensor while concurrently disabling the RGB sensor. During this process, the IR cut filter plays a role; when activated, it blocks IR, allowing only visible light to be captured by the RGB sensor. Conversely, when deactivated, IR is captured by the IR sensor. Consequently, the resultant video bitstream will contain pictures of varying modality types as illustrated in Figure 2. Our proposal helps to indicate the picture modality, thereby enabling the selection of an appropriate network model to perform the machine vision task on the decoded picture, as illustrated in Figure 3. This will improve the accuracy of the machine vision algorithm.

Figure.2: An example of the video bitstream containing visible (VIS) pictures and infrared (IR) pictures.

Figure.3: An example of using picture modality to select an appropriate network for the decoded picture.

Example 2

For real-world applications, it is necessary to utilize image sensors that operate in different spectrums to enhance the performance and reliability of various vision tasks. Pictures captured in different spectrums can be coded in a multi-layered bitstream. As shown in Figure 4, visible pictures may be coded as primary pictures, whereas thermal infrared pictures may be coded as auxiliary pictures. The primary pictures and the auxiliary pictures are incorporated to provide rich and accurate information about the scene. Hence, signaling picture modalities in the bitstream enable the decoder to select an appropriate model for each modality type of picture, which may improve the machine analysis accuracy.  The picture modality types for the multi-layered structure can be specified in the SDI SEI message as proposed in option 3. It is also possible to specify picture modality types for the multilayer structure using option 1 or option 2.

For instance, UAVs (drones) equipped with thermal infrared sensors and cameras are used to detect cracks and defects in structures; hidden cracks and defects are invisible to human vision, however, when a structure is heated or cooled, the areas with cracks or defects will change the temperature at a different rate compared to the undamaged areas, creating a thermal contrast that can be detected by thermal infrared sensors; the thermal infrared pictures are used for crack detection and the detection result (the identification and location of the cracks) will be displayed on the visible pictures for human vision. The surface crack can be detected using visible pictures. The detection results on both infrared pictures and visible pictures can be fused to generate a more comprehensive detection result.

Figure.4: Another example of the video bitstream containing visible (VIS) pictures as primary pictures and thermal infrared (TIR) as auxiliary pictures.

# Reference

[1] M. El Guendy and X. He, "Emerging Image Sensor Technologies 2024-2034: Applications and Markets," IDTechEx, Sep. 2023. [Online].

[2]	Miska M. Hannuksela, Jie Chen, Sachin Deshpande, Sean McCarthy “Technologies under consideration for future extensions of VSEI (draft 1)”, JVET-AE2032, 31st Meeting, Geneva, CH, 11–19 July 2023.

# Patent rights declaration(s)

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).