| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0150-v1 |
| --- | --- |

| Title: | AhG11: on HOP batch size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin, T. Dumas P. Bordes | Author(s) or Contact(s): | first.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

A Unified Filter Architecture for the High-performance Operation Point (HOP) was defined at JVET-AD0380. It includes filter architecture, training process and training test set. Training strategy and full results were presented in JVET-AF0041. This contribution gives additional results on the impact of the batch size on the training results. It is reported that smaller batch size improves the results and also allows reducing the requirements on the GPU memory size. Results for a smaller batch size are {} xxx

# Introduction

At meeting AD, the HOP model design and training procedure were decided in JVET-AD0380. The training procedure is decomposed into 3 stages. The first 2 stages were successfully analyzed and validated in JVET-AE0191. The last stage strategy was refined in JVET-AE0289 and JVET-AE0291. Full results are available in JVET-AF0041.

During LOP model training, the impact of batch size was studied in JVET-AF0042/JVET-AF0043. It was shown that a larger batch size slightly improves the results and decreases the number of epochs needed to train the model.

This contribution studies the impact of the batch size on the HOP model. It turns out that a smaller batch size seems more adequate in the case of HOP training. It also allows to train on GPU with less memory (e.g. 16GB).

# Simulations

Three hyper-parameters are tested, all other aspects are the same, following the official HOP training procedure and dataset:

- Batch size of 64: the default batch size

- Batch size of 128: similar to LOP

- Batch size of 32: smaller batch size.

Partial training is performed, that is training only from stage 3, using official stage 3 dataset.

## Validation loss

Validation loss for stage III is shown in the graph above.

From the graph, it seems that a batch size of 32 is slightly better than the default size of 64. On the opposite, a batch size of 128 is slightly worse than the default size of 64.

## Intermediate performance results

xxx

## Final results

### Batch size 32

xxx

### Anchor batch size 64

The following results were obtained using NNVC-6.0 (commit bf862678) in VTM mode, with HOP Stage III model trained by Interdigital. This version corresponds to the new anchor of NNVC for HOP tests.

Table  NNVC 6.0 VTM vs NNVC 6.0 VTM + HOP float

|  | NNVC 6.0 VTM vs NNVC-6.0rc VTM+HOP float |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -7.18% | -18.19% | -21.99% | -10.64% | -21.11% | -25.77% |
| Class A2 | -7.13% | -19.97% | -18.55% | -10.82% | -23.70% | -22.55% |
| Class B | -7.02% | -18.74% | -19.64% | -9.67% | -26.01% | -24.26% |
| Class C | -8.12% | -18.58% | -21.17% | -10.50% | -27.46% | -27.27% |
| Class E | -10.55% | -20.81% | -21.89% |  |  |  |
| Overall | -7.90% | -19.16% | -20.57% | -10.31% | -24.96% | -25.02% |
| Class D | -7.95% | -17.88% | -21.47% | -11.92% | -26.82% | -27.43% |
| Class F | -5.28% | -15.73% | -15.93% | -5.83% | -18.50% | -17.02% |

Using a naïve quantization (all quantizers equal to 1<<11), the following results are obtained.

Table  NNVC 6.0 VTM vs NNVC 6.0 VTM + HOP int16

|  | NNVC 6.0 VTM vs NNVC-6.0 VTM+HOP int16 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -7.20% | -18.27% | -22.13% | -10.52% | -20.62% | -25.85% |
| Class A2 | -7.09% | -20.09% | -18.85% | -10.79% | -23.75% | -22.69% |
| Class B | -6.98% | -19.22% | -19.78% | -9.65% | -25.88% | -24.33% |
| Class C | -8.08% | -18.41% | -21.09% | -10.49% | -27.44% | -27.39% |
| Class E | -10.45% | -20.65% | -21.76% |  |  |  |
| Overall | -7.86% | -19.27% | -20.64% | -10.27% | -24.82% | -25.12% |
| Class D | -7.92% | -17.77% | -21.44% | -11.88% | -26.75% | -27.45% |
| Class F | -5.27% | -15.78% | -15.91% | -5.81% | -18.45% | -17.28% |

### Batch size 128

As results for batch size 128 are worse, a full test has not been performed.

# Conclusion

Adapting the batch size for HOP model training to 32 is proposed. It is reported that it gives slightly better results and also allows to train the model on smaller GPU, using less than 16GB of GPU memory.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).