| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0155-v1 |
| --- | --- |

| Title: | AhG11: on HOP luma/chroma balance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin, T. Dumas P. Bordes | Author(s) or Contact(s): | first.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

A Unified Filter Architecture for the High-performance Operation Point (HOP) was defined at JVET-AD0380. It includes filter architecture, training process and training test set. Training strategy and full results were presented in JVET-AF0041. This contribution reports additional results on the impact of the luma/chroma balance. Both dataset adaptation and training adaptation are studied. xxx

# Introduction

At meeting AD, the HOP model design and training procedure was decided in JVET-AD0380. The training procedure is decomposed into 3 stages. The first 2 stages were successfully analyzed and validated in JVET-AE0191. The last stage strategy was refined in JVET-AE0289 and JVET-AE0291. Full results are available in JVET-AF0041. During LOP model training, the impact of the luma/chroma balance was studied in JVET-AF0042/JVET-AF0043. It turns out that a larger weight for luma (12 instead of 6) was beneficial. This contribution studies the impact of the luma/chroma balance on the HOP model. xxx

# Simulations

In all the simulations, the official HOP training scripts are used. Only the luma/chroma parameters (either training weights or QP chroma offset) are changed.

## Chroma balance adaptation only

In these tests, the original stage 3 dataset is used and only the chroma balance during the training is changed.

### Validation loss

The figure above shows the validation loss for Y (as YUV loss is not comparable) for several luma/chroma balance.

Note: some curves are partial due to logs overwritten.

In all curves but the 20:1:1, the Y loss is smaller, as expected, as the weight on Y error is larger.

### Impact on the bd-rate

The table below show the impact of the different luma/chroma balance on the final gains for class C and D.

|  | NNVC 6.0-VTM vs NNVC-6.0-VTM+HOP 6:1:1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class C | -8.11 | -18.58 | -21.16 | -10.50 | -27.46 | -27.27 |
| Class D | -7.95 | -17.88 | -21.47 | -11.92 | -26.82 | -27.43 |

|  | NNVC 6.0-VTM vs NNVC-6.0-VTM+HOP QP 6:1:1 offset=1cp |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class C | -9.07 | -10.23 | -12.73 | -11.22 | -17.74 | -17.16 |
| Class D | -8.85 | -9.95 | -13.07 | -12.62 | -17.36 | -17.85 |

|  | NNVC 6.0-VTM vs NNVC-6.0-VTM+HOP 10:1:1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class C |  |  |  | -10.76 | -24.73 | -23.94 |
| Class D |  |  |  | -12.07 | -23.92 | -24.62 |

|  | NNVC 6.0-VTM vs NNVC-6.0-VTM+HOP 12:1:1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |

|  | NNVC 6.0-VTM vs NNVC-6.0-VTM+HOP 16:1:1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |

|  | NNVC 6.0-VTM vs NNVC-6.0-VTM+HOP 20:1:1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class C | -7.51 | -11.38 | -10.61 | -9.38 | -16.91 | -12.95 |
| Class D | -7.49 | -8.96 | -9.70 | -10.71 | -15.12 | -12.06 |

|  | NNVC 6.0-VTM vs NNVC-6.0-VTM+HOP 32:1:1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |

### Impact on full CTC

|  | NNVC 6.0-VTM vs NNVC-6.0-VTM+HOP 6:1:1 (float) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  | -10.64% | -21.11% | -25.77% |
| Class A2 |  |  |  | -10.82% | -23.70% | -22.55% |
| Class B |  |  |  | -9.67% | -26.01% | -24.26% |
| Class C |  |  |  | -10.50% | -27.46% | -27.27% |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  | -10.31% | -24.96% | -25.02% |
| Class D |  |  |  | -11.92% | -26.82% | -27.43% |
| Class F |  |  |  | -5.83% | -18.50% | -17.02% |

|  | NNVC 6.0-VTM vs NNVC-6.0-VTM+HOP 6:1:1 QP offset=1 (float) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  | -13.09% | -10.68% | -19.07% |
| Class A2 |  |  |  | -13.00% | -15.28% | -14.85% |
| Class B |  |  |  | -10.35% | -14.95% | -12.92% |
| Class C |  |  |  | -11.33% | -17.86% | -17.45% |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  | -11.69% | -14.94% | -15.74% |
| Class D |  |  |  | -12.61% | -16.96% | -17.94% |
| Class F |  |  |  | -7.02% | -11.68% | -10.50% |

|  | NNVC 6.0-VTM vs NNVC-6.0-VTM+HOP 10:1:1 (float) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |

|  | NNVC 6.0-VTM vs NNVC-6.0-VTM+HOP 32:1:1 (float) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |

## Dataset adaptation

In a second experiment, the official dataset for stage 3 was recomputed. All encoder parameters are the same as for the official HOP stage 3 dataset generation, except the QP chroma offset which was set to 1.

By increasing the QP chroma offset, the quality on Y, U and V of the dataset are more balanced.

Partial training is then performed, that is training only from stage 3, using official stage 3 dataset.

After generating the new dataset, several trainings have been performed.

### Validation loss

As with the original dataset, the loss on Y is better when the weight on Y is larger.

### Impact on the bd-rate

### Impact on full CTC

# Conclusion

Adapting the luma/chroma balance for HOP model training is proposed. xxx

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).