| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0160-v1 |
| --- | --- |

| Title: | Non-EE2: On DMVR Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar, Yuwen He, Kai Zhang, Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Email: | mehdi.salehifar@bytedance.com yuwen.he@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This proposal introduces improvements for DMVR. Specifically, DMVR is extended to non-equal POC distance cases and 16×16 BDOF DMVR is added. Also, a mean removed formula is introduced for BDOF DMVR.

Simulation results on top of the ECM-10.0 are reported as:

RA: -0.17%, -0.20%, -0.23%, 102.1%, 102.9%.

# Introduction

In ECM-10.0, a multi-pass DMVR is applied. In the first pass, bilateral matching (BM) is applied to each CU to get a block level refined MV pair. In the second pass, the block is divided into 16×16 subblocks and on top of the block level refined MV pair obtained in the first pass, BM is applied on each 16×16 subblock to get a 16×16 subblock level refined MV pair. Finally, in the last stage, an iterative BDOF based MV refinement is applied on the 4×4 or 8×8 subblocks depending on the PU size, and the iteration stage [1, 2].

# Proposed method

In this contribution, it is proposed to extend DMVR to non-equal POC distance cases and add 16x16 BDOF DMVR.

It is also proposed to use the mean removed equations to derive the BDOF MV refinement parameters as:

(Gx.Gx+R1) * vx + Gx.Gy * vy = dI . Gx.  (Gx.Gx+R1) * vx + Gx.Gy * vy = dI . Gx - dM . Gx

Gx.Gy * vx  + (Gy.Gy+R1) * vy= dI . Gy   Gx.Gy * vx + (Gy.Gy+R1) * vy = dI . Gy - dM . Gy

where Gx/Gy are the summation of the 2 horizontal/vertical gradients derived for each reference block.

Summations () are weighted sums, where weights depend on the position in the target region .

dM is the new parameter, which is derived by the mean value differences.

# Experimental results

The proposed method has been implemented on top of the ECM-10.0 [3]. Testing results under RA configuration is summarized in Table I. The simulations were performed following the common test conditions [4]. More detailed results can be found in the attached spreadsheet.

Table I: Performance of proposed method compared with ECM-10.0

*The results in v1 are approximated, final result will be reported in v2.

# Conclusions

In this contribution, additional improvement for DMVR is proposed. It provides a reasonable coding gain. It is recommended to study this new method in next round of EE2.

# References

[1] M. Salehifar, Y. He, K. Zhang, L. Zhang, “EE2-2.6: High-Precision MV Refinement for BDOF,” JVET-AD0195, Apr. 2023.

[2] 	M. Salehifar, Y. He, K. Zhang, H. Liu, L. Zhang, J. Chen, R.-L. Liao, X. Li, Y. Ye, “EE2-3.5: Iterative BDOF pass in multi-pass DMVR”, JVET-AE0065, July 2023.

[3] ECM-10.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-10.0

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, July 2023.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).