| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0164-v3 |
| --- | --- |

| Title: | AHG12: CABAC inter/intra model switch |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Lo Bianco F. Galpin C. Salmon-Legagneur | Tel: Email: | Federico.lobianco@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to

- Use intra CABAC models to encode residual information of CU in inter slices

- Use inter CABAC models to encode residual information of some CU in intra slices

This contribution provides the individual test results for the following configurations:

Test 1: Use intra CABAC models to encode residual information of intra-predicted CU in inter slices

RA { } LDB {}

Test 2: As in Test 1, and also use inter CABAC models to encode residual information of IBC or IntraTMP-predicted CU in intra slices

AI {} RA {}  LDB { }

Test 3: As in Test 2, but QtCbf flag is excluded from the syntax elements for which a model switch is allowed

AI {} RA {} LDB {}

# Introduction

In current CABAC design, residual information (e.g. residual coefficients, QtCbf, SigCoeffGroup etc.) is coded using CABAC models which depend on the slice type, but not on the prediction mode of the current CU.

This contribution asserts that, for some prediction modes used in inter slices (namely, intra prediction), intra CABAC models may better represent the statistics of the residual information; conversely, for some prediction modes used in intra slices (namely, those more affine to inter prediction, like IBC or IntraTMP), inter CABAC models may better represent the statistics of the residual information.

# Proposal

It is proposed that, for CU which are intra-predicted, intra CABAC models are used independently of the slice type. Conversely, it is proposed that, for CU which are predicted with a prediction mode similar to inter prediction (IBC and IntraTMP in the proposed Tests), inter CABAC models are used to code residual information independently of the slice type.

The following tests have been conducted.

Test 1: Use intra CABAC models to encode residual information of intra-predicted CU in inter slices.

Test 2: As in Test 1, and also use inter CABAC models to encode residual information of IBC or IntraTMP-predicted CU in intra slices.

Test 3: As in Test 2, but QtCbf flag is excluded from the syntax elements for which a model switch is allowed.

It is remarked that no retraining of CABAC parameters was performed.

# Experimental results

## Test 1 results

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | 0.03% | -1.58% | -1.13% | 105.2% | 97.2% | #NUM! |
| Class C | 0.06% | -1.17% | -1.21% | 110.6% | 115.9% | #NUM! |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | 0.00% | -1.00% | -0.81% | 106.2% | 109.4% | #NUM! |
| Class F | 0.00% | -0.52% | -0.43% | 109.7% | 111.0% | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class C | -0.08% | -0.56% | -0.40% | 106.2% | 99.0% | #NUM! |
| Class E | -0.09% | 0.72% | -0.07% | 104.8% | 98.4% | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | 0.02% | -0.80% | -0.40% | 105.7% | 106.2% | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

## Test 2 results

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | -0.06% | 0.08% | 0.02% | 106.5% | 96.9% | #NUM! |
| Class C | -0.04% | 0.13% | 0.09% | 109.2% | 105.3% | #NUM! |
| Class E | -0.10% | 0.05% | -0.18% | 115.6% | 115.7% | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | -0.04% | 0.16% | -0.04% | 110.6% | 106.0% | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | -0.04% | -1.56% | -1.13% | 106.5% | 97.3% | #NUM! |
| Class C | 0.04% | -1.09% | -1.39% | 111.1% | 116.9% | #NUM! |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | -0.01% | -0.87% | -0.84% | 107.3% | 112.3% | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.03% | -0.49% | -1.00% | 73.3% | 70.5% | #NUM! |
| Class C | -0.11% | -0.39% | -0.58% | 106.4% | 96.8% | #NUM! |
| Class E | 0.00% | 1.31% | -0.95% | 106.7% | 101.4% | #NUM! |
| Overall | -0.05% | 0.00% | -0.85% | 91.2% | 85.8% | #NUM! |
| Class D | -0.14% | -0.77% | -0.60% | 106.3% | 107.5% | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

## Test 3 results

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | -0.08% | 0.07% | -0.15% | 108.0% | 99.3% | #NUM! |
| Class C | -0.05% | 0.10% | 0.12% | 109.5% | 106.2% | #NUM! |
| Class E | -0.11% | 0.00% | -0.02% | 115.6% | 115.4% | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | -0.03% | 0.27% | -0.07% | 105.6% | 101.0% | #NUM! |
| Class F | -0.13% | 0.04% | 0.23% | 106.5% | 103.3% | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | -0.12% | 0.00% | -0.05% | 106.1% | 97.2% | #NUM! |
| Class C | -0.09% | -0.08% | -0.16% | 110.3% | 115.1% | #NUM! |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | -0.10% | -0.18% | -0.07% | 107.4% | 112.0% | #NUM! |
| Class F | -0.14% | -0.02% | 0.11% | 109.3% | 110.6% | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class C | -0.05% | -0.28% | -0.30% | 106.8% | 99.4% | #NUM! |
| Class E | -0.07% | 1.19% | 0.18% | 108.1% | 103.3% | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | -0.05% | -0.61% | -0.75% | 107.1% | 108.2% | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

# Conclusion

It is suggested to study the proposed tool in Exploration Experiment.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).