| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0174-v1 |
| --- | --- |

| Title: | AHG9: On the SEI processing order SEI message and NNPF groups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Cornelius Hellge Thomas Schierl | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In this document a proposal is described that suggests modifications to the SEI processing order SEI message to allow referencing payload type and Id for association of an SEI message, as well as to specify a length of the prefix of an SEI indicting the bits that are used to identify the SEI message which a particular order id refers to. The proposal also suggests to allow that an SEI processing order SEI message can reference another SEI processing order SEI message and thereby the processing order specified in the latter is inherited by the first SEI processing order SEI message. Besides, a group activation SEI message is proposed that activates all NNPF filters that are associated with an SEI processing order SEI message.

# Introduction

During the previous JVET meeting (31st JVET meeting), several proposals discussed different aspects about the SEI processing order SEI message and grouping aspects of neural network post filter SEI messages.

During the discussion of some of the contributions regarding grouping of neural network post filter SEI messages (e.g., JVET-AE0052 or JVET-AE0063) it was encouraged, as stated in the meeting notes, to study whether the discussed functionality could be achieved by extending the concept of SEI processing order SEI messages.

In this contribution, a proposal is made that aims at extending the SEI processing order SEI messages to cover the discussed functionalities and solve the following issues:

- Allow directly referencing NNPF_id -s (done generically for any SEI) within a SEI processing SEI message=> an association mode is added to indicate whether for the association a) the payload_type is used b) the payload_type + an id is used (see specification in the first aspect of the proposed text)

c) the payload_type + prefix is used.

- Different post-processing versions:

There are cases, in which different combination of post processing steps may be offered so that different receivers with different capabilities or end-devices with different needs can do one or another (alternatives to each other). This is illustrated in Figure 1. => Several variants of the SEI processing SEI message should be allowed (see specification in the first aspect of the proposed text).

Although, wrapping of an SEI message within the SEI processing SEI message is a useful mechanism, if the wrapped SEI message is included into several “alternative” SEI processing SEI messages, this feature can easily lead to a big overhead.

  - Referencing another SEI processing SEI message from an SEI processing SEI message inheriting the described processing order from the referred to SEI message can solve this issue (see specification in the first aspect of the proposed text). In this case, some constraints on the values of po_sei_processing_order[ i ] need to be relaxed.

  - When at a post-processing step (order) different alternatives are offered and each of the alternative SEI messages is wrapped, the implicit constraint of having at least two SEI messages within an SEI processing SEI message leads to having to have more than one (as many as possible combinations) SEI processing SEI message wrapping a same SEI message (see Figure 2), which leads to a big overhead. => allow to have only one SEI message within the SEI processing SEI message as a particular case (see specification in the second aspect of the proposed text).

- Activation of several NNPF filters included into an SEI processing SEI message can be done separately but could lead to unnecessarily having many activation SEI messages. A more compact representation allowing also an activation with a single SEI message in addition to the option of doing separately could avoid unnecessary overhead. (see specification in the third aspect of the proposed text)

- Further aspects of the current SEI processing order SEI message

  - Currently, two SEI messages with same po_sei_processing_order[ i ] are not allowed to have the same payload type when there is no prefix, but when there is prefix even though this is the same, it is allowed. => Suggestion to also disallow the prefix to be exactly the same for two SEI messages, when these have the same payload type (see specification in the first aspect of the proposed text).

  - Currently, there is a constraint that reads that when there is more than one SEI processing order SEI message in a CVS, all shall be the same. It is asserted, as discussed in 1), that the content provider might envision different post-processing steps for different end-devices. => Add a further condition on that constraint that includes an “id”. (see specification in the first aspect of the proposed text).

  - The use of the prefix of an SEI message for associating SEI messages to the SEI processing order SEI message allows also identifying characteristics of the associated SEI messages. However, when also used for identifying characteristics of the associated SEI messages not all fields might be constant within the CVS and the values of the bytes from prefix could be updated within the CVS => indicate which part is used for association (stays constant). (see specification in the first aspect of the proposed text).

Figure 1: Variants for post-processing (different combinations)

Figure 1: Wrapping issue for different combinations

# Proposal

In the first aspect of this proposal, the following highlighted changes are proposed:

- An id for allowing different SEI processing order SEI messages

- Allowing indicating an associated SEI message with a payload type and id, with a specific case for referring to a SEI processing order SEI message inheriting the processing order of SEIs associated to the referred to SEI processing order SEI message.

- Indicating the length in bits of the prefix that is used for associating an SEI message to the SEI processing order SEI message

- Modification of few constraints as discussed in Section 1.

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| if( po_sei_wrapping_flag[ i ] ) { |  |
| reserved_alignment_6bits | u(6) |
| sei_message( ) |  |
| } else { |  |
| po_sei_association_idc[ i ] | u(2) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(12) |
| if( po_sei_association_idc[ i ] = = 1) |  |
| po_sei_payload_id[ i ] | u(8) |
| if( po_sei_association_idc[ i ] = = 2) { |  |
| po_num_prefix_bytes_minus1[ i ] | b(7) |
| po_all_prefix_bytes_used_for_association_flag[ i ] | u(1) |
| if( po_all_prefix_bytes_used_for_association_flag[ i ]) |  |
| po_prefix_association_length_minus1[ i ] | u(16) |
| for( j = 0; j < po_num_prefix_bytes_minus1[ i ] + 1; j++ ) |  |
| po_prefix_byte[ i ][ j ] | b(8) |
| } |  |
| } |  |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| } |  |

The SEI processing order SEI message carries information indicating the preferred processing order, as determined by the encoder (i.e., the content producer), for different types of SEI messages that may be present in a CVS.

When an SEI processing order SEI message is present in any access unit of a CVS, an SEI processing order SEI message shall be present in the first access unit of the CVS. The SEI processing order SEI message persists in decoding order from the current access unit until the end of the CVS. When there are multiple SEI processing order SEI messages present in a CVS with the same po_id, they shall have the same content.

…

po_id contains an identifying number that may be used to identify a PO SEI message.

…

po_sei_association_idc[ i ], when equal to 0, indicates that the i-th SEI message for which preferred processing order information is provided in the SEI processing order SEI message is identified by the syntax element po_sei_payload_type[ i ]. po_sei_association_idc[ i ] equal to 1 indicates that the i-th SEI message for which preferred processing order information is provided in the SEI processing order SEI message is identified by the syntax elements po_sei_payload_type[ i ] and po_sei_payload_id[ i ]. po_sei_association_idc[ i ] equal to 2 indicates that the i-th SEI message for which preferred processing order information is provided in the SEI processing order SEI message is identified by the syntax elements po_sei_payload_type[ i ], po_prefix_association_length_minus1[ i ] and po_prefix_byte[ i ][ j ]. The value 3 for po_sei_association_idc[ i ] is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document.

…

po_sei_payload_type[ i ] specifies the payloadType value of the i-th SEI message type for which preferred processing order information is provided in the SEI processing order SEI message. For any two different non-negative integer values of m and n, the values of po_sei_payload_type[ m ] and po_sei_payload_type[ n ] shall not be identical unless po_sei_association_idc[ m ] and po_sei_association_idc[ n ] are both equal to 1 and po_sei_payload_id[ m ] and po_sei_payload_id[ n ] are different or po_sei_association_idc[ m ] and po_sei_association_idc[ n ] are both equal to 2 and the value of po_prefix_byte[ m ][ j ] and po_prefix_byte[ m ][ j ] is different for at least one j in the range of 0 to min(po_num_prefix_bytes_minus1[ m ], po_num_prefix_bytes_minus1[ n ])po_sei_prefix_flag[ m ] and po_sei_prefix_flag[ n ] are both equal to 1.

po_sei_payload_id[ i ] specifies the payloadId value with payloadType equal to po_sei_payload_type[ i ] of the i-th SEI message for which preferred processing order information is provided in the SEI processing order SEI message.

When po_sei_payload_type[ i ] indicates a payloadType equal to the payload type of the SEI processing order SEI message, po_sei_association_idc[ i ] shall be equal to 1 and the same preferred processing order of the SEI messages specified within the SEI processing order SEI message with po_id equal to po_sei_payload_id[ i ] are inferred for the current SEI processing order SEI message.

po_num_prefix_bytes_minus1[ i ] plus 1, when present, specifies the number of bytes associated with the i-th SEI message for which preferred processing order information is provided in the SEI processing order SEI message. When not present, the value of po_num_prefix_bytes[ i ] is inferred to be equal to 0.

po_all_prefix_bytes_used_for_association_flag[ i ] equal to 1 indicates that all prefix bytes po_prefix_byte[ i ][ j ] are equal to the j-th byte value of any SEI message corresponding to i-th SEI message in the SEI processing order SEI message. po_all_prefix_bytes_used_for_association_flag[ i ] equal to 0 indicates that

po_prefix_association_length_minus1[ i ] plus1 specifies the length in bits, prefixBitLen, up to which the bytes po_prefix_byte[ i ][ j ] coincide with the SEI payload of any SEI message corresponding to i-th SEI message in the SEI processing order SEI message.

When po_prefix_association_length_minus1[ i ] is not present and po_sei_association_idc[ i ] is equal to 2, the value of prefixBitLen is derived as 8 * (po_num_prefix_bytes_minus1[ i ] + 1 )

po_prefix_byte[ i ][ j ], when present, specifies the j-th byte value of an SEI message corresponding to i-th SEI message in the SEI processing order SEI message the i-th SEI message.

po_sei_processing_order[ i ] indicates the preferred order of processing of the i-th SEI message type for which preferred processing order information is provided in the SEI processing order SEI message. For any two different integer values of m and n that are greater than or equal to 0, po_sei_processing_order[ m ] less than po_sei_processing_order[ n ] indicates any SEI message type with payloadType equal to po_sei_payload_type[ m ] and, when present, po_sei_payload_id[ m ] or bytes po_prefix_byte[ m ][ p ] for p ranging from 0 to po_num_prefix_bytes_minus1[ m ] − 1, inclusive, should be processed before any SEI message type with payloadType equal to po_sei_payload_type[ n ], and, when present, po_sei_payload_id[ n ] or bytes po_prefix_byte[ n ][ q ] for q ranging from 0 to po_num_prefix_bytes_minus1[ n ] − 1, inclusive, and po_sei_processing_order[ m ] equal to po_sei_processing_order[ n ] indicates that there is no preferred order of processing between the SEI message types. When there are multiple SEI messages with the same values of po_sei_payload_type[ i ], po_num_prefix_bytes [ i ], and bytes po_prefix_ byte[ i ][ j ] for j ranging from 0 to po_num_prefix_bytes [ i ] − 1, inclusive, they shall have the same value of po_sei_processing_order[ i ].

po_sei_processing_order[ 0 ] shall be equal to 0, and fFor i greater than 0, when po_sei_payload_type[ i − 1 ] does not indicate a payloadType equal to the payload type of the SEI processing order SEI message, po_sei_processing_order[ i ] shall be equal to po_sei_processing_order[ i − 1 ] or po_sei_processing_order[ i − 1 ] + 1. For i greater than 0, when po_sei_payload_type[ i − 1 ] indicates a payloadType equal to the payload type of the SEI processing order SEI message, po_sei_processing_order[ i ] shall be equal to or greater than the value of po_sei_processing_order[ po_num_sei_messages_minus2 + 1 ] of the SEI processing order SEI message with po_id equal to po_sei_payload_id[ i ] of the current SEI processing order SEI message.

The value of po_sei_processing_order[ po_num_sei_messages_minus2 + 1 ] shall not be equal to 0.

In the second aspect of this proposal, the following highlighted changes are proposed:

- Allowing a single SEI message to be indicated within the SEI processing order SEI message, in which case the SEI message is wrapped => and some syntax elements are inferred without the need of signalling them.

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus1 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus1 + 1; i++) { |  |
| if( po_num_sei_messages_minus1 > 0) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| if( po_sei_wrapping_flag[ i ] ) |  |
| reserved_alignment_6bits | u(6) |
| } |  |
| if( po_sei_wrapping_flag[ i ] ) |  |
| sei_message( ) |  |
| } else { |  |
| po_sei_association_idc[ i ] | u(2) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(12) |
| if( po_sei_association_idc[ i ] = = 1) |  |
| po_sei_payload_id[ i ] | u(8) |
| if( po_sei_association_idc[ i ] = = 2) { |  |
| po_num_prefix_bytes_minus1[ i ] | b(7) |
| po_all_prefix_bytes_used_for_association_flag[ i ] | u(1) |
| if( po_all_prefix_bytes_used_for_association_flag[ i ]) |  |
| po_prefix_association_length_minus1[ i ] | u(16) |
| for( j = 0; j < po_num_prefix_bytes_minus1[ i ] + 1; j++ ) |  |
| po_prefix_byte[ i ][ j ] | b(8) |
| } |  |
| } |  |
| if( po_num_sei_messages_minus1 > 0) |  |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| } |  |

po_num_sei_messages_minus12 plus 12 indicates the number of SEI messages that have a processing order indicated in the SEI processing order SEI message.

…

po_sei_importance_flag[ i ] indicates the degree of importance determined by the encoder for the SEI message with index i. When the po_sei_payload_type[ i ] is equal to the payload type of the SEI processing order SEI message, and the associated SEI processing order SEI message has po_num_sei_messages_minus1 equal to 0, po_sei_importance_flag[ i ] indicates the degree of importance determined by the encoder for the SEI message in sei_message( ) of the associated SEI processing order SEI message.

If the decoding system cannot interpret or does not support any indicated SEI message that has po_sei_importance_flag[ i ] equal to 1, it should ignore the entire SEI processing order SEI message.

When the po_sei_payload_type[ i ] is equal to the payload type of the SEI processing order SEI message, and the associated SEI processing order SEI message has po_num_sei_messages_minus1 greater than 0, po_sei_importance_flag[ i ] shall be equal to 1 if there is at least one SEI message j in the associated SEI processing order SEI message that has po_sei_importance_flag[ j ] equal to 1, with j ranging from 0 to po_num_sei_messages_minus1 of the associated SEI processing order SEI message. Otherwise, when the po_sei_payload_type[ i ] is equal to the payload type of the SEI processing order SEI message po_sei_importance_flag[ i ] shall be equal to 0.

…

If po_sei_wrapping_flag[ i ] is equal to 0, an SEI message should be present outside of the SEI processing order SEI message with payloadType equal to po_sei_payload_type[ i ]. When not present, the value of po_sei_wrapping_flag[ i ] is inferred to be equal to 1. However, if po_sei_wrapping_flag[ i ] is equal to 0 and no SEI message is present with payloadType equal to po_sei_payload_type[ i ], the following applies:

- If po_sei_importance_flag[ i ] is equal to 1, the decoder should ignore the entire SEI processing order SEI message.

- Otherwise, the decoder should ignore all data associated with the loop variable value of i.

NOTE – po_sei_wrapping_flag[ i ] equal to 1 enables SEI messages to be carried within the SEI processing order SEI message to prevent such SEI messages from being incorrectly interpreted by decoders that do not process the SEI processing order SEI message. Thus, po_sei_wrapping_flag[ i ] equal to 1 is intended to be used when po_sei_wrapping_flag[ i ] equal to 0 can lead to unintended results being produced by such decoders.

In the third aspect of this proposal, the NNPF group activation SEI message from JVET-AE0052 is proposed with the following highlighted changes:

- A group activation for all NNPF filters in an SEI processing order SEI message is defined by indicating an nnpfga_target_id equal to po_id.

- Persistence for each NNPF is defined until a new NNPFGA with the same id is present or an NNPFA for the corresponding NNPF is present, i.e. NNPFGA and NNPFA can be simultanesouly used for the NNPF filters associated within an SEI processing order SEI message.

- Since signalling for each NNPF input or output parameters would be almost equally big than using separate NNPFA SEI messages, a set of parameters configurations is indicated and provided and a mapping to such parameters is done for each NNPF filter within a SEI.

| nn_post_filter_group_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfga_target_id | ue(v) |
| nnpfga_cancel_flag | u(1) |
| if( !nnpfga_cancel_flag ) { |  |
| nnpfga_persistence_flag | u(1) |
| nnpfga_num_filter_params_minus1 | ue(v) |
| for( i = 0; i <= nnpfga_num_filter_params_minus1 ; i++ ) { |  |
| nnpfga_target_base_flag[ i ] | u(1) |
| nnpfga_input_all_pics_flag[ i ] | u(1) |
| if( !nnpfga_input_all_pics_flag[ i ] ) { |  |
| nnpfga_num_input_pics_minus1[ i ] | ue(v) |
| for( j = 0; j <= nnpfga_num_input_pics_minus1[ i ]; j++ ) |  |
| nnpfga_input_pic_skip_count[ i ][ j ] | ue(v) |
| } |  |
| nnpfga_num_output_entries[ i ] | ue(v) |
| for( j = 0; j < nnpfga_num_output_entries[ i ]; j++ ) |  |
| nnpfga_output_flag[ i ][ j ] | u(1) |
| } |  |
| nnpfga_num_filters_minus2 | ue(v) |
| for( i = 0; i <= nnpfga_num_filters_minus2 + 1; i++ ) { |  |
| nnpfga_filter_param_idx[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

The neural-network post-filter group activation (NNPFGA) SEI message activates or de-activates a group of NNPF, which consists of all NNPF associated with an SEI message identified by an entry with po_sei_payload_type[ i ] equal to the payload type of the NNPF SEI message in an SEI processing order SEI message with po_id equal to nnpfga_target_id, for post-processing filtering of a set of pictures in the order as indicated by the SEI processing order SEI message.

[NOTE: The following syntax elements are copied from JVET-AE0052 with relevant changes highlighted]

nnpfga_target_id indicates the target NNPFG, which is specified by the NNPFGC SEI message that pertains to the current picture andSEI processing order SEI message that have has po_id equal to nnpfga_target_id.

The value of nnpfga_target_id shall be in the range of 0 to 28 − 1, inclusive.

An NNPFGA SEI message with a particular value of nnpfga_target_id shall not be present in a current PU unless there is an SEI processing order SEI message with po_id equal to the particular value of nnpfga_target_id present in the current PU or in a PU that precedes the current PU in decoding order within the current CLVS.

When a PU contains both a SEI processing order SEI message with a particular value of po_id and an NNPFGA SEI message with nnpfga_target_id equal to the particular value of po_id, the SEI processing order SEI message shall precede the NNPFGA SEI message in decoding order.

nnpfga_cancel_flag equal to 1 indicates that the persistence of the group of NNPF established by any previous NNPFGA SEI message with the same nnpfga_target_id as the current SEI message is cancelled, i.e., the group of NNPF is no longer used unless it is activated by another NNPFGA SEI message with the same nnpfga_target_id as the current SEI message and nnpfga_cancel_flag equal to 0. nnpfga_cancel_flag equal to 0 indicates that the group of NNPF is activated for use in the order as specified by the SEI processing order SEI message with po_id equal to nnpfga_target_id.

nnpfga_persistence_flag specifies the persistence of the group of NNPF as specified by the SEI processing order SEI message with po_id equal to nnpfga_target_id for the current layer.

nnpfga_persistence_flag equal to 0 specifies that the group of NNPF may be used for post-processing filtering for the current picture only.

nnpfga_persistence_flag equal to 1 specifies that the each NNPF within the group of NNPF may be used for post-processing filtering in the order as specified by the SEI processing order SEI message with po_id equal to nnpfga_target_id for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with a NNPFGA SEI message with the same nnpfga_target_id as the current SEI message that follows the current picture in output order.

–	A picture in the current layer associated with a NNPFA SEI message with a nnpfa_target_id as the NNPF SEI message that follows the current picture in output order.

nnpfga_num_filter_params_minus1 specifies the number of set of filter parameters nnpfga_target_base_flag[ i ], nnpfga_num_output_entries[ i ], nnpfga_output_flag[ i ][ j ] present in the neural network post filter group activation SEI message.

nnpfga_target_base_flag[ i ] equal to 1 specifies that a NNPF is the base NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] in an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id. nnpfga_target_base_flag[ i ] equal to 0 specifies a NNPF is the NNPF specified by the last NNPFC SEI message that has nnpfc_id equal to nnpfgc_member_id[ i ] in an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id, precedes the first VCL NAL unit of the current picture in decoding order, and is not a repetition of the NNPFC SEI message that contains the base NNPF.

nnpfga_num_output_entries[ i ] specifies the number of nnpfga_output_flag[ i ][ j ] syntax elements present in the NNPFGA SEI message. The value of nnpfga_num_output_entries[ i ] shall be in the range of 0 to NumInpPicsInOutputTensor, inclusive, for an NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] of an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id.

nnpfga_output_flag[ i ][ j ] equal to 1 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ j ] derived for a NNPF is output by the NNPF process activated by this loop entry, where the NNPF process is specified in the semantics of the NNPFC SEI message. nnpfga_output_flag[ i ][ j ] equal to 0 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ j ] derived for a NNPF is not output by the NNPF process activated by this loop entry. When nnpfga_num_output_entries[ i ] is less than NumInpPicsInOutputTensor derived for the i-th NNPF of the target NNPFG, nnpfga_output_flag[ i ][ j ] is inferred to be equal to 1 for each value of i in the range of nnpfga_num_output_entries[ i ] to NumInpPicsInOutputTensor − 1, inclusive.

Let NnpfgaOutputPicList, which is the list of pictures output by NNPF process of the NNPFG in output order, be initially empty and formed in decreasing order of n in the range of 0 to nnpfga_num_filters_minus2 + 1, inclusive, by including each picture that is output by the NNPF process of the n-th loop entry that has no corresponding picture already present in NnpfgaOutputPicList.

nnpfga_num_filters_minus2 plus 2 indicates the number of NNPFs in the SEI processing order SEI message with po_id equal to nnpfga_target_id that this SEI message activates. The value of nnpfga_num_filters_minus2 shall be equal to the value of nnpfgc_num_members_minus2 in an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_idnumber of associated SEI messages with po_sei_payload_type[ i ] equal to the payload type of the NNPF SEI message in the SEI processing order SEI message with po_id equal to nnpfga_target_id.

nnpfga_filter_param_idx[ i ] specifies the index, filterParamIdx, into the list of set of filter parameters, of the set of filter parameters that applies to the i-th NNPF filter activated by the i-th loop entry.

The value of nnpfga_num_output_entries[ filterParamIdx ] shall be in the range of 0 to NumInpPicsInOutputTensor, inclusive, for the i-th NNPF filter activated by the i-th loop entry.

When nnpfga_num_output_entries[ filterParamIdx ] is less than NumInpPicsInOutputTensor derived for the i-th NNPF activated by the i-th loop entry, nnpfga_output_flag[ m ][ j ] is inferred to be equal to 1 for each value of m in the range of nnpfga_num_output_entries[ filterParamIdx ] to NumInpPicsInOutputTensor − 1, inclusive, for the i-th NNPF activated by the i-th loop entry.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).